3GPP TSG-RAN WG2 Meeting #129	R2-2500839

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.7.3

Source:	China Telecom

Title:	Remaining Issues on RRM Measurement Enhancements

Document for:	Discussion and Decision

- Introduction

In the RAN2#127bis meeting, the following agreements were made for RRM measurement gaps related enhancements.

Agreements on RRM measurement gaps impacts - No MG-specific enhancements is needed on DSR operation. - RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped - No MG-specific enhancements is needed for DRX operation.

The intention of this paper is to discuss the remaining issues related to the RRM measurement.

2 Discussion

In the RAN1#118 meeting, a DCI-based solution was agreed to enable Tx/Rx transmission during particular gap/restriction caused by RRM measurements.

For solutions based on triggering/enabling by network signalling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it.

The network can use DCI to indicate the UE to skip a particular gap/restriction caused by RRM measurement dynamically. Obviously, the DCI-based solution is suitable for dynamic data scheduling, such as uncertain data transmission or XR traffic with significant jitter.

However, for dynamic scheduling, the network needs to send DCI to the UE before each gap/restriction occasion. Consequently, the UE needs to monitor PDCCH continuously to determine whether to skip the upcoming gap/restriction. Obviously, the DCI-based solution offers flexibility at the expense of extra signalling overhead and complexity.

Observation 1: DCI-based solution may be suitable for dynamic data scheduling.

In addition, there are concerns regarding MG skipping when DRX is configured. Specifically, the UE may not receive the MG skipping indication during the off-duration. In some cases, the MG may overlap with the DRX on-duration from the outset, leaving the network insufficient time to schedule the DCI indication. This scenarios is particularly likely when the DRX periodicity is larger than the MG periodicity.

Observation 2: If the MG overlaps with the DRX on-duration from the outset, the network may not have enough time to indicate the UE to skip the MG.

Since both DRX and MG are configured by the network in a semi-static way, semi-static solutions may be more suitable for resolving overlaps between the MG and DRX. For instance, the network can send a semi-static configuration to UE to indicate when MG skipping should occur. Based on this configuration, the UE can skip the MG without requiring a DCI reception. This approach ensures that the limited time between the MG and DRX on-duration will not affect MG skipping.

Proposal 1: Semi-static solutions can be employed to resolve conflicts between DRX and MG.

In Rel-18, semi-static scheduling (i.e. CG or SPS) is used for periodic XR traffic to reduce the signalling overhead. With the DCI-based solution, the network has to schedule a DCI before the gap/restriction occasion coming. And the UE needs to monitor the PDCCH continuously. Obviously, dynamic DCI scheduling will result in huge extra signalling overhead and energy consumption, which is contrary to the original intention of semi-static scheduling (i.e. CG/SPS).

Observation 3: For periodic XR traffic with semi-static CG/SPS scheduling, the DCI-based solution results in huge signalling overhead and energy consumption.

As MG configuration is also configured in a semi-static way, semi-static solutions are more suitable to address the collision between the MG and CG/SPS configuration. In this way, there is no need for the network to schedule the DCI continuously, which can reduce the signalling overhead, energy consumption and implementation complexity.

Proposal 2: Semi-static solutions can be applied to resolve conflicts between CG/SPS and MG.

- Conclusions

Based on the above analysis, we have the following observations and proposals:

Observation 1: DCI-based solution may be suitable for dynamic data scheduling.

Observation 2: If the MG overlaps with the DRX on-duration from the outset, the network may not have enough time to indicate the UE to skip the MG.

Observation 3: For periodic XR traffic with semi-static CG/SPS scheduling, the DCI-based solution results in huge signalling overhead and energy consumption.

Proposal 1: Semi-static solutions can be employed to resolve conflicts between DRX and MG.

Proposal 2: Semi-static solutions can be applied to resolve conflicts between CG/SPS and MG.

- References

- R2-25xxxxx, Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA