3GPP TSG-RAN WG2 Meeting #129	R2-2500840

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.7.4.1

Source:	China Telecom

Title:	Remaining Issues on LCP Enhancements

Document for:	Discussion and Decision

- Introduction

In the RAN2#127bis meeting, the following agreements were made for LCP prioritization.

Agreements on LCP enhancements As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

The paper aims to discuss the details of additional priority for LCH and provide some considerations.

2 Discussion

In the current LCP mechanism, the Prioritized Bit Rate (PBR) is introduced to ensure the minimum data rate for each logical channel. Specifically, the UE maintains a variable Bj for each logical channel j. During the first round of resource allocation, UL grant resources are assigned to logical channels in decreasing order of Bj, where Bj >0. Subsequently, Bj is reduced by the total size of the MAC SDUs used in the MAC PDU for logical channel j. In the second round of resource allocation, the remaining UL resources are allocated to logical channels in strict decreasing order, without considering the size of Bj. This ensure that, in the second round, the UE maximizes data transmission for high-priority logical channels.

As a result, some LCH priority-adjusted data may not receive sufficient resources in the first round of resource allocation if Bj is smaller than the LCH priority-adjusted data volume. In certain cases, this may delay LCH priority-adjusted data transmission. To address this issue, some companies proposed to increase the Bj or set PBR to infinity for the LCH priority-adjusted data to ensure the LCH priority-adjusted data can be transmitted timely. However, this approach is unfair for other data, as it could result in delays or even discarding of other data. Therefore, in our understanding, neither Bj nor PBR should be adjusted for the LCH priority-adjusted data.

Proposal 1: Bj or PBR should not be adjusted for LCH priority-adjusted data.

Furthermore, the method for configuring additional priority for LCH with delay-critical data remains unclear. According to the TS38.331, logical channel parameters, including LCH priority, are configured via RRC signalling, LogicalChannelConfig. To reduce the specification impact, the additional priority of LCH with delay-critical data could also be configured through RRC signalling.

Proposal 2: Additional LCH priority for delay-critical data could be configured via RRC signalling.

In the RAN2#127 meeting, the intra-UE prioritization issues were discussed and the following agreements were reached.

FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3).

Per TS38.321, the mechanism of intra-UE prioritization procedure is outlined as follows.

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

It can be observed that both overlapping uplink grants and SRs cases are considered in the legacy intra-UE prioritization mechanism. Taken the LCH priority-adjusted data into consideration, whether the legacy LCH priority or the additional LCH priority should be applied in the intra-UE prioritization need further discussion. Clearly, there are two options for intra-UE prioritization.

- Opt 1: The legacy LCH priority is used in intra-UE prioritization, regardless of whether the LCH priority is adjusted due to LCH priority-ajusted data.

- Opt 2: The additional LCH priority is considered in intra-UE prioritization. If the LCH priority is adjusted to additioanl LCH priority due to the LCH priority-adjusted data, the additional LCH priority will be used in intra-UE prioritization.

In our understanding, Opt 2 is preferred. If the LCH priority is adjusted to the additional priority, it is more reasonable to apply the additional LCH priority in the intra-UE prioritization procedure. Otherwise, the LCH priority used for resource allocation would differ from the actual LCH priority. As a result, in case of overlapping uplink grants or SRs, the LCH priority-adjusted data may not be transmitted timely. Therefore, the additional priority of LCH should be considered in the intra-UE prioritization case.

Proposal 3: The additional priority of LCH should be considered in intra-UE prioritization case.

- Conclusions

Based on the above analysis, we have the following proposals:

Proposal 1: Bj or PBR should not be adjusted for LCH priority-adjusted data.

Proposal 2: Additional LCH priority for delay-critical data could be configured via RRC signalling.

Proposal 3: The additional priority of LCH should be considered in intra-UE prioritization case.

- References

- R2-25xxxxx, Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA