3GPP TSG-RAN WG2 Meeting #129	                                                           R2-2500843

Athens, Greece, Feb. 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1237 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the IAB-MT capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Samsung, Qualcomm Incorporated, Nokia, NEC, Fujitsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. In 38.300, the following description implies SRB2 is mandatory for NCR-MT: 8/8a. The gNB performs the reconfiguration to setup SRB2 and DRBs for UE, or SRB2 and optionally DRBs for IAB-MT. 2. Besides, according to the following descriptions in RRC spec, SRB2 configuration is essential for the IAB-MT to initiate RRC connection re-establishment and to be RRC connection suspended: TS 38.331 5.3.7 RRC connection re-establishment 5.3.7.1 General The purpose of this procedure is to re-establish the RRC connection. A UE in RRC_CONNECTED, for which AS security has been activated with SRB2 and at least one DRB setup or, for IAB, SRB2, may initiate the procedure in order to continue the RRC connection. … If AS security has not been activated, the UE shall not initiate the procedure but instead moves to RRC_IDLE directly, with release cause 'other'. If AS security has been activated, but SRB2 and at least one DRB or, for IAB, SRB2, are not setup, the UE does not initiate the procedure but instead moves to RRC_IDLE directly, with release cause 'RRC connection failure'. 5.3.8 RRC connection release 5.3.8.1 General The purpose of this procedure is: - to release the RRC connection, which includes the release of the established radio bearers, BH RLC channels as well as all radio resources; or - to suspend the RRC connection only if SRB2 and at least one DRB or, for IAB, SRB2, are setup, which includes the suspension of the established radio bearers. 5.3.8.2 Initiation The network initiates the RRC connection release procedure to transit a UE in RRC_CONNECTED to RRC_IDLE; or to transit a UE in RRC_CONNECTED to RRC_INACTIVE only if SRB2 and at least one DRB or, for IAB, SRB2, is setup in RRC_CONNECTED; or to transit a UE in RRC_INACTIVE back to RRC_INACTIVE when the UE tries to resume; or to transit a UE in RRC_INACTIVE to RRC_IDLE when the UE tries to resume. The procedure can also be used to release and redirect a UE to another frequency. But in case IAB-MT supports neither drb-IAB-r16 nor non-DRB-IAB-r16, NW will configure neither DRB nor SRB2 to IAB-MT. In that case, it would be problematic as IAB-MT would neither be able to initiate RRC connection re-establishment nor be able to be RRC connection suspended. Thus, it is necessary to clarify SRB2 is mandatory for IAB-MT. It is noted SRB2 is mandatory for NCR-MT. In this case, if IAB-MT doesn’t support DRB, it should support SRB2 configuration without DRB in order to support SRB2 configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that if drb-IAB-r16 is absent, non-DRB-IAB-r16 shall be reported. Impact analysis Impacted functionality: IAB-MT Impacted 5G architecture options: NR SA Inter-operability: If the IAB-MT is implemented according to this CR but the network is not, there is no inter-operability issue. If the network is implemented according to this CR but the IAB-MT is not, there will be confusion to NW if both drb-IAB-r16 and non-DRB-IAB-r16 are absent. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will cause unexpected error if both drb-IAB-r16 and non-DRB-IAB-r16 are absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.15.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

4.2.15	IAB Parameters

4.2.15.4	PDCP Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| drb-IAB-r16 Indicates whether the IAB-MT supports DRB configuration including split DRB with one UL path, (de)ciphering on DRB and PDCP status reporting. | IAB-MT | No | No | No |
| non-DRB-IAB-r16 Indicates whether the IAB-MT supports SRB2 configuration without a DRB, as specified in TS 38.331 [9]. This field shall be reported if drb-IAB-r16 is absent. | IAB-MT | No | No | No |

End of Change