3GPP TSG-RAN WG2 Meeting #129	R2-2500844

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.7.4.2

Source:	China Telecom

Title:	Leftover Issues for DSR Enhancement

Document for:	Discussion and Decision

- Introduction

The following agreements were achieved for the DSR enhancement in previous meetings.

- RAN2#127bis agreements DSR MAC CE details A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds - RAN2#128 agreements Thresholds configuration Let the network configure the triggering and reporting thresholds without constraints. RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. Non-delay critical data ahead in the queue The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

This paper aims to discuss the remaining issues for DSR enhancements and give some insights.

2 Discussion

Per TS38.321, the legacy DSR MAC CE in Rel-18 is illustrated in Figure 1. It can be observed that only one pair of remaining time and buffer size information can be reported for each LCG. In Rel-19, an enhanced MAC CE is required to report multiple pairs of remaining time threshold and buffer size information for each LCG. As shown in Figure 1, a reserved bit R is allocated for each LCG. One possible approach to indicate multiple pairs is to redefine the reserved R bit as an extension E bit to indicate whether the next pair and the current pair belong to the same LCG. If the next pair and the current pair are related to the same LCG, these pairs of information will be reported for the LCG.

Observation: One simple way to indicate multiple pairs for each LCG is to redefine the reserved R bit as an extension E bit to indicate that the next pair and the current pair belong to the same LCG.

Figure 1: Legacy DSR MAC CE

Figure 2: Rel-19 Enhanced DSR MAC CE

Specifically, the extension E bit value 1 indicates that the next pair and the current pair belongs to the same LCG, while the extension E bit value 0 indicates that the next pair and the current pair are associated with different LCG. As illustrated in Figure 2, LCG 0 and LCG 7 are configured with 2 pairs and 3 pairs remaining time and buffer size information respectively, while LCG 1 is configured with a single pair of information. Therefore, one extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE.

Proposal 1: One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE.

As is well known, the legacy DSR MAC CE can indicate at most one reporting threshold for each LCG. Therefore, once an LCG is configured with more than one reporting thresholds, the enhanced new DSR MAC CE will be used. In other words, new DSR MAC CE will be used when at least one LCG is configured with multiple reporting thresholds.

Proposal 2: New DSR MAC CE will be used when at least one LCG is configured with multiple reporting thresholds.

Regarding the coexistence of the new DSR MAC CE and legacy DSR MAC CE, if all LCGs are configured with at most one pair of remaining time and buffer size information to be reported, the legacy DSR MAC CE could be used. Otherwise, new DSR MAC CE will be employed.

Furthermore, if the proposed new MAC CE format is adopted, the legacy DSR MAC CE can be considered a special case of the new DSR MAC CE, e.g., when all extension bits E are set to 0. The new DSR MAC CE  can be used to indicate one or multiple reporting thresholds for each LCG. Therefore, there is no need to configure both the legacy and new DSR MAC CE simultaneously.

Proposal 3: The legacy DSR MAC CE can be considered a special case of the proposed new DSR MAC CE, so there is no need to configure both the legacy and new DSR MAC CEs simultaneously.

- Conclusions

Based on the above analysis, we have the following observation and proposals:

Observation: One simple way to indicate multiple pairs for each LCG is to redefine the reserved R bit as an extension E bit to indicate that the next pair and the current pair belong to the same LCG.

Proposal 1: One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE.

Proposal 2: New DSR MAC CE will be used when at least one LCG is configured with multiple reporting thresholds.

Proposal 3: The legacy DSR MAC CE can be considered a special case of the proposed new DSR MAC CE, so there is no need to configure both the legacy and new DSR MAC CEs simultaneously.

- References

- R2-25xxxxx, Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA