3GPP TSG-RAN WG2 Meeting #129	R2-2500846

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.7.5

Source:	China Telecom

Title:	Leftover Issues for RLC AM Enhancements

Document for:	Discussion and Decision

- Introduction

The topic of RLC enhancements was extensively discussed in the RAN2#128 meeting, and the following agreements were achieved.

Agreements on AL-FEC (related to LS from SA2) - There is no consensus in RAN2 that AL-FEC ratio information is useful for the gNB for both RLC AM and RLC UM. - RAN2 understanding is that in case this information would be provided to the gNB, it is up to gNB how/whether to consider it, i.e. no impact on RAN2 specifications. Agreements on unnecessary RLC retransmissions avoidance - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced - Special handling to avoid PDCP control PDU discard is not needed. - A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. - The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report. Agreements on timely RLC retransmissions - Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

This paper continues to discuss the remaining issues on timely RLC retransmission and unnecessary retransmission avoidance.

- Discussion

2.1 Timely RLC retransmissions

Generally, the UE uses polling mechanism to request the network to trigger a status report. Upon receiving the status report, the UE triggers the retransmission. One potential approach to ensure timely RLC retransmission is to enhance the polling mechanism, enabling frequent status reports. According to TS38.322, the parameters of pollPDU or pollByte can be configured with smaller values to facilitate quicker status report. However, since the status report is triggered only after the expiration of the t-StatusProhibit timer, shorter pollPDU or pollByte values should work together with a shorter t-StatusProhibit timer value to achieve rapid status report.

Nevertheless, frequent status reports increase transmission overhead, particularly for non-delay critical data. To balance transmission overhead and latency, an optimization is to configure separate parameters for different types of data. For instance, smaller values of pollPDU or pollByte could be configured for delay-critical data, while normal values would be configured for non-delay critical data.

Proposal 1: To configure separate parameters (e.g. pollPDU or pollByte values) for delay-critical data and normal data to balance transmission latency and overhead.

To ensure the timely transmission of delay-critical data, the remaining time can be considered as an additional condition for triggering polling. For instance, polling can be triggered if the remaining time falls below the threshold or if the data volume of delay-critical SDUs exceeds the threshold.

Proposal 2: The remaining time can be considered an additional polling triggering condition.

In XR, a PDU set containing one or more PDUs is introduced to carry the payload of one unit of information generated at the application level. Per TS38.322, the UE polls the gNB after a configured number of PDUs or bytes have been transmitted. It is likely that the UE may trigger a poll in the middle of a PDU set. As a result, the status report may only contain the reception status of a part of PDUs in a PDU set. If the other part of the PDUs has not been retransmitted promptly, the entire PDU set including previously successfully transmitted PDUs may be discarded, resulting in resource wastage.

Therefore, for XR traffic, the polling mechanism may need to be enhanced to allow the UE to trigger polling based on the PDU set. For instance, the polling bit could be set based on the PDU set traffic pattern. In this way, the UE can receive the status report of one or more PDU sets.

Proposal 3: Polling enhancement is needed to enable the UE to poll the RLC status based on the PDU set.

2.2 Avoid unnecessary retransmission

For the combined Tx and Rx approach, it has been agreed that Tx stops (re)transmission of an outdated SDU based on either the PDCP layer discard indication or the number of retransmissions. However, it is undetermined how the RLC Tx side discards outdated SDUs. According to TS38.322, SDU discarding at the RLC Tx side is determined by the PDCP layer indication, as described below. To minimize the impact on the specification, in the combined approach, the RLC Tx side could also discard outdated SDUs based on the PDCP layer indication.

## 5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

Proposal 4: The RLC Tx side discards outdated SDUs based on the PDCP layer indication.

- Conclusions

Based on the above analysis, we have the following proposals:

Proposal 1: To configure separate parameters (e.g. pollPDU or pollByte values) for delay-critical data and normal data to balance transmission latency and overhead.

Proposal 2: The remaining time can be considered an additional polling triggering condition.

Proposal 3: Polling enhancement is needed to enable the UE to poll the RLC status based on the PDU set.

Proposal 4: The RLC Tx side discards outdated SDUs based on the PDCP layer indication.

- References

- R2-25xxxxx, Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA