3GPP TSG-RAN WG2 Meeting #129	R2-2500847

Athens, Greece, February 17 – 21, 2025

Agenda item:	8.1.2.3

Source: 	Qualcomm Incorporated

Title: 	LCM for positioning use case

Document for:		Discussion and Decision

# 1.	Introduction

During the post-meeting email discussion "[POST128][026][AIML] LCM Procedure for Positioning Case1 (Ericsson)" [1] some progress has been made on Case 1. The following observations and proposals were suggested by the moderator:

- Observation 1 Criteria for introducing a new method include: new measurements, assistance data, incompatibility of assumptions/constraints in legacy methods, forward compatibility for future AI/ML enhancements. - Observation 2 There may not be one golden rule to decide whether to introduce new method or enhance legacy and thus a case-by-case decision has to be made. - Observation 3 The way forward is that companies preferring fallback configuration should provide how much latency saving can be done; i.e. motivate companies who have disagreed to introduce this option. However, additional signalling overhead with fallback configuration should be considered since assistance data for two methods may have to be provided. - Observation 4 If fallback is requested, it is requested in Step 5 LPP request location information message. - Observation 5 Fallback configurations should be legacy methods (e.g., DL-TDOA, DL-AoD) to ensure continuity of service. - Observation 6 Since, there is clear majority to let RAN1 progress and send RAN2 the parameter list, we can wait for RAN1 to comeback. However, RAN2 can send an LS to RAN1 with RAN2 agreements. - Observation 7 No need to send an LS to RAN1 for UE side additional condition. - Proposal 1 Introduce AI/ML positioning Case 1 as a new positioning method. - Proposal 2 Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation. The detail stage 3 message extension can be discussed while drafting the stage 3 CR. - Proposal 3 UE autonomous switching between AI/ML and non-AI/ML methods is not allowed. - Proposal 4 The content of error cause is discussed while drafting stage3 CRs. - Proposal 5 UE receives the needed assistance data for calculating UE location for AI/ML in step 3 (ProvideAssistanceData) and UE receives the instruction to perform the interference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list. - Proposal 6 UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control.

In this contribution, we discuss the remaining aspects on "fallback configuration" for Case 1 per Observation 3-5 above and the applicability of the above proposals to the 2nd priority Cases 2a/2b.

# 2.	Discussion

## 2.1	Fallback Operation for Case 1

3GPP currently supports 12 LPP-based positioning methods which can be used individually or in combination (the latter is also referred to as "hybrid positioning") as defined in e.g., TS 38.305 clause 4.3 [2]:

Hybrid positioning using multiple methods from the list of positioning methods above is also supported.

In LPP, this is implemented by a SEQUENCE of requested positioning method(s) in the LPP Request Location Information message:

RequestLocationInformation-r9-IEs ::= SEQUENCE {

commonIEsRequestLocationInformation

CommonIEsRequestLocationInformation	OPTIONAL,	-- Need ON

a-gnss-RequestLocationInformation	A-GNSS-RequestLocationInformation	OPTIONAL,	-- Need ON

otdoa-RequestLocationInformation	OTDOA-RequestLocationInformation	OPTIONAL,	-- Need ON

ecid-RequestLocationInformation		ECID-RequestLocationInformation		OPTIONAL,	-- Need ON

epdu-RequestLocationInformation		EPDU-Sequence						OPTIONAL,	-- Need ON

...,

[[

sensor-RequestLocationInformation-r13

Sensor-RequestLocationInformation-r13

OPTIONAL,	-- Need ON

tbs-RequestLocationInformation-r13	TBS-RequestLocationInformation-r13	OPTIONAL,	-- Need ON

wlan-RequestLocationInformation-r13	WLAN-RequestLocationInformation-r13	OPTIONAL,	-- Need ON

bt-RequestLocationInformation-r13	BT-RequestLocationInformation-r13	OPTIONAL	-- Need ON

]],

[[	nr-ECID-RequestLocationInformation-r16

NR-ECID-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-Multi-RTT-RequestLocationInformation-r16

NR-Multi-RTT-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-DL-AoD-RequestLocationInformation-r16

NR-DL-AoD-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-DL-TDOA-RequestLocationInformation-r16

NR-DL-TDOA-RequestLocationInformation-r16

OPTIONAL	-- Need ON

]]

}

For example, the UE may receive a location request for GNSS and DL-TDOA positioning. The UE may then combine, all the GNSS and DL-TDOA results, if possible, and determine a single location estimate for the target UE. However, it may also be possible that a UE is in a blocked environment (e.g., indoors) and may not receive GNSS signals in this example but may be able to receive sufficient DL-PRS signals from base stations. The UE would then determine the location estimate via DL-TDOA only. The LPP Provide Location Information message (which also implements a SEQUENCE of positioning methods) indicates (indirectly) which positioning method(s) have been used:

ProvideLocationInformation-r9-IEs ::= SEQUENCE {

commonIEsProvideLocationInformation

CommonIEsProvideLocationInformation	OPTIONAL,

a-gnss-ProvideLocationInformation	A-GNSS-ProvideLocationInformation	OPTIONAL,

otdoa-ProvideLocationInformation	OTDOA-ProvideLocationInformation	OPTIONAL,

ecid-ProvideLocationInformation		ECID-ProvideLocationInformation		OPTIONAL,

epdu-ProvideLocationInformation		EPDU-Sequence						OPTIONAL,

...,

[[

sensor-ProvideLocationInformation-r13

Sensor-ProvideLocationInformation-r13

OPTIONAL,

tbs-ProvideLocationInformation-r13	TBS-ProvideLocationInformation-r13	OPTIONAL,

wlan-ProvideLocationInformation-r13	WLAN-ProvideLocationInformation-r13	OPTIONAL,

bt-ProvideLocationInformation-r13	BT-ProvideLocationInformation-r13	OPTIONAL

]],

[[	nr-ECID-ProvideLocationInformation-r16

NR-ECID-ProvideLocationInformation-r16		OPTIONAL,

nr-Multi-RTT-ProvideLocationInformation-r16

NR-Multi-RTT-ProvideLocationInformation-r16 OPTIONAL,

nr-DL-AoD-ProvideLocationInformation-r16

NR-DL-AoD-ProvideLocationInformation-r16	OPTIONAL,

nr-DL-TDOA-ProvideLocationInformation-r16

NR-DL-TDOA-ProvideLocationInformation-r16	OPTIONAL

]]

}

For the GNSS and DL-TDOA example, the UE would include the location coordinates in the CommonIEsProvideLocationInformation and the additional information (e.g., time stamp, etc.) in the A-GNSS-ProvideLocationInformation and NR-DL-TDOA-ProvideLocationInformation. In this way, the LMF knows which location method(s) have been used by the device to determine the reported location estimate (note, some use cases may require the network to report the used location method(s) to the client as additional information; e.g., emergency calls).

Assuming Moderator Proposal 1 in section 1 above can be agreed, AI/ML positioning would be another method in the list of requested positioning methods. For example:

RequestLocationInformation-r9-IEs ::= SEQUENCE {

commonIEsRequestLocationInformation

CommonIEsRequestLocationInformation	OPTIONAL,	-- Need ON

a-gnss-RequestLocationInformation	A-GNSS-RequestLocationInformation	OPTIONAL,	-- Need ON

otdoa-RequestLocationInformation	OTDOA-RequestLocationInformation	OPTIONAL,	-- Need ON

ecid-RequestLocationInformation		ECID-RequestLocationInformation		OPTIONAL,	-- Need ON

epdu-RequestLocationInformation		EPDU-Sequence						OPTIONAL,	-- Need ON

...,

[[

sensor-RequestLocationInformation-r13

Sensor-RequestLocationInformation-r13

OPTIONAL,	-- Need ON

tbs-RequestLocationInformation-r13	TBS-RequestLocationInformation-r13	OPTIONAL,	-- Need ON

wlan-RequestLocationInformation-r13	WLAN-RequestLocationInformation-r13	OPTIONAL,	-- Need ON

bt-RequestLocationInformation-r13	BT-RequestLocationInformation-r13	OPTIONAL	-- Need ON

]],

[[	nr-ECID-RequestLocationInformation-r16

NR-ECID-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-Multi-RTT-RequestLocationInformation-r16

NR-Multi-RTT-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-DL-AoD-RequestLocationInformation-r16

NR-DL-AoD-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-DL-TDOA-RequestLocationInformation-r16

NR-DL-TDOA-RequestLocationInformation-r16

OPTIONAL	-- Need ON

]],

[[

nr-AI-ML-RequestLocationInformation-r19

NR-AI-ML-RequestLocationInformation-r19

OPTIONAL	-- Need ON

]]

}

Similar for the Provide Location Information, capabilities, and assistance data messages. Therefore, "fallback" can be supported as today by requesting/allowing one or more positioning methods in a LPP Request Location Information message. The UE will try to fulfil the location request meeting the required QoS. The LMF selects a method or methods supported by UE and network which are most likely able to fulfil the client requested QoS.

Therefore, "fallback configuration" is up to the network (and network and UE capabilities), but seems not requiring any additional changes to the specification.

Proposal 1: 	Assuming "AI/ML positioning Case 1" is specified as a new NR positioning method, "fallback" to non-AI/ML positioning is supported by including multiple positioning methods in a LPP Request Location Information message. No additional specification work is foreseen specifically for supporting "fallback operation".

The NR positioning methods Multi-RTT, DL-TDOA, and DL-AoD all require DL-PRS assistance data, which are implemented in LPP IE NR-DL-PRS-AssistanceData. The IE NR-DL-PRS-AssistanceData can be included in each of NR-DL-TDOA-ProvideAssistanceData, NR-DL-AoD-ProvideAssistanceData, and NR-Multi-RTT-ProvideAssistanceData. However, if multiple NR positioning methods are requested (e.g., DL-TDOA and DL-AoD hybrid positioning), the IE NR-DL-PRS-AssistanceData is provided only once, and an indexing/pointer scheme is used in the message which does not include the IE NR-DL-PRS-AssistanceData. For example:

NR-DL-TDOA-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r16		NR-DL-PRS-AssistanceData-r16		OPTIONAL,	-- Need ON

nr-SelectedDL-PRS-IndexList-r16		NR-SelectedDL-PRS-IndexList-r16		OPTIONAL,	-- Need ON

...

}

NR-DL-TDOA-ProvideAssistanceData field descriptions

nr-DL-PRS-AssistanceData This field specifies the assistance data reference and neighbour TRPs and provides the DL-PRS configuration for the TRPs. Note, if this field is absent but the nr-SelectedDL-PRS-IndexList field is present, the nr-DL-PRS-AssistanceData may be provided in IE NR-Multi-RTT-ProvideAssistanceData or NR-DL-AoD-ProvideAssistanceData.

Since AI/ML positioning is also a NR positioning method and may require the same NR-DL-PRS-AssistanceData, the additional overhead of requesting "AI/ML Positioning" and another NR positioning method (e.g., DL-TDOA) can be small.

Observation 1:		Assuming "AI/ML positioning Case1" is specified as a new NR positioning method and following existing principles for NR positioning, the overhead for requesting multiple NR positioning methods as "fallback" can be reduced by providing common assistance data only once.

## 2.2	Case 2a/2b Aspects

For 2nd priority Case 2a, RAN1 made the following agreements regarding UE reported measurements:

Agreement [3] For AI/ML assisted positioning Case 2a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via DL RSTD or UE Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

Therefore, the location information in a location report for Case 2a are the existing measurement types for DL-TDOA and Multi-RTT positioning (RSTD and UE Rx-Tx Time Difference, respectively), together with other existing attributes (e.g., LOS/NLOS indicator). Hence, there seems no need to introduce a new LPP positioning method for Case 2a. The Request Location Information can be a request for DL-TDOA or Multi-RTT positioning, which – if needed – could include an indication to request AI/ML enhanced measurements. Similar, the Provide Location Information message can indicate whether the provided measurements are AI/ML generated or not. A fallback to non-AI/ML based measurements would then not mean a change of selected positioning method. E.g., if requested AI/ML enhanced measurements cannot be obtained, a UE could report the 'normal' (non-AI/ML enhanced) measurements (instead of reporting an error cause).

Proposal 2:	AI/ML positioning Case 2a is introduced as an enhancement of DL-TDOA and Multi-RTT positioning method.

For Case 2b, no RAN1 agreements are available yet. However, it is expected that some "channel impulse response" information will be reported, similar to LPP IE NR-AdditionalPathList. Any "additional path" measurement is related to a timing measurement, and therefore, also for Case 2b it is expected that the existing timing measurements (RSTD, UE Rx-Tx Time Difference) are reported in addition to any new/additional path measurement information. Hence, also for Case 2b there seems no need to introduce a new LPP positioning method.

Proposal 3:	It is assumed that AI/ML positioning Case 2b can be introduced as an enhancement of DL-TDOA and Multi-RTT positioning method. Final decision depends on further RAN1 progress.

With the above discussion, the suggested Moderator Proposal 2 for Case 1 in section 1 above can be updated for Case 2a/2b as follows.

Proposal 4:	Existing LPP procedures related to Location Information Transfer for DL-TDOA and Multi-RTT positioning (NR-DL-TDOA-RequestLocationInformation/NR-DL-TDOA-ProvideLocationInformation and NR-Multi-RTT-RequestLocationInformation/NR-Multi-RTT-ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation for Case 2a. The detail stage 3 message extension depends on RAN1 parameter.

Proposal 5:	Existing LPP procedures related to Location Information Transfer for DL-TDOA and Multi-RTT positioning (NR-DL-TDOA-RequestLocationInformation/NR-DL-TDOA-ProvideLocationInformation and NR-Multi-RTT-RequestLocationInformation/NR-Multi-RTT-ProvideLocationInformation messages) are used for providing the measurements for LMF sided model operation for Case 2b. The detail stage 3 message extension depends on RAN1 parameter.

For Case 1, it was discussed/suggested that "UE autonomous switching between AI/ML and non-AI/ML methods is not allowed" (Moderator Proposal 3 in section 1). It seems then consequent that this "UE autonomous switching" is also not allowed for Case 2a. With Proposals 2 and 4 above, this could be implemented similar to the existing LocationInformationType [4] in LPP. The location request may indicate whether "AI/ML enhanced measurements required", "AI/ML enhanced measurements desired, but non-AI/ML enhanced allowed", etc.

Proposal 6: 		For Case 2a, UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.

Proposal 7:	For Case 2a, the LPP 'method'-RequestLocationInformation messages (with 'method' equal to 'DL-TDOA' and 'Multi-RTT') can include an indication whether AI/ML enhanced measurements are requested or not, and whether fallback to non-AI/ML enhanced measurements is allowed or not.

According to Moderator Proposal 4 for Case 1 in section 1 above, "The content of error cause is discussed while drafting stage3 CRs". With the above Proposals 2-5 above, there seems no strict need for new Error Causes. However, this can also be discussed after further RAN1 input when drafting Stage 3 CRs.

Proposal 8:	For Case 2a/2b, existing LPP error cause IEs NR-DL-TDOA-Error and NR-Multi-RTT-Error are used for reporting error reasons. Any additional error causes are discussed while drafting Stage 3 CRs.

Moderator Proposals 5 and 6 in section 1 above are general/existing LPP procedures/behaviour and can be augmented for Case 2a/2b as follows.

Proposal 9:	For Case 2a, the UE receives the needed assistance data for performing measurements for AI/ML in Step 3 (NR-DL-TDOA-ProvideAssistanceData, NR-Multi-RTT-ProvideAssistanceData) and the UE receives the instruction to perform the interference in step 5 (NR-DL-TDOA-RequestLocationInformation, NR-Multi-RTT-RequestLocationInformation). The content of assistance Data and the content of request location information is based upon RAN1 parameter list.

Proposal 10:	For Case 2b, the UE receives the needed assistance data for performing measurements in Step 3 (NR-DL-TDOA-ProvideAssistanceData, NR-Multi-RTT-ProvideAssistanceData) and the UE receives the instruction to report the results in step 5 (NR-DL-TDOA-RequestLocationInformation, NR-Multi-RTT-RequestLocationInformation). The content of assistance Data and the content of request location information is based upon RAN1 parameter list.

Proposal 11:	For Case 2a, the UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control.

# 3.	Summary

In this contribution, we discussed some LCM aspects for AI/ML positioning considering the discussions in "[POST128][026][AIML] LCM Procedure for Positioning Case1 (Ericsson)". The following observations and proposals were made.

Case 1:

Proposal 1: 	Assuming "AI/ML positioning Case 1" is specified as a new NR positioning method, "fallback" to non-AI/ML positioning is supported by including multiple positioning methods in a LPP Request Location Information message. No additional specification work is foreseen specifically for supporting "fallback operation".

Observation 1:		Assuming "AI/ML positioning Case1" is specified as a new NR positioning method and following existing principles for NR positioning, the overhead for requesting multiple NR positioning methods as "fallback" can be reduced by providing common assistance data only once.

Case 2a:

Proposal 2:	AI/ML positioning Case 2a is introduced as an enhancement of DL-TDOA and Multi-RTT positioning method.

Proposal 4:	Existing LPP procedures related to Location Information Transfer for DL-TDOA and Multi-RTT positioning (NR-DL-TDOA-RequestLocationInformation/NR-DL-TDOA-ProvideLocationInformation and NR-Multi-RTT-RequestLocationInformation/NR-Multi-RTT-ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation for Case 2a. The detail stage 3 message extension depends on RAN1 parameter.

Proposal 6: 		For Case 2a, UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.

Proposal 7:	For Case 2a, the LPP 'method'-RequestLocationInformation messages (with 'method' equal to 'DL-TDOA' and 'Multi-RTT') can include an indication whether AI/ML enhanced measurements are requested or not, and whether fallback to non-AI/ML enhanced measurements is allowed or not.

Proposal 9:	For Case 2a, the UE receives the needed assistance data for performing measurements for AI/ML in Step 3 (NR-DL-TDOA-ProvideAssistanceData, NR-Multi-RTT-ProvideAssistanceData) and the UE receives the instruction to perform the interference in step 5 (NR-DL-TDOA-RequestLocationInformation, NR-Multi-RTT-RequestLocationInformation). The content of assistance Data and the content of request location information is based upon RAN1 parameter list

Case 2b:

Proposal 3:	It is assumed that AI/ML positioning Case 2b can be introduced as an enhancement of DL-TDOA and Multi-RTT positioning method. Final decision depends on further RAN1 progress.

Proposal 5:	Existing LPP procedures related to Location Information Transfer for DL-TDOA and Multi-RTT positioning (NR-DL-TDOA-RequestLocationInformation/NR-DL-TDOA-ProvideLocationInformation and NR-Multi-RTT-RequestLocationInformation/NR-Multi-RTT-ProvideLocationInformation messages) are used for providing the measurements for LMF sided model operation for Case 2b. The detail stage 3 message extension depends on RAN1 parameter.

Proposal 10:	For Case 2b, the UE receives the needed assistance data for performing measurements in Step 3 (NR-DL-TDOA-ProvideAssistanceData, NR-Multi-RTT-ProvideAssistanceData) and the UE receives the instruction to report the results in step 5 (NR-DL-TDOA-RequestLocationInformation, NR-Multi-RTT-RequestLocationInformation). The content of assistance Data and the content of request location information is based upon RAN1 parameter list.

Proposal 11:	For Case 2a, the UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control.

Case 2a/2b:

Proposal 8:	For Case 2a/2b, existing LPP error cause IEs NR-DL-TDOA-Error and NR-Multi-RTT-Error are used for reporting error reasons. Any additional error causes are discussed while drafting Stage 3 CRs.

# References

[1]	R2-2500968, "Summary of LCM Procedure for Positioning Case1 (Ericsson)".

[2]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[3]	RP-240499, "Status Report to TSG RAN#103".

[4]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".