3GPP TSG-RAN WG2 #129	R2-2500852

Athens, Greece, 17th – 21st February 2025

Agenda Item:	8.3.2.1 Simulation results

Source:	Ericsson

Title:	Simulation results for spatial domain RRM measurement predictions

Document for:	Discussion

# Introduction

The study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved in RAN plenary #102 [1]. In this contribution, the simulation results for the spatial domain prediction evaluation are discussed.

# Discussion

Case-6 is recognized as middle priority case, where we predict spatial cell(s) in one frequency but neighbor spatial cells. All below results are for 2 GHz frequency. In the below subsections, we evaluated both sub-case 2 and sub-case 3, we also evaluate with and without assistant information (i.e., UE-Location). We also evaluate spatial clustering approach vs singular prediction scenarios.

- Spatial-Cluster: the cells within input-cluster and output-cluster are spatial cells. That is given a 7-sites, and 3-sector per-site, if we divide similarly, we can have a maximum of about 11 input cells and 10 output cells (both input and output cells have to sum-up to 21, and they belong to a single frequency, i.e., 2 GHz). There are many approaches for clustering, but we select the following

  - SC-M1: This method selects even cells as input and odd cells as output (which we use in the below evaluation). Example of InputOutput cells indexing is: In= [0, 2, 4, 6, 8]  Out=[1, 3, 5, 7, 9].

## Investigation of Clustering for Sub-case 2 and Sub-case 3

Table 1 shows evaluation of different cluster size (using SC-M1) for both input and output for sub-case 2 and sub-case 3, without assistance information. In this table, larger cluster size improves prediction performance. For instance, row-6, 7, 8, 9, 10 (which represent cluster-size=11,6,5,3,1, respectively) shows degraded RSRP difference of 1.08, 2.12, 2.89, 5.22, 7.56 dB, respectively. Furthermore, Table 8 shows that performance of sub-case 2 outperforms that of sub-case 3 for all cluster-sizes, as shown when comparing row-1<->row-6: RSRP difference = 0.55 dB<->1.08 dB, row-2<->row-7: RSRP difference= 1.07 dB<->2.12 dB.

Table 1. Spatial prediction - Evaluation of clustering for two-subcases.

| Row | Sub-Case | Input Cells @ 2GHz | Output Cells @ 2GHz | RSRP difference (dB) |
| --- | --- | --- | --- | --- |
| 1 | 2 | 11-Even Spatial Cells [0, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20] | 10-Odd Spatial cells [1, 3, 5, 7, 9, 11, 13, 15, 17, 19] | 0.55 |
| 2 | 2 | 6-Even Spatial Cells [0, 2, 4, 6, 8, 10] | 6-Odd Spatial Cells [1, 3, 5, 7, 9, 11] | 1.07 |
| 3 | 2 | 5-Even Spatial Cells [0, 2, 4, 6, 8] | 5- Odd Spatial Cells [1, 3, 5, 7, 9] | 1.60 |
| 4 | 2 | 3-Even Spatial Cells [0, 2, 4] | 3- Odd Spatial Cells [1, 3, 5] | 3.74 |
| 5 | 2 | 1 Spatial Cell [0] | 1 Spatial Cell [2] | 7.07 |
| 6 | 3 | 11-Even Spatial Cells [0, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20] | 10-Odd Spatial cells [1, 3, 5, 7, 9, 11, 13, 15, 17, 19] | 1.08 |
| 7 | 3 | 6-Even Spatial Cells [0, 2, 4, 6, 8, 10] | 6-Odd Spatial Cells [1, 3, 5, 7, 9, 11] | 2.12 |
| 8 | 3 | 5-Even Spatial Cells [0, 2, 4, 6, 8] | 5- Odd Spatial Cells [1, 3, 5, 7, 9] | 2.89 |
| 9 | 3 | 3-Even Spatial Cells [0, 2, 4] | 3- Odd Spatial Cells [1, 3, 5] | 5.22 |
| 10 | 3 | 1 Spatial Cell [0] | 1 Spatial Cell [2] | 7.56 |

Given Odd/Even Clustering method (cells with even index as input and cells with odd index as output), larger cluster size improves prediction performance.

Prediction performance of Sub-case 2 outperforms that of Sub-case 3.

In RAN2#128 it was agreed that RAN2 will focus on frequency domain for cluster based approach as the benefits in the temporal domain are unclear. However, the results in the table above show that there are significant gains also for spatial domain predictions. Therefore, it should be captured that cluster based approach is beneficial for spatial domain predictions. A larger cluster size improves the prediction performance.

Capture in the TR that cluster as input (i.e. measurements from multiple cells) can improve the prediction accuracy of spatial domain predictions compared to single cell as input.

Capture in the TR that a larger cluster size improves the spatial domain prediction performance.

## Impact of Assistance Information

Table 2 shows evaluation result of cluster based (SC-M1) spatial prediction in sub-case 2 for the case of using assistance information (i.e., UE-Location) and no assistance information. Using UE-Location as assistance information improve performance considerably especially when cluster size is low, as show in row-8 (RSRP difference=7.98 dB) in comparison to row-4 (RSRP difference = 0.55 dB). However, such improvement is not as clear when applied to larger cluster size, as shown when comparing row-1 (RSRP difference=0.39 dB) and row-5 (RSRP difference=0.55 dB).

Table 2. Spatial Prediction - Comparing with and without assistance information.

| Row | Input Cells @ 2GHz | Output Cells @ 2GHz | Assist. Info. | ML Loss |
| --- | --- | --- | --- | --- |
| 1 | 11-Even Spatial Cells [0, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20] | 10-Odd Spatial Cells [1, 3, 5, 7, 9, 11, 13, 15, 17, 19] | UE Location | 0.39 |
| 2 | 6-Even Spatial Cells [0, 2, 4, 6, 8, 10] | 6-Odd Spatial Cells [1, 3, 5, 7, 9, 11] | UE Location | 0.42 |
| 3 | 5-Even Spatial Cells [0, 2, 4, 6, 8] | 5-Odd Spatial Cells [1, 3, 5, 7, 9] | UE Location | 0.52 |
| 4 | Single-Even Spatial Cell [0] | Single-Odd Spatial Cell [1] | UE Location | 0.55 |
| 5 | 11-Even Spatial Cells [0, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20] | 10-Odd Spatial Cells [1, 3, 5, 7, 9, 11, 13, 15, 17, 19] | NO | 0.55 |
| 6 | 6-Even Spatial Cells [0, 2, 4, 6, 8, 10] | 6-Odd Spatial Cells [1, 3, 5, 7, 9, 11] | NO | 1.07 |
| 7 | 5-Even Spatial Cells [0, 2, 4, 6, 8] | 5-Odd Spatial Cells [1, 3, 5, 7, 9] | NO | 1.60 |
| 8 | Single-Even Spatial Cell [0] | Single-Odd Spatial Cell [1] | NO | 7.98 |

Based on the above discussion, we have the following observations:

Using UE location as assistance information improves the prediction accuracy.

The assistance information improves the prediction accuracy when the cluster includes few cells. For bigger cluster, the assistance information has less effect in improving the prediction accuracy.

Capture in the TR that using UE location as assistance information improves the prediction accuracy of spatial domain predictions when the cluster size is small.

## Definition of Measurement Reduction Rate in Spatial domain (MRRS)

In RAN2#126, the following formulas were agreed:

Measurement reduction rate in temporal domain (MRRT) assuming same length of measurement time instances:

MRRT= skipped measurement time instances / total measurement time instances

Measurement reduction rate in spatial domain (MRRS):

MRRS = skipped beams to be measured/ total beams to be measured

The agreed formula for MRRS has some issues. One issue is that the value of skipped instances/total instances is the same if the ratio between the skipped and total instances is the same, even if the values are different. As illustrated in the example in Table 3, if the skipped number is 5 and the total number is 10, it will give the same MRRS as if the skipped number is 7 and the total number is 14. To be able to compare between different experiments, different number of input samples should be reflected in the MRRS.

Another issue is that the current formula will have different denominators if the sum of the skipped and the measured instances are different, meaning that the ratio is not compared in the same way between different experiments. There is a need to have a fixed reference value so that the denominator is fixed to be able to compare the ratios correctly, i.e. a fixed number of total measured instances. In the table below, a comparison between the MRRS using fixed denominator compared with varying denominator. It can be seen that the MRRS values are not correct if there is no scaling factor used to compare different experiments.

Table 3. Motivating Scenarios for using proposed MRRS.

|  |  |  | Denominator |  | MRRS |  |
| --- | --- | --- | --- | --- | --- | --- |
| ID | Input (# RSRP) | Output (# RSRP) | ALL (Fixed) | ALL (#Input+ #Output) | Proposed MRRS, Fixed=21 | Legacy MRRS, ALL (#Input+ #Output) |
| Exp1 | 5 | 5 | 21 | 10 | 76.19% | 50.00% |
| Exp2 | 7 | 7 | 21 | 14 | 66.67% | 50.00% |
| Exp3 | 4 | 7 | 21 | 11 | 141.67% | 63.64% |

When defining MRRS we need to consider multiple factors:

- Fair comparison across multiple experiments, hence a need for fixed scaling factor (FSF), that has the same value across all experiments.

  - For Spatial prediction we choose the fixed scaling factor to be 21 (as it represents 3-cells*7-sites).

- A need to reflect the difference between number of measured samples (References signals) as input to the number of predicted samples (not measured reference signals). Lets call this Input-to-Output ratio (IOR) = IOR= #Cells@Input#Cells@Output

Considering the above, we propose to use the following formula to calculate MRRS (Measurement Reduction Rate in Spatial prediction):

MRRS % = SkippedMeasurement All Cells*IOR*100  =   FSF- Measured FSF*IOR*100

Update the MRRS formula to: 
MRRS = (FSF – Measured) / FSF * IOR * 100, where
FSF is a fixed scaling factor (e.g. 21) and IOR = #input cells / #output cells

## Investigation of UMi vs UMa Performance

Table 4 shows evaluation results of UMa versus UMi performance when the number of output cells (predicted) is fixed and the number of cells in input is variable.

Table 4. Performance Evaluation UMa vs UMi, and MRRS for experiment with variable input and fixed output

| Input Cells @2GHz | Output Cells @2GHz | Environment | RSRP Difference (dB) | Environment | RSRP Difference (dB) | MRRS |
| --- | --- | --- | --- | --- | --- | --- |
| 11 | 4 | UMa | 0.17 | UMi | 0.28 | 17.32% |
| 9 | 4 |  | 0.18 |  | 0.31 | 25.40% |
| 7 | 4 |  | 0.21 |  | 0.38 | 38.10% |
| 6 | 4 |  | 0.23 |  | 0.44 | 47.62% |
| 5 | 4 |  | 0.27 |  | 0.55 | 60.95% |
| 3 | 4 |  | 0.91 |  | 1.78 | 114.29% |
| 1 | 4 |  | 7.00 |  | 8.56 | 380.95% |

From Table 4, we observe:

Prediction with UMi performs worse than UMa predictions.

Prediction with UMi requires higher number of input cells to achieve similar prediction performance as in UMa.

We can achieve high MRRS rate, up-to 114%, while keeping RSRP prediction below 1dB (for UMa)

The lower the input size (number of cells), given fixed output, the worse the model performance.

Table 5 shows evaluation results for Umi environment, varying the number of output cells while keeping fixed the number of cells in input

Table 5. Evaluation of variable output with fixed input size, given UMi environment.

| Input Cells @2GHz | Output Cells @2GHz | RSRP Difference (dB) | MRRS |
| --- | --- | --- | --- |
| 7 | 4 | 0.38 | 38.10% |
| 7 | 7 | 0.41 | 66.67% |
| 7 | 10 | 0.44 | 95.24% |
| 3 | 3 | 1.59 | 85.71% |
| 3 | 4 | 1.78 | 114.29% |
| 3 | 10 | 2.04 | 285.71% |

From Table 5 we observe:

Increasing the size of prediction outputs (number of cells) worsens the performance given a fixed number of input cells.

The worsening behaviour when increasing the number of prediction outputs (given a fixed input) has higher impact when the number of inputs is small (e.g., 3 input cells).

# Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	Given Odd/Even Clustering method (cells with even index as input and cells with odd index as output), larger cluster size improves prediction performance.

Observation 2	Prediction performance of Sub-case 2 outperforms that of Sub-case 3.

Observation 3	Using UE location as assistance information improves the prediction accuracy.

Observation 4	The assistance information improves the prediction accuracy when the cluster includes few cells. For bigger cluster, the assistance information has less effect in improving the prediction accuracy.

Observation 5	Prediction with UMi performs worse than UMa predictions.

Observation 6	Prediction with UMi requires higher number of input cells to achieve similar prediction performance as in UMa.

Observation 7	We can achieve high MRRS rate, up-to 114%, while keeping RSRP prediction below 1dB (for UMa)

Observation 8	The lower the input size (number of cells), given fixed output, the worse the model performance.

Observation 9	Increasing the size of prediction outputs (number of cells) worsens the performance given a fixed number of input cells.

Observation 10	The worsening behaviour when increasing the number of prediction outputs (given a fixed input) has higher impact when the number of inputs is small (e.g., 3 input cells).

Based on the discussion in the previous sections we propose the following:

Proposal 1	Capture in the TR that cluster as input (i.e. measurements from multiple cells) can improve the prediction accuracy of spatial domain predictions compared to single cell as input.

Proposal 2	Capture in the TR that a larger cluster size improves the spatial domain prediction performance.

Proposal 3	Capture in the TR that using UE location as assistance information improves the prediction accuracy of spatial domain predictions when the cluster size is small.

Proposal 4	Update the MRRS formula to:  MRRS = (FSF – Measured) / FSF * IOR * 100, where FSF is a fixed scaling factor (e.g. 21) and IOR = #input cells / #output cells

# References

RP-234055,	SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

RAN2 Chairman’s Notes, RAN2#126