3GPP TSG-RAN WG2 Meeting #129	R2-2500853

Athens, Greece, 17th – 21st February, 2025

Agenda item:	8.7.4.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on additional priority based LCP enhancements in XR

Document for:	Discussion and Decision

# 1. Introduction

For additional priority based LCP enhancements, RAN2#128 further discussed the related issues and had the following agreements:

- As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

In this contribution, we will further discuss details of the additional priority handling and other issues based on the above agreements.

# 2.	Discussion

## 2.1	Additional priority handling

### 2.1.1	Triggering of additional priority

In RAN2#127bis meeting, a separate per LCH remaining time threshold is agreed for the additional priority handling:

Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

However, it has not been discussed when and how this new LCP threshold is applied to the remaining time of the running PDCP discardTimers among all the PDCP SDUs buffered for a logical channel.

In existing DSR procedure, the trigger condition for DSR is applied to the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel as follow, and this is applied to the buffered PDCP SDUs that have not been transmitted in any MAC PDU:

3GPP TS 38.321 V18.3.0 (2024-09) ### 5.4.9 Delay status reporting …… If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel.

For LCP enhancement in Rel-19, similar approach should be applied, but there is no need to limit this mechanism only to PDCP SDUs “that have not been transmitted in any MAC PDU”. In the DSR triggering condition, the reason for this constraint is to avoid a situation where a PDCP SDU is already transmitted successfully in a MAC PDU but its discardTimer is still running and the PDCP SDU is not yet discarded from the PDCP buffer. For LCP enhancement, the constraint “have not been transmitted in any MAC PDU” should be removed. Specifically, assuming that there is an RLC data PDU for initial transmission with remaining time of discardTimer larger than the remaining time threshold when LCP is performed, it is obvious that the default priority is used for the corresponding LCH. And for RLC AM, if the RLC data PDU is not sent successfully and is pending for retransmission, and the remaining time of discardTimer is below the per-LCH configured remaining time threshold when LCP is performed, it is reasonable to use the additional priority for the LCH with the RLC data PDU pending for retransmission, otherwise the RLC data PDU may fail to obtain resources and be discarded if discardTimer expires. Therefore, the condition for using the additional priority for an LCH should be among all the UL data available for transmission.

Proposal 1:	When LCP procedure is initiated for a new transmission, additional priority is applied for the LCH when the smallest remaining value of the running PDCP discardTimers of the PDCP SDU among all the UL data available for transmission in the LCH is below the LCP remaining time threshold for the LCH.

Moreover, 38.321 specifies that the reported Remaining Time field in DSR MAC CE is the shortest remaining value of running PDCP discardTimer at the DSR MAC CE transmission time as below:

- Remaining Time: This field indicates the shortest remaining value of running PDCP discardTimer (described in clause 7.3 in TS 38.323 [4]) among all PDCP SDUs that are buffered for an LCG but have not been transmitted in any MAC PDU, at the time of the first symbol of the first PUSCH transmission that includes this DSR MAC CE. The length of this field is 6 bits. This field is present only if the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the value r in this field indicates a remaining time within the range of (r, r + 1] msec;

Similar mechanism can also be applied to LCP enhancement. Specifically, when LCP procedure is performed, UE can decide whether to use the additional priority based on the remaining time of discardTimer at the time of the scheduled MAC PDU transmission, instead of the time of an UL grant reception or the time of performing LCP.

Proposal 2:	The smallest remaining value of the running PDCP discardTimer of the PDCP SDU among all the UL data available for transmission in the LCH (to be compared with the LCP remaining time threshold for deciding on whether additional priority is applied for an LCH during the resource allocation of the LCP procedure) is determined at the time of scheduled MAC PDU transmission.

To specify the triggering condition for applying the additional priority for an LCH, a text proposal for TS 38.321 specification can be found in Appendix 1.

### 2.1.2	Configuration of additional priority

To configure the additional priority, one approach is to configure only one additional priority for each LCH as discussed in [2] and [3]. In this approach, as long as there is LCH priority-adjusted data buffered on an LCH, the single additional priority for the LCH is applied. However, such additional priority is configured based on the remaining time threshold (e.g., the lower the remaining time threshold, the higher the additional priority) for each LCH, regardless of the data arrival time for each LCH, which may result in an LCH with lower remaining time using a lower priority just because of its higher remaining time LCP threshold.

Taking the following Figure (1) and Table (1) as an example, assuming that the data arrival on each LCH is shown in Figure (1), and the default priority, additional priority for each LCH is shown in Table (1), e.g., considering that LCH1 has higher remaining time threshold than LCH2 (18ms and 12ms), then LCH1 can be configured with lower additional priority than LCH2. For LCH1 and LCH2, assuming that data#1 arrives at LCH1 at T1, data#2 arrives at LCH2 at T2. Upon reception of the UL grant at T3, the remaining time of data#1 and data#2 are both below the configured remaining time threshold. UE performs the LCP procedure based on the additional priority of LCH1 and LCH2. Considering that LCH2 has higher priority, UE transmit data#2 first, and then data#1. However, at T3, the remaining time of data#1 is 8ms, and the remaining time of data#2 is 12ms, data#1 which has less remaining time should be transmitted first, otherwise, data#1 may be discarded if the UL grant is not enough.

To avoid such case, it would be beneficial to allow the NW to configure a list of additional priorities for a UE together with corresponding remaining time ranges. Based on such list, the UE would determine the additional priority for each LCH by considering the remaining time of data in each LCH. With such solution, the LCH priority-adjusted data with lower remaining time can be allocated with resources first.

|  | Default priority | Additional priority |
| --- | --- | --- |
| LCH1 | 5 | 4 |
| LCH2 | 5 | 3 |

Figure (1)                                                                                                Table (1)

Observation 1:	Depending on when the grant is provided from the network, there may be situation where the “LCH priority-adjusted data” on an LCH with lower additional priority has lower remaining time than the data in another LCH with higher additional priority. If NW configures only one additional priority for each LCH based on the remaining time threshold, the “LCH priority-adjusted data” on the LCH with lower remaining time may be impacted, e.g. its PDB may expire.

Proposal 3:	To maximize the chances that the resource allocation goes to LCH with lower remaining time, NW can configure a list of additional priorities, UE determines the additional priority for each LCH based on the smallest remaining time of data buffered on each LCH according to the configured list of additional priorities.

### 2.1.3	Bj enhancement

In existing LCP procedure, resource allocation procedure is performed with consideration of Bj, i.e., tokens in the bucket. In the Initial resource allocation stage, only LCH(s) with Bj > 0 are allocated resources in a decreasing priority order, which means that the LCH priority-adjusted data may not be allocated with resources in the Initial resource allocation procedure if Bj is smaller than the LCH priority-adjusted data volume. In such case, the LCH priority-adjusted data may be further delayed or even be discarded because of the Bj limitation if there are no remaining resources.

To avoid the Bj limitation impact on LCH priority-adjusted data transmission, some companies proposed to raise Bj [4] or set PBR to infinity [5]. There is no doubt that the LCH priority-adjusted data transmission can be assured if Bj is temporarily raised or PBR is set to infinity. Our understanding of Bj and PBR is that it is calculated in real time at every resource allocation during LCP procedure. It is unclear how Bj can be temporarily raised and what value it should be raised to. For PBR setting to infinity, which is related to LCH configured with LCP enhancement, it is not fair to other LCH(s) without LCP enhancement configured which can also be of high priority.

Considering that the PBR and BSD parameters are configured by NW with consideration of the traffic on each LCH, it may be a corner case that the configured PBR and BSD differ greatly from the buffered LCH priority-adjusted data volume. Therefore, even if the LCH priority-adjusted data cannot be allocated resources because of Bj limitation, the difference between Bj and LCH priority-adjusted data is small. If any enhancement is needed, it should not impact UE complexity or fairness.

Observation 2:	LCH priority-adjusted data may not get through if Bj is not enough and could cause further delay.

Observation 3:	Considering that the PBR and BSD parameters are configured by NW with consideration of the traffic on each LCH, enhancement methods based on setting PBR to infinity or temporarily raising the Bj may result in either unfairness to other LCHs or complexity to the LCP procedure.  If an enhancement is needed, fairness and low complexity need to be considered.

To ensure that LCH with LCH priority-adjusted data can be allocated with resources, the following approach with consideration of fairness and complexity can be considered. Specifically, at the start of the Initial resource allocation, UE can select an LCH as long as there is buffered LCH priority-adjusted data, regardless of the value of Bj, through which the LCH(s) with LCH priority-adjusted data would have opportunity to obtain resources.

Furthermore, in current spec, Bj can be negative as described in 38.321 below:

##### 5.4.3.1.3 Allocation of resources 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); [Initial resource allocation] 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally. [Remaining resource allocation] NOTE 1: The value of Bj can be negative.

During the Initial resource allocation, Bj may be updated to a value less than 0 if the volume of LCH priority-adjusted data is larger than initial value of Bj. But considering that the spec already supports Bj with a negative value, so UE can keep performing the resource allocation for an LCH with LCH priority-adjusted data in the Initial resource allocation stage, even if Bj < 0 and there is still LCH priority-adjusted data in the buffer. After that, Bj will be increased as time elapses, which has less impact on data on other LCH without LCH priority-adjusted data. In addition, UE doesn't need to decide which value the Bj should be temporarily raised.

Proposal 4:	In the initial resource allocation, resource should also be allocated for logical channel selected for an UL grant containing LCH priority-adjusted data regardless of the value of Bj.

Based on the above Observations and Proposals, a corresponding text procedure is provided in Appendix.

Proposal 5:	Considering the TP in Appendix 1 as baseline for LCP enhancement.

### 2.1.4	Impact on SRB

With respect to impacts on traffic from SRBs, NW can configure a higher priority for the LCH associated with SRBs. For example, by NW implementation, NW can ensure that the priority for LCH associated with SRBs is higher than that with LCH priority-adjusted data, even when the priority of the LCH with LCH priority-adjusted data has been adapted.

Proposal 6:	For additional priority based LCP enhancement, if there is a concern on traffic from SRBs delayed due to LCHs with LCH priority-adjusted data, it can be left to network implementation to ensure LCH of SRBs are always higher than LCHs with LCH priority-adjusted data.

### Additional priority fallback

RAN2#128 discussed the issues related to additional priority fallback and had the following agreements:

- As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

With the introduction of the additional priority fallback capability, UE can fallback to default priority when there is no LCH priority-adjusted data in the 2nd round of LCP procedure. However, when fallback condition is met, it is currently whether UE can enable the fallback capability by itself or that it needs gNB to enable it. Considering that the LCP enhancement related configuration, e.g., additional priority and remaining time threshold for LCP enhancement, are configured by gNB, it is more reasonable for UE to perform the fallback capabilityl only if gNB enable it.

Proposal 7:	If UE supports the capability of additional priority fallback, UE can perform the additional priority fallback after NW enables it via RRC configuration.

## 2.2	Impact of additional priority handling on Intra-UE prioritization

With respect of additional LCH priority, RAN2#127 discussed the issues of intra-UE prioritization, and have the following FFS:

FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3).

In legacy intra-UE prioritization procedure, for the MAC entity configured with lch-basedPrioritization, the following mechanism is used to determine the priority of an UL grant as defined in 38.321. There is also similar mechanism on deciding on the priority of a SR based on the priority of the logical channel triggering the SR.

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

Considering that an LCH may be adjusted to use the additional priority when there is LCH priority-adjusted data on the LCH, it is more reasonable to use the LCH’s additional priority when deciding the priority of its associated uplink grant when the LCH contains LCH priority-adjusted data. The overlapping cases involve not only the UL-SCH grant, but also the SR resources. Therefore, additional LCH priority should also be considered when deciding the priority of SR which is triggered by LCH with LCH priority-adjusted data, as well as when deciding the priority of an UL-SCH grant which is associated with LCH(s) with LCH priority-adjusted data. Otherwise, the LCH priority actually used for resource scheduling will be different from the LCH priority used when deciding the UL grant and SR priority, which would result in LCH priority-adjusted data being delayed further if its UL grant or SR resources is not selected.

Moreover, we think that this should be considered for both new transmission and retransmission case. In the retransmission case, when the MAC PDU to transmit is already stored in the HARQ buffer, it is possible that the multiplexed data in the MAC PDU may become LCH priority-adjusted data. This should be considered also when determining the priority of the UL grant.

Observation 4:	In the retransmission case, when the MAC PDU to transmit is already stored in the HARQ buffer, it is possible that the multiplexed data in the MAC PDU may become LCH priority-adjusted data. This should be considered also when determining the priority of UL grant.

Proposal 8:	The additional LCH priority should be considered for intra-UE prioritization while determining the priority of UL grant for new transmission, UL grant for retransmission and the priority of SR.

Proposal 9:	Considering the TP in Appendix 2 as baseline for intra-UE prioritization for LCP enhancement.

# 3. Conclusion

In this contribution, we analyzed the potential aspects for LCP enhancements. The following observations and proposals were made:

Triggering of additional priority

Proposal 1:	When LCP procedure is initiated for a new transmission, additional priority is applied for the LCH when the smallest remaining value of the running PDCP discardTimers of the PDCP SDU among all the UL data available for transmission in the LCH is below the LCP remaining time threshold for the LCH.

Proposal 2:	The smallest remaining value of the running PDCP discardTimer of the PDCP SDU among all the UL data available for transmission in the LCH (to be compared with the LCP remaining time threshold for deciding on whether additional priority is applied for an LCH during the resource allocation of the LCP procedure) is determined at the time of scheduled MAC PDU transmission.

Configuration of additional priority

Observation 1:	Depending on when the grant is provided from the network, there may be situation where the “LCH priority-adjusted data” on an LCH with lower additional priority has lower remaining time than the data in another LCH with higher additional priority. If NW configures only one additional priority for each LCH based on the remaining time threshold, the “LCH priority-adjusted data” on the LCH with lower remaining time may be impacted, e.g. its PDB may expire.

Proposal 3:	To maximize the chances that the resource allocation goes to LCH with lower remaining time, NW can configure a list of additional priorities, UE determines the additional priority for each LCH based on the smallest remaining time of data buffered on each LCH according to the configured list of additional priorities.

Bj enhancement

Observation 2:	LCH priority-adjusted data may not get through if Bj is not enough and could cause further delay.

Observation 3:	Considering that the PBR and BSD parameters are configured by NW with consideration of the traffic on each LCH, enhancement methods based on setting PBR to infinity or temporarily raising the Bj may result in either unfairness to other LCHs or complexity to the LCP procedure.  If an enhancement is needed, fairness and low complexity need to be considered.

Proposal 4:	In the initial resource allocation, resource should also be allocated for logical channel selected for an UL grant containing LCH priority-adjusted data regardless of the value of Bj.

TP for Additional priority handling

Proposal 5:	Considering the TP in Appendix 1 as baseline for LCP enhancement.

Impact on SRB

Proposal 6:	For additional priority based LCP enhancement, if there is a concern on traffic from SRBs delayed due to LCHs with LCH priority-adjusted data, it can be left to network implementation to ensure LCH of SRBs are always higher than LCHs with LCH priority-adjusted data.

Additional priority fallback

Proposal 7:	If UE supports the capability of additional priority fallback, UE can perform the additional priority fallback after NW enables it via RRC configuration.

Impact of additional priority handling on Intra-UE prioritization

Observation 4:	In the retransmission case, when the MAC PDU to transmit is already stored in the HARQ buffer, it is possible that the multiplexed data in the MAC PDU may become LCH priority-adjusted data. This should be considered also when determining the priority of UL grant.

Proposal 8:	The additional LCH priority should be considered for intra-UE prioritization while determining the priority of UL grant for new transmission, UL grant for retransmission and the priority of SR.

Proposal 9:	Considering the TP in Appendix 2 as baseline for intra-UE prioritization for LCP enhancement.

# 4. Reference

[1]. RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2]. R2-2408426	Discussion on LCP enhancements of XR traffic	Xiaomi Communications	RAN2#127bis 202410

[3]. R2-2408916	LCP enhancements	Ericsson	RAN2#127bis 202410

[4]. R2-2408121	Discussion on LCP enhancement for XR	vivo	RAN2#127bis 202410

[5]. R2-2409149	Discussion on LCP enhancement for XR	LG Electronics Inc.	RAN2#127bis 202410

# 5. Appendix

## 5.1 Appendix 1

#### 5.4.3.1	Logical Channel Prioritization

##### 5.4.3.1.1	General

The Logical Channel Prioritization (LCP) procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

-	additionalPriorityList which is used if there is data with remaining time of discardTimer below the configured additionalPriorityThreshold.

-	additionalPriorityThreshold which sets the remaining time threshold to use the additional priority for an LCH.

-	lcp-DefaultPriorityFallback which is used to determine whether the priority of the logical channel can fallback to the default priority indicated by priority when there is no delay critical data in the logical channel during the remaining resource allocation phase of the resource allocation among logical channels.

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission;

-	allowedCG-List which sets the allowed configured grant(s) for transmission;

-	allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission;

-	allowedHARQ-mode which sets the allowed UL HARQ mode for transmission.

The following UE variable is used for the Logical channel prioritization procedure:

-	Bj which is maintained for each logical channel j.

The MAC entity shall initialize Bj of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented;

1>	if the value of Bj is greater than the bucket size (i.e. PBR × BSD):

2>	set Bj to the bucket size.

NOTE:	The exact moment(s) when the UE updates Bj between LCP procedures is up to UE implementation, as long as Bj is up to date at the time when a grant is processed by LCP.

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

1>	select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	configuredGrantType1Allowed, if configured, is set to true in case the UL grant is a Configured Grant Type 1; and

2>	allowedServingCells, if configured, includes the Cell information associated to the UL grant. Does not apply to logical channels associated with a DRB configured with PDCP duplication within the same MAC entity (i.e. CA duplication) when CA duplication is deactivated for this DRB in this MAC entity; and

2>	allowedCG-List, if configured, includes the configured grant index associated to the UL grant; and

2>	allowedPHY-PriorityIndex, if configured, includes the priority index (as specified in clause 9 of TS 38.213 [6]) associated to the dynamic UL grant; and

2>	allowedHARQ-mode, if configured, includes the allowed UL HARQ mode for the HARQ process associated to the UL grant.

NOTE:	The Subcarrier Spacing index, PUSCH transmission duration, Cell information, and priority index are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

##### 5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover.

The MAC entity shall, when a new transmission is performed:

1>	determine the priority for the logical channels according to clause 5.4.3.1.4.

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 or with LCH priority-adjusted data regardless of the value of Bj are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2>	if any resources remain:

3>	if lcp-DefaultPriorityFallback is configured:

4> determine the priority for the logical channels according to clause 5.4.3.1.4.

3> all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

NOTE 1:	The value of Bj can be negative.

If the MAC entity is requested to simultaneously transmit multiple MAC PDUs, or if the MAC entity receives the multiple UL grants within one or more coinciding PDCCH occasions (i.e. on different Serving Cells), it is up to UE implementation in which order the grants are processed.

The UE shall also follow the rules below during the scheduling procedures above:

-	the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity;

-	if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible;

-	the UE should maximise the transmission of data;

-	if the MAC entity is given a UL grant size that is equal to or larger than 8 bytes (when eLCID is not used) or 10 bytes (when eLCID is used) while having data available and allowed (according to clause 5.4.3.1) for transmission, the MAC entity shall not transmit only padding BSR and/or padding.

The MAC entity shall:

1>	if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

1>	else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	MAC CE for C-RNTI, or data from UL-CCCH;

-	MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation;

-	MAC CE for Sidelink Configured Grant Confirmation;

-	MAC CE for LBT failure;

-	MAC CE for SL LBT failure according to clause 5.31.2;

-	MAC CE for Timing Advance Report;

-	MAC CE for Delay Status Report;

-	MAC CE for SL-BSR prioritized according to clause 5.22.1.6;

-	MAC CE for SL-PRS Resource Request;

-	MAC CE for (Extended) BSR, with exception of BSR included for padding;

-	MAC CE for (Enhanced) Single Entry PHR, or MAC CE for (Enhanced) Multiple Entry PHR or MAC CE for Single Entry PHR with assumed PUSCH, or MAC CE for Multiple Entry PHR with assumed PUSCH, or MAC CE for Enhanced Single Entry PHR for multiple TRP or MAC CE for Enhanced Multiple Entry PHR for multiple TRP, or MAC CE for Enhanced Single Entry PHR for multiple TRP STx2P or MAC CE for Enhanced Multiple Entry PHR for multiple TRP STx2P;

-	MAC CE for Positioning Measurement Gap Activation/Deactivation Request;

-	MAC CE for the number of Desired Guard Symbols;

-	MAC CE for Case-6 Timing Request;

-	MAC CE for (Extended) Pre-emptive BSR;

-	MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding;

-	MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for Recommended bit rate query;

-	MAC CE for BSR included for padding;

-	MAC CE for SL-BSR included for padding.

NOTE 2:	Prioritization among MAC CEs of same priority is up to UE implementation.

The MAC entity shall prioritize any MAC CE listed in a higher order than 'data from any Logical Channel, except data from UL-CCCH' over NR sidelink transmission.

##### 5.4.3.1.4	Priority determination

For each logical channel selected in clause 5.4.3.1.2:

1>	if the logical channel is configured with additionalPriorityThreshold:

2>	if the smallest remaining value of the running PDCP discardTimers of the PDCP SDU among all the UL data available for transmission buffered for the logical channel is below additionalPriorityThreshold of the LCH at the time of scheduled MAC PDU transmission, and the UL data available for transmission are not yet in the MAC PDU generation:

3>	select one additional priority from the configured additionalPriorityList for the LCH based on the smallest remaining value of the running PDCP discardTimers of the PDCP SDU among all the UL data available for transmission buffered for the logical channel;

2>	else:

3>	use the priority for the logical channel.

1>	else:

2>	use the priority for the logical channel.

NOTE:	UE may select one additional priority from the configured additionalPriorityList for an LCH based on the smallest remaining value of the running PDCP discardTimers of the PDCP SDU among all the UL data available for transmission buffered for the logical channel to maximize the chances that the resource allocation goes to LCH with lower remaining time.

## 5.2 Appendix 2

### 5.4.1	UL Grant reception

Uplink grant is either received dynamically on the PDCCH, in a Random Access Response, configured semi-persistently by RRC or determined to be associated with the PUSCH resource of MSGA as specified in clause 5.1.2a. The MAC entity shall have an uplink grant to transmit on the UL-SCH. To perform the requested transmissions, the MAC layer receives HARQ information from lower layers. An uplink grant addressed to CS-RNTI with NDI = 0 is considered as a configured uplink grant. An uplink grant addressed to CS-RNTI with NDI = 1 is considered as a dynamic uplink grant.

For a BWP configured with sTx-2Panel, the MAC entity considers the PUSCH duration of one uplink grant overlaps with the PUSCH duration of another uplink grant if they are overlapping in time and associated with an srs-ResourceSetId corresponding to the same coresetPoolIndex.

If the MAC entity has a C-RNTI, a Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer or a running cg-SDT-TimeAlignmentTimer and for each grant received for this PDCCH occasion:

1>	if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or

1>	if an uplink grant has been received in a Random Access Response:

2>	if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant:

3>	consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI.

2>	if the uplink grant is for MAC entity's C-RNTI, and the identified HARQ process is configured for a configured uplink grant:

3>	start or restart the configuredGrantTimer for the corresponding HARQ process, if configured;

3>	stop the cg-RetransmissionTimer for the corresponding HARQ process, if running.

2>	stop the cg-SDT-RetransmissionTimer for the corresponding HARQ process, if running.

2>	stop the cg-RRC-RetransmissionTimer for the corresponding HARQ process, if running.

2>	if the uplink grant has been received on the PDCCH for the MAC entity's C-RNTI after the first PUSCH transmission to the Serving Cell; and

2>	if the uplink grant is for a new transmission on the same HARQ process used for the first PUSCH transmission to the Serving Cell:

3>	if there is an ongoing RACH-less handover procedure:

4>	consider the RACH-less handover to be successfully completed and indicate to upper layers.

3>	else if there is an ongoing RACH-less LTM cell switch:

4>	consider the LTM cell switch to be successfully completed and indicate it to upper layers.

2>	deliver the uplink grant and the associated HARQ information to the HARQ entity.

1>	else if an uplink grant for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	start or restart the configuredGrantTimer for the corresponding HARQ process, if configured;

3>	stop the cg-RetransmissionTimer for the corresponding HARQ process, if running;

3>	stop the cg-SDT-RetransmissionTimer for the corresponding HARQ process, if running;

3>	stop the cg-RRC-RetransmissionTimer for the corresponding HARQ process, if running;

3>	deliver the uplink grant and the associated HARQ information to the HARQ entity;

3>	if a logical channel associated with a DRB configured with survivalTimeStateSupport is multiplexed in the MAC PDU stored in the HARQ buffer for the corresponding HARQ process:

4>	trigger activation of PDCP duplication for all configured RLC entities of the DRB.

2>	else if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate configured grant Type 2 deactivation:

4>	trigger configured uplink grant confirmation.

3>	else if PDCCH contents indicate configured grant Type 2 activation:

4>	trigger configured uplink grant confirmation;

4>	store the uplink grant for this Serving Cell and the associated HARQ information as configured uplink grant;

4>	initialise or re-initialise the configured uplink grant for this Serving Cell to start in the associated PUSCH duration and to recur according to rules in clause 5.8.2;

4>	stop the configuredGrantTimer for the corresponding HARQ process, if running;

4>	stop the cg-RetransmissionTimer for the corresponding HARQ process, if running.

For each Serving Cell and each configured uplink grant, if configured and activated and available for use as specified in clause 5.8.2, the MAC entity shall:

1>	if the MAC entity is configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or the PUSCH duration of a MSGA payload for this Serving Cell; or

1>	if the MAC entity is not configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response or the PUSCH duration of a MSGA payload for this Serving Cell:

2>	set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration;

2>	if, for the corresponding HARQ process, the configuredGrantTimer is not running and cg-RetransmissionTimer is not configured and cg-SDT-RetransmissionTimer is not configured, and cg-RRC-RetransmissionTimer is not configured (i.e. new transmission):

3>	if the configured uplink grant is for the initial transmission for the CG-SDT with CCCH message; or

3>	if there is an ongoing CG-SDT procedure and PDCCH addressed to the MAC entity's C-RNTI has been received; or

3>	if the configured uplink grant is for the first PUSCH transmission during an ongoing RACH-less LTM cell switch; or

3>	if the configured uplink grant is for the first PUSCH transmission during an ongoing RACH-less handover procedure; or

3>	if there is no ongoing CG-SDT nor ongoing RACH-less LTM cell switch nor ongoing RACH-less handover procedure:

4>	consider the NDI bit for the corresponding HARQ process to have been toggled;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

2>	else if the cg-RetransmissionTimer for the corresponding HARQ process is configured and not running, then for the corresponding HARQ process:

3>	if the configuredGrantTimer is not running, and the HARQ process is not pending (i.e. new transmission):

4>	consider the NDI bit to have been toggled;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

3>	else if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant (i.e. retransmission on configured grant):

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

2>	else if the cg-SDT-RetransmissionTimer is configured and not running for the corresponding HARQ process; or

2>	if the cg-RRC-RetransmissionTimer is configured and not running for the corresponding HARQ process:

3>	if the configured uplink grant is for the first PUSCH transmission at RACH-less LTM cell switch (i.e., initial new transmission); or

3>	if the configured uplink grant is for the initial transmission of RACH-less handover (i.e., initial new transmission); or

3>	if the configured uplink grant is for the initial transmission for the CG-SDT with CCCH message (i.e., initial new transmission); or

3>	if the configuredGrantTimer is not running or not configured, and PDCCH addressed to the MAC entity's C-RNTI has been received after the initial transmission of the CG-SDT with CCCH message (i.e., subsequent new transmission):

4>	consider the NDI bit to have been toggled;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

3>	if PDCCH addressed to the MAC entity's C-RNTI has not been received and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant for initial transmission of CG-SDT with CCCH message or for its retransmission (i.e., retransmission for initial CG-SDT transmission); or

3>	if RACH-less handover is not successfully completed and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant for initial transmission of RACH-less handover or for its retransmission (i.e., retransmission for initial RACH-less handover transmission); or

3>	if RACH-less LTM cell switch is ongoing and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant for first PUSCH transmission at RACH-less LTM cell switch or for its retransmission (i.e., retransmission for initial transmission at RACH-less LTM cell switch):

4>	consider the NDI bit to have not been toggled;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

For configured uplink grants that are not part of a multi-PUSCH configured grant and neither configured with harq-ProcID-Offset2 nor with cg-RetransmissionTimer, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes

For configured uplink grants that are not part of a multi-PUSCH configured grant and configured with harq-ProcID-Offset2, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

For a multi-PUSCH configured grant (as specified in clause 5.8.2) configured with neither harq-ProcID-Offset2 nor cg-RetransmissionTimer, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [nrofSlotsInCG-Period× floor (CURRENT_symbol / periodicity) + ID_OFFSET] modulo nrofHARQ-Processes

For a multi-PUSCH configured grant configured with harq-ProcID-Offset2, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [nrofSlotsInCG-Period × floor (CURRENT_symbol / periodicity) + ID_OFFSET] modulo nrofHARQ-Processes + harq-ProcID-Offset2

where, if cg-SDT-PeriodicityExt (as defined in TS 38.331 [5]) is not configured, CURRENT_symbol = (SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8]; alternatively, if cg-SDT-PeriodicityExt (as defined in TS 38.331 [5]) is configured, CURRENT_symbol = ((H-SFN × numberOfSFNperH-SFN + SFN) × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSFNperH-SFN, numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive frames per H-SFN, the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8]. For a multi-PUSCH configured grant, ID_OFFSET equals 0 for the first configured uplink grant within a periodicity of the configuration and K for the Kth (1 ≤ K < nrofSlotsInCG-Period) valid configured uplink grant after the first configured uplink grant within the same periodicity. A configured uplink grant in a multi-PUSCH configured grant is considered valid if it satisfies the conditions specified in clause 6.1 in TS 38.214 [7].

For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation selects an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize the HARQ Process ID with the highest priority, where the priority of HARQ process is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection among initial transmission and retransmission with equal priority, the UE shall prioritize retransmissions before initial transmissions. The priority of a HARQ Process for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than the priority of a HARQ Process for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU. If the MAC entity is not configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize retransmissions before initial transmissions. The UE shall toggle the NDI in the CG-UCI for new transmissions and not toggle the NDI in the CG-UCI in retransmissions.

NOTE 1:	If a configured uplink grant is associated with a multi-PUSCH configured grant, CURRENT_symbol refers to the symbol index of the first transmission occasion in the first configured uplink grant within the same periodicity. Otherwise, CURRENT_symbol refers to the symbol index of the first transmission occasion of a bundle of configured uplink grant.

NOTE 2:	A HARQ process is configured for a configured uplink grant where neither harq-ProcID-Offset nor harq-ProcID-Offset2 is configured, if the configured uplink grant is activated and the associated HARQ process ID is less than nrofHARQ-Processes. A HARQ process is configured for a configured uplink grant where harq-ProcID-Offset2 is configured, if the configured uplink grant is activated and the associated HARQ process ID is greater than or equal to harq-ProcID-Offset2 and less than sum of harq-ProcID-Offset2 and nrofHARQ-Processes for the configured grant configuration.

NOTE 3:	If the MAC entity receives a grant in a Random Access Response (i.e. MAC RAR or fallbackRAR), or addressed to Temporary C-RNTI or determines a grant as specified in clause 5.1.2a for MSGA payload and if the MAC entity also receives an overlapping grant for its C-RNTI or CS-RNTI, requiring concurrent transmissions on the SpCell, the MAC entity may choose to continue with either the grant for its RA-RNTI/Temporary C-RNTI/MSGB-RNTI/the MSGA payload transmission or the grant for its C-RNTI or CS-RNTI.

NOTE 4:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned Serving Cell is used to calculate the HARQ Process ID used for configured uplink grants.

NOTE 5:	If cg-RetransmissionTimer is not configured, a HARQ process is not shared between different configured grant configurations in the same BWP.

For the MAC entity configured with lch-basedPrioritization, apply the value of additionalPriority, if configured, as the priority for the LCH for which the running PDCP discardTimer of an PDCP SDU of the LCH that is multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or can be multiplexed(i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU has the remaining value below additionalPriorityThreshold.

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

For the MAC entity configured with lch-basedPrioritization, if the corresponding PUSCH transmission of a configured uplink grant is cancelled by CI-RNTI as specified in clause 11.2A of TS 38.213 [6] or cancelled by a high PHY-priority PUCCH transmission as specified in clause 9 of TS 38.213 [6], this configured uplink grant is considered as a de-prioritized uplink grant. If this de-prioritized uplink grant is configured with autonomousTx, the configuredGrantTimer for the corresponding HARQ process of this de-prioritized uplink grant shall be stopped if it is running. If this de-prioritized uplink grant is configured with autonomousTx, the cg-RetransmissionTimer for the corresponding HARQ process of this de-prioritized uplink grant shall be stopped if it is running.

When the MAC entity is configured with lch-basedPrioritization, apply the value of additionalPriority, if configured, as the priority for the logical channel triggering an SR, if the running PDCP discardTimer of an PDCP SDU buffered for the LCH has the remaining value below additionalPriorityThreshold.

When the MAC entity is configured with lch-basedPrioritization, for each uplink grant delivered to the HARQ entity and whose associated PUSCH can be transmitted by lower layers, the MAC entity shall:

1>	if this uplink grant is received in a Random Access Response (i.e. in a MAC RAR or fallback RAR), or addressed to Temporary C-RNTI, or is determined as specified in clause 5.1.2a for the transmission of the MSGA payload:

2>	consider this uplink grant as a prioritized uplink grant.

1>	else if this uplink grant is addressed to CS-RNTI with NDI = 1 or C-RNTI:

2>	if there is no overlapping PUSCH duration of a configured uplink grant which was not already de-prioritized, in the same BWP, whose priority is higher than the priority of the uplink grant; and

2>	if there is no overlapping PUCCH resource with an SR transmission which was not already de-prioritized and the simultaneous transmission of the SR and the uplink grant is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, and the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant:

3>	consider this uplink grant as a prioritized uplink grant;

3>	consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s);

3>	consider the other overlapping SR transmission(s), if any, as a de-prioritized SR transmission(s), except for the SR transmission(s) whose simultaneous transmission is allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup;

3>	if the de-prioritized uplink grant(s) is a configured uplink grant configured with autonomousTx whose PUSCH has already started:

4>	stop the configuredGrantTimer for the corresponding HARQ process of the de-prioritized uplink grant(s);

4>	stop the cg-RetransmissionTimer for the corresponding HARQ process of the de-prioritized uplink grant(s).

1>	else if this uplink grant is a configured uplink grant:

2>	if there is no overlapping PUSCH duration of another configured uplink grant which was not already de-prioritized, in the same BWP, whose priority is higher than the priority of the uplink grant; and

2>	if there is no overlapping PUSCH duration of an uplink grant addressed to CS-RNTI with NDI = 1 or C-RNTI which was not already de-prioritized, in the same BWP, whose priority is higher than or equal to the priority of the uplink grant; and

2>	if there is no overlapping PUCCH resource with an SR transmission which was not already de-prioritized and the simultaneous transmission of the SR and the uplink grant is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, and the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant:

3>	consider this uplink grant as a prioritized uplink grant;

3>	consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s);

3>	if the de-prioritized uplink grant(s) is a configured uplink grant configured with autonomousTx whose PUSCH has already started:

4>	stop the configuredGrantTimer for the corresponding HARQ process of the de-prioritized uplink grant(s);

4>	stop the cg-RetransmissionTimer for the corresponding HARQ process of the de-prioritized uplink grant(s).

3>	consider the other overlapping SR transmission(s), if any, as a de-prioritized SR transmission(s), except for the SR transmission(s) whose simultaneous transmission is allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup.

NOTE 6:	If the MAC entity is configured with lch-basedPrioritization and if there is overlapping PUSCH duration of at least two configured uplink grants whose priorities are equal, the prioritized uplink grant is determined by UE implementation.

NOTE 7:	If the MAC entity is not configured with lch-basedPrioritization and if there is overlapping PUSCH duration of at least two configured uplink grants, it is up to UE implementation to choose one of the configured uplink grants.

NOTE 8:	If the MAC entity is configured with lch-basedPrioritization, the MAC entity does not take UCI multiplexing according to the procedure specified in TS 38.213 [6] into account when determining whether the PUSCH duration of an uplink grant overlaps with the PUCCH resource for an SR transmission.