3GPP TSG-RAN WG2 Meeting #129	R2-2500854

Athens, Greece, 17th – 21st February, 2025

Agenda item:	8.7.4.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on DSR enhancements in XR

Document for:	Discussion and Decision

# 1. Introduction

For DSR enhancements, RAN2 made the following agreements in RAN2#127bis and RAN2#128:

- RAN2#127bis: Thresholds configuration We do not change the definition of delay-critical data For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. DSR MAC CE details For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - • Buffer size of data volume in each portion - • Shortest remaining time among PDCP SDUs buffered in each portion. There is no need to include PSI in the enhanced DSR MAC CE. A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds - RAN2#128: Thresholds configuration Let the network configure the triggering and reporting thresholds without constraints. RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. Non-delay critical data ahead in the queue The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

In this contribution, we will further discuss DSR enhancement according to the current progress.

# 2.	Discussion

## 2.1	DSR MAC CE format

In RAN2#127bis, the issues of new DSR MAC CE format are discussed and RAN2 agreed the following, where the use of one-bit indication related to the pair of remaining time and buffer size information needs further discussion.

A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

With respect of the one-bit indication for remaining time and buffer size information pair (name as information pair for short) associated with a reporting threshold, RAN2 needs to further discuss whether this one-bit indication is used to indicate the existence of a certain information pair associated with a reporting threshold (Option 1), or to indicate the existence of next information pair associated with a reporting threshold (Option 2).

For Option 1, a separate field for each information pair in an LCG needs to be introduced, indicating the existence of the remaining time and buffer size information pair for a specific threshold range. Figure (1a) show an example of the new DSR MAC CE format in Option 1, assuming that 4 multiple thresholds are configured for each LCG, e.g., a one-byte length new field is introduced for each LCG. For LCG0, if the value of Rangex bit is 1, then the new DSR MAC CE includes the information pair of Rangex for LCG0, if the value of Rangex bit is 0, then the new DSR MAC CE doesn’t include the information pair of Rangex for LCG0, where Rangex can be regarded as the remaining time range between the two configured thresholds. For example, if the value of Range field is 00001100, then the new DSR MAC CE includes the information pair of Range4 and Range3 for LCG0, and doesn’t include the information pair of other Ranges for LCG0. With such information, NW can know whether each information pair of each LCG exists and the corresponding starting bit if an information pair exist, and thus enabling the parallel decoding. In other words, the NW can obtain the full picture of the reported information as soon as receiving the DSR MAC CE, rather than waiting for the decoding of the MAC CE, so that the NW can schedule the transmission resources for UE in a timely manner.

Figure (1a). Example of new DSR MAC CE format in Option 1

Observation 1:	A one-bit indication to indicate whether a certain pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG, can enable the parallel decoding of the new DSR MAC CE by NW.

For Option 2, the one-bit indication is included in the reported information pair in each LCG, indicating whether the next information pair for the LCG exists or not. Taking the following Figure (1b) as an example, where 4 multiple thresholds are configured for each LCG. For LCG0, if the value of One-bitx bit is 1, then the new DSR MAC CE includes further information pair for LCG0, otherwise, the new DSR MAC CE doesn’t include further information pair for LCG0, meaning that the current information pair is the last one for LCG0. For example, if the value of One-bit1 to One-bit3 in LCG0 is 110, then the new DSR MAC CE includes the information pair of remaining time range 1, 2 and 3 for LCG0, and the next information pair is for next LCG. Applying the reserved bit in such way, can reduce the signaling overhead by 1 byte for each LCG for the new DSR MAC CE format, compared with Option 1. However, with respect of decoding, NW cannot have full picture of the reported information as soon as reception of the DSR MAC CE, considering that NW doesn’t know starting bit of each information pair for each LCG with such dynamic one-bit indication, and thus NW should decode in a serial way. Moreover, NW does not know which range is included in the DSR MAC CE until it decodes the shorting remaining time value.

Figure (1b). Example of new DSR MAC CE format in Option 2

Observation 2:	A one-bit indication to indicate whether a further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG, will cause decoding latency (e.g. NW does not know which range has been included until it decodes the remaining time etc.), and impact the transmission of delay-critical data.

Considering that the transmission latency is the key factor for XR traffic, it is better to adopt a DSR MAC CE format which is beneficial for XR scheduling, thus we propose the following:

Proposal 1:	Include a bitmap in a MAC CE where each bit indicates whether a certain pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

## 2.2	Co-existence of R18 and R19 DSR

In RAN2#127bis, the issue of how to use the new DSR MAC CE format is discussed and RAN2 agree the following FFS:

FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

If NW configures multiple reporting thresholds for at least one of the LCGs, UE can report the delay information using the new DSR MAC CE when DSR is triggered for an LCG configured with multiple reporting thresholds, and the reported information is for multiple reporting range. If DSR is triggered for an LCG configured with multiple reporting thresholds, and UE only has information for one threshold, then the legacy DSR MAC CE format is used (note that if data volume is reported for only one reporting range, it has to be the one containing delay critical data).

Proposal 2:	Use the new DSR MAC CE in case there is at least one LCG configured with multiple reporting thresholds which triggered DSR and the reported information is for multiple thresholds, in other cases, legacy DSR format is used.

In addition, even though the new DSR MAC CE format has larger signalling overhead compared with R18 DSR MAC CE, the new DSR MAC CE at most carries around 100 bytes information, which is not a burden for NW to schedule. Hence, in our view, there is no need to consider the case where UL grant is not enough to carry the new DSR MAC CE.

Proposal 3:	There is no need to consider the case where UL grant is not enough to carry the new DSR MAC CE and no need to specify truncated DSR MAC CE.

To specify the report of R19 DSR MAC CE, a text proposal for TS 38.321 specification can be found in Appendix.

Proposal 4:	Considering the TP in Appendix as baseline for report of DSR MAC CE.

## 2.3	DSR Multiple reporting thresholds related issues

### 2.3.1	Buffer size calculation

In existing R18 DSR procedure, the buffer size in the reported DSR MAC CE is specified in MAC spec as follows, where the buffer size is calculated according to the data volume calculation procedure in RLC and the data volume calculation procedure in PDCP respectively.

3GPP TS 38.321 V18.3.0 (2024-09) #### 6.1.3.72 Delay Status Report MAC CE …… - Buffer Size: The Buffer Size field indicates the total amount of delay-critical UL data for an LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively, after the MAC PDU has been built. If the corresponding LCG is configured with additionalBS-TableAllowed and the amount of delay-critical UL data for an LCG is within the buffer sizes specified in Table 6.1.3.1-3, the MAC entity shall use the buffer sizes specified in Table 6.1.3.1-3 to set the value of this field; otherwise, the MAC entity shall use Table 6.1.3.1-2 instead. This field is indicated in number of bytes. The length of this field is 8 bits;

In RLC spec, the data volume calculation for delay-critical RLC data volume is specified as follows, where “the RLC data PDUs that are pending for retransmission” and “STATUS PDU” are considered as part of the delay-critical data volume calculation.

3GPP TS 38.322 V18.2.0 (2024-12) ## 5.5 Data volume calculation …… For the purpose of MAC delay status reporting, the UE shall consider the following as delay-critical RLC data volume: - delay-critical RLC SDUs and delay-critical RLC SDU segments that have not yet been included in an RLC data PDU; - RLC data PDUs pending for initial transmission, and containing a delay-critical RLC SDU or a delay-critical RLC SDU segment; - RLC data PDUs that are pending for retransmission (RLC AM). In addition, if a STATUS PDU has been triggered and t-StatusProhibit is not running or has expired, the UE shall estimate the size of the STATUS PDU that will be transmitted in the next transmission opportunity, and consider this as part of RLC data volume for MAC buffer status reporting and as part of delay-critical RLC data volume for MAC delay status reporting.

In PDCP spec, the data volume calculation for delay-critical PDCP data volume is specified as follows, where “the PDCP Control PDUs”, “the PDCP SDUs to be retransmitted” and “the PDCP Data PDUs to be retransmitted” are considered as part of the delay-critical data volume calculation.

3GPP TS 38.323 V18.2.0 (2024-06) ## 5.15 Data volume calculation for delay status reporting For the purpose of MAC delay status reporting, the transmitting PDCP entity shall consider the following as delay-critical PDCP data volume: - the delay-critical PDCP SDUs for which no PDCP Data PDUs have been constructed; - the PDCP Data PDUs that contain the delay-critical PDCP SDUs and have not been submitted to lower layers; - the PDCP Control PDUs; - for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13; - for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

For R19 DSR, multiple reporting thresholds are introduced based on which UE can provide NW with more information in DSR MAC CE. With configuration of multiple reporting thresholds, UE will report the buffer size information for each reporting threshold, then one issue RAN2 needs to discuss is buffer size of which reporting threshold should contain the above highlighted PDCP and RLC control information or retransmission data? Considering that the RLC entity and PDCP entity will prioritize the transmission of control information and retransmission over other data transmission, including the delay-critical data with smallest remaining time, it is reasonable for UE calculate the data volume of control information and retransmission as the buffer size of the smallest remaining time range, i.e., the buffer size for the smallest reporting threshold that is configured.

Proposal 5:	The data volume of “PDCP control PDU”, “PDCP SDUs to be retransmitted”, “PDCP Data PDUs to be retransmitted”, “RLC data PDUs that are pending for retransmission” and “RLC STATUS PDU” are calculated to the buffer size for the configured smallest reporting threshold.

In RAN2#128 meeting, the issues of reporting non-delay critical data ahead of delay-critical data is discussed and RAN2 agreed the following:

The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

The case where non-delay-critical data appears ahead of delay-critical data includes the low importance data in front of the delay-critical data. Considering the multiple reporting thresholds and that there is no constraint to the configuration on the triggering threshold and the reporting thresholds, non-delay critical data may be in front of delay critical data in each reporting level for the case when more than one reporting threshold is below the triggering threshold. Taking the below Figure (2) as an example, data1-data7 are delay-critical data buffered on the LCH with different remaining time and belong to different reporting thresholds. For each reporting threshold, there may be non-delay-critical data buffered ahead of delay-critical data, e.g., non-delay-critical data ahead of data4 or non-delay-critical data ahead of data2. In our understanding the previous agreement on inclusion of non-delay critical data ahead of delay critical data in the buffer size calculation for DSR should also cover such case and not just the smallest reporting range in the DSR (i.e., it refers to all reporting levels that are below the triggering threshold).

Figure (2). Remaining time of delay-critical data buffered on the LCH

Proposal 6:	RAN2 confirms that the following agreement is for all the reporting levels in the DSR that are below the triggering threshold:

- The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

### 2.3.2	Number of reporting thresholds

In existing R18 DSR procedure, there is no reporting threshold and the DSR triggering threshold is regarded as the reporting threshold to be reported in the legacy DSR MAC CE. For R19 DSR, with the introduction of reporting thresholds, a separate parameter for reporting thresholds can be introduced for gNB to configure the reporting thresholds. Then, the UE can report the data volume according to the configured reporting thresholds while the triggering threshold would only be used for DSR triggering.

Proposal 7:	In the enhanced DSR MAC CE, data volume is reported only according to reporting thresholds and the triggering threshold is used solely for DSR triggering.

However, the number of the configured reporting threshold needs to be discussed. According to the agreement from a previous meeting, it is left to NW implementation to decide the relationship between reporting threshold and triggering threshold. In our view, it should be possible for the NW to also configure a single reporting threshold different than triggering threshold which is not possible with a legacy DSR MAC CE. This would allow the NW to obtain more delay/data volume information and facilitate scheduling. As on the upper limit for the number of reporting thresholds, it can be decided during Stage-3 implementation

Observation 3:	A single reporting threshold can be used in the enhanced DSR MAC CE to allow the NW to obtain more delay/data volume information and facilitate scheduling, than is if triggering threshold is used in legacy DSR MAC CE.

Proposal 8:	The number of reporting threshold can start from 1, up to X to limit the maximum size of the MAC CE where X can be decided during Stage-3 implementation.

# 3. Conclusion

In this contribution, we analyzed the potential aspects for DSR enhancements. The following observations and proposals were made:

DSR MAC CE format:

Observation 1:	A one-bit indication to indicate whether a certain pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG, can enable the parallel decoding of the new DSR MAC CE by NW.

Observation 2:	A one-bit indication to indicate whether a further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG, will cause decoding latency (e.g. NW does not know which range has been included until it decodes the remaining time etc.), and impact the transmission of delay-critical data.

Proposal 1:	Include a bitmap in a MAC CE where each bit indicates whether a certain pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

Co-existence of R18 and R19 DSR

Proposal 2:	Use the new DSR MAC CE in case there is at least one LCG configured with multiple reporting thresholds which triggered DSR and the reported information is for multiple thresholds, in other cases, legacy DSR format is used.

Proposal 3:	There is no need to consider the case where UL grant is not enough to carry the new DSR MAC CE and no need to specify truncated DSR MAC CE.

Proposal 4:	Considering the TP in Appendix as baseline for report of DSR MAC CE.

Buffer size calculation

Proposal 5:	The data volume of “PDCP control PDU”, “PDCP SDUs to be retransmitted”, “PDCP Data PDUs to be retransmitted”, “RLC data PDUs that are pending for retransmission” and “RLC STATUS PDU” are calculated to the buffer size for the configured smallest reporting threshold.

Proposal 6:	RAN2 confirms that the following agreement is for all the reporting levels in the DSR that are below the triggering threshold:

- The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

Number of reporting thresholds

Proposal 7:	In the enhanced DSR MAC CE, data volume is reported only according to reporting thresholds and the triggering threshold is used solely for DSR triggering.

Observation 3:	A single reporting threshold can be used in the enhanced DSR MAC CE to allow the NW to obtain more delay/data volume information and facilitate scheduling, than is if triggering threshold is used in legacy DSR MAC CE.

Proposal 8:	The number of reporting threshold can start from 1, up to X to limit the maximum size of the MAC CE where X can be decided during Stage-3 implementation.

# 4. Appendix

### 5.4.9	Delay status reporting

The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of LCGs. This delay status for an LCG includes remaining time, which is the smallest remaining value of the running PDCP discardTimers among PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU as specified in clause 7.3 in TS 38.323 [4], and the total amount of delay-critical UL data for the LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively.

RRC controls the DSR procedure by configuring the following parameter:

-	remainingTimeThreshold (per LCG): the threshold on remaining time for triggering a DSR for a logical channel within an LCG.

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG:

1>	if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and

1>	if there is no DSR pending for the logical channel:

2>	trigger a DSR for the logical channel.

If there is at least one DSR pending, the MAC entity shall:

1> 	if dsr-reportingThresList-r19 is configured for the LCG that triggered the DSR and the LCG requires reporting delay information for more than one reporting threshold:

2>	if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the refined DSR MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the refined DSR MAC CE as specified in clause 6.1.3.72.

2>	else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR:

3>	trigger a Scheduling Request.

1>	else:

2>	if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the DSR MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the DSR MAC CE as specified in clause 6.1.3.72.

2>	else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR:

3>	trigger a Scheduling Request.

NOTE 1:	The availability of UL-SCH resources for the transmission of the DSR MAC CE follows the same critieria specified in clause 5.4.5.

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR.

A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs.

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

NOTE 2:	It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate all PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader.