3GPP TSG-RAN WG2 Meeting #129	R2-2500855

Athens, Greece, 17th – 21st Feb 2025

Agenda item:	8.10.2.2

Source:	Samsung

Title:	MRO enhancements for CHO with Candidate SCG(s)

Document for:	Discussion and Decision

# Introduction

As per [1], one of the objectives of R19 SON WI is to support the MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs and subsequent CPAC. From RAN2, this involves identification and specification of necessary UE reporting to enhance the mobility parameter tuning as given below. We discuss the remaining issues for CHO with candidate SCG(s) in this contribution.

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

RAN2 has received LS from RAN3 [2] including the below RAN3 agreements with UE impact for CHO with candidate SCG(s).

RAN3 concluded that the following information can be reported by the UE in RLF Report for failure reason detection and optimization:

- The type of the first fulfilled execution condition (e.g. CPAC or CHO). Fulfilment means all events of the type are met.

- Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

- Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

- The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

First of all, we think that RAN2 can confirm that the UE logs this information as provided in the RAN3 LS. RAN2 has also agreed to log the measurement results for candidate PCells and candidate PSCells. We think that the existing MeasResultNR can be reused for logging the agreed information from RAN3. Our understanding is that once the UE includes the fields firstTriggeredEvent and timeBetweenEvents in MeasResultsNR for candidate PCells and PSCells in RLF report, it is equivalent to including the identifier of the PCells and PSCells which met the configured CHO conditions and CPAC conditions. For e.g. the timeBetweenEvents is included only when all eventsof the type are met.  Thus the UE can report the fulfilled execution conditions before T310/T312 expiry or handover failure using the existing information elements.

Proposal 1: RAN2 confirms the support of all the information in RAN3 LS for CHO with candidate SCG(s).

Proposal 2: UE logs the following information in RLF report in MeasResultNR using existing fields:

-	Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

-	Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

-	The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

UE can report the first fulfilled execution condition, i.e. CHO or CPAC outside the measurement results. It is possible that there can be some CHO conditions which are met before CPAC conditions  are fulfilled and some CPAC conditions are met before some CHO conditions are fulfilled. However, we can take a simple approach and log the first fulfilled execution condition outside the measurement results.

Proposal 3: UE logs the type of the first fulfilled execution condition (e.g. CPAC or CHO) in RLF report outside measurement results.

# Conclusion

Proposal 1: RAN2 confirms the support of all the information in RAN3 LS for CHO with candidate SCG(s).

Proposal 2: UE logs the following information in RLF report in MeasResultNR using existing fields:

-	Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

-	Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

-	The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

Proposal 3: UE logs the type of the first fulfilled execution condition (e.g. CPAC or CHO) in RLF report outside measurement results.

# References

[1] RP-234038: New WID on Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4

[2] R3-247908: LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT