3GPP TSG-RAN WG2 Meeting #129						            R2-2500858

Athens, Greece, 17th – 21nd February, 2025

Title: 		Discussion on LCM for UE-sided model for Beam Management use case

Source: 	Huawei, HiSilicon

Agenda item:	8.1.2.2

Document for:	Discussion/Decision

# Introduction

In RAN2 #128 meeting, RAN2 made the following agreements for the UE-side model for BM case, including the functionality applicability report and status exchange of functionality applicability report aspects.

Agreements

- When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can report it to the network.   FFS detailed design

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

- FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

- Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

- Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

In RAN2 #127 meeting, RAN2 had sent a LS[1] to RAN1, and in RAN1 #119 meeting RAN1 has updated the reply[2] and made several agreements below.

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

In this paper, we discuss the FFSs highlighted in the above related to applicable reporting with consideration of the LS from RAN1 and performance monitoring based on latest RAN1 progress.

# Discussion

## Applicable functionality determination

According to the LS[2], RAN1 discussed two directions A) and B) for the functionality configuration in Step 3 as highlighted in yellow and green respectively:

- In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction

It can also be observed from the above that both Direction A) and B) can be used on their own to configure a functionality in Step 3, but it is also possible to apply both at the same time in Step 3 (e.g.  a functionality is partially defined by Direction A and partially defined by Direction B, some functionalities in Direction A and some functionalities in Direction B, same functionality is defined in both Direction A as well as Direction B).

Observation#1: Based on RAN1 LS, the functionalities can be configured/signalled in the following options in Step 3:

Option 1: Only Direction A)

Option 2: Only Direction B)

Option 3: Mix of Direction A and B(e.g. a functionality is partially defined by Direction A and partially defined by Direction B, some functionalities in Direction A and some functionalities in Direction B or even the same functionality is defined in both Direction A as well as Direction B)

RAN2 is tasked to decide on where to include the functionality configuration for Direction A) and/or B) in the RRC message. Since the detailed information is yet to be discussed in RAN1, the best RAN2 can do is to design based on the what we know so far from the RAN1 LS.

For Option 1 (Direction A only), the gNB provides one or more of CSI-ReportConfig for inference operation to the UE in Step 3.  Since it is for inference operation, the functionality configuration (i.e. Set A and B signaled by one or more of CSI-ReportConfig) are automatically available for inference, if UE reports the functionality configuration as applicable in either the UAI or RRC Reconfiguration Complete (i.e. periodic CSI reports are automatically activated and SP and aperiodic can be activated with L1/L2 signalling by the network). From this, Direction A) should be part of the existing CSI reporting framework/configuration, which is in CSI-MeasConfig where the CSI-ReportConfig are included. Further changes to the contents of CSI-ReportConfig (e.g. how to indicate Set A and B and linking of them) can be left to RAN1 as seen in the RAN 1 LS below.

Answer to Q6: Please refer to the agreements related to the Questions from RAN 2. The content of inference configuration as CSI-ReportConfig is to be designed later in RAN1.

For Option 2 (Direction B only), the gNB provides only the parameter settings for Set A and B for a functionality and is not a full CSI report configuration for inference as indicated in the RAN1 LS where it is mentioned that it is for applicable reporting only and not for inference. Even if the UE reports the configuration corresponding to a functionality as applicable, the network will have to send another RRC reconfiguration (i.e. in Step 5) to provide the CSI-ReportConfig configurations for inference. So, if only Direction B is applied, it is more logical to configure the parameter settings for Set A and B in OtherConfig together with the configuration for the UAI reporting for reporting applicability.

For Option 3 (both Direction A and B), as mentioned, there can be different interpretations on this:

Interpretation#1: Some functionalities in Direction A and some functionalities in Direction B or even the same functionality is defined in Direction A as well as Direction B

Interpretation#2: A functionality is partially defined by Direction A and partially defined by Direction B

With Interpetation#1, it will be the same as Option 1 for Direction A and Option 2 for Direction B. However, if it is Interpretation#2 (e.g. other than providing one or more of CSI-ReportConfig for inference operation, the gNB may provide other parameter settings (e.g. Report content related information, time instances of BM-Case 2 etc.)).  With the details still yet to be defined in RAN 1, it is still unclear from the RAN1 LS which interpretation RAN1 is progressing.  RAN1 may also down select to 1 direction at the end. Hence, RAN2 can wait for further RAN1 progress.

Proposal 1: As baseline, the RRC signalling for Direction A and B can be as follow:

- Option 1: Only Direction A): One or more CSI-ReportConfig configurations for inference is included as part of the existing CSI reporting framework/configuration.

- Option 2: Only Direction B): One set or multiple sets of inference related parameters for applicability report is included in OtherConfig together with the configuration of UAI for applicability reporting

For Option 3: Mix of Direction A) and B) (which can be interpreted differently as e.g. a functionality is partially defined by Direction A and partially defined by Direction B, or some functionalities in Direction A and some functionalities in Direction B or even the same functionality is defined in Direction A as well as Direction): Wait for further progress/clarification from RAN1.

Associated ID is agreed as a working assumption by RAN1 to be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, when needed. The network side additional condition alignment can be done in the following ways.

- First, the UE trains the model based on the dataset and associates it with the related associated ID, if provided by the network.

- The UE reports the applicable functionality to the NW, which may be associated with the NW-side additional condition identified by the associated ID.

- NW selects the applicable functionality and determines the inference configuration based on the NW-side additional condition.

In the LS [1], RAN2 asked RAN1 in Q4-1 that:

- Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition?

Based on the reply of Q4-1 below and agreement in RAN1 #119 above:

Answer to Q4-1: There is no consensus yet on whether it is mandatory or optional. There is no conclusion yet on whether it is feasible or not for UE to decide the applicability without NW-side additional condition, and RAN 1 is discussing the related issues.

The NW-side additional condition can be indicated in step 3 optionally in two options depending on the Direction A and B.

Option1 (for Direction A): associated ID may be added in existing L1 CSI-RS framework

Option2 (for Direction B): associated ID may be added in RRC IE. e.g., in OtherConfig or outside L1 CSI-RS framework.

For Option 2 (for Direction B), the associated ID can also be optionally indicated by the UE to the NW in Step 4. In our view, associated ID is optional for the whole LCM process, i.e., even for training phase the NW might not provide it and the UE should still be able to determine the applicability of the functionality based on implementation, but this needs to be concluded by RAN1 based on the above reply from RAN1.

Observation#2: Based on the RAN1’s agreement for Direction A and B, the NW-side additional condition can be indicated in Step 3 optionally. For Direction B, the associated ID can also be optionally indicated by the UE to the NW in Step 4.

Based on Observation#2, the UE reporting behavior can be as follows depending on whether the network indicates or not the NW-side additional condition in Step 3 in all options or whether the UE indicates back or not in Step 4 for Options 2:

- Case 1: If the network indicates the associated ID, the network can indicate the network-side additional condition it is interested in. In this case, the UE reports only the applicable functionalities corresponding to the indicated associated ID(s).

- Case 2: If the network does not indicate the associated ID, the UE can determine which of the functionalities are applicable from UE perspective considering model availability and UE side additional conditions and report those functionalities together with the corresponding associated IDs, if available. Based on this, the network can select and configure the selected applicable functionality for inference according to the current network-side addition condition.

For Case 1, NW may indicate the interested associated IDs when requesting the applicable functionality reporting, and then the UE only needs to report the applicable functionalities associated with the indicated associated IDs. In this case, the UE can do the filtering based on the associated ID information, so that it does not need to report all the applicable functionality information stored. However, if the NW-side additional condition changes to the one not requested earlier from the UE, the network will have to ask the UE to report the applicable functionalities again, with the new associated ID.

For Case 2, when the NW side additional condition changes, the NW can do an update based on the associated ID reported earlier. However, in some situations, it may cause some signaling overhead in case the network is able to currently support only a very few NW-side additional conditions/associated IDs. For, example, if the UE has applicable functionality for associated ID #1, #2, and #3, but the current NW supports only ID#4, then the UE reporting of other IDs is meaningless.

Based on the above analysis, both Case 1 and Case 2 are useful and the choice between them will depend on the NW implementation and current NW-side additional conditions. We think it should be left to NW implementation which option to use at a certain time. If the NW indicates associated ID(s) to UE, the UE checks the indicated associated ID for determination of applicable functionalities; otherwise, the UE only checks UE-side additional conditions and model availability and reports applicable functionalities together with the supported associated IDs, if available.

Proposal 2: The NW-side additional condition is optional in Step 3. The NW can decide whether to provide the NW-side additional condition by implementation, and the applicable functionalities determination can be supported in the following ways:

- In case the network does not provide the UE with associated IDs, the UE performs applicable functionality filtering without considering NW-side additional conditions and reports the (UE-considered) applicable functionalities together with their associated IDs, if available.

- In case the network provides the UE with associated IDs, the UE performs applicable functionality filtering considering them and only reports the functionalities meeting the NW-side additional condition.

## Applicable functionality reporting procedure

As in the RAN1 LS, the report of applicability depends on the configuration defining the functionality in Step 3 based on Direction A) and B) as follow:

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed

In RAN2#127bis, RAN2 agree to that the applicable reporting can be in UAI or in the RRC Reconfiguration Complete message and the aim is to align the report of applicability for both messages.

UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

The above agreement is assumed to be applied to both Direction A) and B).

Observation 3: Regardless of whether Direction A) and/or B) is applied in Step 3. UAI and RRCReconfigurationComplete messages can be used to report applicable functionality and RAN2 aim to align the design on how the applicable functionality are signaled in both messages.

However, the design of the signaling on reporting applicability for Direction A) or B) may not be the same. For Direction A) in step3, the UE may report a bitmap or CSI-ReportConfig IDs corresponding to the inference configuration to indicate the applicability of the functionality. If gNB provides Direction B) where one set or multiple sets of inference related parameters are signaled for applicability report, the UE may indicate the set of the inference related parameters of the applicable functionalities (e.g. a Set ID).  To align the design on how the applicable functionality are signaled in both UAI and RRC Reconfiguration Complete message, the UAI message containing the applicable reporting can be carried as container in RRC Reconfiguration Complete.

Proposal 3: To align the design on how the applicable functionality are signaled in both UAI and RRC Reconfiguration Complete message, the UAI message containing the applicable reporting can be carried as container in RRC Reconfiguration Complete.

Proposal 4: The applicability reporting in the UAI message may indicate either a certain CSI config (for Direction A) and/or some set of parameters (for Direction B)) as applicable (e.g. using a bitmap, or set of indexes etc. which can be further discussed in a stage-3 design)

For Direction A) where one or more CSI-ReportConfig is provided in Step 3 for inference, RAN1 agrees that:

- Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

Also, RAN1 has conclusion below, if CSI-ReportConfig for inference configuration is provided in step5:

- Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig

For non-applicable aperiodic CSI Report and semi-persistent CSI Report, it seems fine for the UE to keep them in the corresponding CSI-ReportConfig configuration since gNB will not trigger or activate them.  It can be left to the gNB to decide whether to deconfigure at Step 5 or keep them when they become applicable in the future. As for non-applicable periodic CSI Report, we need to consider whether to de-configure it to follow the note above as applicable periodic CSI Report is considered activated after RRCReconfigurationComplete. For this case, UE can autonomously release those periodic CSI report configurations that are not applicable after step4, or wait until Step 5 for the NW to release those configurations.  Note that this may occur not just during initial configuration case in Step 3. An applicable periodic CSI Report may become non-applicable some time future after initial configuration. RAN2 agree that UE doesn’t autonomously deactivate or deconfigure an active functionality as follow:

When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

Sticking to the same principle as this agreement, UE should not autonomously release non-applicable periodic CSI report configurations after Step 4.

Proposal 5: if Direction A (i.e. CSI-ReportConfig is provided) in step3, UE doesn’t autonomously release those CSI report configurations that are non-applicable after Step4 but leave them to gNB to decide whether to de-configure the non-applicable CSI-ReportConfig at Step5 (e.g. for periodic CSI Report configurations) or keep the configurations (in the case of aperiodic or semi-persistent CSI Report configurations).

## Applicability status changes

In this section, the reporting of the changes of the applicability of a functionality is discussed based on the following agreements.

- When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can report it to the network.   FFS detailed design

- FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

It seems a common understanding that a functionality as indicated by a UE may not remain applicable or non-applicable all the time. UE internal conditions and/or NW side conditions and/or the availability of the functionality can impact the status of the functionality. The status of a functionality may change from “non-applicable” to “applicable” or from “applicable” to “non-applicable”. Once an applicable functionality is activated, the functionality has to be deactivated if it becomes non-applicable, otherwise system performance will degrade. So, when applicable functionality was requested by the network before, if the previous applicable functionality becomes non-applicable, this status change shall be reported to network, because the network should know as soon as possible to avoid performance degrade and to avoid too frequent signaling with prohibit timer as usual.

There is an FFS on whether status change information needs to be signaled explicitly or implicitly. We believe that the non-applicable functionalities do not need to be indicated explicitly (i.e., the UE always reports only applicable functionalities) and NW can assume that functionalities not reported in the applicable functionality reporting in Step 4 as non-applicable. Here is a simple example as the format below. it is assumed that the NW sends functionality related config in step 3, and then the UE sends the applicability report in RRC complete message at first time, and then sends UAI for possible updates. For green highlighted part, the UE should report the relevant functionality explicitly.

|  | T0 (Complete) | T1 (UAI) | T2 (UAI) | T3 (UAI) |
| --- | --- | --- | --- | --- |
| Func#1 | Applicable | Non-applicable | Applicable | Applicable |
| Func#2 | Non-applicable | Non-applicable | Applicable | Non-applicable |
| Func#3 | Non-applicable | Applicable | Non-applicable | Non-applicable |

Proposal 6a: When the status of the functionality which has been previously requested by the network changes from applicable to non-applicable or vice-versa, UE shall report the status change information to network.

Proposal 6b: UE always reports only applicable functionalities in the applicable functionality reporting, regardless of whether it is reported for the first time or reported upon a status change.

However, it should be noted that as opposed to the changes of NW side conditions, UE internal conditions are unseen and unpredicted from NW perspective. Even if UE could have the flexibility to timely indicate the status changes of the applicable functionality due to UE internal conditions, signaling overhead is introduced, especially for the cases where the changes among different functionality status are frequent. On the other hand, if status updates frequency is limited by the network (e.g. using a prohibit timer), then there may be a delay in UE reporting a functionality as non-applicable. In this case, the NW may need to configure more resources for performance monitoring, but this will introduce additional resource overhead.

Observation 4: Changes among different functionality statuses have an impact on functionality management. Frequent changes may result in frequent activation/deactivation or may require more resources for performance monitoring, which will introduce signaling/resource overhead.

From network perspective, it is preferred to choose such functionalities for inference which are likely to be used for a longer time and which are not prone to frequent status changes, e.g. due to dynamic changes of UE-side conditions. To help the network to choose proper configurations, it would be useful if on top of bare applicability information, the UE also provided also some kind of “stability of applicability” indication. For example, if the UE knows a certain inference operation is likely to exceed its processing power or that the model is only applicable for the current UE speed, the network should be aware of this. Such indication does not have to mention an explicit reason why the functionality status may change, but instead the UE could for example indicate the time during which the UE commits to keep the applicability of the functionality. Such information would allow the network to achieve a better tradeoff between signaling/resource overhead and system performance guarantee and would be helpful for performing appropriate functionality management. For example, if the applicable status can be maintained for a relatively long time, less resources for functionality monitoring are needed. If the applicable status cannot be maintained for a relative long time or cannot be determined reliably, the NW may decide not to activate the functionality, even though it is applicable for the time being, or the NW may still choose to activate the functionality, but configure more resources for performance monitoring.

Observation 5: Frequent functionality applicability changes/updates may have a detrimental impact on the signaling overhead, system performance and complicate AIML functionality management by the network.

Proposal 7: RAN2 should investigate the means to avoid the need of frequent applicability status updates and ensure the stability of inference operation once it is activated (e.g., UE can provide some kind of “stability of applicability” indication for each applicable functionality etc.).

As a commonly used method to avoid the frequent reporting, a prohibit timer can be configured. However, with the prohibit timer, the NW may not get the applicability information timely and cause some problem, as shown in the following cases.

- Case 1: if a functionality becomes “non-applicable” but cannot be updated because the prohibit timer is still running, meanwhile the NW decides to configure this functionality for inference, the inference configuration can fail. In this case, the UE needs to indicate the inference configuration failure to the NW after receiving the inference configuration.

- Case 2: a functionality becomes “applicable” but cannot be updated timely, the NW cannot configure the functionality for inference immediately if the NW is interested in this functionality.

From the two cases, it can cause different results if the NW cannot know the applicability update from “applicable to non-applicable” and from “non-applicable to applicable”. Thus, we need to configure different prohibit timer for these different cases. For example, if the NW wants to avoid the first cases and can endure the second case, the NW can configure the value of the prohibit timer for “applicable to non-applicable” update as 0 or a small value, and configure a proper value (which is larger than the prohibit timer for “applicable to non-applicable”) for the prohibit timer for “non-applicable to applicable” update.

Observation 6: The prohibit timer can be applied to avoid the frequent report for the applicability update of functionalities. However, if the UE cannot report the “applicable to non-applicable” update timely, the NW may configure a non-applicable functionality for inference.  If the UE cannot report the “non-applicable to applicable” update timely, the NW cannot configure the functionality for inference immediately when the NW is interested in this functionality.

Proposal 8: The NW can configure different values of the prohibit timer for “applicable to non-applicable” update and “non-applicable to applicable” update to be applied for functionalities that are not currently configured for inference operation. If a functionality that is currently configured for inference operation becomes non-applicable, the UE should immediately report the status change.

## Performance monitoring

In RAN1 #118bis meeting, RAN1 has agreed Type 1 Option 2 where UE-assisted performance metric is reported to NW for performance monitoring. In RAN1 #119, RAN1 further agreed that the monitoring Type 1 Option 2 support the reusing of CSI framework for the configuration for monitoring result report in L1 signaling:

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues UE measures the dedicated resource set(s) for monitoring.

Hence, for the time being, there seems to be no RAN2 impact other than ASN.1 impact.

Proposal 9: For monitoring Type 1 Option 2 of UE-side model monitoring, there is no RAN2 impact currently based on RAN1 progress so far other than possible ASN.1 impact.

# Conclusion

In this paper, our observations and proposals are summarized as follows:

Applicable functionality determination

Observation#1: Based on RAN1 LS, the functionalities can be configured/signalled in the following options in Step 3:

- Option 1: Only Direction A)

- Option 2: Only Direction B)

- Option 3: Mix of Direction A and B (e.g., a functionality is partially defined by Direction A and partially defined by Direction B, some functionalities in Direction A and some functionalities in Direction B or even the same functionality is defined in Direction A as well as Direction)

Proposal 1: As baseline, the RRC signalling for Direction A and B can be as follow:

- Option 1: Only Direction A): One or more CSI-ReportConfig configurations for inference is included as part of the existing CSI reporting framework/configuration.

- Option 2: Only Direction B): One set or multiple sets of inference related parameters for applicability report is included in OtherConfig together with the configuration of UAI for applicability reporting

For Option 3: Mix of Direction A) and B) (which can be interpreted differently as e.g. a functionality is partially defined by Direction A and partially defined by Direction B, or some functionalities in Direction A and some functionalities in Direction B or even the same functionality is defined in Direction A as well as Direction): Wait for further progress/clarification from RAN1.

Observation#2: Based on the RAN1’s agreement for Direction A and B, the NW-side additional condition can be indicated in Step 3 optionally. For Direction B, the associated ID can also be optionally indicated by the UE to the NW in Step 4.

Proposal 2: The NW-side additional condition is optional in Step 3. The NW can decide whether to provide the NW-side additional condition by implementation, and the applicable functionalities determination can be supported in the following ways:

- In case the network does not provide the UE with associated IDs, the UE performs applicable functionality filtering without considering NW-side additional conditions and reports the (UE-considered) applicable functionalities together with their associated IDs, if available.

- In case the network provides the UE with associated IDs, the UE performs applicable functionality filtering considering them and only reports the functionalities meeting the NW-side additional condition.

Applicable functionality reporting procedure

Observation#3: Regardless of whether Direction A) and/or B) is applied in Step 3. UAI and RRCReconfigurationComplete messages can be used to report applicable functionality and RAN2 aim to align the design on how the applicable functionality are signaled in both messages.

Proposal 3: To align the design on how the applicable functionality are signaled in both UAI and RRC Reconfiguration Complete message, the UAI message containing the applicable reporting can be carried as container in RRC Reconfiguration Complete.

Proposal 4: The applicability reporting in the UAI message may indicate either a certain CSI config (for Direction A) and/or some set of parameters (for Direction B)) as applicable (e.g. using a bitmap, or set of indexes etc. which can be further discussed in a stage-3 design)

Proposal 5: if Direction A (i.e., CSI-ReportConfig is provided) in step3, UE doesn’t autonomously release those CSI report configurations that are non-applicable after Step4 but leave them to gNB to decide whether to de-configure the non-applicable CSI-ReportConfig at Step5 (e.g. for periodic CSI Report configurations) or keep the configurations (in the case of aperiodic or semi-persistent CSI Report configurations).

Applicability status changes

Proposal 6a: When the status of the functionality which has been previously requested by the network changes from applicable to non-applicable or vice-versa, UE shall report the status change information to network.

Proposal 6b: UE always reports only applicable functionalities in the applicable functionality reporting, regardless of whether it is reported for the first time or reported upon a status change.

Observation 4: Changes among different functionality statuses have an impact on functionality management. Frequent changes may result in frequent activation/deactivation or may require more resources for performance monitoring, which will introduce signaling/resource overhead.

Observation 5: Frequent functionality applicability changes/updates may have a detrimental impact on the signaling overhead, system performance and complicate AIML functionality management by the network.

Proposal 7: RAN2 should investigate the means to avoid the need of frequent applicability status updates and ensure the stability of inference operation once it is activated (e.g., UE can provide some kind of “stability of applicability” indication for each applicable functionality etc.).

Observation 6: The prohibit timer can be applied to avoid the frequent report for the applicability update of functionalities. However, if the UE cannot report the “applicable to non-applicable” update timely, the NW may configure a non-applicable functionality for inference.  If the UE cannot report the “non-applicable to applicable” update timely, the NW cannot configure the functionality for inference immediately when the NW is interested in this functionality.

Proposal 8: The NW can configure different values of the prohibit timer for “applicable to non-applicable” update and “non-applicable to applicable” update to be applied for functionalities that are not currently configured for inference operation. If a functionality that is currently configured for inference operation becomes non-applicable, the UE should immediately report the status change.

Performance monitoring

Proposal 9: For monitoring Type 1 Option 2 of UE-side model monitoring, there is no RAN2 impact currently based on RAN1 progress so far other than possible ASN.1 impact.

# References

- R2-2407848, LS on applicable functionality reporting for beam management UE-sided model

- R1-2410898, Reply LS on applicable functionality reporting for beam management UE-sided model