3GPP TSG-RAN WG2 #129	R2-2500864

# Athens, Greece, Feb 17th– 21st, 2025

Agenda Item:	8.13.3

Source:	Ericsson, Apple, AT&T, InterDigital Inc, FirstNet, Qualcomm Incorporated

Title:	Discussion on control plane procedures

Document for:	Discussion, Decision

# 1	Introduction

For the WI on NR sidelink multiple-hop relay, the below objectives on control plane procedures have been defined in the WID [1].

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3]

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

We discuss aspects on control plane procedures in this paper. All discussions refer to L2 U2N Remote UE or L2 U2N Relay UE unless otherwise stated.

# 2	Discussion

## 2.1 Agreements

Regarding whether each intermediate relay UE needs to be in RRC_CONNECTED when the remote UE goes to RRC_CONNECTED, RAN2 has made the below agreements in RAN2#128

- Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED.

  - 1. Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell.

  - 2. Continue to discuss whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment.

- All agreements in this WI apply to both these cases unless otherwise specified.

## 2.2 RRC connection management

Although RAN2 has agreed the baseline that all relay UEs need to move to RRC_CONNECTED in the last relay UE’s serving cell when the remote UE goes to RRC_CONNECTED, we see it is necessary to further discuss whether an intermediate relay UE is allowed to remain in RRC_IDLE/RRC_INACTIVE state when the remote UE triggers connection establishment due to below reasons

- The baseline procedure may suffer from high signalling overhead and long latency

- The baseline procedure restricts all relay UEs in the same serving cell, which limits the number of multi-cell scenarios which are supported

In addition, according to the discussions in RAN2#128, the approach allowing an intermediate UE to stay in a non-RRC_CONNECTED state can reduce the signalling overhead and latency. Meanwhile, companies raised concerns for the approach that

- It may require more design complexity and specification efforts for RAN2.

- An intermediate relay UE in RRC_IDLE/RRC_INACTIVE may not be authorized by the network.

The baseline procedure may suffer from high signalling overhead and long latency.

The baseline procedure restricts all relay UEs in the same serving cell, which limits the number of multi-cell scenarios which are supported.

the approach allowing an intermediate UE to stay in non-RRC_CONNECTED state can give more flexibility for RAN2 design.

Companies raised concerns on the approach that allows a non RRC_CONNECETED intermediate relay UE in the path:

  - It may require more design complexity and specification efforts for RAN2.

  - An intermediate relay UE in RRC_IDLE/RRC_INACTIVE may not be authorized by the network.

Therefore, it is recommended that RAN2 further works on the approach that allows a RRC_IDLE/RRC_INACTIVE intermediate UE in the path aiming for limited additional design complexity and specification efforts.

RAN2 to further work on the approach that allows a RRC_IDLE/RRC_INACTIVE intermediate UE in the path, aiming for limited additional design complexity and specification efforts for RAN2.

Based on the above analyses, it is reasonable to first allow an RRC_INACTIVE intermediate relay UE in the selected path, since we have demonstrated the feasibility of an intermediate UE in RRC_INACTIVE  supporting the authorization procedure with the core network if necessary. The UE can be authorized when the intermediate UE was in RRC_CONNECTED (i.e., before transiting to RRC_INACTIVE).

In addition, whether there is a concern on authorization to allow an RRC_IDLE intermediate UE can be checked with SA2 and SA3. RAN2 can only work on mechanisms to support RRC_IDLE intermediate UE if SA2 and SA3 indicates there is no authorization concern to doing so.

RAN2 to work on mechanisms to allow an intermediate relay UE to stay in RRC_INACTIVE when the remote UE triggers RRC connection establishment.

RAN2 to send an LS to SA2 and SA3 to query whether there is any authorization concern to support an intermediate relay UE to stay in RRC_IDLE when the remote UE triggers RRC connection establishment.

To support an intermediate relay UE to stay in RRC_INACTIVE when the remote UE triggers RRC connection establishment, if the (pre)configuration allows, each intermediate relay UE decides by itself to operate as an intermediate relay UE without moving to RRC_CONNECTED. By allowing this approach, we can achieve the below merits

- Signaling overhead and latency due to connection establishment for remote UE can be reduced.

- The procedure is simpler.

- Additional spec efforts is lower

- The (pre)configuration allows the network to enable or disable the approach. The gNB can send the configuration to the intermediate UE when the intermediate UE was in RRC_CONNECTED before transiting to RRC_INACTIVE.

For Approach 2, if the (pre)configuration allows, each intermediate relay UE decides by itself to operate as an intermediate relay UE without moving to RRC_CONNECTED

## 2.3 E2E QoS Split

End-to-end QoS is guaranteed in relaying by splitting the QoS requirements (i.e., latency) between the different hops. For a relay UE in RRC_CONNECTED when the U2N Remote UE is in RRC_CONNECTED,  it is natural that the gNB performs the QoS splitting for each hop, considering this was done for single hop U2N relays in R17 where the L2 U2N relay also needed to be RRC_CONNECTED.

In approach 2, only the Last Relay UE needs to be in RRC_CONNECTED.  Since the Uu interface is managed by the network, it should be the network to determine the QoS split (i.e., the portion of the latency) associated with the Uu hop (i.e., last hop).

For an intermediate relay UE in RRC_INACTIVE, the network determines the QoS split (i.e., latency allocation) associated with the Uu hop of the last relay UE.

One remaining question is how to perform the splitting over the path between the Last Relay UE and the U2N Remote UE.  If the relays are all in RRC_CONNECTED, the situation is the same as the legacy in Rel-17 for U2N relay, and the network can perform the splitting.  On the other hand, if an intermediate relay is in RRC_INACTIVE, there are two options for QoS split for the PC5 hops/links (i.e., hops between remote UE and last relay UE).

Option 1: Network performs the split.

Option 2: Relay UE serving that link, performs the split.

Option 1 is more simpler, and Remote UE’s serving gNB should be the anchor node.

Option 2 is extended by existing U2U relay.

Option 1 is preferred and also aligned with the baseline procedure, i.e., Approach 1.

For Approach 2, the network (gNB) determines QoS split of the links between an intermediate relay in RRC_INACTIVE and its neighbor UE (remote or another relay UE) in the path.

In the baseline procedure, i.e., Approach 1, it is the gNB that performs QoS splits among the links. The gNB may consider measurement results of the remote UE and each relay UE if they are available. When the gNB performs QoS split for a remote UE during its connection establishment procedure, it is very likely that the gNB doesn’t have measurement results of links/hops reported by each UE, because it takes time for the gNB and each UE to obtain measurement results, i.e.,

- The gNB needs to send measurement configurations to each UE, especially for the link(s) which are newly established.

- After reception of measurement configurations from the gNB, each UE needs to perform measurements and sends measurement reports to the gNB.

It is only therefore feasible for the gNB to perform the QoS split among links/hops considering the number of hops. This would be the case for both Approach 1 and Approach 2. This is also believed to be sufficient for the gNB to derive a stable QoS split among the links/hops in the path.

Regardless of Approach 1 or Approach 2, when the gNB performs QoS split for a remote UE during its connection establishment procedure, it is very likely that the gNB doesn’t have measurement results of links/hops reported by each UE, since it takes time for the gNB and each UE to obtain measurement results.

It is only therefore feasible for the gNB to perform the QoS split among links/hops considering the number of hops. This would be the case for both Approach 1 and Approach 2. This is also believed to be sufficient for the gNB to derive a stable QoS split among the links/hops in the path.

Based on the above analyses,  it is sufficient for QoS split in Approach 2 that the last relay UE reports the number of hops for the associated relay path(s) to the gNB via the SUI message, which requires least additional spec changes. In addition, RAN2 can further discuss how the last relay UE knows the number of hops for the associated path(s).

For Approach 2 QoS split, it is sufficient that the last relay UE reports the number of hops for the associated relay path(s) to the gNB via the SUI message, which requires  least additional spec changes.

For Approach 2, the last relay UE reports the number of hops for the associated relay path(s) to the gNB for QoS split purpose via the SUI message.

In Approach 2 (either Approach 2-1 or Approach 2-2), the last relay UE is required to report the number of hops in addition to the remote UE’s L2 ID to the gNB. Based on the number of hops reported in the SUI message, the gNB splits the QoS among hops and allocates a local ID for the remote UE. The last relay UE also needs to forward the local ID of the remote UE and QoS split for hops/links to intermediate relay UEs in RRC_INACTIVE.

To avoid introducing big changes to the PC5 interface, it is preferred to define SRAP control PDUs to signal relevant information, e.g., local ID, number of hops and QoS split.

It is beneficial for PC5 signalling overhead reduction to rely on control PDUs to forward the local ID of a remote UE, number of hops on a relay path and QoS Split.

For Approach 2, adopt PC5 RRC or new control PDUs in the SRAP layer to forward path related information between UEs on the path, e.g., the local ID of the remote UE, number of hops on the relay path and QoS split.

## 2.4 PC5 RLC channel

In Rel-17 U2N relay, a U2N remote UE sends RRCSetupRequest message via specified PC5 RLC channel to the U2N relay UE. The U2N relay UE is triggered to go to RRC CONNECTED, if it is already not in RRC CONNECTED. Last relay UE sends SUI message to gNB.  Based on that the received SUI message, the gNB configures Uu RLC channel for SRB0. The gNB replies with an RRCSetup message towards the Remote UE to the U2N relay UE on the configured Uu RLC channel. The U2N relay UE forwards the RRCSetup message on the specified PC5 RLC channel. It is sufficient to reuse the same mechanism for SRB0 messages in case of multi-hop U2N relay. This is the same for both Approach 1 and 2.

In Approach 2, each intermediate relay UE in RRC_INACTIVE uses specific PC5 Relay RLC channels to forward SRB0 messages received from the remote UE.

Regarding RLC channel configuration on the PC5 links for SRB1, SRB2 and DRB(s), it is reasonable to reuse the same mechanisms as in the legacy for U2U relay in Rel-18.

Same as in legacy for U2U relay, each intermediate relay UE in RRC_INACTIVE derives its PC5 RLC channel configuration for relaying SRB1, SRB2 and DRB(s) based on SIB or pre-configurations in Approach 2.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The baseline procedure may suffer from high signalling overhead and long latency.

Observation 2	The baseline procedure restricts all relay UEs in the same serving cell, which limits the number of multi-cell scenarios which are supported.

Observation 3	the approach allowing an intermediate UE to stay in non-RRC_CONNECTED state can give more flexibility for RAN2 design.

Observation 4	Companies raised concerns on the approach that allows a non RRC_CONNECETED intermediate relay UE in the path:

a.	It may require more design complexity and specification efforts for RAN2.

b.	An intermediate relay UE in RRC_IDLE/RRC_INACTIVE may not be authorized by the network.

Observation 5	For an intermediate relay UE in RRC_INACTIVE, the network determines the QoS split (i.e., latency allocation) associated with the Uu hop of the last relay UE.

Observation 6	Regardless of Approach 1 or Approach 2, when the gNB performs QoS split for a remote UE during its connection establishment procedure, it is very likely that the gNB doesn’t have measurement results of links/hops reported by each UE, since it takes time for the gNB and each UE to obtain measurement results.

Observation 7	It is only therefore feasible for the gNB to perform the QoS split among links/hops considering the number of hops. This would be the case for both Approach 1 and Approach 2. This is also believed to be sufficient for the gNB to derive a stable QoS split among the links/hops in the path.

Observation 8	For Approach 2 QoS split, it is sufficient that the last relay UE reports the number of hops for the associated relay path(s) to the gNB via the SUI message, which requires  least additional spec changes.

Observation 9	It is beneficial for PC5 signalling overhead reduction to rely on control PDUs to forward the local ID of a remote UE, number of hops on a relay path and QoS Split.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to further work on the approach that allows a RRC_IDLE/RRC_INACTIVE intermediate UE in the path, aiming for limited additional design complexity and specification efforts for RAN2.

Proposal 2	RAN2 to work on mechanisms to allow an intermediate relay UE to stay in RRC_INACTIVE when the remote UE triggers RRC connection establishment.

Proposal 3	RAN2 to send an LS to SA2 and SA3 to query whether there is any authorization concern to support an intermediate relay UE to stay in RRC_IDLE when the remote UE triggers RRC connection establishment.

Proposal 4	For Approach 2, if the (pre)configuration allows, each intermediate relay UE decides by itself to operate as an intermediate relay UE without moving to RRC_CONNECTED

Proposal 5	For Approach 2, the network (gNB) determines QoS split of the links between an intermediate relay in RRC_INACTIVE and its neighbor UE (remote or another relay UE) in the path.

Proposal 6	For Approach 2, the last relay UE reports the number of hops for the associated relay path(s) to the gNB for QoS split purpose via the SUI message.

Proposal 7	For Approach 2, adopt PC5 RRC or new control PDUs in the SRAP layer to forward path related information between UEs on the path, e.g., the local ID of the remote UE, number of hops on the relay path and QoS split.

Proposal 8	In Approach 2, each intermediate relay UE in RRC_INACTIVE uses specific PC5 Relay RLC channels to forward SRB0 messages received from the remote UE.

Proposal 9	Same as in legacy for U2U relay, each intermediate relay UE in RRC_INACTIVE derives its PC5 RLC channel configuration for relaying SRB1, SRB2 and DRB(s) based on SIB or pre-configurations in Approach 2.

# 4	References

[1] RP-241609, “New WID on NR sidelink multi-hop relay”, 3GPP TSG RAN Meeting #104, Shanghai, China, 17 – 20 June 2024.