3GPP TSG-RAN WG2 Meeting #129	R2-2500867

Athens, Greece, 17th – 21st Feb 2025

Agenda item:	8.10.5.3

Source:	Samsung

Title:	MHI Enhancement for SCG Deactivation/Activation

Document for:	Discussion and Decision

# Introduction

RAN2 has send a LS to RAN3 [1] for MHI enhancement for SCG activation/deactivation whether a network solution is good enough, or a UE solution is required.

RAN3 has send a reply LS in [2] as below.

RAN3 thanks RAN2 for the LS on MHI enhancement solution for SCG deactivation/activation. RAN3 has agreed to support a network-based solution for enhancing SCG activation/deactivation. RAN3 understands that UE based solution may also be beneficial in order to monitor SCG deactivation/activation across RRC connected->idle->connected transitions.

We discuss the issue further based on the reply LS.

# Discussion

From the RAN3 reply LS [2], we note that the RAN3 has already decided to go for the network based solution and will enhance the UHI. It is indicated that there is one scenario where the UE based solution may be beneficial - i.e.  if the UE has transitioned across RRC connected->idle->connected, the network may not have the activation status of the PSCells before moving to RRC_IDLE through UHI. The argument is that such an information is helpful to determine whether to configure the PSCells or not while transitioning to RRC_CONNECTED or during first  handover in RRC_CONNECTED. We think that this is not a very strong use case, as when the PSCell was deactivated most of the time while being RRC_CONNECTED previously, the network strategy itself is to configure the PSCell and keep in deactivated, for some reasons such as to minimize latency over SCG bearers. So we propose to first discuss whether there is a real need for logging the SCG activation related information.

Proposal 1: RAN2 to discuss if the UE based solution is required in addition to the network based solution considering the limited benefit.

As there is network-based solution available which covers most of the cases, we expect that most of the networks will not need this data through a UE based solution. Thus, if the RAN2 decides that UE based solution is required, we propose that this should be based on the UE capabilities and the network configuration.

Proposal 2: If the UE based solution is required, it needs to be based on the UE capability and network configuration.

It is also very straight forward that for a network which configures the SCG based on how long the SCG was activated, the time spend with the SCG as activated is the decision making information than the actual time the SCG was present. In the current specifications, timeSpent is used to log the time the UE has spend without  a PSCell or the time the UE has spend in a PSCell. The same field can be used for logging the time spend in activated state.

i.e. if the network configures the UE to report the activation related information, UE can log the time the UE has spend in a PSCell with SCG activated, instead of the time the UE has spend in a PSCell. If the network doesn’t configure the UE to report the activation related information, UE can log the time the UE has spend in a PSCell as in the current specification (ie. Including both activated and deactivated states). Since MHI is really an optional way as UHI is primarily available, we think that all gNBs in the network can handle this information in a similar way. If some gNBs in a network need to work with time UE has stayed in the PSCell and some gNBs need the activation/deactivation time information, even an indication on what is reported in MHI can be considered.

Proposal 3: If the UE based solution is required, UE reports timeSpend as the time the UE has spend in PSCell with SCG activated, based on network configuration.

# Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: RAN2 to discuss if the UE based solution is required in addition to the network based solution considering the limited benefit.

Proposal 2: If the UE based solution is required, it needs to be based on the UE capability and network configuration.

Proposal 3: if the UE based solution is required, UE reports timeSpend as the time the UE has spend in PSCell with SCG activated, based on network configuration.

# References

[1] R2-2407664: LS on MHI enhancement solution for SCG deactivation/activation

[2] R3-247906: Reply LS on MHI enhancement solution for SCG deactivation/activation