3GPP TSG-RAN WG2 Meeting #129			         R2-2500868

Athens, Greece, Feb 17th – 21th, 2025

Agenda Item:	8.4.2

Source: 	LG Electronics Inc.

Title:         	LP-WUS operation in RRC_IDLE and RRC_INACTIVE

Document for: 	Discussion and Decision

Introduction

In this contribution, we will continue the discussion on the procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, based on the agreements made by RAN1, RAN2, and RAN4 so far.

Discussion

- LP-WUS band issue

RAN1 informed RAN2 of their common understanding that UE may not support LP-WUS reception on all the bands supported by the UE. Regarding the LP-WUS separate band issue, RAN made following way forward [1] in RAN#106:

- For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656).  For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered.

- Detailed discussions and final solution selection is left up to WGs.

Since the inter-frequency LP-WUS operation has been excluded, the use of LP-WUS is limited to cases where the UE is camping on a frequency supported by LP-receiver. We have introduced mechanisms where the UE prioritizes a specific frequency for various purposes, e.g. MBS or HSDN, to facilitate moving to that frequency. Similarly, if UE is informed of neighbor cells supporting LP-WUS and is allowed to prioritize the corresponding frequency, it could have more opportunities to use LP-WUS.

The network may not operate LP-WUS on the frequencies supported by the UE’s LP-receiver, hence the network first should inform UE of the frequencies where the LP-WUS is transmitted. However, although a frequency is indicated as a LP-WUS frequency by network, not all UEs should prioritize this frequency. The frequencies supported by the LP-receiver may vary depending on the UE implementation. If the UE’s LP-receiver does not support the frequency, the UE should not prioritize the frequency. That is, UE should be allowed to prioritize only the LP-WUS frequencies that its LP-receiver supports.

Proposal 1	[Option 1] UE should be allowed to prioritize only the LP-WUS frequencies that its LP-receiver supports.

Even though an UE camps on a LP-WUS cell, if the entry condition of using LP-WUS is not met, the UE cannot use LP-WUS. In such case, the UE should be allowed to prioritize another LP-WUS frequency. However, prioritizing another LP-WUS frequency also does not guarantee the use of LP-WUS. The best cell on that frequency may not support the LP-WUS, or the entry condition of the best cell also may not be met. Therefore, UE should be allowed to prioritize a LP-WUS frequency, only if the UE can use LP-WUS by camping on that frequency.

Proposal 2	[Option 1] If the LP-WUS capable UE is not using LP-WUS in current serving cell and can use LP-WUS by camping on a frequency, the UE may consider that frequency to be the highest priority.

Alternatively, the network can provide a separate frequency priority for LP-WUS capable UEs via system information. In this approach, for a LP-WUS frequency, the network may set the legacy priority to a relatively low priority and set the LP-WUS specific priority to a relatively high priority. However, since the frequencies supported by the LP-receiver may be different per UE, LP-WUS capable UEs should not apply the LP-WUS specific priority for all LP-WUS frequencies. If UEs camp on a LP-WUS frequency not supported by their LP-receiver, it may lead to unnecessary overload on that frequency. If the LP-WUS specific frequency priority is provided by network, the UE should determine whether to apply the LP-WUS specific priority or the legacy priority based on its LP-receiver capability. That is, UE should apply the LP-WUS specific priority for frequencies supported by its LP-receiver, but apply the legacy priority for frequencies not supported by its LP-receiver.

Proposal 3	[Option 2] If a LP-WUS specific frequency priority is provided by network, UE should determine whether to apply the LP-WUS specific priority or the legacy priority based on its LP-receiver capability.

- LP-WUS configuration

There has been extensive discussion on whether the entry/exit condition of using LP-WUS is mandatory or optional present over the past several meetings, but RAN2 have yet to reach a conclusion. The proponents of the optional entry/exit condition for using LP-WUS are divided into three camps, each with a different intended UE behaviour when the exit/entry condition is absent in the LP-WUS related configuration, as follows:

- Camp 1. Leave it up to UE implementation, if the entry/exit conditions are absent

- Camp 2. Use default entry/exit condition, if the entry/exit conditions are absent

- Camp 3. Assume that the entry condition is always met, if the entry/exit conditions are absent

In fact, this is not simply an issue of whether to support the optional configuration. Rather, we need to discuss separately whether to support UE behaviour that each camp considers when the entry condition is absent.

Issue #1: Whether the decision on the entry/exit condition of using LP-WUS can be up to UE

The first camp argues that smart UEs can make sensible UE implementation when to start/stop LP-WUS monitoring. However, if UE conservatively determines the entry/exit condition to ensure reliable LP-WUS reception, the MR may lose the opportunity to enter the ultra-deep-sleep and the power saving gain from using LP-WUS will somewhat decreases. Conversely, if the threshold for the entry condition is set too low, or if the threshold for the exit condition is set too high, the UE will monitor LP-WUS with low reliability and may miss paging. Since only a few bits are expected to configure the entry/exit condition, it seems unnecessary to tolerate such potential issues.

Observation 1	The entry/exit condition determined by UE implementation may cause a decrease in UE power saving through LP-WUS monitoring and a degradation in paging reception performance.

Proposal 4	The entry/exit condition for using LP-WUS cannot be decided by UE implementation.

Issue #2: Whether to support default entry/exit condition of using LP-WUS

Since the LP-WUS transmission operates on a cell basis, the appropriate entry/exit conditions may vary for each cell. We are not sure a single default condition can be used in any cell. Considering that only 8bits are needed to configure an RSRP/RSRQ threshold and the entry/exit condition would consist of up to four RSRP/RSRQ thresholds, i.e. MR RSRP, MR RSRQ, LR RSRP and LR RSRQ, signalling gain that can be achieved by not configuring the entry/exit conditions would be trivial. Furthermore, although it is FFS which SIB to use to convey the LP-WUS related configuration, it will be transmitted in on-demand manner.

Proposal 5	Do not support default entry/exit condition for using LP-WUS.

Issue #3: Need for entry/exit condition of using LP-WUS, when LP-WUS and the cell have the same coverage

Although it is FFS whether to support inter-frequency LP-WUS, at least in intra-frequency LP-WUS case, good SSB quality can guarantee good LP-WUS reception quality. Therefore, if the LP-WUS coverage is the same as or larger than the cell coverage and LP-WUS is transmitted on the intra-frequency, the network is not required to provide the entry/exit condition, and UE can assume the entry condition is always met.

Proposal 6	The entry/exit condition for using LP-WUS can be absent, e.g. if the LP-WUS coverage is the same as or larger than the cell coverage and LP-WUS is transmitted on the intra-frequency.

Proposal 7	If the entry/exit condition for using LP-WUS is absent, UE considers the entry condition is always met.

Regarding the LP-WUS related configuration, there is another FFS if dedicated configuration is needed. It is obvious the LP-SS/LP-WUS configuration is cell specific. There is no reason to support dedicated signalling-based configuration for them. However, the LP-WUS configuration may need to be activated/de-activated per UE. For instance, if a UE has a very high paging probability, the power saving gain the UE can achieve through LP-WUS monitoring would be minimal. Additionally, if the LP-WUS is transmitted every time a paging message is sent to such UE, it will cause frequency false alarm for other UEs belonging to the same paging subgroup, significantly reducing their power saving gain. The network can increase the power saving gain of UEs by disabling the WUS monitoring for UEs with a high paging probability. Therefore, we propose followings:

Proposal 8	Do not support LP-WUS related configuration using dedicated signalling.

Observation 3	By disabling WUS monitoring for UEs with a high paging probability, the power saving gain of other UEs using LP-WUS can be increased.

Proposal 9	The LP-WUS monitoring can be activated/de-activated per UE, e.g. via RRC Release message.

- Entry/exit condition of using LP-WUS

To ensure the UE doesn't miss paging, UE should be allowed to use LP-WUS only when the LP-WUS is sufficiently reliable. Regarding the entry/exit condition for LP-WUS monitoring, followings were agreed:

RAN2#127

Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

RAN2#127bis

If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

Although LR threshold is included in the entry condition of using LP-WUS, it does not seem reasonable to force UE to perform LR measurements upon acquisition of the entry condition. The power consumed for LR measurements may not be significant, so some UEs might start LR measurements as soon as it receives the entry condition, meanwhile other UEs that want to minimize the LR measurement may decide whether to perform the LR measurement based on the MR measurement results, e.g. if MR measurement becomes better than the MR threshold in the entry condition. Therefore, we propose to leave when to start the LR measurement outside the LP-WUS coverage up to UE implementation.

Proposal 10	It is up to UE implementation when to start LR measurements.

MR can wake up while performing LP-WUS monitoring using LR for various reasons, e.g. due to the detection of LP-WUS, for performing RRM measurement, for SI request/reception, or for transitioning to RRC_CONNECTED. While MR is active, LR may not need to monitor LP-WUS. However, except the case when transitioning to RRC_CONNECTED, the activated MR will rapidly accomplish its task, e.g. monitoring PO or RRM measurement, then return to sleep. Hence, UE does not need to stop monitoring LP-WUS in such cases. If LR is turned off while MR is active, and then is turned back on after the MR activity ends, MR cannot transition to sleep mode until LR is ready to monitor LP-WUS. This could delay the entry of MR into sleep state. Therefore, unless significant benefit of stopping LP-WUS monitoring during MR activity is revealed, MR activity should not be specified as an exit condition of LP-WUS monitoring.

Proposal 11	If the entry condition of using LP-WUS is met and the LP-WUS indicates no Paging, MR is only allowed to skip PEI-O/PO monitoring but still should perform other existing activities, such as RRM measurements or UL transmission.

If the proposal 8 is accepted, it is obvious that MR wakes up for other activities even though exit condition is met. Therefore, other activities of MR do not need to be specified as exit condition.

Proposal 12	The MR activation caused by RRM measurements or UL transmission is not specified as exit condition of using LP-WUS.

RAN2 agreed that the exit condition of using LP-WUS is only based on the LR measurement. This is under the assumption that the UE has no MR measurement results while using LP-WUS. However, if LP-WUS coverage is larger than the case 3 coverage as shown in figure1, the relaxed MR measurements are available inside the case3 coverage and even normal MR measurements are available outside the case 3 coverage. No MR measurement is only inside the case1 coverage. UEs outside case1 coverage can derive the MR measurement results.

Figure 1

Therefore, we need to re-discuss whether the exit condition of using LP-WUS is still based on the LR measurements only, even though serving cell measurement based on MR is available outside case 1.

Observation 5	In case 3 (RRM relaxation) defined by RAN4, UE has valid MR measurement results and it can be used to determine whether to stop using LP-WUS.

Proposal 13	Discuss whether the exit condition of using LP-WS is only based on LR measurements, even though MR measurement is available, e.g. in RRM relaxation case 3.

Some companies want to have a unified condition for using LP-WUS and RRM offloading. The main reason of that is MR can maintain a longer sleep state, maximizing power saving gain, if it does not perform both paging monitoring and serving cell measurements. Considering that the RRM offloading/relaxation is not suitable for UEs at cell edge, the LP-WUS coverage will be larger than the RRC offloading/relaxation coverage. Using the same condition for using LP-WUS and RRM offloading/relaxation implies that if RRM offloading is not suitable, LP-WUS would also not be used even though it is still reliable. However, the relaxed requirement for serving cell measurement is being defied in RAN4 and in case 3 MR can have more opportunities to sleep by not monitoring PEI-O/PO.

Observation 6	Reliable LP-WUS reception may be possible even outside case1/3 coverage, and MR can get more opportunity to sleep by skipping paging monitoring even without RRM offloading/relaxation.

Proposal 14	The condition for using LP-WUS is not associated with the conditions for RRM offloading/relaxation.

- LP-WUS subgrouping

RAN2#127bis:

For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping.

Some companies suggested to slightly modify the existing formula, e.g. use different UE ID, for UE_ID based PEI subgrouping so that UE can have different subgroup ID for PEI subgroup and LP-WUS subgroup, that is useful to reduce false alarm. However, if the maximum number of subgroups per PO is 32 as assumed by RAN1, there is no need to modify the existing formula, because the UE-ID based PEI subgroup ID is derived using modulo arithmetic between UE ID and the number of subgroups for UE_ID based subgrouping. Although the same UE ID is used, different number of subgroups will be configured for PEI subgrouping and LP-WUS subgrouping. Therefore, there is no reason to modify the existing formula for UE_ID based PEI subgrouping to apply it to the LP-WUS subgrouping, and we propose to reuse the existing formula as it is for UE_ID based LP-WUS subgrouping.

Proposal 15	For UE_ID based LP-WUS subgrouping, the existing formula for UE_ID based PEI subgrouping is reused as it is.

In the PEI subgrouping, the CN assigned subgroup is prioritized over the UE_ID based subgroup. That is, UE determines the UE_ID based subgroup ID only when the CN assigned subgroup ID is not assigned, or the serving cell support the UE_ID based subgrouping. There is no difference between the PEI subgrouping and the LP-WUS subgrouping, except they are transmitted on different channel, so the existing principles used for PEI subgrouping can be directly applied to the LP-WUS subgrouping.

Proposal 16	Reuse existing principles of PEI subgrouping as follows:

      - If the LP-WUS subgroup ID is not assigned by CN, or if the serving cell supports only UE_ID based LP-WUS subgrouping, UE uses a LP-WUS subgroup ID based on UE_ID.

      - If a LP-WUS subgroup ID is assigned by CN, and if the serving cell supports the CN assigned LP-WUS subgrouping, UE uses the LP-WUS subgroup ID assigned by CN.

Conclusion

Regarding LP-WUS band issue:

Proposal 1	[Option 1] UE should be allowed to prioritize only the LP-WUS frequencies that its LP-receiver supports.

Proposal 2	[Option 1] If the LP-WUS capable UE is not using LP-WUS in current serving cell and can use LP-WUS by camping on a frequency, the UE may consider that frequency to be the highest priority.

Proposal 3	[Option 2] If a LP-WUS specific frequency priority is provided by network, UE should determine whether to apply the LP-WUS specific priority or the legacy priority based on its LP-receiver capability.

Regarding LP-WUS configuration:

Proposal 4	The entry/exit condition for using LP-WUS cannot be decided by UE implementation.

Proposal 5	Do not support default entry/exit condition for using LP-WUS.

Proposal 6	The entry/exit condition for using LP-WUS can be absent, e.g. if the LP-WUS coverage is the same as or larger than the cell coverage and LP-WUS is transmitted on the intra-frequency.

Proposal 7	If the entry/exit condition for using LP-WUS is absent, UE considers the entry condition is always met.

Proposal 8	Do not support LP-WUS related configuration using dedicated signalling.

Proposal 9	The LP-WUS monitoring can be activated/de-activated per UE, e.g. via RRC Release message.

Regarding entry/exit condition of using LP-WUS:

Proposal 10	It is up to UE implementation when to start LR measurements.

Proposal 11	If the entry condition is met and the LP-WUS indicates no Paging, MR is only allowed to skip PEI-O/PO monitoring but still should perform other existing activities, such as RRM measurements or UL transmission.

Proposal 12	The MR activation caused by RRM measurements or UL transmission is not specified as exit condition of using LP-WUS.

Proposal 13	Discuss whether the exit condition of using LP-WS is only based on LR measurements, even though MR measurement is available, e.g. in RRM relaxation case 3.

Proposal 14	The condition for using LP-WUS is not associated with the conditions for RRM offloading/relaxation.

Regarding LP-WUS subgrouping

Proposal 15	For UE_ID based LP-WUS subgrouping, the existing formula for UE_ID based PEI subgrouping is reused as it is.

Proposal 16	Reuse existing principles of PEI subgrouping as follows:

      - If the LP-WUS subgroup ID is not assigned by CN, or if the serving cell supports only UE_ID based LP-WUS subgrouping, UE uses a LP-WUS subgroup ID based on UE_ID.

      - If a LP-WUS subgroup ID is assigned by CN, and if the serving cell supports the CN assigned LP-WUS subgrouping, UE uses the LP-WUS subgroup ID assigned by CN.

Reference

[1] RP-243266	Way forward on Rel-19 LP-WUS separate band issues	RAN2 chair (InterDigital)