3GPP TSG-RAN WG2 Meeting #129			         R2-2500871

Athens, Greece, Feb 17th – 21th, 2025

Agenda Item:	8.5.3

Source: 	LG Electronics Inc.

Title:         	On-demand transmission of SIB1

Document for: 	Discussion and Decision

Introduction

In this contribution we will discuss remaining issues on on-demand SIB1.

Discussion

- OD-SIB1 in RRC_CONNECTED

Regarding the OD-SIB1 in RRC_CONNECTED, followings were agreed in RAN#106:

- On RRC Connected mode - The scope of WID will not be updated to include RRC Connected. - However, RAN2 will address the specific case of RLF, to use the same procedure as idle/inactive to request OD-SIB1 transmission. No need to change the scope of WI to indicate this

RAN2 has thus far limited our discussion on the OD-SIB1 to cell reselection procedure, and agreed to use the cell re-selection criteria as the WUS triggering condition in OD-SIB1 acquisition. However, the target cell of the RRC connection re-establishment is selected by performing the cell selection, not cell re-selection. When UE performs the cell selection for RRC connection re-establishment, it has no serving cell and the cell reselection criteria cannot be fulfilled. Therefore, RAN2 should define a new WUS triggering condition for this scenario.

In this scenario, the UE can camp on the NES cell only if at least the cell selection criteria are fulfilled. This means that the UE should not be allowed to transmit WUS to the NES Cell, if it cannot satisfy the cell selection criteria. However, the UE even cannot check whether the NES cell satisfy the cell selection criteria or not before acquiring SIB1, because the essential information, i.e. Qrxlevmin, Qrxlevminoffset, and p-Max, of the NES cell is transmitted in SIB1. Therefore, the essential information for cell selection criteria for the NES cell should be provided to UE in advance, e.g. in the WUS configuration. Then, the UE is able to filter out unsuitable NES cells which does not satisfy the cell selection criteria and save time by transmitting WUS only to the suitable NES cell. Since the cell selection is a very delay-sensitive procedure, such a new WUS triggering condition should be defined.

Proposal 1	In OD-SIB1 request for cell selection, the UE transmits WUS to a NES cell, only if the cell selection criteria are fulfilled for the NES cell.

- Collision between Paging Occasion and Random Access Occasion for WUS transmission

If the SIB1 request condition is met, the UE transmits MSG1 to the neighbor NES cell and then monitors the neighbor NES cell to receive MSG2 . Until now, there has been no cases where UEs in RRC_IDLE/INACTIVE perform the random access procedure on the neighbor cell. UE in RRC_IDLE/INACTIVE should monitor periodically the serving cell for paging reception. If UE transmits MSG1 to the neighbor NES cell or monitors the neighbor NES cell for MSG2 during the paging occasion, the UE could miss the paging. Obviously, the paging reception from serving cell is much more important than acquiring SIB1 of the neighbor cell. The MSG1 transmission and MSG2 reception for OD-SIB1 of neighbor cell should not interfere with the paging reception from serving cell.

Observation 1	The paging reception from serving cell should be prioritized over the MSG1 transmission and MSG2 reception from neighbor NES cell.

Therefore, UE should select a PRACH occasion which does not overlap with the paging occasion, and the RAR window associated with the selected PRACH occasion also should not overlap with the paging occasion.

Proposal 2	[Alt 1] When determining the PRACH occasions of the neighbor NES cell for WUS transmission, the UE takes into account the possible overlapping between the paging occasion and PRACH occasion/RAR window.

Alternatively, UE can select PRACH resource regardless of the paging occasion. After that, if the RAR window for MSG reception is overlapped with the paging occasion, the UE should monitor the serving cell for paging.

Proposal 2`	[Alt 2] If RAR window for SIB1 request and the paging occasion are overlapped, the UE monitors serving cell for paging.

- Broadcast status of OD-SIB1

When a neighbor NES Cell satisfies the SIB1 request condition, if SIB1 of a neighbor NES Cell is already being broadcast and the UE is aware of this, it can save its power by acquiring the SIB1 from the NES Cell without transmitting WUS. For the same reason, network informs UEs of si-BroadcastStatus for each SI message in the existing SI request procedure. In the last meeting, RAN2 agreed to adopt similar approach to on-demand SIB1.

Agreement in RAN2#127bis 8. If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

If si-BroadcastStatus of SIB1 of the neighbor NES cell is provided by Cell A, the UE can check if SIB1 of the neighbor NES Cell is being broadcast before requesting SIB1 from that cell. However, cell A also can indicate the broadcast status implicitly without introducing si-BroadcastStatus for SIB1 by not providing the WUS configuration. Whether the broadcast status of OD-SIB1 is indicated via explicit indication or implicitly via the absent of the WUS configuration, the UE can know the broadcast status by readings SIB x of Cell A. If the WUS configuration is not provided, UE will consider the cell as a normal cell and will not trigger WUS transmission.

Proposal 3	No enhancements to inform UEs of the broadcast status of OD-SIB1 of neighbor NES cell. (The WUS configuration can be absent while OD-SIB1 is being broadcast.)

- Validity of WUS configuration

Agreement in RAN2#128 9. UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

In the previous meeting, RAN2 discussed the validity of the WUS configuration, but it is still FFS whether SIB x including the WUS configuration can be configured as an area specific SIB. We already assume that one NES cell can be associated with more than one anchor cells. The NES cell will provide the same WUS configuration to all associated anchor cells. Though the SIB x is configured as an area specific SIB, if the WUS configuration has changed, the UE can detect this change through the value tag of SIB x and re-acquire the updated WUS configuration. therefore, there is no need to restrict SIB x to being cell specific SIB.

Proposal 4	The SIB x including the WUS configuration can be configured as an area specific SIB.

Conclusion

Proposal 1	In OD-SIB1 request for cell selection, the UE transmits WUS to a NES cell, only if the cell selection criteria are fulfilled for the NES cell.

Proposal 2	[Alt 1] When determining the PRACH occasions of the neighbor NES cell for WUS transmission, the UE takes into account the possible overlapping between the paging occasion and PRACH occasion/RAR window.

Proposal 2`	[Alt 2] If RAR window for SIB1 request and the paging occasion are overlapped, the UE monitors serving cell for paging.

Proposal 3	No enhancements to inform UEs of the broadcast status of OD-SIB1 of neighbor NES cell. (The WUS configuration can be absent while OD-SIB1 is being broadcast.)

Proposal 4	The SIB x including the WUS configuration can be configured as an area specific SIB.