3GPP TSG-RAN WG2 Meeting #129		R2-2500874

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item	: 8.6.3

Source	: Fraunhofer HHI, Fraunhofer IIS

Title	: L1 event-triggered measurement reporting for LTM

Document for	: Discussion

# Introduction

RAN2#128 agreed the following [1]:

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT. - TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam. - TTT is applied to the leaving condition. - Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation. - Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1. - SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE. - For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE. - For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE. - A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals. - Support the truncated measurement report MAC CE.

RAN2#127bis agreed the following [2]:

- MR can be sent when the leaving condition is met, based on NW configuration. - Event triggered periodic MR can be supported, based on NW configuration. - For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3. - For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed. - For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed. - The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1. - TTT operates only based on a timer (like TTT used in L3 event triggered MR). - Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5). - Basic information included in MR MAC CE: Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams Triggered event information (e.g., ReportConfigID) MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW. - Additional information included in MR MAC CE: The information and quantity of current beam, based on NW configuration. - The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. - NW can configure a dedicated SR configuration for MR MAC CE transmission.

RAN2#127 agreed the following [3]:

- Event LTM1 is not defined. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. - Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. - R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. - MAC layer handles the event evaluation and measurement report triggering. - Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

RAN2#126 agreed the following [4]:

1. Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. 2. For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. 3. Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1. 4. Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. 5. RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. 6. For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

This paper discusses further aspects related to measurement enhancement for LTM.

# Reference Signal Configuration

RAN2#128 reached an agreement that Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation. There are scenarios where the UE may need to evaluate both reference signals simultaneously to ensure accurate event detection. Based on the consensus from the previous meeting, either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, a proper configuration of the UE with both SSB and CSI-RS is crucial to ensure alignment with the reference signal selection strategy.

Furthermore configuring the UE with both SSB and CSI-RS as reference signals provides greater flexibility since neither the network nor the UE can predict in advance which RS type will satisfy the event condition. For example, in handover scenarios where one cell primarily uses SSB while the other cell relies on CSI-RS, the UE must be capable of evaluating both to ensure a smooth transition. Similarly, in beam failure recovery cases, if the serving beam degrades, the UE needs to assess both RS types to determine the best recovery option.

Therefore, the UE should be configured to measure and evaluate both SSB and CSI-RS reference signals for LTM event evaluation. This approach ensures maximum adaptability and guarantees that mobility decisions are based on the most suitable RS type in any given scenario.

Proposal 1: UE is configured with both SSB and CSI-RS reference signals for LTM event evaluation purposes.

# LTM Event Reporting Procedures

## Report Configuration

RAN2#128 agreed that a single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals. This decision can be evaluated by considering scenarios where the UE is configured to measure both SSB and CSI-RS, as these reference signals serve distinct purposes. For example, SSB is primarily used for evaluating cell quality over wider beams, making it suitable for initial cell search and broad coverage assessments. CSI-RS, on the other hand, provides higher resolution measurements due to its use of narrower beams, making it more effective for precise channel state information.

Given these differences, we propose that the gNB can configure the UE to report both SSB and CSI-RS in a single measurement report, ensuring a more comprehensive network evaluation while significantly reducing overhead. Instead of requiring separate reports for different reference signals, consolidating them into a single report minimizes signaling overhead, improving overall network efficiency. This approach also optimizes resource utilization. Additionally, a unified reporting structure simplifies processing for both the UE and gNB, reducing complexity and enhancing system performance.

Proposal 2: UE is configured to report both CSI-RS and SSB beams within a single measurement report.

According to the agreement in the previous RAN2 meeting, for event-triggered L1 LTM measurement reporting, the network controls whether beams that do not satisfy the event criteria can still be reported, following the N-beam configuration in MR MAC CE. Different scenarios and options should be considered for these N beams to ensure flexibility and efficiency in reporting.

In the existing LTM reporting framework, the following constraints apply:

- A maximum of four reference signals can be reported per cell.

- Up to four cells may be reported in a single L1 measurement report instance.

These specifications are outlined in TS 38.212 v18.2.0:

The number of reported cells and beams per cell is determined by configuration, meaning the UE always reports a fixed number of beams, four, for each configured cell. However, this approach is inefficient for several reasons.

First, some cells have stronger signals than others, making equal reporting unnecessary. Second, RAN2 has agreed that the NW controls if the beam(s) not satisfying the event can still be reported. This may necessitate reporting more than four beams per cell in certain scenarios to ensure a more comprehensive measurement.

Therefore, we propose allowing the number of beams per cell to vary. The UE should have the capability to report a different number of beams for different cells, improving flexibility and efficiency.

Proposal 3: For event-triggered L1 LTM measurement reporting, the number of beams (M) reported can vary for each cell, where N = M × L gives the total number of reported beams.

Similarly, this approach can also be applied to the number of reported cells (L).

Proposal 4: For event-triggered L1 LTM measurement reporting, the total number of reported cells (L) can vary, maintaining the total reported beams as N = M × L.

## Event ID for Triggered Measurements

In the context of triggered event reporting, it is logical to establish event definitions alongside an event ID, using the L3 measurement framework as a reference, as previously agreed. Therefore, we propose to define an event ID which is more suitable and streamlined approach for identifying triggered events.

Proposal 5: The event ID should be used as the identifier for triggered events.

## Report Content

In RAN2#127bis, the following is agreed:

Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) - MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). - N is configurable by NW.

A decision is required on the preferred method for representing the beam index in LTM MR CE. The two possible approaches are:

- Using SSBRI/CRI for N beams

- Using a combination of LTM configuration ID and SSB/CSI-RS ID

Both methods introduce the same overhead in terms of signaling. However, considering that the SSBRI/CRI-based approach is already used in Release 18 and offers a straightforward solution, we propose adopting this method for consistency and simplicity.

Proposal 6: SSBRI and CRI should be used to represent the beam ID in LTM MR MAC CE.

## Differential RSRP Reporting

For measurement metric representation, a differential format approach can be used to minimize the overall report message size. Reporting one absolute value for the largest RSRP of SSB or CSI-RS should be considered to ensure an accurate reference point for differential reporting. One absolute value for the largest RSRP of the SSB or CSI-RS measured beam and the remaining measured beams using differential L1-RSRP values are reported.

Proposal 7: Configure the UE to report measurements in a differential L1-RSRP format to reduce overhead.

- A single absolute value is reported for the highest RSRP of the SSB and/or CSI-RS beam.

- Use differential L1-RSRP values for the remaining measured beams,

- FFS further quantization details, e.g., step size, range, bit width.

## RSRP Measurement Reporting

The reporting range for SS-RSRP and CSI-RSRP in L3 reporting, along with the mapping of measured quantities, is defined in Table 10.1.6.1-1 of TS 38.133. Since this table is already used for the R18 LTM measurement report by CSI, it can be reused for LTM event-triggered measurement reporting.

Currently, LTM event-triggered measurements require an efficient and standardized way to report SS-RSRP and CSI-RSRP values. Reusing the existing reporting range and mapping definitions from TS 38.133 ensures alignment with current measurement frameworks, reduces implementation complexity, and maintains compatibility across different reporting layers. This approach also minimizes the need for additional specifications or modifications in reporting methodologies.

Proposal 8: The L1 reporting measured quantity values for SS-RSRP and CSI-RSRP should be utilized for LTM event-triggered measurement reporting at higher layers, e.g., L2.

# Conclusion

Proposal 1: UE is configured with both SSB and CSI-RS reference signals for LTM event evaluation purposes.

Proposal 2: UE is configured to report both CSI-RS and SSB beams within a single measurement report.

Proposal 3: For event-triggered L1 LTM measurement reporting, the number of beams (M) reported can vary for each cell, where N = M × L gives the total number of reported beams.

Proposal 4: For event-triggered L1 LTM measurement reporting, the total number of reported cells (L) can vary, maintaining the total reported beams as N = M × L.

Proposal 5: The event ID should be used as the identifier for triggered events.

Proposal 6: SSBRI and CRI should be used to represent the beam ID in LTM MR MAC CE.

Proposal 7: Configure the UE to report measurements in a differential L1-RSRP format to reduce overhead.

- A single absolute value is reported for the highest RSRP of the SSB and/or CSI-RS beam.

- Use differential L1-RSRP values for the remaining measured beams,

- FFS further quantization details, e.g., step size, range, bit width.

Proposal 8: The L1 reporting measured quantity values for SS-RSRP and CSI-RSRP should be utilized for LTM event-triggered measurement reporting at higher layers, e.g., L2.

# References

[1] Chair notes, 3GPP RAN2#128, Orlando, USA,18th – 22nd, 2024

[2] Chair notes, 3GPP RAN2#127bis, Hefei, China, Oct 14th – 18th, 2024.

[3] Chair notes, 3GPP RAN2#127, Maastricht, NL, August 19th – 23rd, 2024.

[4] Chair notes, 3GPP RAN2#126, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.