3GPP TSG-RAN WG2 Meeting #129	R2-2500888

Athens, Greech, 17 – 21 February 2025

Agenda item:	8.1.3

Source:	Nokia

Title:	Data Collection for Training of NW-side ML models

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This contribution elaborates on the issues related to logged measurement configuration and reporting, multiple configurations, availability indicators and ongoing event-based triggering of logging for beam management data collection for network-side models. The contribution also provides a generic overview for the data collection procedure and pre-requisite for enabling logging.  Below we listed the agreements from RAN2#128 and RAN2#127bis:

RAN2#128 Agreements on NW side data collection

- Duration is not supported

- Measurements on aperiodic CSI resources are not reported for NW sided data collection.

- Data collection is controlled by the network.   The UE will not autonomously stop when low power state is detected.

- The UE reports to the network when the power state is low.  We will not specify how the UE determines low power state.   The network should de-configure the data collection (this can be captured in stage 2).

- The UE reports to the network when buffer is or may become full.  FFS when it reports (before and/or after).

- The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory).  FFS how this is signalled and if the reporting can be part of availability indication.

RAN2#127bis Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for NW sided/UE sided BM model training, at least L1 RSRPs and/or beam IDs needs to be collected by UE. FFS if other data needs to be collected based on RAN1 progress.

# 2	Discussion

## 2.1	Framework and Immediate MDT procedure scope

For network-side data collection related to beam management, RAN2#126 agreed to enhance Immediate Minimization of Drive Test (MDT). The aim is to establish common framework for collecting measurements for both gNB-centric and OAM-centric approaches. The basic procedures of an OAM centric network-side data collection can be realized with the following steps illustrated in Figure 1:

Step 1, Step 7: RAN2 can assume that data collection configuration activation will reuse the existing procedure of activating Immediate MDT through MDT Activation and MDT Record procedures.

Step 2: UE capability information (e.g. logging support) exchange ensures relevant UEs selection.

Step 3: CSI framework configuration triggers data collection between the gNB and the UE to measure, log and report L1 measurements via RRC message.

Step 4-5: The UE measures L1 measurements and logs in AS layer if logging condition (periodic or event-based trigger) is configured, otherwise UE measures and reports without logging.

Step 6: UE reports the logged data.

Figure 1: Basic procedure of an OAM centric data collection.

Proposal 1: RAN2 to adopt the above steps of Figure 1 as a baseline procedure for OAM centric data collection.

These steps can set the basis for the common procedure to enable data collection configuration and reporting for beam management for both: gNB-centric and OAM-centric approach.

Observation 1: The scope of the MDT Activation and Reporting extensions for Immediate MDT procedures to enable data collection for beam management is in RAN3 and SA5 realm.

In the following, we discuss configuration and reporting of logged measurements.

## 2.2	Logged Measurement Configuration

### 2.2.1	Logged Measurement Configuration Activation

Data collection for the training of network-side models for beam management will rely on UEs which receive CSI measurement (resource and reporting) configurations for the set(s) of RS resources (e.g., NZP-CSI-RS resources or SSB-RS resources). These measurements would serve as inputs and represent outputs, or predictions, of a NW-side model. This assumption serves as the basis for data collection, also in line with RAN1 agreements from RAN1#116bis:

Observation 2: For network-side model for beam management Case 1 and beam management Case 2, RAN1 agreed to rely on existing CSI framework.

The UE performs required measurements on the configured RS resources. The configuration of RS resources (NZP-CSI-RS-Resource(s) and NZP-CSI-RS-ResourceSet(s)) specified in CSI-MeasConfig are required for AI/ML beam management, as these RS resources specify necessary reference signals to be measured.

Observation 3: For data collection for network-side models, legacy CSI-MeasConfig can be reused to configure measurement resources (e.g. resource type, resource set) as a pre-requisite for logging.

The CSI-ReportConfig Information Element (IE) within CSI-MeasConfig is used to configure either a periodic or semi-persistent report sent via PUCCH in the cell where the CSI-ReportConfig was configured, or for establishing a semi-persistent or aperiodic report communicated through PUSCH when triggered by DCI received in that specific cell. The reporting conditions are determined based on reportConfigType of CSI-ReportConfig. Even if some the fields may not be relevant for data collection for network-side models, the reuse of CSI-ReportConfig would be beneficial to avoid the definition of the same fields in other IEs and to harmonize the data collection configurations with the eventual inference configurations which would configure measurements on the same CSI-RS resources.

On the other hand, RAN2 agreed to report the logged data in RRC messages. So, the key principle is to reuse this CSI-ReportConfig to convey the CSI-RS resource set for measurement without enabling the legacy reporting. The UE can log the measurements based on the configured CSI-RS resource sets associated with CSI-ReportConfig and network can retrieve the logged data via on-demand reporting.

Proposal 2: Legacy CSI-ReportConfig is reused to provide configuration for measurement and logging of CSI-RS  for network-side models, i.e., added or modified with csi-ReportConfigToAddModList, and released with csi-ReportConfigToRemoveList.

If the CSI-ReportConfig is reused, then an indication is needed for the data collection related report configurations to enable the UE to differentiate the logging configuration from legacy report configurations for L1 measurement reports (denoted as “Logging flag” in Figure 2 below).

Figure 2: Configurations for logging of measurements for data collection for NW side models (red indicates new additions)

Proposal 3: RAN2 to define a new flag in the CSI-ReportConfig to indicate that instead of legacy reporting the logging of the measurements is configured.

### 2.2.2	Triggering Event Based Logging of Measurements

During RAN2#128, it was proposed by some companies to introduce a trigger for the logging of measurements for NW-side data collection similar to Event A1 (serving cell becomes better than absolute threshold) and Event A2 (serving cell becomes worse than absolute threshold). The L3 serving cell measurements would be used to judge the signal level (to avoid collecting low SNR measurements) and to approximately locate the UE in a particular area of the cell where the dataset is lacking in data. An email discussion [POST128][019][AI PHY] was triggered to discuss the topic.

In this section, we will explore the reuse of the existing A1 and A2 events and related configurations such as the amount of averaging or filtering (nrofCSI-RS-ResourcesToAverage) and limiting of measurements to specific SSBs (ssb-ToMeausure). The definition of these parameters can be found in TS 38.331:

nrofCSI-RS-ResourcesToAverage Indicates the maximum number of measurement results per beam based on CSI-RS resources to be averaged. The same value applies for each detected cell associated with this MeasObjectNR.

ssb-ToMeasure

The set of SS blocks to be measured within the SMTC measurement duration. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not to be measured while value 1 indicates that the corresponding SS/PBCH block is to be measured (see TS 38.215 [9]). When the field is not configured the UE measures on all SS blocks. Regardless of the value of this field, SS/PBCH blocks outside of the applicable smtc are not to be measured. See TS 38.215 [9] clause 5.1.1.

The email discussion showed that there is significant agreement that some kind of L3 event should be defined to trigger the logging of L1 CSI-RS measurements. Without running simulations, it would be difficult to confirm that the bias introduced, and noted by many companies, by configuring a UE to collect data in specific areas would adversely affect model performance.

Observation 4: Configuring a UE with too much specificity in its data logging configuration could introduce bias in the resulting dataset.

However, it could make sense to find a middle ground wherein the trigger can still facilitate localized data collection. The measurements which are collected are on fine beams used for data transmission, and these beams are usually located within the footprint of a single coarse SSB beam. The RRM measurement reporting framework already supports limiting measurements to specific SSBs.

Note that, RRM measurement reports are configured as a combination of a ReportConfigNR and a MeasObjectNR identified by a MeasId and stored in a list of MeasIdToAddMods containing the ReportConfigId, MeasObjectId, and MeasId.

Given that it is possible to control the amount of L3 measurement averaging and to limit measurements to specific SSBs, L3 measurement event triggers are sufficient to roughly determine whether a UE is a good candidate to configure for data collection to help fill in missing data from a dataset. Note as well that the specification supports this type of configuration today, meaning that as long as the configuration doesn’t require a configurable link to a specific L1 measurement configuration, there would be no specification impact to use Event A1 and Event A2 as triggers.

Observation 5: There is no specification impact to configure a MeasId with a MeasObjectNR configuring CSI-RS averaging to a desired value and a specific set of SSBs to measure and a ReportConfigNR with Event A1 or Event A2.

If we combine the RRM MeasurementReport triggered by Event A1 or Event A2, configured with limited CSI-RS averaging, and limited to a configurable set of SSB indices with a command or configuration to activate or deactivate measurement logging, then L3 event-triggered measurement logging could be implemented without any specification impact over that which is required to implement CSI-RS measurement logging, and its activation and deactivation.

For periodic CSI-RS measurement logging, similar principles could be used to configure L3 event-triggered measurement logging. Upon triggering of Event A1 or Event A2, UE can send L3 measurement reports corresponding to the event and network can configure CSI-ReportConfig with periodic reporting for logging (Step 5). Therefore, the L1 measurements can be logged without any specification impact.

The call flow of Figure 2.3-1 illustrates the procedure.

Figure 2.3-1: L3 Event-Triggered L1 CSI-RS Logging Activation with Event A1.

Proposal 4: To support L3 event-triggered measurement logging for NW-side AI/ML beam management data collection, MeasurementReports resulting from the legacy ReportConfigNR, configured with events A1 and A2, can be used by the gNB to trigger the activation or deactivation of CSI-RS measurement logging.

Proposal 5: The NW can activate or deactivate logging by initiating a subsequent (to L3-trigger)  procedure or command, where the exact mechanism to support the activation and deactivation of logging is FFS.

### 2.2.3	Multiple Configurations

At RAN2#127 the following agreements were made on multiple measurement configuration:

RAN2 #127 Agreements

1. As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

According to the above agreement, UE will receive logging and measurement configurations from the network. If UE is configured with different RSes in multiple configurations, the CSI computation complexity of the UE can increase. Since this will impact UE behaviour at the physical layer, it would require input from RAN1. In our understanding, UE shall report what it is being configured by the network, and it will not require any additional spec impact. Moreover, we observe that CSI framework enables multiple instances of the configurations (of measurement resources and reporting types), with certain allowable maximum number of configurations (cf. TS 38.331). Since data logging will be based on RRC configuration, we should ensure that the total number of CSI-RS resources and applied configurations do not exceed the maximum allowable values, even if shared with other LCM purposes (such as inference).

Observation 6: Multiple instances of configurations are supported by adequate RRC configurations.

Proposal 6: RAN2 assumes that configurations for NW-side training data collection share total allowable number of CSI-RS resources and reporting configurations including for other purposes (e.g. legacy CSI-RS, Rel-19 CSI-RS for inference) and do not exceed the existing allowable maximum values.

### 2.2.4	Logged Measurements Configuration Release

The CSI-MeasConfig used to configure reference signals belonging to the serving cell uses regular RRC configuration approach with “ToAddModList” and “ToReleaseList” to configure or de-configure resources. It is a straightforward implication that a removal operation for the CSI-RS measurements resources means that the UE cannot continue with data collection, because the resources are released. In addition, the solutions for AI/ML-enabled beam management for Rel-19 are limited to UE in RRC CONNECTED state by Work Item objectives, thus, regular operations for RRC configuration release will also impose the resources configuration release when the UE leaves RRC CONNECTED mode.

Proposal 7: UE will follow the legacy procedures to release the CSI-MeasConfig configurations. FFS: UE behavior of handling the logged measurements.

## 2.3	Availability indicator

Triggering for availability indicator:

During RAN2#127bis discussions, we identified that the availability indicator can be used:

a) to indicate data is available,

b) to indicate logging discontinuity due to UE’s internal conditions (e.g., buffer full, low power state),

c) to retrieve remaining data if logged data is large to fit in one response message.

In general, data availability indicator is used to assist the network in triggering the transmission of logged data from UE. This has been essential for Logged MDT, as the UE performs logged measurements data collection in IDLE/INACTIVE modes and could reselect to a gNB unaware of the previous logged MDT configuration. During Rel-19 data collection for network-side ML models, data will be logged and reported in RRC CONNECTED mode. Therefore, if the UE is configured with periodic logging, then, network can roughly estimate and expect when it is optimal to retrieve the logged data from the UE (for instance, considering the minimum UE’s buffer size (e.g., 64Kbytes), logging interval (e.g., CSI-ResourcePeriodicityAndOffset), and allowable maximum size per RRC message (e.g. 9Kbytes)). The availability indicator would create an unnecessary overhead in this case. By the reuse of the UEInformationRequest/ UEInformationResponse procedure, the network can request, and UE can respond without the availability indication.

Observation 7: For periodic logging, UEInformationRequest/UEInformationResponse procedure can be used to retrieve logged data without availability indication.

In case of UE configured with event-based logging, the network may not know the frequency that events were triggered within UE and that the UE will have data available to report. In such situations, it might be more useful to indicate to network that data is available for retrieval. Another alternative is that if network is aware of the event, it can estimate how much data is available during the active period. However, this is to be clarified in the ongoing email discussion whether to continue periodical logging after event is being triggered.

Observation 8: In event-based logging, availability indicator can be used to indicate the availability of the data.

During RAN2#128, the following agreements were made to enable the UE to report the following status:

- The UE reports when the power state is low.

- The UE reports to the network when buffer is or may become full. (FFS when it reports)

- The UE can report the reason for triggering of indication for the status (low power state, low memory)

Each of these agreements is relevant to the configuration of measurement logging and buffer retrieval. The indication of UE state that necessitates the reduction or disabling of measurement logging should be in the same message as is used to indicate that the buffer is or may become full so that the network can retrieve the logged measurements before deactivating measurement logging.

Proposal 8: The indication of UE internal state that necessitates the reduction of or disabling of measurement logging should be signalled in the same message as is used to indicate that the buffer is or may become full.

To retrieve a large amount of logged data in the UE buffer, the logged data needs to be segmented and one RRC message may not be sufficient. In accordance with RAN2#127 agreement, it is stated that “The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)”. Therefore, it is up to UE to determine the segmentation of logged data such that the entries do not exceed the maximum PDCP SDU. The remaining data that is available, UE can indicate using an availability indicator.

On the other hand, as for the logged MDT, the network can receive a part of the total log data with data availability indication. For every request from network, (e.g., via UEInformationRequest), UE can, in response, indicate the availability of more data by setting logMeasAvailable to true within UEInformationResponse message. In case of multiple RRC transmissions, FIFO order is followed such that UE can provide oldest available log entries in the earliest message [cf. TS 37.420, clause 5.1.1.3.2]. For network side models in beam management, the same logic can be applicable. If UE logs a large data in RRC CONNECTED mode and the total log does not fit in one RRC message, then UE can indicate the availability of more data. Network can receive part of this data with every request.

To indicate the remaining data availability for logged MDT, the availability indicator can be provided in logMeasAvailable within UEInformationResponse in case of UE has not transferred the total log in one RRC message. This indicator is just a one bit of information. We can leverage the similar principle in network side models, where, with every request, network can receive part of the log data with such availability indicator and the specification impact is straightforward.

Proposal 9: The availability indicator is a flag for UE to indicate that remaining data is available in buffer.

Signalling for availability indicator:

Based on the above analysis on the usage of availability indicator, we further propose:

Proposal 10: To support retrieval of logged data during event-based logging, use UEAssistantInformation (UAI) for availability indicator.

Proposal 11: To support retrieval of logged data due to logging discontinuity issues (e.g., buffer full, low power state), use UEAssistantInformation (UAI) for availability indicator.

Proposal 12: To support retrieval of remaining logged data, include data availability indicator in UEInformationResponse message.

# 3	Conclusion

This document has made the following observations:

Observation 1: The scope of the MDT Activation and Reporting extensions for Immediate MDT procedures to enable data collection for beam management is in RAN3 and SA5 realm.

Observation 2: For network-side model for beam management Case 1 and beam management Case 2, RAN1 agreed to rely on existing CSI framework.

Observation 3: For data collection for network-side models, legacy CSI-MeasConfig is reused to configure measurement resources (e.g. resource type, resource set) as a pre-requisite for logging.

Observation 4: Configuring a UE with too much specificity in its data logging configuration could introduce bias in the resulting dataset.

Observation 5: There is no specification impact to configure a MeasId with a MeasObjectNR configuring CSI-RS averaging to a desired value and a specific set of SSBs to measure and a ReportConfigNR with Event A1 or Event A2.

Observation 6: Multiple instances of configurations are supported by adequate RRC configurations.

Observation 7: For periodic logging, UEInformationRequest/UEInformationResponse procedure can be used to retrieve logged data without availability indication.

Observation 8: In event-based logging, availability indicator can be used to indicate the availability of the data.

And proposed the following:

Proposal 1: RAN2 to adapt the above steps of Figure 1 as a baseline procedure for OAM centric data collection.

Proposal 2: Legacy CSI-ReportConfig is reused to provide configuration for measurement and logging of CSI-RS  for network-side models, i.e., added or modified with csi-ReportConfigToAddModList, and release with csi-ReportConfigToReleaseList.

Proposal 3: RAN2 to define a new flag in the CSI-ReportConfig to indicate that instead of legacy reporting the logging of the measurements is configured.

Proposal 4: To support L3 event-triggered measurement logging for NW-side AI/ML beam management data collection, MeasurementReports resulting from the legacy ReportConfigNR, configured with events A1 and A2, can be used by the gNB to trigger the activation or deactivation of CSI-RS measurement logging.

Proposal 5: The NW can activate or deactivate logging by initiating a subsequent (to L3-trigger)  procedure or command, where the exact mechanism to support the activation and deactivation of logging is FFS.

Proposal 6: RAN2 assumes that configurations for NW-side training data collection share total allowable number of CSI-RS resources and reporting configurations including for other purposes (e.g. legacy CSI-RS, Rel-19 CSI-RS for inference) and do not exceed the existing allowable maximum values.

Proposal 7: UE will follow the legacy procedures to release the CSI-MeasConfig configurations. FFS: UE behavior of handling the logged measurements.

Proposal 8: The indication of UE internal state that necessitates the reduction of or disabling of measurement logging should be signalled in the same message as is used to indicate that the buffer is or may become full.

Proposal 9: The availability indicator is a flag for UE to indicate that remaining data is available in buffer.

Proposal 10: To support retrieval of logged data during event-based logging, use UEAssistantInformation (UAI) for availability indicator.

Proposal 11: To support retrieval of logged data due to logging discontinuity issues (e.g., buffer full, low power state), use UEAssistantInformation (UAI) for availability indicator.

Proposal 12: To support retrieval of remaining logged data, include data availability indicator in UEInformationResponse message.