3GPP TSG-RAN WG2 Meeting #129	R2-2500889

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.1.4

Source:	Nokia

Title:	UE side data collection

WID/SID:	FS_NR_AIML_air_Ph2- Release 19

Document for:	Discussion and Decision

# 1	Introduction

SID for AI/ML for NR air interface in [1] involves objective to examinate UE-sided model training data as follows:

Study CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

- For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

- Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

In accordance with [2], RAN#106 down selected studied options in TR 38.843, to focus study for UE side data collection to Options 2 and 3, as they fulfil agreed criteria for UE-side model training, outlined in [3].

This contribution includes observations and proposals on the UE-side data collection, for AI/ML-based beam management and AI/ML-based positioning enhancements with alignment with the choice of gathering and transferring the data from the UE under Option 2 and Option 3.

# 2	Discussion

## 2.1	Scope for UE side data collection

UE-side Data Collection focuses on gathering raw information from the device to train and fine-tune ML models. In particular, for the agreed RAN2 Rel-19 scope, the focus of UE-side Data Collection considers offline training. Furthermore, according to narrowed down options by RAN#106 [2], UE-side Data Collection considers offline training at different endpoints for data being gathered from the UE. Thus, the training data will be produced by the UE, with destinations being either gNB or OAM for Beam Management or LMF for Positioning.

Proposal 1: RAN2 to study for UE-side data collection measurement configuration should focus on enhancements to RRC for beam prediction and LPP for positioning.

## 2.2	Data types and visibility

### 2.2.1	Standardized data with full visibility

RAN1 has addressed the specifics of data types to be collected for training UE models for AI/ML-based beam management and AI/ML-based positioning enhancements, which include datasets as shown in Table 2.2.1-1.

Table 2.2.1-1: Data types for UE-side data collection

| Use case | UE raw measurements | Labels (Data accuracy assessment) |
| --- | --- | --- |
| Beam Management data [5] | L1-RSRP Beam Ids | n/a |
| Positioning data (Case 1) [6] | Part A: Channel measurement Quality indicator of channel measurement Time stamp of channel measurement | Part B: Ground truth label (or its approximation) Quality indicator of label Time stamp of label |

Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification.

The key content for data collection considers the UE measurements that are in standardized format and can be decoded by data collection terminating entity and MNO (e.g. RSRP measurement results and its ranges defined in TS 38.331 and TS 38.133).

In addition, for Positioning use case, the content for data collection considers labels, which includes ground truth of the raw UE measurements. The labels, including ground truth, quality indicator or time stamp of label could take a form of known position estimate. The existing positioning methods (e.g. GNSS, DL-TDoA) enable obtaining such positioning estimates in various ways. For instance, GNSS positioning supports MeasurementReferenceTime, Uncertainty and Confidence variants, or DL-TDoA positioning supports MeasurementReferenceTime, NR-TimeStamp, NR-TimingQuality (contents of NR-DL-TDOA-SignalMeasurementInformation in TS37.355). Aligning a legacy position estimate with measurements on positioning reference signals, could be one straightforward way to assign labels to a set of measurements, resulting in achieving standardized labels.

Observation 1: Labels for training UE-side AI/ML positioning models could be obtained through legacy positioning estimates available in the existing positioning methods such as GNSS or DL-TDoA.

We note that by the reuse of existing information for both UE raw measurements and positioning labels, the data content remains in clearly defined format within 3GPP specifications and ability to interpret and process the data with full MNO control is ensured.

Therefore, standardized data types for AI/ML-based beam management, this is records on L1-RSRP and beams indices should reuse known data formats on UE information on beams reference signals and ids (e.g. RRC Information Elements). Likewise, standardized data types for AI/ML-based positioning enhancements, including UE measurements and positioning labels should reuse legacy UE data formats on positioning reference signals and legacy position estimates (i.e., LPP Information Elements for Positioning). However, it is necessary to examine how to correlate traditional positioning estimates with accuracy labels and their relationship to measurements from positioning reference signals, in order to verify if existing positioning methods address all required assumptions regarding data types.

Proposal 2: Standardized data types for UE-side data collection, which include measurements on reference signals should reuse existing and known formats from RRC and LPP protocols.

Proposal 3: For positioning use case, RAN2 should study how to map legacy position estimates to accuracy labels and their association with measurements on positioning reference signals in order to identify cases that require new information definitions, if any.

### 2.2.2 	Data with no full visibility

Known and standardized data structures ensures the controllability and privacy requirements. For that reason, only approaches supporting full visibility on data content (Option 2 and Option 3) have been approved by RAN [2]. However, in the course of UE-side Data Collection studies there has been no clear consensus reached on partial visibility of data for these two selected options. Option 2 for which considered protocol for transferring data is CN protocol (NAS or LPP) and first terminating entity is CN is suitable for positioning use case. Option 3, for which considered protocol for transferring data is RRC and terminating entity is inside OAM, is suitable for beam management. In any of the options, we assume MNO interest in better insights into data than the ones represented by standardized data types may relate to some proprietary data such as UE or gNB RF hardware or firmware versions. Such additional data, with some known properties, even though with detailed formats of decoding varying or defined outside RAN specifications could be considered. However, we assume only data content that can be made interpretable to MNO or the MNO can gain access to, can be considered as authorized for UE-side data collection.

We note that Trace functionality enables already vendor-specific extensions (see vendorExtensions TS32.423) that facilitates vendor-specific information exchange between the gNB and OAM. This can realize partial visibility over network interfaces without impacting the UE and Uu interface. In case a similar need is there for Option 2, it would require further studies on a feasibility, including impacts to involved CN entity (i.e. LMF).

Observation 2: Trace Functionality supports collection of data from network where format is not fully standardized (partial visibility) through vendor-specific Trace data.

Nonetheless, in either scenario, any other than standardized over RRC and LPP data from the UE cannot be retrieved, as neither RRC nor LPP facilitate device-specific data transfer. Consequently, while MNOs seek a clear connection between device-specific data or attributes and network configurations (e.g. NW additional conditions), they should gain awareness regarding the characteristics of the devices without risking security and privacy, i.e. only by implementation.

Proposal 4: RAN2 to study the need for a vendor-specific container signaling over RRC and LPP for beam management and positioning use cases and its impact on privacy and data size.

## 2.3	Data Collection Activation

To enable the key content for data collection, i.e. the standardized UE measurements, the gNB has to configure signals for measurements, e.g., CSI-RS and PRS for the training data collection activation. Although, RAN2#128 already agreed that for UE-side data collection, the UE can send a request for data collection, the network needs to enquiry UE capabilities before activation and generate appropriate configuration accordingly. That is, regardless of the pre-configured UE request, the same pre-requisite for activation of UE-side data collection can be assumed as for NW-side data collection. Resources and signals configuration for measurement needs to be reused for both cases: UE-side and NW-side data collection.

Proposal 5: For Beam Management, the same RRC configuration framework is used to configure measurement resources for UE-side data collection as for NW-side data collection.

Unlike BM management, AI/ML-based positioning use case, LPP message can be used to collect data from UE and that can be viewed as both activating and collecting the data at the same time. However, until further progress is made on positioning labels verification, it remains unclear whether existing positioning methods address all required assumptions regarding data types and whether additional configuration (on top of the LPP configuration of required PRS) would be required for UE-side data collection activation for case 1.

Observation 3: For Positioning use case, due to pending studies on data types, until the study phase of the WI is concluded, it is only possible to assume that LPP configuration for positioning reference signals (PRS) is pre-requisite for UE-side data collection.

## 2.4	Data transmission protocols

Considering the way of data transfer, TSG RAN has agreed to the following:

From RAN perspective, transfer of data over UP for Solution 2 in SA2 should be studied and provide feedback on fulfillment of visibility and controllability requirements.  Transfer of data over UP for Solution 3 can be studied by SA5

RAN has already concluded that transfer of data over CP for Solution 2 and 3 can meet the visibility and controllability RAN requirements, even though some scalability concerns have been raised for high amount of data transfer.  RAN is not expecting SA2 and SA5 to study the transfer of data over CP for now.

For the two identified RAN uses cases for AI/ML data collection: beam management and positioning, RRC Signalling and LPP Signalling, respectively, facilitate exchange of information between the UE and the gNB/LMF through Control Plane.  This is:

- UE data required for Beam Management can be transferred through CP (RRC Signalling)

- UE data required for Positioing can be transferred through CP (LPP Signalling)

UP-based solutions came into play for transferring AI/ML data during Rel-19 studies [4], to overcome a burden to the physical layer or control planes of the gNB and the LMF and to avoid potential AI/ML impacts to traffic essential for normal network operation. UP has been recognized as a candidate for the transmission of larger volumes of data and measurements that can be sent without latency requirements, i.e., those used for data collection for offline training.

In the current stage, without gaining more comprehensive understanding on data types, including additional attributes required to standardize UE measurements, their volumes and potential implications of non-fully visible data sizes, involved in UE-side data collection, it is too premature to determine alternative transfer method to the available CP-based data transfer.

In particular, we observe that if all necessary data type for UE-side data collection for Positioning is concluded and recognized as implementable in LPP signalling, it can be potentially feasible to be used in UP-based data transfer, because SA2 introduced a 3GPP solution for user-plane positioning in Rel-18 through LCS-UPP layer (on top of LPP, based on TS38.305).  Otherwise, we do not consider studies should focus on a completely new UP method designs.

Proposal 6: UE-side data collection is always configured to the UE through Control Plane (RRC or LPP) regardless of a use case and which transfer method is decided.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: RAN2 to study for UE-side data collection measurement configuration should focus on enhancements to RRC for beam prediction and LPP for positioning.

Proposal 2: Standardized data types for UE-side data collection, which include measurements on reference signals should reuse existing and known formats from RRC and LPP protocols.

Proposal 3: For positioning use case, RAN2 should study how to map legacy position estimates to accuracy labels and their association with measurements on positioning reference signals in order to identify cases that require new information definitions, if any.

Proposal 4: RAN2 to study the need for a vendor-specific container signalling over RRC and LPP for beam management and positioning use cases and impact thereof on privacy and data size.

Proposal 5: For Beam Management, the same RRC configuration framework is used to configure measurement resources for UE-side data collection as for NW-side data collection.

Proposal 6: UE-side data collection is always configured to the UE through Control Plane (RRC or LPP) regardless of a use case and which transfer method is decided.

# 4	References

[1] RP-243245, “New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm, December 2024

[2] RP-243316, LS on AI/ML UE sided data collection, RAN, December 2024

[3] RP-242389, “LS on AIML data collection”, RAN, September 2024

[4] TR 38.843 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, Version 18.0.0, 3GPP.

[5] R1-2310681, Reply LS on Data Collection Requirements and Assumptions, RAN1

[6] RAN1#116bis Report