3GPP TSG-RAN WG2 Meeting #129					           R2-2500891

Athens, Greece, 17 – 21 February, 2025

Agenda Item:		8.3.2

Source:			Huawei, HiSilicon

Title:			Model generalization evaluation for RRM measurement prediction

Document for:		Discussion and Decision

# 1   Introduction

In the previous meetings RAN2 analysed RRM measurement prediction accuracy when using AIML models in different scenarios. The following scenarios were prioritized for evaluation:

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

In this Tdoc we analyse a generalization performance of AIML models for RRM measurement prediction across different UE speeds for scenario 2 and 4, as per the agreements made by RAN2.

# 2   Simulation results

## 2.1 General

Our previous RSRP prediction accuracy results were provided in [1]. We evaluated AIML performance using LSTM model and we continue to use this model for the generalization performance valuation, as presented in this Tdoc. All the simulation assumptions are aligned with RAN2 agreements and outlined in detail in the excel sheet attached to this Tdoc.

In RAN2#128 meeting, RAN2 made agreements on generalization based on UE speed for both FR1 and FR2. Two generalization cases were agreed, i.e.

- GC#1 where the model is trained using one UE speed and tested also for one, but different, UE speed

- GC#2 where the model is trained using different UE speeds and tested for one specific UE speed

RAN2 also captured the “generalization tables” summarizing the test cases to be performed, but during the discussion after the meeting, an error in the agreed tables was identified and we in our evaluations we use the refined table, as presented below.

Table 1. Evaluation combinations for FR1 and FR2 generalization study on UE speed

|  | Training @Dataset: S1 | Training @Dataset: S2 | Training @Dataset: S3 | Inference @Dataset:S1 | Inference @Dataset:S2 | Inference @Dataset:S3 |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | Yes |  |  | Yes |  |  |
| GC#1 |  | Yes |  | Yes |  |  |
| GC#1 |  |  | Yes | Yes |  |  |
| GC#2 | Yes | Yes | Yes | Yes |  |  |
| Baseline |  | Yes |  |  | Yes |  |
| GC#1 | Yes |  |  |  | Yes |  |
| GC#1 |  |  | Yes |  | Yes |  |
| GC#2 | Yes | Yes | Yes |  | Yes |  |
| Baseline |  |  | Yes |  |  | Yes |
| GC#1 | Yes |  |  |  |  | Yes |
| GC#1 |  | Yes |  |  |  | Yes |
| GC#2 | Yes | Yes | Yes |  |  | Yes |

For FR1, the UE speeds for evaluation are 30 km/h, 60km/h and 90km/h while for FR2, it is 60 km/h, 90km/h and 120km/h.

## 2.2 Generalization evaluation for FR1

The performance of model generalization for FR1 was evaluated for Scenario 2, i.e. intra-frequency Case B with the following settings:

- MRRT=50%

- OW=600ms

- PW=200ms

- RRM sub use case 2, non-sliding L1 filtering

- UE speeds: 30, 60 and 90km/h

The remaining simulation assumptions were aligned with those agreed by RAN2 earlier and details can be found in the excel sheet attached to this Tdoc. The results are presented in tables below.

Table 2. Generalization evaluation results for various UE speed combinations for FR1

|  | Configuration#A |  |  | Configuration#B Results are an average difference for L3 RSRP [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Training @30km/h | Training @60km/h | Training @90km/h | Inference @30km/h | Inference @60km/h | Inference @90km/h |
| Baseline | Yes |  |  | 0.96 |  |  |
| GC#1 |  | Yes |  | 1.15 |  |  |
| GC#1 |  |  | Yes | 1.29 |  |  |
| GC#2 | Yes | Yes | Yes | 1.13 |  |  |
| Baseline |  | Yes |  |  | 1.59 |  |
| GC#1 | Yes |  |  |  | 1.62 |  |
| GC#1 |  |  | Yes |  | 1.75 |  |
| GC#2 | Yes | Yes | Yes |  | 1.65 |  |
| Baseline |  |  | Yes |  |  | 1.93 |
| GC#1 | Yes |  |  |  |  | 2.06 |
| GC#1 |  | Yes |  |  |  | 2.03 |
| GC#2 | Yes | Yes | Yes |  |  | 1.95 |

As we can see the RSRP differences for the studied generalization cases are always worse than the baseline case where the model is trained and used for inference for the same UE speed. However, the differences are not significant and range from 0.02 to 0.33 dB, depending on the tested UE speed and generalization case. We can also see that GC#2 performance is very close to baseline performance with the difference of RSRP ranging from 0.02 to 0.17 dB, depending on the UE speed used for inference, the difference being higher for lower UE speeds. This seems to be an intuitive result as in GC#2 the training dataset comprises at least partially the data for the inferred UE speed. On the other hand, for GC#1 we can observe that the closer the training UE speed is to the inferred UE speed, the better the prediction accuracy, which is mainly visible for 90 km/h inference speed. This also seems to be a rather intuitive result.

Based on the presented results, we can make the following observations:

Observation 1: For FR1, The RSRP prediction accuracy is highest if the training and inference are performed using the same UE speed (i.e. baseline case).

Observation 2: For FR1, AIML model generalizes relatively well for different UE speeds and the drop of prediction accuracy ranges from a few hundredths to a few decimals of a dB.

Observation 3: For FR1, GC#2 performs better than GC#1 for all inferred UE speeds.

Observation 4: For FR1, the closer training UE speed is to the inferred UE speed, the better the prediction accuracy.

## 2.2 Generalization evaluation for FR2

The performance of model generalization for FR2 was evaluated for Scenario 4, i.e. intra-frequency Case A with the following settings:

- OW=600ms

- PW=200ms

- RRM sub use case 2, non-sliding L1 filtering

- UE speeds: 60, 90 and 120km/h

The remaining simulation assumptions were aligned with those agreed by RAN2 earlier and details can be found in the excel sheet attached to this Tdoc. The results are presented in tables below.

Table 2. Generalization evaluation results for various UE speed combinations for FR1

|  | Configuration#A |  |  | Configuration#B Results are an average difference for L3 RSRP [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Training @60km/h | Training @90km/h | Training @120km/h | Inference @60km/h | Inference @90km/h | Inference @120km/h |
| Baseline | Yes |  |  | 1.49 |  |  |
| GC#1 |  | Yes |  | 1.96 |  |  |
| GC#1 |  |  | Yes | 2.04 |  |  |
| GC#2 | Yes | Yes | Yes | 1.73 |  |  |
| Baseline |  | Yes |  |  | 1.83 |  |
| GC#1 | Yes |  |  |  | 1.85 |  |
| GC#1 |  |  | Yes |  | 1.94 |  |
| GC#2 | Yes | Yes | Yes |  | 1.93 |  |
| Baseline |  |  | Yes |  |  | 2.02 |
| GC#1 | Yes |  |  |  |  | 2.15 |
| GC#1 |  | Yes |  |  |  | 2.34 |
| GC#2 | Yes | Yes | Yes |  |  | 2.06 |

The observations for FR2 evaluation are similar to those we made for FR1. Unsurprisingly, the baseline case performed best, but the model generalized relatively well, especially with GC#2, even though the prediction accuracy differences were higher than for FR1 and ranged from 0.04 to 0.55 dB depending on the tested UE speed and generalization case. With GC#2 performance drop compared to the baseline case was between 0.04 and 0.24 dB, depending on the UE speed used for inference, the difference being higher for lower inference UE speeds. For GC#1 in FR2 an observation that the closer the training UE speed is to the inferred UE speed, the better the prediction accuracy, which we proposed to capture for FR1, does not hold for FR2 in all cases.

Based on the presented results, we can make the following observations:

Observation 5: For FR2, the RSRP prediction accuracy is highest if the training and inference are performed using the same UE speed (i.e. baseline case).

Observation 6: For FR2, AIML model generalizes relatively well for different UE speeds and the drop of prediction accuracy ranges from a few hundredths to a few decimals of a dB.

Observation 7: For FR2, GC#2 performs better than GC#1 for all inferred UE speeds.

# 3   Summary

Based on the simulations presented in this paper, the following observations are made:

Generalization for different UE speeds in FR1

Observation 1: For FR1, The RSRP prediction accuracy is highest if the training and inference are performed using the same UE speed (i.e. baseline case).

Observation 2: For FR1, AIML model generalizes relatively well for different UE speeds and the drop of prediction accuracy ranges from a few hundredths to a few decimals of a dB.

Observation 3: For FR1, GC#2 performs better than GC#1 for all inferred UE speeds.

Observation 4: For FR1, the closer training UE speed is to the inferred UE speed, the better the prediction accuracy.

Generalization for different UE speeds in FR2

Observation 5: For FR2, the RSRP prediction accuracy is highest if the training and inference are performed using the same UE speed (i.e. baseline case).

Observation 6: For FR2, AIML model generalizes relatively well for different UE speeds and the drop of prediction accuracy ranges from a few hundredths to a few decimals of a dB.

Observation 7: For FR2, GC#2 performs better than GC#1 for all inferred UE speeds.

# References

- R2-2410539	Simulation results for RRM measurement prediction	Huawei, HiSilicon

# Annex A – simulation assumptions

Below are the company selectable assumptions chosen by us.

| Configurable/selectable simulation assumption |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Spatial consistency | Whether LOSsoft is modeled or not | UE trajectory model | UE trajectory boundary processing | L1/L3 filtering (sliding/non-sliding) | Number of Tx beams | Number of Rx beams |
| Procedure B in TR38.901 | Not modelled | Option 1 in 38.843 section 6.3.1 | Wrap-around model | Both non-filtered and non-sliding L1 filtering cases were simulated | FR1: 4 FR2: 64 | FR1: 1 FR2: 8 |

All the simulation assumptions are listed in the table below with the selectable ones highlighted in yellow.

Table 1: Summarized simulation assumptions

| Parameter | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz, 15/30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading without LOSsoft and inter-frequency correlation; without Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading without LOSsoft; without Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage |
| System BW | 20MHz | 80MHz |
| UE speed | 30,60,90 km/h for study targeting measurement reduction | 60,90,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) |
| UE Antenna radiation pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 44dBm | 40 dBm |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | Procedure B in TR38.901 | Procedure B in TR38.901 |
| UE trajectory model | Option 1 in 38.843 section 6.3.1 | Option 1 in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | wrap round model | wrap round model |