3GPP TSG-RAN WG2 Meeting #129					           R2-2500892

Athens, Greece, 17 – 21 February, 2025

Agenda Item:		8.3.3

Source:			Huawei, HiSilicon

Title:			Simulation results for measurement event prediction

Document for:		Discussion and Decision

# 1   Introduction

In the previous meetings RAN2 studied the prediction accuracy of L3 RSRP measurements using AIML models in different scenarios. The highest priority scenarios are mentioned below.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

As a next step, RAN2 decided to verify how AIML models can be utilized to predict mobility events, with a focus on A3 event first. RAN2 also decided to prioritize so called “indirect method” where the AIML model is used to predict signal level (e.g. RSRP/SINR) while the occurrence of the measurement event is determined based on such predicted signal level, without further involvement of the AIML model. This will allow to verify how the inaccuracies related with the measurement predictions translate into accuracy of mobility event predictions and mobility performance metrics such as handover failure rate and number of handovers.

In this paper, we provide our evaluation results related to Case 2 and Case 4 from the table above, using an indirect measurement event prediction method.

# 2   Measurement event prediction accuracy

## 2.1 Simulation assumptions

Simulation assumptions and evaluation methodology agreed previously for RRM measurement prediction were reused, as captured in [1]. Furthermore, we used the following settings specifically for measurement event prediction simulations purpose (also aligned with RAN2 agreements).

Table 1. Simulation assumptions for FR1 to FR1 intra-frequency temporal domain case B (Case 2)

| Parameter | Baseline value | Additional values |
| --- | --- | --- |
| A3 event offset (db) | 2 | N/A |
| TTT (ms) | 320 | N/A |
| UE speed (km/h) | 30 | 60, 90 |
| OW length (ms) | MRRT 50%: 120 MRRT 66%: 160 | N/A |
| PW length (ms) | 40 | MRRT 50%: 120, 200 MRRT 66%: 80, 200, 320 |
| Max ETD (ms) | 80 | N/A |
| MRRT (%) | 50 | 66 |
| L1 filtering | Sliding window |  |

Table 2. Simulation assumptions for FR2 to FR2 intra-frequency temporal domain case A (Case 4)

| Parameter | Baseline value | Additional values |
| --- | --- | --- |
| A3 event offset (db) | 2 | N/A |
| TTT (ms) | 320 | N/A |
| UE speed (km/h) | 90 | 60, 120 |
| OW length (ms) | 400 | N/A |
| PW length (ms) | 400 | 80 |
| Max ETD (ms) | 80 | N/A |
| L1 filtering | Sliding window |  |

Additional information about the simulation setup, e.g. AIML model details, can be found in the Annex and in the excel sheet attached to this Tdoc.

## 2.2 Simulation results

To evaluate the event prediction accuracy, RAN2 decided to use F1score which is defined as follows:

F1 score = 2*Precision*Recall/(Precision + Recall), where:

- Precision = n3/(n1+n3)

- Recall = n3/(n2+n3)

Counters n1, n2, n3 are defined as follows:

- Counter n1 (false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, maximum ETD]

- Counter n2 (missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, maximum ETD]

- Counter n3 (true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD]

The results for different PW values and different UE speeds are presented in Table 3 for Case 2 and in Table 4 for Case 4.

Table 3. Measurement event prediction accuracy for FR1 to FR1 intra-frequency temporal domain case B (Case 2)

|  | UE speed: 30km/h |  |  |  | UE speed: 60km/h |  |  |  | UE speed: 90km/h |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Precision | Recall | F1 score | RSRP diff | Precision | Recall | F1 score | RSRP diff | Precision | Recall | F1 score | RSRP diff |
| MRRT: 50% OW: 120ms PW: 40ms | 0.96 | 0.953 | 0.956 | 0.1745 | 0.93 | 0.9 | 0.915 | 0.297 | 0.83 | 0.83 | 0.83 | 0.397 |
| MRRT: 50% OW: 120ms PW: 120ms | 0.73 | 0.69 | 0.709 | 0.7 | 0.68 | 0.65 | 0.665 | 1.15 | 0.596 | 0.558 | 0.576 | 1.49 |
| MRRT: 50% OW: 120ms PW: 200ms | 0.19 | 0.16 | 0.174 | 1.259 | 0.14 | 0.13 | 0.135 | 2.004 | 0.088 | 0.07 | 0.078 | 2.535 |
| MRRT: 66% OW: 160ms PW: 80ms | 0.89 | 0.861 | 0.875 | 0.1423 | 0.834 | 0.816 | 0.825 | 0.246 | 0.816 | 0.773 | 0.794 | 0.343 |
| MRRT: 66% OW: 160ms PW: 200ms | 0.27 | 0.25 | 0.26 | 0.6 | 0.22 | 0.19 | 0.204 | 1 | 0.23 | 0.18 | 0.202 | 1.295 |
| MRRT: 66% OW: 160ms PW: 320ms | 0.053 | 0.046 | 0.049 | 1.163 | 0.076 | 0.045 | 0.057 | 1.861 | 0.075 | 0.047 | 0.058 | 2.296 |

Table 4. Measurement event prediction accuracy for FR2 to FR2 intra-frequency temporal domain case A (Case4)

|  | UE speed: 60km/h |  |  |  | UE speed: 90km/h |  |  |  | UE speed: 120km/h |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Precision | Recall | F1 score | RSRP diff | Precision | Recall | F1 score | RSRP diff | Precision | Recall | F1 score | RSRP diff |
| OW: 400ms PW: 80ms | 0.8 | 0.8 | 0.8 | 0.431 | 0.882 | 0.789 | 0.833 | 0.531 | 0.778 | 0.778 | 0.778 | 0.6 |
| OW: 400ms PW: 400ms | 0.059 | 0.05 | 0.054 | 3.313 | 0.0556 | 0.0526 | 0.054 | 4.195 | 0.056 | 0.056 | 0.056 | 4.934 |

As we can see in the presented results, the accuracy of measurement event prediction is relatively good only in case of short prediction window value:

- For Case 2 with MRRT=50% and PW=40 ms, F1 score equals 0.956 for UE speed=30km/h, 0.915 for UE speed=60km/h and 0.83 for UE speed=90km/h

- For Case 2 with MRRT=66%, PW=80ms, F1 score equals 0.875 for UE speed=30km/h, 0.825 for UE speed=60km/h and 0.794 for UE speed=90km/h

- For Case 4 with PW=80ms, F1 score equals 0.8 for UE speed=60km/h, 0.833 for UE speed=90km/h and 0.778 for UE speed=120km/h

Furthermore, it can be observed that the F1 score decreases rather quickly when PW length is extended, e.g.:

- For Case 2 with MRRT=50%, if the PW is increased from 40ms to 120ms, F1 score drops from 0.956 to 0.709

- For Case 2 with MRRT=66%, if the PW is increased from 80ms to 200ms, F1 score drops from 0.875 to 0.26

- For Case 4, if the PW is increased from 80ms to 400ms, F1 score drops from 0.8 to 0.054

Observation 1: Relatively good F1 score values (e.g. above 0.8) can only be achieved for very short PW length values, i.e. up to 80ms in the simulated scenarios.

Observation 2: PW length has a significant impact on measurement event accuracy (F1 score) which decreases quickly when PW length is increased.

Since indirect measurement event prediction method was used in our simulations, the above effects can likely be linked with the increasing RSRP prediction errors. It can be noted that F1 score is very vulnerable to the RSRP prediction error and even relatively small increase in RSRP prediction error leads to rather significant drop of F1 score, e.g.

- For Case 2 with MRRT=50%, the PW increase from 40ms to 120ms corresponds to RSRP prediction error increase from 0.17 to 0.7 (delta=0.53), which leads to F1 score drop from 0.956 to 0.709

- For Case 2 with MRRT=66%, the PW increase from 80ms to 200ms corresponds to RSRP prediction error increase from 0.14 to 0.6 (delta=46), which leads to F1 score drop from 0.875 to 0.26

- For Case 4, the PW increase from 80ms to 400ms corresponds to RSRP prediction error increase from 0.43 to 3.31 (delta=2.88), which leads to F1 score drop from 0.8 to 0.054

Observation 3: RSRP prediction error has a significant impact on measurement event accuracy (F1 score) which decreases quickly when RSRP prediction error increases.

Based on the same results, it can also be seen that RSRP prediction error alone cannot be directly correlated with achievable F1 score for measurement event prediction, i.e. similar value of RSRP prediction error can translate into very different measurement event prediction accuracy depending on other factors, e.g.

- For Case 2 with MRRT: 50%, OW: 120ms, PW: 120ms, UE speed: 30km/h, the RSRP prediction error=0.7 dB while F1 score=0.709

- For Case 2 with MRRT: 66%, OW: 160ms, PW: 200ms, UE speed: 30km/h, the RSRP prediction error=0.6 dB while F1 score=0.26

Observation 4: RSRP prediction error alone cannot be directly correlated with achievable F1 score for measurement event prediction which depends also on other factors, e.g. PW length, MRRT.

Other than that, a set of rather intuitive observations can be made about the effect of UE speed and MRRT on the prediction accuracy:

Observation 5: F1 score decreases with an increasing UE speed.

Observation 6: F1 score decreases with an increasing MRRT value.

# 3	Mobility performance

## 3.1 Simulation assumptions

As per RAN2 agreements, the methodology of handover failure determination in our simulations followed the methodology from TR 36.839 [2], in particular section 5.2.1.3:

#### 5.2.1.3 Handover/PDCCH failure modelling Definition 3: A handover failure is counted if a RLF occurs in state 2, or a PDCCH failure is detected in state 2 or state 3. For calculating the handover failures for the two states: - In state 2: when the UE is attached to the source cell, a handover failure is counted if one of the following criteria is met: 1) Timer T310 has been triggered or is running when the HO_CMD is received by the UE (indicating PDCCH failure) or 2) RLF is declared in the state 2 - In state 3: after the UE is attached to the target cell a handover failure is counted if the following criterion is met: - target cell downlink filtered average (the filtering/averaging here is same as that used for starting T310) wideband CQI is less than the threshold Qout (-8 dB) at the end of the handover execution time (Table 5.1.4.1) in state 3. For the purpose of PDCCH failure condition monitoring in state 2: The L1 sample rate is once every 10ms and the L1 samples are filtered by a linear filter with a sliding window of 200ms (i.e. 20 samples). For the purpose of PDCCH failure condition monitoring in state 3: The L1 sample rate should be at least two samples during the 40ms (i.e. the handover execution time) and averaged over the number of samples. NOTE 1: The handover failure definition 3 above is different from the handover failure definition in TS 36.331 [5]. It serves the purpose of evaluating the handover performance at both serving and target cell while the definition in TS 36.331 [5] is from the UE point of view which only captures the failures in target cell. NOTE 2: PDCCH failure condition detection is based on power measurement with the simplified model for saving simulation time to allow the calibration with various sets of configurations and more efficient simulations. Definition 4: The handover failure rate is defined as: Handover failure rate = (number of handover failures) / (Total number of handover attempts). The total number of handover attempts is defined as: Total number of handover attempts = number of handover failures + number of successful handovers. The number of handover failures is in Definition 3.

The methodology is further explained in the figures below, also copied from [2]:

Figure 5.2.1.3.1: A handover failure is declared when the criterion 1) is met in state 2. Figure 5.2.1.3.2: A handover failure is declared when the criterion 2) is met in state 2.

To verify the impact of using predicted measurements/measurement event on handover KPIs, in addition to the parameters mentioned in section 2.1 of this document, following settings were used in order to be able to determine handover failure as per the methodology from TR 36.839 described above:

| Parameter | Value |
| --- | --- |
| Qin threshold | -6db |
| Qout threshold | -8db |
| Sample rate (TIndication_interval) | 40ms(FR1) |
| Qin evaluation period | 100ms |
| Qout evaluation period | 200ms |
| T310 | 1000ms |
| N310 | 1 |
| N311 | 1 |
| Max ETD | 80ms |
| Handover preparation time | 40ms |
| Handover execution time | 40ms |

## 3.2 Simulation results

RAN2 agreed to utilize two KPIs for evaluation of impact that using predicted measurement event has on mobility performance (also the same as the ones in TR 36.839):

- Handover failure rate, defined as: HOF rate = (number of handover failures) / (Total number of handover attempts).

- Total number of HO attempts per UE per second which includes both successful and failed handover attempts

Table below presents simulation results for the setup described above for FR1 to FR1 intra-frequency temporal domain case B (Case 2) with MRRT=50%, OW=120ms, PW=40ms and for two UE speeds: 30 and 90 km/h.

Table 5. Comparison of HO failure rate and total number of HO attempts between FR1 to FR1 intra-frequency temporal domain case B prediction and non-AIML based measurements

|  | UE speed=30 km/h |  |  | UE speed=90km/h |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | HO failure rate | Total number of HO attempts per UE per second | F1 score for measurement event prediction | HO failure rate | Total number of HO attempts per UE per second | F1 score for measurement event prediction |
| Baseline, i.e. KPIs when actual measurement results are used | 0% | 0.5 | N/A | 4.678% | 0.57 | N/A |
| AIML predicted measurement results: MRRT=50%, OW=120ms, PW=40ms | 0% | 0.4967 | 0.956 | 5.263% | 0.57 | 0.83 |

As can be seen, in both baseline scenario and scenario with AIML aided measurements the observed HOF rate was equal to 0 for UE speed=30km/h, i.e. no HO failures occurred. For UE speed of 90 km/h, the HOF rate slightly increased when AIML based event prediction was used, to be precise there was one more HOF throughout the simulation which lasted 300s. This does not seem to be a big deterioration while it should be noted that F1 score for this scenario was equal to 0.83 as compared to ~0.96 for the scenario with 30km/h UE speed.

When it comes to the number of handovers, it is also the same or almost the same for AI predicted measurements and legacy measurements (there was one handover less in the simulation for the case of UE speed=30km/h).

Even though it would be worth to verify the findings for other scenarios, based on the above results and analysis, we can draw the following conclusions:

Observation 7: In intra-frequency case B temporal domain prediction in FR1 with MRRT=50% and PW=40ms, the handover KPIs (HOF rate, total number of HOs) are the same or almost the same as in the baseline scenario (i.e. when relying only on actual/non-predicted measurement results).

Observation 8: When using AIML based event prediction, HOF rate and the number of handovers KPIs can be comparable to the baseline (non-AIML) scenario even with F1 score of 0.83.

Observation 9: AIML model based measurement predictions give a possibility to reduce the measurement effort of the UE (e.g. by 50% in the simulated scenario) without impacting the handover performance.

# 3   Summary

Based on the simulations presented in this paper, the following observations are made:

Observation 1: Relatively good F1 score values (e.g. above 0.8) can only be achieved for very short PW length values, i.e. up to 80ms in the simulated scenarios.

Observation 2: PW length has a significant impact on measurement event accuracy (F1 score) which decreases quickly when PW length is increased.

Observation 3: RSRP prediction error has a significant impact on measurement event accuracy (F1 score) which decreases quickly when RSRP prediction error increases.

Observation 4: RSRP prediction error alone cannot be directly correlated with achievable F1 score for measurement event prediction which depends also on other factors, e.g. PW length, MRRT.

Observation 5: F1 score decreases with an increasing UE speed.

Observation 6: F1 score decreases with an increasing MRRT value.

Observation 7: In intra-frequency case B temporal domain prediction in FR1 with MRRT=50% and PW=40ms, the handover KPIs (HOF rate, total number of HOs) are the same or almost the same as in the baseline scenario (i.e. when relying only on actual/non-predicted measurement results).

Observation 8: When using AIML based event prediction, HOF rate and the number of handovers KPIs can be comparable to the baseline (non-AIML) scenario even with F1 score of 0.83.

Observation 9: AIML model based measurement predictions give a possibility to reduce the measurement effort of the UE (e.g. by 50% in the simulated scenario) without impacting the handover performance.

# References

- R2-2410186 Text proposal of 38.744	OPPO		pCR	Rel-19	38.744	0.0.5

- 3GPP TR 36.839	Evolved Universal Terrestrial Radio Access (E-UTRA); Mobility enhancements in heterogeneous networks

# Annex A – simulation assumptions

Below are the company selectable assumptions chosen by us.

| Configurable/selectable simulation assumption |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Spatial consistency | Whether LOSsoft is modeled or not | UE trajectory model | UE trajectory boundary processing | L1/L3 filtering (sliding/non-sliding) | Number of Tx beams | Number of Rx beams |
| Procedure B in TR38.901 | Not modelled | Option 1 in 38.843 section 6.3.1 | Wrap-around model | Both non-filtered and non-sliding L1 filtering cases were simulated | FR1: 4 FR2: 64 | FR1: 1 FR2: 8 |

All the simulation assumptions are listed in the table below with the selectable ones highlighted in yellow.

Table 1: Summarized simulation assumptions

| Parameter | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz, 15/30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading without LOSsoft and inter-frequency correlation; without Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading without LOSsoft; without Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage |
| System BW | 20MHz | 80MHz |
| UE speed | 30,60,90 km/h for study targeting measurement reduction | 60,90,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) |
| UE Antenna radiation pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 44dBm | 40 dBm |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | Procedure B in TR38.901 | Procedure B in TR38.901 |
| UE trajectory model | Option 1 in 38.843 section 6.3.1 | Option 1 in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | wrap round model | wrap round model |