3GPP TSG-RAN WG2 Meeting #129								R2-2500893

Athens, Greece, 17th – 21st February, 2025

Title: 	Discussion on NW-sided data collection for training

Source: 	Huawei, HiSilicon

Agenda item:	8.1.3

Document for:	Discussion/Decision

# Introduction

In the previous meetings, the following agreements were achieved for NW-sided data collection :

Agreements from RAN2#127bis

1	Periodic logging is supported for training data collection procedure in R19

2	Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

3	Periodic reporting of logged data is not supported.

4	On-demand reporting of the logged measurements will be specified

5	UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

6	The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

7	Low priority SRB will be used.    FFS new SRB or use of SRB4

8	For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

Agreements from RAN2#128

1. Focus on the following three radio condition event based logging

§ L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2)

§ Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value)

§ L1 beam level measurement

2. Measurements on aperiodic CSI resources are not reported for NW sided data collection.

3. Data collection is controlled by the network.   The UE will not autonomously stop when low power state is detected.

4. The UE reports to the network when the power state is low.  We will not specify how the UE determines low power state.   The network should de-configure the data collection (this can be captured in stage 2).

5. The UE reports to the network when buffer is or may become full.  FFS when it reports (before and/or after).

6. The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory).  FFS how this is signalled and if the reporting can be part of availability indication.

7. Duration is not supported

This paper analyzes the network-sided data collection topic further, with a focus on the FFS points mentioned above.

# Discussion

## Data availability indication

Regarding the on-demand reporting, it has been agreed that the UE should send the data available indication to the NW when the collected data is available on the UE side. There is an FFS about the trigger condition for the data available indication as following in [2].

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

Furthermore, in RAN2#128 some further agreements were made:

- The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after).

In our understanding, the indication of the “full buffer” is the baseline data availability indication trigger. This is because data volume is the most important metric for data logging and the NW can use it to control how much data the UE should collect, e.g. depending on how much data the NW needs to receive from the UE to efficiently train a model. Furthermore, it is expected that the UE memory used for storing the collected measurements will be limited, so it is useful for the network to know when the UE’s memory is running out. Based on this the network can retrieve the data from the UE allowing it to collect more measurements, if needed.

When it comes to the FFS on when the report from the UE should be sent, we think it has to be sufficiently early to allow the UE to continue data collection without an interruption, if needed. If the UE reported data availability only once its buffer becomes full, the UE would have to stop collecting additional data until the network collects the buffered one.

Observation 1: The data collected by the UE needs to be indicated to the network before UE’s buffer becomes full and early enough for the network to be able to collect it and allow the UE continue data collection without an interruption.

Furthermore, the network may have a preference to collect data more frequently, not waiting for the UE’s buffer to become full, e.g. to avoid having to collect big amount of data at the same time.

Observation 2: The network may have a preference to collect data more frequently, not waiting for the UE’s buffer to become full, e.g. to avoid having to collect big amount of data at the same time.

For the reasons mentioned above, we think it is reasonable to introduce a configurable data volume threshold used by the UE to send a data available indication to the NW when the volume of collected data exceeds this threshold. With such threshold, the NW would have flexibility to set it either to a value lower than or equal to UE’s maximum buffer size.

Proposal 1: For the on-demand data retrieval mechanism, the UE should send the data availability indication to the NW when the volume of the collected data reaches a threshold configured by the network.

As for how the UE sends the data available indication, we think the UEAssistanceInformation message can be used.

Proposal 2: The UEAssistanceInformation message is used for sending data availability indication.

## Low power state indication

In the last meeting, RAN2 agreed that the UE reports to the network when the power state is low and then the NW should de-configure the data collection [1]. Similarly as data available indication, we believe low power state indication can be sent by the UE using UEAssistanceInformation message.

Proposal 3: The UEAssistanceInformation message is used for sending low power state indication.

Furthermore, there is the following FFS on whether the low power indication can be the same as availability indication:

The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory).  FFS how this is signalled and if the reporting can be part of availability indication.

Considering that UE may be already in low power state when receiving a data collection request from the network, it may happen that there is no data to be collected from the UE. Hence, low power state indication and data availability indication can be separated.

Proposal 4: Low power state indication and data availability indications are separately signaled in the RRC message.

After receiving the low power indication from the UE, there may be two ways for the NW to de-configure the data collection job. Firstly, the network may remove the measurement and logging configuration on the UE side through the RRC reconfiguration message. Secondly, the NW may indicate the UE to suspend the running data collection procedure and recover it when the UE exits low power state. In case the UE recovers from the low power state quickly, then the NW would not have to configure the data collection configuration again when the UE exits the low power state, so a process can be continued.

Proposal 5: When the NW receives the low power state indication from a UE, the NW may use RRC Reconfiguration to either release the data collection or suspend it without removing it from UE’s RRC configuration.

Proposal 6: When the UE exits low power state, it indicates it to the network and the network may either configure a new data collection configuration or resume a suspended configuration.

Another issue may happen if the NW tries to configure data collection configuration to the UE which is already in a low power state. In this case, the UE will have a preference not to collect the data. Thus, the UE that is already in a low power state should send low power state indication to the NW when it receives data collection configuration.

Proposal 7:  The UE that is in a low power state should send a low power state indication to the NW when it receives the data collection configuration (e.g. using the RRCReconfigurationComplete or UAI message).

## Configuration details

Another FFS is whether multiple configurations can be provided to the UE as follows and whether they can be dynamically activated and deactivated [3].

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

A single UE may support data collection for multiple AI use cases, e.g. CSI prediction, beam management, and positioning. Thus, we think it should be possible for the network to configure one data collection configuration per use case. We currently do not see a need to have multiple configurations for the same use case simultaneously at a single UE.

Proposal 8: The NW can configure one data collection configuration per use case, i.e. multiple configurations for the same use case are not supported.

Regarding whether dynamic activation/deactivation of data collection should be supported. Since we do not believe multiple configurations for the same use case are needed, then also dynamic activation/deactivation is not required. Even when multiple configurations per use case would be supported, dynamic activation/deactivation is still not needed in our view. Data collection for model training is normally a continuous process and it is unclear what the purpose would be of activating and deactivating it dynamically. If the network no longer requires more data from the UE for a certain data collection configuration, the network may simply de-configure the data collection configuration using RRC signaling. Another potential reason to have dynamic activation/deactivation would be to allow the network to rely on semi-persistent CSI resources for data collection. Related to this, RAN2 made the following agreement during RAN2#128:

2. Measurements on aperiodic CSI resources are not reported for NW sided data collection.

Similarly, we do not see a reason to rely on semi-persistent resources and believe periodic CSI resources fit data collection purpose best. Based on the above consideration, we propose the following:

Proposal 9: Only periodic CSI resources are used for NW sided data collection, i.e. neither aperiodic nor semi-persistent CSI resources are supported for this purpose.

Proposal 10: Dynamic activation/deactivation of the data collection configuration is not supported (i.e. it is sufficient to rely on RRC signaling).

If multiple data collection configurations are supported, even if the UE has training data for different use cases, the NW may not need all the collected data from the UE at the same time. For example, the gNB may be only interested in BM-related training data collection for fine-tuning the BM model(s), and it may not need training data for other use cases. Thus, we think that the NW can be able to request some data from the UE.

Proposal 11: The NW should be able to request the UE to report the data only for a certain data logging configuration (e.g. corresponding to a specific use case).

## SRB for data reporting

At RAN2#127bis meeting, it was agreed that low priority SRB will be used and whether to use new SRB or existing SRB4 will be further discussed.

The feature of NR QoE measurement reporting was introduced since Rel-17. RRC messages containing the QoE reports (generated in UE application layer) may be quite large, and then it may lead to frequent RRC message transmission in Uu interface. In LTE Rel-15, a separate SRB4 was introduced, and the network is able to configure SRB4 to a different logical channel group with the lowest priority among SRBs and DRBs. For NR QoE, SRB4 was also introduced for the same purpose.

For SRB4, currently it is mainly for QoE reporting. If SRB4 is also supported for NW-sided data reporting, our analysis is shown as below:

- Pros:  very few specification impacts

- Cons:  if both QoE reporting and NW-sided data reporting are enabled at the same time, there is still competition between the two types of reports, which results in more latency. Furthermore, there are some UE actions specified in relation to QoE based on whether SRB4 is configured or not, so this may cause some misinterpretations, e.g. when the NW wants to use SRB4 for AI, but the UE performs some actions related to QoE.

For new SRB approach, it can follow the principle of SRB4 where the LCH priority of the new SRB is configurable. Our analysis is shown as below:

- Pros:  only NW-sided data is transmitted via the new SRB, so it avoids competition with other reports. It also avoids potential issues with AIML/QoE feature interworking, as mentioned above.

- Cons: some specification impacts

In the future, there may be more use cases for NW-sided data reporting, and the requirements may be different from QoE reporting. In this case, we think new SRB approach may be better and it has better scalability and backwards/forwards compatibility.

Proposal 12: RAN2 to adopt a new SRB for NW-sided data reporting with a configurable priority.

## Collected data handling during handover and state transitions

It may happen that while the UE is collecting the data, a handover needs to be performed. In such case, the source gNB may not have time to fetch the collected data from the UE as delaying a handover procedure may lead to a handover failure.

Observation 3: When a handover needs to be performed, the source gNB may not have time to fetch the collected data from the UE as delaying the handover procedure may lead to a handover failure.

On the other hand, if such data is not delivered to the source gNB, it may often happen that data from the cell edge is under-represented in the dataset collected by the gNB. Hence, we believe it is important that the data collected by the UE under the coverage of the source gNB can be fetched, even after the handover.

Proposal 13: It should be possible for the source gNB to receive the data collected by the UE even after handover to another gNB.

To achieve this, a simplest approach seems to be to allow the UE to send the data availability indication to the target gNB during the handover, e.g. in RRCReconfigurationComplete message. Based on this, the target gNB may fetch the data and forward it to the source gNB, e.g. using Xn signalling.

Proposal 14: The UE may send data availability indication to the target gNB during a handover, if the data collected in the source gNB was not fetched by the source gNB before the handover.

Proposal 15: After reception of such indication, the target gNB can fetch the data from the UE and forward it to the source gNB.

When it comes to data retention during a state transition from RRC CONNECTED to RRC IDLE or RRC INACTIVE, in the normal scenario, the gNB should be able to simply fetch the data from the UE before releasing the RRC connection. However, there may be a scenario where the UE goes to RRC IDLE due to RLF. In such case, it would be beneficial for the gNB to be able to retrieve the collected data. It may actually be also useful for the network to understand what led to an RLF. Therefore, we think the UE may retain the collected data when moving to RRC IDLE due to RLF and RAN2 should discuss the means for this data to be delivered to the source gNB.

Proposal 16: The gNB can fetch the collected data from the UE before moving the UE to RRC IDLE/INACTIVE state. FFS whether the collected data should be retained by the UE when the UE moves to RRC IDLE due to RLF, in order for the UE to deliver it to the network after establishing a new RRC connection.

# Conclusion

Based on the discussion in the Tdoc the following, we provided the following observations and proposals:

Data availability indication

Observation 1: The data collected by the UE needs to be indicated to the network before UE’s buffer becomes full and early enough for the network to be able to collect it and allow the UE continue data collection without an interruption.

Observation 2: The network may have a preference to collect data more frequently, not waiting for the UE’s buffer to become full, e.g. to avoid having to collect big amount of data at the same time.

Proposal 1: For the on-demand data retrieval mechanism, the UE should send the data availability indication to the NW when the volume of the collected data reaches a threshold configured by the network.

Proposal 2: The UEAssistanceInformation message is used for sending data availability indication.

Low power state indication

Proposal 3: The UEAssistanceInformation message is used for sending low power state indication.

Proposal 4: Low power state indication and data availability indications are separately signaled in the RRC message.

Proposal 5: When the NW receives the low power state indication from a UE, the NW may use RRC Reconfiguration to either release the data collection or suspend it without removing it from UE’s RRC configuration.

Proposal 6: When the UE exits low power state, it indicates it to the network and the network may either configure a new data collection configuration or resume a suspended configuration.

Proposal 7:  The UE that is in a low power state should send a low power state indication to the NW when it receives the data collection configuration (e.g. using the RRCReconfigurationComplete or UAI message).

Configuration details

Proposal 8: The NW can configure one data collection configuration per use case, i.e. multiple configurations for the same use case are not supported.

Proposal 9: Only periodic CSI resources are used for NW sided data collection, i.e. neither aperiodic nor semi-persistent CSI resources are supported for this purpose.

Proposal 10: Dynamic activation/deactivation of the data collection configuration is not supported (i.e. it is sufficient to rely on RRC signaling).

Proposal 11: The NW should be able to request the UE to report the data only for a certain data logging configuration (e.g. corresponding to a specific use case).

SRB for data reporting

Proposal 12: RAN2 to adopt a new SRB for NW-sided data reporting with a configurable priority.

Collected data handling during handover and state transitions

Observation 3: When a handover needs to be performed, the source gNB may not have time to fetch the collected data from the UE as delaying the handover procedure may lead to a handover failure.

Proposal 13: It should be possible for the source gNB to receive the data collected by the UE even after handover to another gNB.

Proposal 14: The UE may send data availability indication to the target gNB during a handover, if the data collected in the source gNB was not fetched by the source gNB before the handover.

Proposal 15: After reception of such indication, the target gNB can fetch the data from the UE and forward it to the source gNB.

Proposal 16: The gNB can fetch the collected data from the UE before moving the UE to RRC IDLE/INACTIVE state. FFS whether the collected data should be retained by the UE when the UE moves to RRC IDLE due to RLF, in order for the UE to deliver it to the network after establishing a new RRC connection.

# References

- [1] RAN2#128 meeting minutes

- [2] RAN2#127bis meeting minutes

- [3] RAN2#127 meeting minutes