3GPP TSG-RAN WG2 Meeting #129	R2-2500895

Athens, Greece, 17th – 21st February, 2025

Agenda item:	7.0.2.16

Source: 	Huawei, Hisilicon

Title: 	Discussion on PDCP SN gap report handling during UE mobility

Document for:	Discussion and Decision

# 1. Introduction

There is an LS from RAN3 on PDCP SN gap report handling during UE mobility [1], focusing on how to let the target gNB get aware of the packets discarded before handover so that it can reduce the latency. Three alternatives have been discussed in RAN3 and one of them relies on RAN2 work. RAN3 asks RAN2 to evaluate whether the alternative can be supported or not. In this contribution, we will give our views on the related alternative and provide the draft reply LS.

# 2. Discussion

The main part of the LS in R3-247891 [1] is excerpted as below.

# 1. Overall Description: For Rel-18 XR, it is unclear how to handle the PDCP SN gap report during UE’s inter-gNB mobility. If a UE discarded some UL packets before handover, the UE reports the PDCP SN gap report to the source gNB, but the target gNB may not be aware of those discarded packets if no PDCP SN gap is reported to target gNB. This may result in unnecessary delay in the target gNB. The following solutions were discussed in RAN3: - Alternative 1: The source gNB forwards the PDCP SN gap report information to the target gNB. - Alternative 2: The source gNB indicates the discarded SDUs as successfully received via the UL received status in the SN STATUS TRANSFER sent to the target gNB. - Alternative 3: The UE sends the PDCP SN gap report again after connecting to the target gNB. RAN3 would like to ask RAN2 to evaluate whether Alternative 3 can be supported. # 2. Actions: # To RAN2: ACTION: RAN3 kindly asks RAN2 to provide feedback on whether Alternative 3 can be supported.

According to the LS, RAN3 has some concern on PDCP SN gap report during UE mobility. Specifically, for uplink transmission, if the UE discards some packets before handover, it will send the PDCP SN gap report to the source gNB, so that the source gNB can update the receiving window according to the SN gap report. For AM DRBs, during the handover procedure, the source gNB forwards the buffered packets to the target gNB and sends the SN STATUS TRANSFER message to the target gNB. The SN STATUS TRANSFER message indicates the PDCP SNs corresponding to those packets that haven't been received, so that the target gNB could ask UE for retransmission of these packets after handover by PDCP status report. However, among the packets indicated as not received in the SN STATUS TRANSFER, there may be some packets which have already been discarded by the UE before handover. The target gNB is not aware of the discarding and will wait for the discarded packets by starting the t-reordering timer. However, it is impossible for the target gNB to receive the discarded packets anymore, which means it has to wait for the t-reordering timer to expire before pushing the receiving window and deliver the packets after the discarded ones. This will result in a larger delay for the received packets. And it actually wastes the resource for transmitting the PDCP status report.

For example, the following figure shows a possible situation at the source gNB. The UE indicated to the source gNB that the PDCP SDU 1/2/6/7 were discarded. According to the report, the source gNB updated the RX_DELIV and RX_NEXT. Then during handover, the source gNB will forward the SDU 8 to the target gNB and indicate that SDU 4/5/6/7 are not received yet. However, since SDU 6/7 are already discarded, the target gNB cannot receive them from the UE anymore, which will introduce unnecessary waiting delay.

Fig.1 The source gNB received the PDCP SN gap report before handover.

To resolve the issue, the following solutions were considered in RAN3.

- Alternative 1: The source gNB forwards the PDCP SN gap report information to the target gNB.

- Alternative 2: The source gNB indicates the discarded SDUs as successfully received via the UL received status in the SN STATUS TRANSFER sent to the target gNB.

- Alternative 3: The UE sends the PDCP SN gap report again after connecting to the target gNB.

Among the three alternatives, alternative 1 and alternative 2 are within RAN3 scope while alternative 3 requires RAN2 work. To continue the discussion, RAN3 asks RAN2 to provide feedback on alternative 3.

Alternative 3 is not supported by the current PDCP specification. According to the description in TS 38.323 [2], as excerpted below, the UE will trigger a PDCP SN gap report only when it discards the PDCP SDU. After handover, the UE will not trigger another PDCP SN gap report to indicate the discarded PDUs which have already been included in a previous PDCP SN gap reported to the source gNB. Thus, the alternative 3 does not work according to the existing specification.

Observation 1: It is not supported by the current specification to trigger PDCP SN gap report based on UE handover.

---------------------Excerpted from TS 38.323--------------------- ### 5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - the PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by RLC to lower layers. …

While some companies may think alternative 3 can be achieved based on UE implementation, we think it is incorrect. There is only a NOTE saying the UE can limit the frequency of sending SN gap report based on implementation in the PDCP specification, which actually intends to reduce the overhead of SN gap report when it is triggered frequently. The triggering of SN gap report is clearly standardized as shown above, where handover is not considered. UE is not expected to trigger and send SN gap report without meeting the conditions.

Observation 2: UE should not send the SN gap report of the previous discarded packets after handover based on the implementation.

With the above analysis, we can conclude that, to support alternative 3, RAN2 would have to update the UE behaviour for PDCP SN gap triggering during handover. This will impact the R18 UEs and may cause some inconsistency to the UE implementation. After such a long time after R18 specifications freeze, we do not think such changes to UE behaviour can be made, especially considering the fact that this is not the only solution to resolve the issue according to RAN3 LS. Some network-based solutions are also feasible and can achieve the same purpose. Thus, to avoid an issue of impacting the UE behaviour and having an inconsistent UE behaviour in the field, we prefer to leave the issue to RAN3 by investigating the network-based solutions. A draft reply LS to RAN3 is provided in the annex part based on the above discussion.

Proposal 1: RAN2 to reply to RAN3 that alternative 3 is not preferred by RAN2 since it is not supported by the current specification and would impact UE behaviour, which may lead to inconsistent UE behaviour in the field.

# 3. Conclusion

In this contribution, we analyzed the LS on PDCP SN gap report. The following proposals and observations were provided:

Observation 1: It is not supported by the current specification to trigger PDCP SN gap report based on UE handover.

Observation 2: UE should not send the SN gap report of the previous discarded packets after handover based on the implementation.

Proposal 1: RAN2 to reply to RAN3 that alternative 3 is not preferred by RAN2 since it is not supported by the current specification and would impact UE behaviour, which may lead to inconsistent UE behaviour in the field.

# 4. Reference

[1]. R3-247891, TSG RAN WG3, LS on PDCP SN gap report handling during UE mobility.

[2]. 3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

# Annex. Draft reply LS

3GPP TSG-RAN WG2 Meeting #129	   R2-xxxxxx

Athens, Greece, 17th – 21st February, 2025

Title:	[Draft] Reply on PDCP SN gap report handling during UE mobility

Response to:	R3-247891

Release:	Rel-18

Work Item:	NR_XR_Ph3-Core

Source:	Huawei, HiSilicon (to be RAN2)

To:	RAN3

Cc: 	-

# Contact Person:

# Name:	Dawid Koziol

# Email:		dawid.koziol AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Overall Description:

RAN2 thanks RAN3 for investigating SN gap report handling during UE mobility and providing the solutions for the identified issue.

From RAN2 perspective, alternative 3, i.e., the UE sends the PDCP SN gap report again after connecting to the target gNB, is not preferred. It is not supported by the current RAN2 specification. To support it, the UE behaviour would have to be changed, which is not expected by RAN2 at this stage and may lead to inconsistent UE behaviour in the field.

# 2. Actions:

# To RAN3:

ACTION: RAN2 kindly asks RAN3 to take the above feedback into consideration.

# 3. Date of Next RAN2 Meetings:

RAN2#129bis	7th Apr – 11th Apr 2025				China

RAN2#130	19th May – 23rd May 2025				Malta