3GPP TSG-RAN WG2 Meeting #129								R2-2500897

Athens, Greece, 17 – 21 February, 2025

Title: 	Discussion on SDT data transmission restrictions

Source: 	Huawei, HiSilicon

Agenda item:	8.18

Document for:	Discussion/Decision

# Introduction

During RAN2#128 meeting, it was proposed in [1] to address an issue related to limited data volume that can be transmitted during an SDT procedure. RAN2 agreed to work on resolving this issue, but no concrete solution was agreed. In this paper, we elaborate this topic and compare possible solutions.

# Discussion

As mentioned in [1], the issue with the current SDT procedure is twofold:

- The bandwidth available for transmission in DL is limited to CORESET#0 which means maximum 48PRBs can be allocated by the scheduler.

- There is no support for CSI reporting during SDT meaning that the scheduler needs to employ conservative MCS to ensure successful transmission.

Due to these two issues, the volume of data that can be sent during an SDT procedure is limited.

In [1], the solution that was proposed relied on UE sending CQI report in msg3 to the network. According to the proponents, such report will allow the gNB to utilize less conservative MCS for DL transmissions, which in turn would enhance the data volume that could be sent to the UE during a single SDT procedure. However, during the discussion other companies expressed the view that sending a CQI report in msg3 would solve the issue to a limited extent only. According to them, it would be better to lift the restriction of SDT BWP being limited to CORESET#0 only.

In our view both these approaches have their merits, in particular:

- CQI based approach is already being discussed by RAN1 in the context of LTM. If CQI reporting would be enabled for msg3/msgA for the sake of LTM, it may be easily, or even wihtout any additional efforts, be usable for other scenarios, e.g. different types of handovers and for the sake of SDT. This alternative allows to sue available radio resources in a more spectrally-efficient way.

- Extending SDT BWP approach on the other hand would allow to have more physical resources available for transmissions throughout the SDT procedure. However, the potential issue is that the UE may need to switch a BWP while still being in RRC INACTIVE. Also, even when the BWP restriction is lifted, the available resources still cannot be used efficiently without giving a gNB an opportunity to utilize more refined MCS.

Observation 1: CQI reporting in msg3 is already being considered by RAN1 for LTM and it can potentially address other scenarios as well, such as SDT.

Observation 2: Extending SDT BWP would make more resource available for SDT transmissions, but these resources would still have to utilize conservative MCS, which is not spectrum efficient.

As can be seen, both of these approaches have their merits and are not mutually exclusive. Furthermore, it is evident that both of them have an impact on RAN1 specifications and cannot be decided solely in RAN2. Based on this we propose to raise the issue to RAN1 and ask them for their views on using CQI reporting in msg3/msgA and extending the SDT BWP.

Proposal 1: RAN2 to send an LS to RAN1 asking about:

- The possibility of sending CQI report in msg3/msgA during an SDT procedure

- The possibility to lift the restriction of DL SDT BWP being limited to CORESET#0 only.

A draft of the LS is provided in the Annex.

# Conclusion

Based on the analysis in this paper, we propose the following:

Proposal 1: RAN2 to send an LS to RAN1 asking about:

- The possibility of sending CQI report in msg3/msgA during an SDT procedure

- The possibility to lift the restriction of DL SDT BWP being limited to CORESET#0 only.

A draft of the LS is provided in the Annex.

# References

- [1] R2-2410542 	Quality Indication in Msg3 for SDT 		Ericsson

# Annex. Draft LS to RAN1

3GPP TSG-RAN WG2 Meeting #129	   R2-xxxxxx

Athens, Greece, 17th – 21st February, 2025

Title:	LS on SDT resources restriction

Response to:	-

Release:	Rel-19

Work Item:	TEI19

Source:	RAN2

To:	RAN1

Cc: 	-

# Contact Person:

# Name:	Dawid Koziol

# Email:		dawid.koziol AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Overall Description:

As part of TEI19 discussions, RAN2 agreed to address an issue of limited resources being available for the DL transmissions during SDT procedure. Furthermore, RAN2 identified two potential approaches to solve this issue (which are not necessarily mutually exclusive):

- Enable CQI to be reported by the UE in msg3/msgA which would allow the gNB to utilize more refined MCS for SDT transmissions.

- Lift the restriction of SDT BWP being limited to CORESET#0 only which would allow the gNB to schedule the UE with more physical resources for SDT transmissions.

RAN2 believes that both of these approaches have RAN1 impact and might be or have been discussed by RAN1 already. Thus, RAN2 would like to request RAN1 feedback on the feasibility and potential impact of these approaches in RAN1.

# 2. Actions:

# To RAN1:

ACTION: RAN2 kindly asks RAN1 to provide feedback on the feasibility and potential impact of the approaches mentioned in the LS on RAN1 specifications.

# 3. Date of Next RAN2 Meetings:

RAN2#129bis	7th Apr – 11th April 2025				China

RAN2#130		19th	May – 23rd May 2025				Malta