3GPP TSG-RAN2 Meeting #129	R2-2500900

Athens, Greece, Feb. 17th – 21st, 2025

Source: 			ZTE Corporation

Title: 	Discussion on model generalization of RRM measurement prediction

Agenda item:	8.3.2.2

Document for: 	Discussion and Decision

- Introduction

In the RAN2 128 meeting, we have agreed to study the model generalization for RRM measurement prediction over UE speed and frequency, as follows:

- Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following cases, - Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B. - Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B. - Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B. - Study generalization over UE speeds - Companies that would like to study inter-frequency generalization can start with input 2GHz and output 4GHz, and 4GHz to 2GHz. FFS if we introduce a third frequency.

In this document, we provide the simulation results for model generalization over UE speed and frequency, and share the corresponding views.

- Discussion

## 2.1 Evaluation results for model generalization of UE speed

In the last meeting, we have agreed to start the study with generalization issue with RRM measurement prediction in temporal domain. So, in this section, we provide the evaluation results for model generalization of UE speed with temporal domain prediction case B in FR1 scenario and temporal domain prediction case A in FR2 scenario.

- Evaluation results for model generalization of UE speed in FR1 scenario

The simulation assumptions for intra-freq temporal domain prediction case B are reused here. Some of the simulation assumptions are shown in the table 1:

Table 1: Simulation assumptions for model generalization simulation of UE speed in FR1 scenario

| Parameters |  |
| --- | --- |
| MRRT | 50% |
| PW length (ms) | 200 |
| OW length (ms) | 600 |
| RRM sub use case | Option 2 |
| Filtering option | Option 3 |
| Skipping pattern |  |

The simulation results for model generalization of UE speed in FR1 scenario are shown in the table 2 and table 3. For the table 2, in one simulation case, the AI models are trained using the different dataset with different UE speed, but are tested using the dataset with the same UE speed. From table 2, we can find that even the AI models are trained using different dataset with different UE speed, the average L3 cell RSRP difference are nearly the same if the AI models are tested using the dataset with the same UE speed. For example, if the AI models are tested using inference dataset with 30km/h UE speed (i.e. case 1), the maximum average L3 cell level RSRP difference dB is 0.543dB with the AI model that is trained using datasets with 90km/h UE speed, and the minimum average L3 cell level RSRP difference is 0.521dB with the AI model that is trained using datasets with 30km/h UE speed, the difference between max and minimum average L3 cell level RSRP difference is only 0.022dB. In our understanding, this is because not only the UE speed can impact the channel quality change, but the UE movement direction can also impact. In other words, even with the same UE speed, the experienced channel quality change for different UE are also different. So, it is possible that the training dataset with different UE speed have similar channel quality variation characteristics.

Table 2: the evaluation results for model generalization of UE speed in FR1 scenario with the same inference speed

| Simulation Case | Training/Inference speed | Average L3 cell level RSRP difference (dB) |
| --- | --- | --- |
| Case 1 | Baseline Training @30km/h+Inference @30km/h | 0.521 |
|  | GC #1 Training @90km/h+Inference @30km/h | 0.543 |
|  | GC #2 Training @30/60/90 km/h+Inference @30km/h | 0.537 |
| Case 2 | Baseline Training @60km/h+Inference @60km/h | 0.624 |
|  | GC #2 Training @30/60/90 km/h+Inference @60km/h | 0.625 |
| Case 3 | Baseline Training @90km/h+Inference @90km/h | 0.701 |
|  | GC #1 Training @30km/h+Inference @90km/h | 0.737 |
|  | GC #2 Training @30/60/90 km/h+Inference @90km/h | 0.704 |

Observation 1: Regarding model generalization in the temporal domain prediction case B, in the case that the AI/ML models are trained using the dataset with different UE speed but tested using the dataset with the same UE speed, the average L3 cell level RSRP differences are nearly the same.

For the table 3, in one simulation case, the AI model is trained using one UE speed, but is tested using the dataset with the different UE speed. It is found that with same AI model, the average L3 cell level RSRP difference increases with the UE speed that AI model is tested with. For example, the AI model is trained using the dataset with 90km/h UE speed, if the AI model is tested using the dataset 30km/h UE speed, the average L3 cell level RSRP difference is 0.543dB; but if the AI model is tested using the dataset with 90km/h UE speed, the average L3 cell level RSRP difference is 0.701dB. In this case, even the AI/ML model is trained and tested using the dataset with the same UE speed, the prediction performance is not better. This is because as we mentioned above, the UE speed that training data with does not have much impact on the prediction performance. In the past meeting, we have agreed that with the increase of UE speed, the prediction accuracy decreases, so, in each simulation case, inference UE speed increases, average L3 cell level RSRP difference increases.

Table 3: the evaluation results for model generalization of UE speed in FR1 scenario with the same training speed

| Simulation Case | Training/Inference speed | Average L3 cell level RSRP difference (dB) |
| --- | --- | --- |
| Case 1 | Baseline Training @30km/h+Inference @30km/h | 0.521 |
|  | GC #1 Training @30km/h+Inference @90km/h | 0.737 |
| Case 2 | Baseline Training @90km/h+Inference @90km/h | 0.701 |
|  | GC #1 Training @90km/h+Inference @30km/h | 0.543 |

Observation 2: Regarding intra-freq temporal domain prediction case B, in the case that the AI/ML models are trained using the dataset with the same UE speed, higher UE speed the AI model is tested using dataset with, the larger average L3 cell level RSRP difference.

- Evaluation results for model generalization of UE speed in FR2 scenario

In this part, we study the model generalization of UE speed in temporal domain prediction case A. In the simulation, the observation window and prediction window are 800ms and 400ms, respectively. The corresponding simulation results are shown in the table 4 and table 5.

The same tendency as in FR1 scenario can be seen in the FR2 scenario. From table 4, it is observed that even the AI models are trained using different dataset with different UE speed, the average L3 cell RSRP difference are nearly the same if the AI models are tested using the dataset with the same UE speed. From table 5, it is found that with the same AI model, higher UE speed the AI model is tested using dataset with, the larger average L3 cell level RSRP difference.

Table 4: the evaluation results for model generalization of UE speed in FR2 scenario with the same inference speed

| Simulation Case | Training/Inference speed | Average L3 cell level RSRP difference (dB) |
| --- | --- | --- |
| Case 1 | Baseline Training @60km/h+Inference @60km/h | 0.680 |
|  | GC #1 Training @120km/h+Inference @60km/h | 0.683 |
|  | GC #2 Training @60/90/120 km/h+Inference @60km/h | 0.681 |
| Case 2 | Baseline Training @90km/h+Inference @90km/h | 0.794 |
|  | GC #2 Training @60/90/120 km/h+Inference @90km/h | 0.795 |
| Case 3 | Baseline Training @120km/h+Inference @120km/h | 0.946 |
|  | GC #1 Training @60km/h+Inference @120km/h | 0.951 |
|  | GC #2 Training @60/90/120 km/h+Inference @120km/h | 0.949 |

Table 5: the evaluation results for model generalization of UE speed in FR2 scenario with the same training speed

| Simulation Case | Training/Inference speed | Average L3 cell level RSRP difference (dB) |
| --- | --- | --- |
| Case 1 | Baseline Training @60km/h+Inference @60km/h | 0.680 |
|  | GC #1 Training @60km/h+Inference @120km/h | 0.951 |
| Case 3 | Baseline Training @120km/h+Inference @120km/h | 0.946 |
|  | GC #1 Training @120km/h+Inference @60km/h | 0.683 |

Observation 3: Observation 1 and 2 for intra-freq temporal domain prediction case B are applicable for intra-freq temporal domain prediction case A.

Proposal 1: Regarding model generalization of UE speed, capture observation 1~3 into the TR.

## 2.2 Evaluation results for model generalization of frequency

The simulation assumptions for frequency domain prediction are reused here. In the GC #2, although the AI/ML model is trained using the dataset with both 4G2G and 2G 4G, one label is used to indicate the data is from dataset with 4G2G or the dataset with 2G 4G. From the table 6, we find that the average L3 cell level RSRP difference in the baseline and GC#2 are almost the same, but the average L3 cell level RSRP difference in the GC #1 are much higher. For example, with the test dataset with 4G2G, the average L3 cell level RSRP difference for the AI model that is trained using dataset with 4G2G is 0.97dB, but the average L3 cell level RSRP difference for the AI model that is trained using dataset with 2G 4G is 11.90 dB, which is much higher than the former case. This maybe mainly because the impact of path loss. Specially, for GC #1, if the AI model is trained using the dataset with 2G->4G, cell results at input shall be about 6dB larger than the cell results at output in terms of path loss; but if the AI model is tested using the dataset with 4G2G, the cell results at input are expected to increase 6dB, but actually decrease 6dB for path loss, so the prediction error is very large.

Table: the evaluation results for model generalization of frequency

| Simulation Case | Training/Inference speed | Average L3 cell level RSRP difference (dB) |
| --- | --- | --- |
| Case 1 | Baseline Training: @4G-->2G + Inference @ 4G-->2G | 0.97 |
|  | GC #1 Training: @2G-->4G + Inference @ 4G-->2G | 11.90 |
|  | GC #2 Training: @2G-->4G / 4G-->2G + Inference @ 4G-->2G | 1.00 |
| Case 2 | Baseline Training: @2G-->4G + Inference: @2G-->4G | 0.96 |
|  | GC #1 Training: @4G-->2G+ Inference: @2G-->4G | 11.28 |
|  | GC #2 Training: @ 2G-->4G / 4G-->2G + Inference: @2G-->4G | 1.00 |

Observation 4: Regarding frequency domain prediction, the prediction performance of GC #1 is much worse than the baseline case; while with one label indicating the data is from dataset with 4G2G or 2G4G, the prediction performance of GC#2 and baseline case are almost the same.

Proposal 2: Capture observation 4 for model generalization of frequency into the TR.

- Conclusion and proposals

In this contribution, we have the following observations and proposals:

For generalization of UE speed

Observation 1: Regarding model generalization in the temporal domain prediction case B, in the case that the AI/ML models are trained using the dataset with different UE speed but tested using the dataset with the same UE speed, the average L3 cell level RSRP differences are nearly the same.

Observation 2: Regarding intra-freq temporal domain prediction case B, in the case that the AI/ML models are trained using the dataset with the same UE speed, higher UE speed the AI model is tested using dataset with, the larger average L3 cell level RSRP difference.

Observation 3: Observation 1 and 2 for intra-freq temporal domain prediction case B are applicable for intra-freq temporal domain prediction case A.

For generalization of frequency

Observation 4: Regarding frequency domain prediction, the prediction performance of GC #1 is much worse than the baseline case; while with one label indicating the data is from dataset with 4G2G or 2G4G, the prediction performance of GC#2 and baseline case are almost the same.

Proposal 1: Regarding model generalization of UE speed, capture observation 1~3 into the TR.

Proposal 2: Capture observation 4 for model generalization of frequency into the TR.

- Reference

- Report of RAN2 127bis meeting.