3GPP TSG-RAN WG2 Meeting #129	R2-2500911

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.1.4

Source:	Samsung

Title:	UE side data collection

Document for:	Discussion & Decision

# Introduction

At RAN#106 (December 2024) a Way Forward was agreed, comprising information for SA which has since been sent by RAN in an LS, and specific directions for RAN2 (which have been made the focus of this sub Agenda Item):

For SA From RAN perspective, transfer of data over UP for Solution 2 in SA2 should be studied and provide feedback on fulfilement of visibility and controllability requirements. Transfer of data over UP for Solution 3 can be studied by SA5 RAN has already concluded that transfer of data over CP for Solution 2 and 3 can meet the visibility and controllability RAN requirements, even though some scalability concerns have been raised for high amount of data transfer. RAN is not expecting SA2 and SA5 to study the transfer of data over CP for now. For RAN2 - During Rel-19 RAN2 continues studies on AIML UE sided data collection (excluding 1b) - Study RAN aspects related to data transfer over UP - Discuss level of NG-RAN involvement in the control and configuration of UE side data collection. - Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion). - Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain. - Discuss on the scalability aspects of CP - RAN2 can Liaise with other WGs if/when needed General RAN will make a final decision on way forward taking into account SA inputs when available. RAN will have a checkpoint in Q2, 2025

And as a reminder, at RAN#105 (September 2024), the following was agreed:

RAN has agreed to the following requirements for data collection for UE sided model training for standardized solution (if standardized) (i.e. Option 1b, 2, 3). Option 1a is not precluded. - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is futureproof and extendable. FFS/study if and how to handle non-standardized data (i.e. partial visibility). FFS controllability on data collection Standardized Solutions should follow the principle of aiming to minimize air interface overhead and impact to NW operation

In this submission we examine the way forward set out in RAN#106 in light of the requirements agreed at RAN#105, and the existing UE side data collection framework with its various data transfer options as captured in R2-2407807.

# On Options 2/3 and their variants Opt A/B/C

Based on RAN#106 agreements, the focus of the (revised) Study in the area of UE side data collection (which no longer has any related normative work expected in R19 due to downscoping of the WID) is on Options 2 and 3, each of which comprise 3 sub-options for data content visibility in MNO: 
- Opt A (Full visibility for standardized data contents), 
- Opt B (Partial visibility for partially standardized data contents), and 
- Opt C (No standardized visibility).

In our understanding, the agreement from RAN#105 that the MNO has full visibility for standardized data corresponds to Opt A, and does not rule out Opt B or Opt C. We further note that the requirements from RAN#105 on user data privacy, anonymity and user consent still need to be clearly defined (and this is beyond the scope of just RAN2) – otherwise, especially for Opt A (but also potentially for Opt B) – any solutions may clash with these requirements. It is unclear to us how in the remaining R19 timeframe the requirements could be clearly defined across all participating WGs, and solutions evaluated against these requirements.

Opt B additionally requires RAN2 (and beyond) to reach a consensus on what exactly is the UE proprietary information that can be included transparently together with the standardized data message, in the limited R19 timeframe.

To sum up, we would like the following to be noted:

Observation 1a. Following RAN#106, the focus of the (revised) Study in the area of data transfer related to UE side data collection (which no longer has any related normative work expected in R19 due to downscoping of the WID) is on Options 2 and 3, each of which comprise 3 sub-options for data content visibility in MNO: Opt A (Full visibility for standardized data contents), Opt B (Partial visibility for partially standardized data contents), and Opt C (No standardized visibility).

Observation 1b. Agreement from RAN#105 that the MNO has full visibility for standardized data (corresponding to Opt A for data content visibility in MNO quoted in Observation 1), does not rule out Opt B or Opt C.

Observation 1c. Opt A and Opt B may clash with requirements on user data privacy, anonymity and user consent, depending on their exact definitions (which are unlikely to be agreed upon in the R19 timeframe).

Observation 1d. Opt B has the additional complexity in that RAN2 would likely be unable to agree what is the scope of the UE proprietary information that can be included transparently together with the standardized data message, in the R19 timeframe.

Based on this, we would like to propose the following for RAN2’s consideration:

Proposal 1. RAN2 to clarify how Opt A and Opt B can satisfy RAN#105 requirements, specifically how each of them can meet simultaneously the visibility requirement on the one hand, and the user data privacy/anonymity on the other hand.

# Closer look at Option 2 and Option 3

As mentioned above, the focus of the (revised) Study in the area of UE side data collection is on Options 2 and 3. It is unclear to us why we would standardise both Options 2 and 3 – they both achieve similar overall goals in terms of full controllability on part of MNO without the need of SLA.

Upon closer inspection, Option 2 appears to have very limited RAN impact, and it could in theory be designed mainly by CT and SA WGs. One of the focus areas for this sub Agenda Item is to discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain, and for Option 2 it is not expected RAN2 would have much to discuss on this topic. And finally, it is expected that SA5 will study UP aspects of Option 3 during first half of 2025. This would give RAN2 a chance to study Option 3 in parallel.

We therefore observe the following:

Observation 2a. Motivation for standardising both Options 2 and 3 is unclear – they both achieve similar overall goals in terms of full controllability on MNO part without the need of SLA.

Observation 2b. Option 2 appears to have very limited RAN impact, and it could in theory be designed mainly by CT and SA WGs.

Proposal 2. RAN2 to focus on studying Option 3 for data transfer.

# Data collection configuration and data transfer

Previous sections and much of the discussion in this Agenda Item focuses on data transfer options. However as acknowledged in e.g. RAN#106, level of NG-RAN involvement in the control and configuration of UE side data collection should also be discussed.

We understand that there is no change in the current scope of Work Item objectives i.e. data collection configuration should continue to be discussed under LCM agenda item, and should be specified in Rel-19 WI regardless of status of data transfer option. It is noted that regardless of which of Option 2 or Option 3 (if either) is standardised for data transfer, RAN2 needs to ensure that UE side data collection in Rel-19 is supported based on at least Option 1a as baseline. We understand that there might be some aspects specific to Option 2 or Option 3 e.g. gNB would need to be provided with required data collection configuration from CN or from OAM in order to enable data collection without UE request, as UE request is optional for Options 2 and 3. RAN2 needs to discuss further how this would work as a part of Option 2 or Option 3. In this regards, we think that gNB should receive assistant information from or be triggered by CN in Option 2 or OAM in Option 3 directly, if gNB is to provide data collection configuration without UE request. If this is indeed the common understanding in RAN2, RAN2 should also request SA2/SA5 to discuss the feasibility of this approach along with the data transfer framework.

Proposal 3. RAN2 to continue discussing general aspects on control and configuration of data collection under LCM agenda item.

Proposal 4. RAN2 to discuss whether Option 2 or Option 3 have any specific aspects of control and configuration different from Option 1a in this agenda item.

Proposal 5. RAN2 to agree that gNB should be provided/triggered by CN (in Option 2) and by OAM (in Option3) if gNB is to provide data collection configuration without UE request.

Proposal 6. If Proposal 5 is agreeable, RAN2 to send an LS to SA2/SA5 asking them to take into account the need for data collection configuration support.

# Conclusion

RAN2 is kindly asked to discuss and agree the following proposals:

Proposal 1. RAN2 to clarify how Opt A and Opt B can satisfy RAN#105 requirements, specifically how each of them can meet simultaneously the visibility requirement on the one hand, and the user data privacy/anonymity on the other hand.

Proposal 2. RAN2 to focus on studying Option 3 for data transfer.

Proposal 3. RAN2 to continue discussing general aspects on control and configuration of data collection under LCM agenda item.

Proposal 4. RAN2 to discuss whether Option 2 or Option 3 have any specific aspects of control and configuration different from Option 1a in this agenda item.

Proposal 5. RAN2 to agree that gNB should be provided/triggered by CN (in Option 2) and by OAM (in Option3) if gNB is to provide data collection configuration without UE request.

Proposal 6. If Proposal 5 is agreeable, RAN2 to send an LS to SA2/SA5 asking them to take into account the need for data collection configuration support.