3GPP TSG-RAN WG2 Meeting #129	R2-2500913

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.13.3

Source:	Samsung

Title:	SRAP design for R19 multi-hop SL relaying

Document for:	Discussion & Decision

# Introduction

At the most recent RAN2 meeting (RAN2#128, November 2024), an initial discussion was held on the mechanics of SRAP operation for R19, and the following was agreed:

Agreements:

Maintain the principle that the SRAP header enables mapping from the e2e bearer ID to the egress PC5 RLC channel.  This does not preclude changes/rewrites/addition of the SRAP header by the intermediate UEs.

Remote UE ID and BEARER ID are included in Rel-19 SRAP header for multi-hop L2 U2N relay.

R19 SL Relaying is in our view a cross between U2N Relaying and U2U Relaying. In R19, the legacy U2N Remote UE now becomes (i.e. needs to incorporate functions of) an intermediate Relay – in other words, it does not just terminate or generate relaying data, but also passes data on further down the chain. Intermediate relays essentially perform a relaying function similar to the U2U Relay UEs. In this paper, we follow this line of reasoning and make some high-level proposals on the SRAP design for R19 MH.

# Discussion

In the previous release we had multi-path (direct/indirect), but not multi-hop. In this release we have multi-hop, but not multi-path. It is reasonable to assume that a multi-path, multi-hop scenario will become of interest in a future release, and the SRAP design should be future proofed for this and other key scenarios. One such measure could be adopting the U2U relaying approach and having two IDs in the SRAP header.

We have two IDs in the SRAP header for U2U Relaying, but that is for a different reason – to identify a specific pair of SRC/DST UEs, which we may not need in R19 MH design as we are focused on extending U2N to multi-hop. However, if we had two UE IDs now, we could reuse the R18 U2U Relay UE design for the R19 multi-hop, since the intermediate relays can then be based on U2U design (and they do connect to two UEs, e.g. two surrounding Relay UEs). Assuming that a single intermediate Relay UE can connect to more than one of intermediate/first Relay UEs, and intermediate/last Relay UEs, the R18 U2U Relay function of identifying different SRC/DST UE pairs becomes applicable to R19 MH. (This is still single-path, since from each UE to the network there is only one path. But the relays could be serving multiple other relays and/or UEs.)

Observation 1. Cases with two indirect paths to the gNB for the same Remote UE are not supported in R19. However, any of the relays may be connecting to multiple other relays and/or UEs, in both UL and DL directions. This means a U2U Relay UE design could be reused for the R19 U2N multi-hop Relay design of intermediate relays.

Observation 2. In future releases, multi-path (two indirect paths) may be supported, and this would again mean that a U2U Relay UE design could be the blueprint for the R19 intermediate (and possibly beyond) relay design.

Proposal 1. Based on Observation 1 and Observation 2, RAN2 is asked to discuss whether SRAP header should carry two UE ID fields.

With multiple ‘child nodes’, the next-hop UE on the DL could be indicated by a combination of two UE IDs of Proposal 1. If we decide to include only one UE ID, then we would need to configure the relay with a next-hop indicator for each UE ID – and this is still essentially some kind of UE ID; in other words, two UE IDs need to be used for routing either way.

Observation 3. If only one UE ID is included in the SRAP header, then the next-hop UE cannot be indicated by a combination of two UE IDs from the header, but rather by a combination of a single UE ID, and a next-hop indicator.

Observation 4. The next-hop indicator (the alternative to two UE IDs in the SRAP header) is still essentially a UE ID, although it is provided as part of relay configuration rather than included in the SRAP header.

Another issue to consider is the SRAP entity modelling. Legacy U2N relaying uses the two-entity model per Relay UE. However, this is unnecessary for R19 MH intermediate relays, where a single SRAP entity is more appropriate. Single entity is again an integral aspect of U2U relaying.

While entity modelling may be an implementation aspect, it will impact the specs and possibly also configuration procedures.

Observation 5. SRAP single-entity model per UE is more appropriate for R19 MH intermediate relays, which is another aspect of U2U Relaying applicable to R19 MH.

Based on Observations 3, 4 and 5, we propose:

Proposal 2. RAN2 to discuss whether to design R19 relays based on extension of legacy U2N Relaying (which may include configuring the next hop indicator), or based on U2U Relaying (which may include adding an additional UE ID in the SRAP header), or a combination of both.

# Conclusion

RAN2 is kindly asked to discuss and agree the following proposals:

Observation 1. Cases with two indirect paths to the gNB for the same Remote UE are not supported in R19. However, any of the relays may be connecting to multiple other relays and/or UEs, in both UL and DL directions. This means a U2U Relay UE design could be reused for the R19 U2N multi-hop Relay design of intermediate relays.

Observation 2. In future releases, multi-path (two indirect paths) may be supported, and this would again mean that a U2U Relay UE design could be the blueprint for the R19 intermediate (and possibly beyond) relay design.

Proposal 1. Based on Observation 1 and Observation 2, RAN2 is asked to discuss whether SRAP header should carry two UE ID fields.

Observation 3. If only one UE ID is included in the SRAP header, then the next-hop UE cannot be indicated by a combination of two UE IDs from the header, but rather by a combination of a single UE ID, and a next-hop indicator.

Observation 4. The next-hop indicator (the alternative to two UE IDs in the SRAP header) is still essentially a UE ID, although it is provided as part of relay configuration rather than included in the SRAP header.

Observation 5. SRAP single-entity model per UE is more appropriate for R19 MH intermediate relays, which is another aspect of U2U Relaying applicable to R19 MH.

Proposal 2. RAN2 to discuss whether to design R19 relays based on extension of legacy U2N Relaying (which may include configuring the next hop indicator), or based on U2U Relaying (which may include adding an additional UE ID in the SRAP header), or a combination of both.