3GPP TSG-RAN WG2 #129	R2-2500919

Athens, Greece, 17th – 21st February 2025

Agenda Item:	8.3.3

Source:	Ericsson

Title:	Evaluation of measurement event prediction

Document for:	Discussion

# 1	Introduction

The study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved in RAN plenary #102 [1]. This contribution evaluates the indirect measurement event prediction by leveraging temporal prediction Case A in FR2 and temporal prediction Case B in FR1 to predict A3 events.

# 2	Discussion

In this contribution, the simulation results for the Event A3 prediction using the indirect approach are provided. To see the relevant agreements and the agreed simulation settings used in our evaluation, see Annex A. Results from both Case A and Case B are presented in this contribution.

## Event prediction using Temporal Prediction Case A

Temporal prediction Case A is related to the case where the UE or network is trying to predict RSRP values into the future, e.g. to help with mobility decisions.

In the indirect approach Case A, the cells predictions (e.g. predicted RSRP) are used in the A3 event inequality formulas (which are defined in TS 38.331), instead of the actual cell quality measurements. An example of the inequality formulas for the predicted event A3, that is used in our evaluation is given below:

Entering condition:

Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off

Leaving condition

Mn[t] + Ofn + Ocn + Hys < Mp[t] + Ofp + Ocp + Off

Wherein Mn[t] and Mp[t] indicate respectively the predicted results of the neighboring and serving cells at the future time instance t. When the entering condition (evaluated based on the actual measurements) is fulfilled (evaluated based on based on the actual measurements) and stays fulfilled during TTT, a predicted A3 event is triggered and the UE may trigger a report to the network.

The model parameters used in the simulations for the FR2 Case A is presented in Table 1. A pseudo code of the AI/ML based Algorithm is also shown in the following.

AI/ML based Indirect A3 event prediction

Input: RSRP measurements, A3 event Conditions, Time-to-Triger (TTT) timer, Prediction and Observation windows Output: Triggering a Measurement Report

- While UE is active Do - Perform RSRP measurements for the serving and 20 neighboring cells - Monitor A3 entering condition for the 20 neighboring cells - If A3 entering condition is fulfilled for the last RSRP samples for at least one of the neighboring cells: - Predict a sequence of RSRP values in the prediction window (equal to TTT) - If no value in TTT fulfil A3-leave condition. - Consider the A3 event fulfilled and trigger a measurement report. - Jump 5 samples in the dataset. - Continue to a. - End If - End if - Step one sample in dataset. - End While

Predictions and evaluation of events are only done when one or more neighbor cells fulfill the entering condition in the last sample in the observation window. If no cell fulfills the entering condition no prediction is made, and the evaluation process of the dataset proceed to check if any neighbor fulfills the entering condition in the next time-step/sample. In the case when a prediction is made, and an event is predicted to be fulfilled the evaluation process jumps the number of samples equal to the TTT to simulated that a handover is made after TTT runs out and then other cells could fulfill the entering condition.

Table 1: Parameters for the model used in the FR2 Case A simulations.

| Settings / Parameters | Value |
| --- | --- |
| AI model | LSTM |
| Flops | 156k |
| Number of parameters | 65k |
| Clustering setting | Input: 21 cells, output: 21 cells |
| AI input/output | Cell level L3 RSRP |
| Observation window | 5 samples, i.e. 400 ms. |

Results from the test with the baseline A3 event parameters are presented in Table 2, the RSRP difference is the average error across all the samples in the prediction window.

Table 2: Results for indirect event prediction based on the temporal prediction Case A in FR2.

| Metric | AI Model | Sample and Hold |
| --- | --- | --- |
| RSRP difference of whole pred. window (dB) | 0.73 | 1.10 |
| RSRP difference of last value in pred. win. (dB) | 1.57 | 2.40 |
| Number of True predictions (True Positive). | 16535 | 16670 |
| Number of False predictions (False Positive). | 378 | 780 |
| Number of Missed predictions (False Negative). | 460 | 0 |
| Average time difference of true and pred. (ms) | 0 | 0 |
| F1-Score | 0.975 | 0.977 |

Some considerations:

The reason for the zero-time difference between ground truth and predictions is because all true events had a TTT timer started by a measurement in the observation window. In general, two scenarios can happen when one or more of the current RSRP samples fulfils the A3 entering condition and TTT timer starts:

Scenario 1: One of the current RSRP samples fulfil the A3 entering condition and persist for the entire holding/prediction window. In this scenario, the Sample and Hold algorithm predicts the A3 event at the exact time instance as the true event, since the entering condition is met based on the ground truth measurements. In other words, the result of Sample and Hold stays aligned with the true measurements. See the Sample and Hold behavior shown with dashed lines in Figure 1 wherein the measured RSRPs during the TTT time window fulfils the A3 execution condition.

Figure 1: Example of when the neighboring cell A3 entry condition remains fulfilled in the entire holding/prediction window. The Sample and Hold algorithm detect the A3 event precisely (True Positive).

Scenario 2: One of the current RSRP samples fulfil A3 entry condition, but then the neighboring cell fulfills the leave condition in the prediction window. This leads to a false prediction (False Positive) by Sample and Hold algorithm, since the true measurements is not aligned with the predictions. Needless to mention that false predictions are not considered when measuring time difference of true ground truth event and predicted event. See the Sample and Hold behavior shown with dashed lines in Figure 2 wherein the measured RSRPs during the TTT time window does not fulfil the A3 execution condition.

Figure 2: Example of a neighboring cell fulfilling the A3 leaving condition in the prediction window and the sample and hold predictor therefore results in a false event prediction (False Positive).

To summarize, if a true event occurs at the end of TTT, Sample and Hold algorithm accurately captures it, however if the event does not occur (i.e., A3 exit condition is met during TTT), the algorithm detects a false event. In other words, due to the design of the Sample and Hold algorithm, there is no concept of inaccurate prediction for it, i.e. Sample and Hold results in either an accurate prediction, or a false prediction.

To have a fair comparison between the Sample and Hold and the AI/ML based prediction algorithm, in our evaluation for the AI/ML based algorithm, we followed a similar principle i.e., if the event is not predicted at the time that actual event was detected by the UE, it will be classified as a missed prediction (i.e., False Negative).

Based on the above results we have the following observations.

Sample and Hold never misses any true A3 event, but it suffers from high False prediction (False Positive).

For Sample and Hold, False predictions occur since it cannot detect A3 exit condition fulfilment, for 780 samples in our simulation.

For event prediction based on Temporal prediction Case A, the AI model misses more A3 events (due to prediction error) compared to Sample and hold. However, the number of false predictions is lower for the AI-model (as it detects A3 exit condition).

Overall, we think the current simulation and setting result in a minimal error for both algorithms (comparing to the total number of A3 events) and if we would like to show the gain of the AI/ML based algorithm more challenging scenarios by considering measurement error and different UE mobility patterns are needed for evaluation.

The current simulation settings resulted in only 780 instances of the A3 event exit condition being met while TTT was running out of the total 17450 events. This suggests that the simulation setting, and scenario are relatively simple, and more complex scenarios may be required to effectively demonstrate the AI/ML benefits.

In light of the above observations, we have the following proposals

RAN2 discuss evaluation of the event prediction in more challenging scenarios e.g., considering measurement error, and different UE mobility/trajectory patterns.

Moreover, we believe false detection of mobility events could impose higher costs on the network by leading to incorrect mobility decisions, whereas missing a mobility event might be mitigated by detecting it in subsequent predictions. We propose RAN2 consider discussing the cost of the false event prediction versus the cost of missing an A3 event either via system level simulation or by discussing and agreeing on some assumptions.

To be able to determine how good the results are, RAN2 discuss the cost of missed events compared to false events in FR2 Case A.

### Impact of the number of cells in input cluster on the prediction accuracy

In the following we discuss impact of the number of input cells (input cluster size) on the accuracy of the predictions. In order to evaluate the impact of the input cluster size we have trained new models for the different number of input cells. The number of input cells were either 8, 12 or all 21 wherein the clusters with 8, or 12 cells the strongest cells at the time of inference are chosen to predict the strongest RSRP values. In Table 3 results from the different input and output cluster sizes are shown.

Table 3: Results for the study of varying number of input cells.

| Metric | AI-Model, 8 input/output cells | AI-Model, 12 input/output cells | AI-Model, 21 input/output cells |
| --- | --- | --- | --- |
| RSRP difference of whole pred. window (dB) | 2.85 | 2.18 | 0.73 |
| Number of True predictions | 27913 | 34876 | 16535 |
| Number of False predictions | 11337 | 7172 | 378 |
| Number of Missed predictions | 982 | 1868 | 460 |
| F1-Score | 0.820 | 0.885 | 0.975 |

Evaluation shows that the number of cells in the cluster used have significant impact on the performance of event predictions. As shown in Table 3, increasing the number of input cells from 8 to 21 substantially enhances the AI/ML model's performance in terms of reducing the number of False event prediction and the Missed event prediction.

Increasing the number of input cells from 8 to 21 substantially enhances the AI/ML model's performance in terms of number of True predictions, reducing the number of False event prediction and Missed event prediction.

Capture the observation in the TR that more input cells increase performance of the event prediction based on Temporal prediction Case A.

## Case B

Case B is the scenario where a model is used to help the UE do fewer real measurements. The predictions do not extend as far into the future as in Case A instead shorter predictions between real measurements are in focus. An example of the inequality formulas for the event A3 prediction using Case B, is given below:

Entering condition:

Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off

Leaving condition

Mn + Ofn + Ocn + Hys < Mp + Ofp + Ocp + Off

Wherein Mn and Mp indicate respectively the prediction or measurement results of the neighboring and serving cells at the current time instance. When the A3 condition remains fulfilled for a Time-To-Trigger time (TTT) based on the current predictions or measurements, the A3 event is triggered, and UE may trigger a report to the network.

Results and parameter from the Case B study will be presented in this section. In Table 3 the parameters for the AI model are presented.

Table 4: Parameters for the model used in the FR1 Case B simulations.

| Settings / Parameters | Value |
| --- | --- |
| AI model | LSTM |
| Flops | 4 610k |
| Number of parameters | 1 310k |
| Clustering setting | Input: 21 cells, output: 21 cells |
| AI input/output | Cell level L3 RSRP |
| Observation window | 4 samples, 320 ms |
| Filtering option | Sliding |

Results from the test with the baseline parameters are presented in Table 4, the RSRP difference is the average error of the prediction. The prediction window in this case is only one sample.

Table 5: Results from the event predictions in FR1 Case B.

| Metric | AI Model | Sample and Hold |
| --- | --- | --- |
| RSRP difference (dB) | 0.230 | 0.512 |
| Number of True estimates | 2675 | 2610 |
| Number of False estimates | 35 | 88 |
| Number of Missed estimates | 57 | 122 |
| Average time difference of true and pred. (ms) | 15 | 53 |
| F1-Score | 0.98 | 0.96 |

Fewer events happening in FR1 compared to FR2. Therefore, less false and missed events are detected in FR1 in both AI based and non-AI based algorithms.

For event prediction based on temporal prediction Case B, AI model has few missed and falsely classified events, but the difference compared to Sample and Hold is not significant.

The Average time difference of true and pred. for the sample and hold model is not zero here since the scenario we are investigating here is measurement reduction and the evaluation of the A3 event is made on a blend of measurements and predictions. Therefore, the error induced by the predictions may create inaccuracy in predicting the A3 event at the right time e.g., in some cases erroneous prediction values fulfill the A3 condition earlier that the actual A3 event and in some other cases later. Also erroneous predicted values may fulfil A3 leaving conditions so the algorithm fails to catch the A3 event resulting in missed events.

For event prediction based on temporal prediction Case B, the error induced by the predictions create inaccuracy in event prediction resulting non-zero time difference between true and predicted events, wherein the AI model performs better than Sample and Hold.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	Sample and Hold never misses any true A3 event, but it suffers from high False prediction (False Positive).

Observation 2	For Sample and Hold, False predictions occur since it cannot detect A3 exit condition fulfilment, for 780 samples in our simulation.

Observation 3	For event prediction based on Temporal prediction Case A, the AI model misses more A3 events (due to prediction error) compared to Sample and hold. However, the number of false predictions is lower for the AI-model (as it detects A3 exit condition).

Observation 4	The current simulation settings resulted in only 780 instances of the A3 event exit condition being met while TTT was running out of the total 17450 events. This suggests that the simulation setting, and scenario are relatively simple, and more complex scenarios may be required to effectively demonstrate the AI/ML benefits.

Observation 5	Increasing the number of input cells from 8 to 21 substantially enhances the AI/ML model's performance in terms of number of True predictions, reducing the number of False event prediction and Missed event prediction.

Observation 6	Fewer events happening in FR1 compared to FR2. Therefore, less false and missed events are detected in FR1 in both AI based and non-AI based algorithms.

Observation 7	For event prediction based on temporal prediction Case B, AI model has few missed and falsely classified events, but the difference compared to Sample and Hold is not significant.

Observation 8	For event prediction based on temporal prediction Case B, the error induced by the predictions create inaccuracy in event prediction resulting non-zero time difference between true and predicted events, wherein the AI model performs better than Sample and Hold.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 discuss evaluation of the event prediction in more challenging scenarios e.g., considering measurement error, and different UE mobility/trajectory patterns.

Proposal 2	To be able to determine how good the results are, RAN2 discuss the cost of missed events compared to false events in FR2 Case A.

Proposal 3	Capture the observation in the TR that more input cells increase performance of the event prediction based on Temporal prediction Case A.

# References

RP-234055,	SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

RAN2 Chairman’s Notes, RAN2#128

# Annex

During the previous RAN2 meetings, the following agreements have been takes with respect of mobility event predictions:

RAN2#125bis - At least measurement event evaluation based on RRM measurement prediction result will be studied. Direct measurement event prediction is also allowed. - Clarifications on what is being as input should be provided with results. - Start with Event A3 as a baseline. RAN2#127bis - Measurement event prediction simulations will at least focus on intra-frequency FR2, case A, and second study goal (i.e. HO KPI improvement). FFS what is KPI. - Companies can bring simulation results for intra-frequency measurement reduction for FR1 and report what they are doing. Focus on temporal case B. - Companies will prioritize simulations on indirect method. Companies can bring simulations on direct method and should report what method is being used. - The Simulation assumption of RRM measurement prediction can be reused unless otherwise specified. - Companies can pick and report what they are using for filtering options (similar to RRM prediction) - Companies will focus on sub-case 2 for measurement event prediction. Companies can simulate other sub-cases if they wish and report what they are using. - Leave the simulation parameter discussion for email discussion. Pick only one value for A3. - It is up to company implementation how to model UE behavior after A3 event is trigger. Focus on intermediate KPIs for this exercise. System level KPI is FFFs - For measurement event prediction, traffic is not simulated. - For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (ex. event A3). - For direct measurement event prediction, the model output is the probability of event occurrence within a time window. - A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT). - As baseline, we will use RLF event prediction KPI: - for indirect: F1 score. The following can be reported: RSRP difference, missed event detection, false event detection. FFS how to define F1 score. - time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction. - for direct: F1 score. The following can be reported: missed event detection, false event detection,

In RAN2#128 meeting, additional agreements on the simulation assumptions have been taken for the indirect event prediction based on the temporal prediction Case A and B.

- Agreed values for Case A. NOTE1 indirect prediction only Parameters; baseline value; Note. A3 event offset (db); 2; Open for 3db. TTT (ms); 320; Open for one shorter value. UE speed (km/h); 90; Open for 60 and 120km/h. OW length (ms, note1); N/A; Up to implementation. PW length (ms, note1); 400; Open for more values. Max ETD (ms, note1); 80; Open for more values. - Agreed values for Case B. NOTE1 indirect prediction only Parameters; baseline value; Note. A3 event offset (db); 2; Open for 3db. TTT (ms); 320; Open for one shorter value. UE speed (km/h); 30; Open for 60 and 90km/h. OW length (ms,note1); N/A; Up to implementation. PW length (ms,note1); 200 (non-sliding) 40ms (sliding); Open for more values. Max ETD (ms, note1); 80; Open for more values. MRRT; 50%; Open for more values. - Company can report which filtering options is being used for the input L3 RSRP of sub-use case 2: option 1, option 2, option 3.