3GPP TSG-RAN WG2 #129	R2-2500920

Athens, Greece, 17th – 21st February 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on AIoT general aspects

Document for:	Discussion

Agenda Item:	8.2.4

- Introduction

The study for Ambient IoT solutions is concluded in RAN#104 and the discussion for normative phase begins in this meeting. We would like to discuss general (and fundamental) aspects of A-IoT AS layer to kick off the Stage-3 discussion.

- Discussion

  - Multiplexing in A-IoT MAC layer

This chapter discusses the relationship of the A-IoT MAC layer to the physical layer.

In the NR MAC [1], a MAC SDU coming down from the upper layer (RLC) or a control signal generated in the MAC layer (MAC CE) is added with a MAC subheader to be a MAC subPDU. Multiple MAC subPDUs generated in this way are concatenated and assigned a padding bit to align them with the transport block size in the physical layer.

We believe that a similar mechanism can be reused in the A-IoT MAC layer. In other words, as shown in the Figure 1, upper layer data coming down from A-IoT NAS or a control signal generated in the A-IoT MAC layer (e.g., A-IoT paging message, A-IoT Msg1, Msg2, etc.) can be added with an A-IoT MAC subheader to be an A-IoT MAC subPDU. Although whether there is a need to concatenate multiple A-IoT MAC subPDUs depends on future discussion, padding bits can be added to one subPDU or multiple concatenated subPDUs to align them to the transport block size in the physical layer.

Figure 1. Structure of A-IoT MAC PDU.

This will more or less clarify the points to be discussed by RAN2 in the future: for example, the formats of the A-IoT subheader, the formats of control signals generated in the A-IoT MAC layer such as A-IoT paging message, Msg1, Msg2, etc, and the functionalities of the A-IoT MAC layer e.g., random access procedure and segmentation.

Proposal 1.	In A-IoT MAC layer, upper layer data and control element generated in A-IoT MAC layer can be added with A-IoT subheader to be A-IoT MAC subPDU.

Proposal 2.	Discuss whether multiple A-IoT MAC subPDUs should be multiplexed for a TB.

Proposal 3.	When AIoT MAC subPDUs cannot fill a TB, padding subPDU is included.

  - A-IoT MAC subheaders

In the NR MAC [1], the MAC subheader for a fixed length MAC payload and the MAC subheader for a flexible length MAC payload are defined separately, as shown in figures below. In addition, a separate MAC subheader is defined for a MAC payload with a particularly long MAC payload among the MAC payloads with a flexible length.

Figure 2. NR MAC subheaders (cited from [1]).

In the A-IoT MAC layer, the length of payload is considered as follows.

- The length of the A-IoT MAC SDU (i.e., upper layer data) is flexible. Depending on future discussion, other messages may also be of flexible length, for example, A-IoT Msg2 for 3-step CBRA may be targeted to multiple devices simultaneously.

- Depending on future discussion, there may be fixed-length messages, for example, A-IoT Msg1 for 3-step CBRA may contain only a 16-bit random ID.

Therefore, in the A-IoT MAC layer, the A-IoT MAC subheader for fixed length A-IoT MAC payloads and the A-IoT MAC subheader for flexible length A-IoT MAC payloads should be defined separately.

Proposal 4.	Separate A-IoT MAC subheaders for fixed length subPDU and flexible length subPDU are introduced.

    - A-IoT MAC subheader for flexible length subPDU

Similar to the discussion above, we aim to define the A-IoT MAC subheader based on the NR MAC subheader as the baseline. Table 1 shows our view on each field included in the NR MAC subheader and how these would be reused in the A-IoT MAC subheader.

Table 1. Fields in NR MAC subheader.

| Field | Purpose in NR MAC subheader | Expectation in A-IoT MAC subheader |
| --- | --- | --- |
| R | Reserved bit. | - |
| F | 1 bit. Indicates length of L field (1 byte or 2 bytes). | Since it is agreed that the maximum TB size of an Ambient IoT system is about 1000 bits (*1), 1 byte of L field (length of payload is up to 256 bytes) is sufficient. |
| L | 1 or 2 bytes. Indicates the length of payload. | 1 byte is sufficient as mentioned above, but may be shorter, i.e., 7 bits (payload is up to 128 bytes). |
| LCID | 6 bits. Indicates logical channel ID for MAC SDU or type of MAC CE. | Since it has been agreed that the AIoT system will support only one logical channel for data (*2), and since limited number of messages would be defined, there is room to shorten LCID. |
| eLCID | 1 or 2 bytes. Extends LCID field. | No need to extend LCID. |
| LX | 1 bit. Extends LCID field without increasing total subheader size. | No need to extend LCID. |
| *1 TR 38.796 6.3.5 [2] - In both R2D and D2R directions, there is no lower bound on the minimum TB size, and a maximum TB size of around 1000 bits can be supported. |  |  |
| *2 TR 38.796 6.3.2 [2] As to the A-IoT required functionalities, at least the following functionalities are NOT supported (see TS 38.300 [153] for references for any legacy NR functionality): … - Multiple A-IoT logical channels for upper layer data |  |  |

As mentioned in Table 1, considering that the TB size in the A-IoT system is expected to be around 1000 bits at most, there is no need to extend the L field from 1 byte to 2 bytes. Therefore, we believe that the F field is unnecessary in the A-IoT MAC subheader.

Proposal 5.	F field for NR MAC subheader is not reused for A-IoT MAC, i.e., A-IoT MAC subheader has a fixed size of L field.

Regarding the LCID, considering that there is a high possibility of multiple types of control signals being generated in the A-IoT MAC sublayer, it would be beneficial to have a field with a similar role to LCID in the A-IoT MAC sublayer. However, since multiple logical channels are not supported for data from the upper layer and the types of control signals generated in the A-IoT MAC layer are expected to be significantly fewer compared to the NR MAC, we would like to discuss the possibility of reducing the number of bits required for the LCID field to less than 6 bits. Additionally, since multiple logical channels are not supported for data from the upper layer, it might not be appropriate to literally call this field the logical channel ID, so we would like to discuss the naming as well.

Proposal 6.	LCID field for NR MAC subheader reused is for A-IoT MAC. Length of LCID field is FFS. How to call the field (other than LCID) is FFS.

As mentioned above, 6 bits are sufficient for the LCID, and the possibility of LCID exhaustion is low. Therefore, there is no need to consider reusing the eLCID field and LX field in the A-IoT MAC layer.

Proposal 7.	eLCID field for NR MAC subheader is not reused for A-IoT MAC.

Proposal 8.	LX field for NR MAC subheader is not reused for A-IoT MAC.

By the way, we understand that it is FFS whether AS ID for the purpose of D2R scheduling and R2D reception is indicated in physical layer or A-IoT MAC layer. Basically RAN2 should wait for RAN1 on this aspect. If the AS ID is concluded to be delivered in A-IoT MAC layer, we believe that A-IoT MAC subheader is a good place to indicate it. The length of the AS ID is yet to be determined, but it needs to be allocated in such a way that there is no overlap among devices within the range of a single reader's coverage. Therefore, it is necessary to ensure a length that can accommodate several hundred to several thousand IDs (the specific value needs a discussion). In other words, the AS ID should have a length of preferably 12 bits (4096 IDs) or more.

Proposal 9.	If AS ID is concluded to be delivered in A-IoT MAC layer, introduce AS ID field in A-IoT MAC subheader.

Proposal 10.	The size of AS ID field is not less than 12 bits.

    - A-IoT MAC subheader for fixed length subPDU

The format of A-IoT MAC subheader for fixed length payload is sufficient if the information regarding the payload length, i.e., the L field, is omitted from the flexible-length A-IoT MAC subheader.

Proposal 11.	For A-IoT MAC subheader for fixed length subPDU, just omit L field from the subheader for flexible length subPDU.

  - AS ID

It is expected to introduce the AS ID attached in higher layer or in physical layer for the purpose of D2R scheduling and R2D reception. TR 38.796 captures three options as follows. It is still FFS whether the AS ID is generated by the device or the reader. Additionally, it is also FFS whether the reader adopts a random ID as the AS ID or generates the AS ID itself.

TR 38.796 6.3.5 [2] - Option 1: a random ID (if used in first D2R message) can be reused; - Option 2: the reader assigns this "AS ID" to the device. It needs to be further discussed via which R2D message. - Option 3: It is up to the reader whether to reuse the random ID (if used in first D2R message) as the "AS ID" or to assign a new "AS ID". It needs to be further discussed via which R2D message.

For Option 1, since the device randomly reserve its AS ID, it is inevitable to collide to another device within the coverage of the same reader. On the other hand, Option 2 allows the reader to assign AS ID without collision because the reader can make sure that the AS ID is unique within all devices that is in distance to communicate with an A-IoT reader. Moreover, since there is no possibility to collide, we can introduce shorter AS ID, which helps every message transmission after random access considering the short TB size in AIoT system.

The reader should assign AS ID to the device via the first R2D message dedicated to the device as depicted in Figure 1, i.e., Msg2 in 3-step CBRA, success indication in 2-step CBRA and CFRA.

Figure 3. First R2D messages dedicated to a device.

Proposal 12.	AS ID is generated by A-IoT reader.

Proposal 13.	AS ID is indicated to each A-IoT device via Msg2 in 3-step CBRA, success indication in 2-step CBRA and CFRA.

Another aspect is when the device discards AS ID. The device needs to retain the AS ID as long as it might receive R2D messages. However, the shorter the device retains the AS ID, the better for operation. This is because the longer the device retains the AS ID, the higher the possibility of collision with AS IDs obtained by other devices (if the device generates the AS ID) or the higher the possibility of AS ID exhaustion (if the reader generates the AS ID).

Figure 4. When the device (and reader) can discard AS ID?

Therefore, it would be beneficial to discuss a mechanism that allows the device and reader to discard the AS ID at the earliest possible timing.

Proposal 14.	RAN2 discuss how to minimize a length of time in which an AS ID is allocated to a device.

  - Disable command

In TR for SA2 study [3], in addition to inventory, read, and write, a service called disable is also envisioned to be supported in A-IoT. Since this service has not been discussed very well in RAN2 so far, we would like to consider it in this chapter.

TR 23.700-13 [3] The following Ambient IoT services are agreed to be supported by the 5GC, which apply to both topology 1 and topology 2: - Inventory: Request to perform an inventory operation. - Read: Request to read information from an AIoT Device. - Write: Request to write information to an AIoT Device. - Disable: Request that an AIoT Device has its capability to transmit RF permanently disabled.

Our understanding is that a device receiving a disable command will send an ACK message to the NW reporting the completion of the disable process and then transition to the disabled state. This is the same behavior as when the device receives a write command. Therefore, from the RAN2 perspective, there is no need to discuss dedicated AS messages or procedures for the disable command.

Proposal 15.	RAN2 does not assume any dedicated AS message or procedure for disable command unless SA2 requires.

- Summary and proposal

Proposal 1.	In A-IoT MAC layer, upper layer data and control element generated in A-IoT MAC layer can be added with A-IoT subheader to be A-IoT MAC subPDU.

Proposal 2.	Discuss whether multiple A-IoT MAC subPDUs should be multiplexed for a TB.

Proposal 3.	When AIoT MAC subPDUs cannot fill a TB, padding subPDU is included.

Proposal 4.	Separate A-IoT MAC subheaders for fixed length subPDU and flexible length subPDU are introduced.

Proposal 5.	F field for NR MAC subheader is not reused for A-IoT MAC, i.e., A-IoT MAC subheader has a fixed size of L field.

Proposal 6.	LCID field for NR MAC subheader reused is for A-IoT MAC. Length of LCID field is FFS. How to call the field (other than LCID) is FFS.

Proposal 7.	eLCID field for NR MAC subheader is not reused for A-IoT MAC.

Proposal 8.	LX field for NR MAC subheader is not reused for A-IoT MAC.

Proposal 9.	If AS ID is concluded to be delivered in A-IoT MAC layer, introduce AS ID field in A-IoT MAC subheader.

Proposal 10.	The size of AS ID field is not less than 12 bits.

Proposal 11.	For A-IoT MAC subheader for fixed length subPDU, just omit L field from the subheader for flexible length subPDU.

Proposal 12.	AS ID is generated by A-IoT reader.

Proposal 13.	AS ID is indicated to each A-IoT device via Msg2 in 3-step CBRA, success indication in 2-step CBRA and CFRA.

Proposal 14.	RAN2 discuss how to minimize a length of time in which an AS ID is allocated to a device.

Proposal 15.	RAN2 does not assume any dedicated AS message or procedure for disable command unless SA2 requires.

- References

[1]	TS 38.321, “NR; Medium Access Control (MAC) protocol specification,” V18.4.0.

[2]	TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR,” V19.0.0.

[3]	TR 23.700-13, “Study on Architecture support of Ambient power-enabled Internet of Things,” V1.3.0.