3GPP TSG RAN WG2 #129	R2-2500922

Athens, Greece, February 17 – 21, 2025

Agenda Item:	8.9.2
Source:	Toyota ITC
Title:	Discussion on Paging and Mode Switching

Document for:	Discussion and Decision

# Introduction

The RAN plenary has approved a work item for Internet of Things (IoT) Non-Terrestrial Networks (NTN) [1]. One area of focus is the store-and-forward operation, where data can be transmitted to a satellite for storage if the feeder link is unavailable. The stored data can then be offloaded to a ground station once the feeder link is restored. This document discusses the paging and mode switching aspects of this feature.

# Discussion

## Paging Monitoring

The store-and-forward feature allows a satellite to function in real-time mode for two-way connections or in store-and-forward mode for one-way traffic from IoT devices to satellite storage. Several contributions have highlighted the energy consumption associated with monitoring paging channels when satellites switch to store-and-forward operating mode. In the absence of incoming mobile-terminated traffic, it is suggested that User Equipment (UE) should not monitor these channels in idle mode.

This contribution aims to address this issue by evaluating energy efficiency solutions while considering the reliability of mobile-terminated data delivery.

Solution A

In [2], it is suggested that when no mobile-terminated data exists for an area, the UE should be notified to save energy by not monitoring paging.

The advantage of this solution is its simplicity, requiring only notification to all UEs. This approach is also network-controlled, which aids in ensuring the reliability of mobile-terminated data delivery.

This method's disadvantage is its reliance on the area. If any mobile-terminated data exists, all UEs within the same area should wait for the data to be delivered to all UEs before ceasing to monitor paging. In addition, multiple transmission attempts may be required for successful data reception to a single UE, causing delays for everyone. A satellite's wide coverage with a large population of UEs further reduces this solution's efficiency.

Solution B

In [3], it is proposed that the last time the feeder link was available should be provided to the UE for deciding whether to monitor paging.

This solution's advantage is its independence from other UEs because the decision is made locally.

The disadvantage of this solution is the dependency between data arrival and feeder link availability. Data might arrive just before the feeder link becomes unavailable, causing uncertainty in the UE's paging monitoring duration. These local UE implementation-specific decisions affect both the solution's energy efficiency and data delivery reliability. If the UEs cease monitoring paging too early, the network may persist in paging unresponsive UEs, leading to resource wastage. Additionally, the UEs may miss mobile-terminated data.

Solution C

To address energy consumption from paging monitoring, the solution should be independent of other UEs, standardized, and controlled by the network. This ensures both reliable data delivery and energy efficiency through configuration. It should be noted that mobile terminated data may experience delays due to the feeder link, leading to data arriving after the mode switch. This situation can cause paging message transmission to a paging occasion to occur post mode switch, resulting in data loss, resource inefficiencies, and increased latency. The solution requires the UE monitoring N paging occasions after entering store-and-forward mode, as configured by the network. The feeder link delay can be compensated by appropriately setting the value of N.

The advantage of this solution is its configurability, making data delivery more reliable than Solution B where the decisions are local, while still providing energy efficient paging monitoring.

The disadvantage is that signaling N needs slightly more bits than indicating presence of mobile-terminated data.

Standardizing Solution C is recommended due to its benefits, including improved energy efficiency and reliable mobile-terminated data delivery.

Proposal 1: The UE is required to monitor N paging occasions after switching from real-time to store-and-forward mode, with N configured by the network.

## Mode Switching

Another important aspect to consider is the impact of switching operating modes. Transitioning to store-and-forward mode halts all real-time traffic, while transitioning back to real-time mode re-enables it. For the real-time UEs, this process may lead to an abrupt surge of traffic, particularly on contention-based resources, due to simultaneous access by many UEs.

One possible solution is access class barring that temporarily blocks some UEs to manage traffic load until the system can handle the load. This mechanism gradually adjusts parameters and eventually grants access to all users as soon as possible. However, the main issue is the potential simultaneous access of many UEs caused by the mode switching rather than the traffic load itself, so other mechanisms should be considered. Additionally, access class barring operates at a very slow pace due to its reliance on a network-driven control-loop mechanism.

To manage real-time UE access upon mode switching, a randomization method could be used. After switching to real-time mode, users would delay their access attempts by drawing a random number within a transmission window and transmitting it when the timer expires. This approach aims to spread out access attempts, mitigating mode switching impacts. If traffic loads become high, access class barring may still be necessary which means that these mechanisms are not mutually exclusive.

Proposal 2: Standardize a signaling mechanism to avoid traffic surges caused by simultaneous access when transitioning from store-and-forward to real-time operating mode.

# Conclusion

This contribution discusses uplink capacity enhancements. Observation and proposals are summarized as follows:

Proposal 1: The UE is required to monitor N paging occasions after switching from real-time to store-and-forward mode, with N configured by the network.

Proposal 2: Standardize a signaling mechanism to avoid traffic surges caused by simultaneous access when transitioning from store-and-forward to real-time operating mode.

# Reference

- RP-240776, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, March 18-21, 2024.

- R2-2408108, “Further consideration on Store and Forward”, Huawei, HiSilicon, Turkcell, China Southern Power Grid, RAN2#127bis

- R2-2408282, “Discussion on the Store and Forward satellite operation”, HONOR