3GPP TSG-RAN Meeting #129	R2-2500924

# Athens, Greece, 17th – 21st February 2025

Agenda Item:	8.7.3

Source:	Ericsson

Title:	RRM Measurement Gap/Restriction Enhancements

Document for:	Discussion, Decision

# 1	Introduction

The enhancement to the measurement gap/restrictions is to minimize its effect on XR the data rate. To this end, RAN1 (being the leading WG) has extensively discussed multiple options and finally landed on the DCI-based skipping solution. Although this was a working assumption pending on RAN4’s analysis, based on their LS reply, RAN4 has confirmed the feasibility of the DCI-based skipping solution and provided some timelines for the cancellation of a measurement gap.

In RAN2, it was agreed that we focus on the solution chosen by RAN1/RAN4 i.e., DCI-based skipping. Thus, we do not see the need to discuss any other solution. In this contribution, we discuss the stage-3 impact of the DCI-based skipping solution and provide our analysis.

# 2	Discussion

## 2.1 	Stage-3 Impacts of Dynamic Skipping Indication Solution

## 5.14 Handling of measurement gaps During an activated measurement gap, except a measurement gap(s) which the lower layer has indicated to be skipped, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]: 1> not perform the transmission of HARQ feedback, SR, and CSI; 1> not report SRS; 1> not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2; 1> if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running, or if there is an ongoing RACH-less LTM cell switch, or if there is an ongoing RACH-less handover: 2> monitor the PDCCH as specified in clauses 5.1.4, 5.1.5, and 5.7. 1> else: 2> not monitor the PDCCH; 2> not receive on DL-SCH.

Although the skipping indication itself is in the scheduling DCI, the MAC layer handles the measurement gaps (MG). The MAC [1] specification captures the procedures a UE need not perform during a MG and hence, the specification needs to be updated if a MG is skipped. Thus, the lower layers should inform the MAC when it receives a scheduling DCI with the skipping indication for a MG.

Lower layers indicate to the MAC if a measurement gap occasion is skipped.

## 2.2 	Measurement Gap Enhancement Solutions

There have been extensive discussions on all possible alternatives to avoid scheduling restriction including the DCI and RRC-based solution. From the previous RAN1 meetings, discussions concluded with the following working assumption that DCI based dynamic solution (Alt 1-1) is agreed to be supported.

Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s). Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Final LS in R1-2407561.

- RAN1 agreement selected the DCI based solution instead of RRC based solution.

Given that the RAN4’s LS response has already confirmed the feasibility of the DCI-based solution, it is not necessary to perform further evaluations. RAN2 should follow RAN1’s suggestion to consider only the DCI-based solution, as it was also agreed in the last meeting that RAN2 will focus its work on supporting the solution chosen from RAN1/RAN4. Hence, RAN2 should not consider other solutions.

Follow and focus only on RAN1’s explicit indication by DCI to skip a particular gap/restriction solution. i.e., DCI based skipping solution.

In addition to the semi-static solution, the other solution discussed was autonomous skipping where the UE can itself choose to skip a particular MG occasion. Companies had the understanding that this approach is applicable to the cases where the configured grant (CG) occasions overlap with an MG occasion. For CG, the baseline assumption is that network configuration can already ensure that there is either no overlap between CG and MG or the overlap can be kept to a minimum such that the XR data rate is unaffected.

- For CG, network configuration can ensure there is no or minimum overlap between CG and MG.

If a UE does want to use a CG occasion overlapping with the MG, companies claim it is most likely due to an urgent requirement to send UL data or buffer status report (BSR) for new traffic arrival. So, a necessary and sufficient condition is that there is the arrival of high priority data in the UE buffer. However, in an autonomous approach, the gNB is unaware of the data arrival at the UE buffer and this cannot be reported to the gNB before the CG occasion which overlaps with the MG. From the gNB’s point of view, if there is no information on the skipping of the MG. Then, normal behaviour is that CG is skipped, and hence does not perform blind decoding during that MG/CG overlap occasion. Thereby, resulting in a lost UL transmission. Although performing more blind decoding can help recover some of these transmissions, such unspecified behaviour also leads to new error cases like false positive detections. For example, the gNB can falsely detected that the UE transmitted on a CG within an MG triggering further unnecessary and wasted transmissions.

- Autonomous UE skipping of MG for overlapping CG occasions results in lost UL transmissions and false detections since gNB assumes there is no transmission during a MG. This approach would also lead to an unacceptable increase of the blind decodings at the gNB.

Figure 1: Timeline for MG skipping

Further, considering the timelines from RAN4’s LS, we see that the cancellation of the MG requires 5 ms with the normal timeline and 3 ms with the aggressive timeline when the DCI-based skipping indication is used. Assuming that the processing time of the DCI itself is ~ 1 ms, the rest of the time is required for the cancellation of the MG. Therefore, as shown in Figure 1, for any data arriving within this timeline cannot be served by the overlapping CG occasion because there is not enough time to cancel the MG. For any data arriving in the purple marked region shown above, there might not be sufficient time for a DCI-based approach or even with autonomous skipping to cancel the MG. But there are easy ways to handle this situation for e.g., the gNB can always configure another CG occasion or send a (pro-active) dynamic grant to cover the data arrival in the purple region. The nature of this data arrival period ~1 ms is however a corner case and thus need not be optimized.

- Such data arrival periods of ~1ms are corner cases and need not optimize for them.

Lastly, if the gNB wants to use the CG resources which overlaps with a MG, it can always proactively send a dynamic grant with DCI indicating skipping the corresponding gap and overriding the CG occasion.

- DCI based solution can override a CG occasion overlapping with measurement gap to ensure required uplink transmissions.

No other solutions are needed for the MG enhancements.

## 2.3 	UAI

On UE assistance information related to UE mobility e.g., whether the UE is static, this information could potentially be useful for the network in deciding the measurement gap configuration/skipping occasions.

- UE mobility information could potentially be useful to configure measurement gaps.

We could also simply reuse the framework of the relaxed measurement criterion for a stationary (e)RedCap UE. In the current procedure, the network provides an RRM-MeasRelaxationReportingConfig for determination of stationarity and the UE can report its rrm-MeasRelaxationFulfilment status in the UAI on whether the stationary criterion is met. The other advantage of reusing the relaxed measurement criterion for a stationary UE is that there are no corresponding RAN4 requirements.

Consider reusing the RRM measurement relaxation criterion for a stationary (e)RedCap UE as framework for providing the UE mobility information in the UAI.

# 3 	Conclusion

Based on the discussion in the previous sections, we have the following observations:

Observation 1	RAN1 agreement selected the DCI based solution instead of RRC based solution.

Observation 2	For CG, network configuration can ensure there is no or minimum overlap between CG and MG.

Observation 3	Autonomous UE skipping of MG for overlapping CG occasions results in lost UL transmissions and false detections since gNB assumes there is no transmission during a MG. This approach would also lead to an unacceptable increase of the blind decodings at the gNB.

Observation 4	Such data arrival periods of ~1ms are corner cases and need not optimize for them.

Observation 5	DCI based solution can override a CG occasion overlapping with measurement gap to ensure required uplink transmissions.

Observation 6	UE mobility information could potentially be useful to configure measurement gaps.

The observations lead to the following proposals:

Proposal 1	Lower layers indicate to the MAC if a measurement gap occasion is skipped.

Proposal 2	Follow and focus only on RAN1’s explicit indication by DCI to skip a particular gap/restriction solution. i.e., DCI based skipping solution.

Proposal 3	No other solutions are needed for the MG enhancements.

Proposal 4	Consider reusing the RRM measurement relaxation criterion for a stationary (e)RedCap UE as framework for providing the UE mobility information in the UAI.

# 4 	References

- TS 38.214, 5G; NR; Medium Access Control (MAC) Protocol Specification, V18.4.0 (2024-12)

# 5	Annex

The following are the agreements from RAN2#127bis on measurement gap enhancements.

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution.

Further, the following agreements were taken in the RAN2#128

Agreements on RRM measurement gaps impacts - No MG-specific enhancements is needed on DSR operation. - RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped - No MG-specific enhancements is needed for DRX operation.

Furthermore, in the LS response from RAN4 to RAN1 with RAN2 in Cc is as follows:

RAN4 discussed the above RAN1 working assumption and confirms that Alt 1-1 is feasible from RAN4 perspective with X=5ms as baseline, and X=3ms with optional UE capability. It is also RAN4 understanding that the time offset is the time from the end of the last symbol of the PDCCH carrying the DCI to the start of the corresponding skipped measurement gap indicated by the DCI, where the start of the measurement gap is determined after applying Measurement Gap Timing Advance (mgta) configured by the network according to TS 38.331. Furthermore, from RAN4 RRM requirements perspective, Alt 1-1 is not supported for EN-DC and NE-DC scenarios. Lastly, in addition to Alt 1-1, RAN4 is also discussing the need to introduce semi-static indication based solutions to skip measurement gap occasions.