3GPP TSG-RAN WG2 Meeting #129	R2-2500925

# Athens, Greece, 17th – 21st February 2025

Agenda Item:	8.7.5

Source:	Ericsson

Title:	Even More Discussions on RLC AM Enhancements

Document for:	Discussion, Decision

# 1	Introduction

In the last meeting, on the topic of avoiding unnecessary RLC retransmissions, for the combined Tx-Rx approach, it as agreed that a new RLC timer is introduced at the Rx-side to identify obsolete RLC SDU(s). This timer is started when there is a gap detected at the RLC layer. It was also agreed that there is no clear understanding on how any indication from the Tx-side would help to solve the issue that cannot already be solved with the local timer and that unless a critical issue is identified, no such indication will be introduced. In this contribution, we get into the specification details for the combined Tx-Rx approach with stage-3 details for the new RLC timer, Tx-side discard mechanism and more.

On the topic of timely RLC retransmissions, the agreement was that both polling enhancements and autonomous retransmissions shall be pursued. But from the agreement it is unclear if both solutions can be configured independently or if there is a unified solution and depending on network configuration either one can be used. In our understanding, this comes down to the triggers used for the two solutions. We believe and as detailed below a common trigger in the remaining time to discard can be used as the framework for a unified solution. Of course, it is still possible to configure either one of the two solutions.

# 2	Discussion

## 2.1 	Unnecessary RLC Retransmission Avoidance

### 2.1.1	Specification Changes for the Combined Tx-Rx Approach

Based on the TP for the combined Tx-Rx based approach as shown in Section 5, we discuss the specifications details of this approach and other stage-3 aspects.

#### 2.1.1.1	Rx-Side Changes

It was already agreed in the last meeting that a new RLC timer is started when a gap is detected at the RLC layer, we propose that the new timer is named t-WinAdvance.

The name t-WinAdvance is to be used for the new RLC timer.

From a specification standpoint, a gap is created when a higher RLC SN than RX_Next has been delivered to the upper layers. Currently, RX_Next_Highest is used to ‘hold the value of the SN following the SN of the RLC SDU with the highest SN among received SDUs’ i.e., it holds the SN which is one higher than the RLC SN submitted to the upper layers. Then, we see that if RX_Next + 1 is less than RX_Next_Highest, this creates a gap indicating that at least one RLC SDU with a SN > RX_Next has been delivered to the upper layer. Thus, this condition should trigger the start of t-WinAdvance.

t-WinAdvance is started when RX_Next_Highest > RX_Next + 1.

The stopping condition for t-WinAdvance is when the gap does not exist, but RX_Next_Highest which triggered the timer to start is constantly updated as and when subsequent RLC SNs are received. Thus, comparing RX_Next with RX_Next_Highest always results in a gap unless there are no missing RLC SNs in between them hence, even if the pending RLC SN is received, the t-WinAdvance is not stopped resulting in error case.  As a result, we propose to introduce a new state variable which holds the previous value of RX_Next_Highest that triggered the t-WinAdvance and propose to name it RX_Win_Advance.

Introduce a new RLC state variable named RX_Win_Advance which holds the value of the SN following the SN of the RLC SDU which triggered t-WinAdvance.

The stopping condition is when the RLC SN gap no longer exists i.e., RX_Next is equal to RX_Win_Advance.

t-WinAdvance can be stopped/reset when the gap no longer exists i.e., RX_Win_Advance is equal to RX_Next.

Upon expiry of t-WinAdvance, RX_Next will be updated to the next RLC SN greater than or equal to RX_Win_Advance yet to be received and RX_Win_Advance will be equal to RX_Next_Highest.

Upon expiry of t-WinAdvance, RX_Next is updated to the first RLC SN >= RX_Win_Advance, which is yet to be received and RX_Win_Advance is updated to RX_Next_Highest.

From an ASN1 perspective, the UE can be configured with the t-WinAdvance timer as part of the DL-AM-RLC IE.

t-WinAdvance can be configured as part of the DL-AM-RLC IE.

#### 2.1.2.2	Tx-Side Changes

The Tx RLC entity stops transmission of outdated SDUs based on the indication from the upper layer e.g., PDCP even for those RLC SDUs or segments thereof have been submitted to the lower layers. As there are no changes to the indication itself, it is possible to reuse the existing or legacy indication.

Reuse the existing/legacy discard indication from the upper layer (e.g., PDCP) to RLC.

Although the indication is the same, as the discard behaviour for Rel-19 is different from legacy, the two cases should be differentiated. This can be done by introducing a new parameter in the UL-AM-RLC IE namely, rlc-pdudiscard. When this parameter is configured, the Tx entity can follow the new behaviour.

Introduce a new parameter rlc-pdudiscard in the UL-AM-RLC IE. When configured, the Tx RLC side can stop transmission of outdated SDUs based on the upper layer indication even if the RLC SDUs or segments thereof have been submitted to the lower layers.

## 2.2 	Timely RLC Retransmissions

The views were split among companies for which solution is used to address the timely RLC retransmission issue. One group of companies wanted to rule out autonomous retransmissions owing to its effect on system capacity whilst pursuing only the polling enhancements. The other group of companies wanted to specify both solutions and leaving it to the network to configure one or the other. Further, as a compromise, a solution was also discussed where the two solutions are combined as follows:

If the remaining time drops below a threshold, poll is triggered. If SR is not received within a time, then Tx can autonomously retransmit the delay critical packet.

As also described in the introduction, the current agreement is unclear in terms of whether a unified framework is used for both solutions or that each solution is designed independently. This largely depends on the proposed triggers for the two solutions as analysed below.

### 2.2.1	Triggers for Polling Enhancements and Autonomous Retransmissions

In order to better understand the autonomous retransmission solution and its interaction with the polling enhancements, we first study the proposed triggers:

- Configured number of failed HARQ retransmissions

- Remaining time of an RLC PDU less than a threshold

Configured number of failed HARQ retransmissions

For (A), the proponents believe that the network can configure a threshold of failed HARQ retransmissions beyond which, the UE can perform an autonomous RLC retransmission i.e., stop the associated HARQ process early and trigger RLC retransmission. However, the early stopping of the HARQ process can already be achieved in the following ways:

- gNB can just configure lesser number of overall HARQ retransmissions. The outcome is the same as if the (lower number of) overall HARQ retransmissions fail, RLC retransmissions are triggered earlier.

- gNB schedules HARQ retransmissions using the NDI bit:

  - If in a scheduling DCI, the NDI bit remains the same (i.e., not flipped), then the gNB implies that the previous HARQ attempt was unsuccessful, and the UE should continue HARQ retransmissions for this process. But if the NDI bit was flipped, then gNB implies that the previous HARQ transmission was successful, and the UE can flush its buffer related to this HARQ process.

  - Thus, if the gNB realizes that subsequent HARQ retransmissions for an RLC PDU will be unsuccessful, it can send a scheduling DCI with the NDI bit flipped (i.e., provide a fake ACK for a HARQ retransmission), thereby flushing the HARQ buffer for this PDU. Then, the gNB sends an RLC status report with the corresponding PDU marked as NACK thereby triggering an early RLC retransmission. Essentially, send a fake ACK for HARQ to trigger early RLC retransmission.

  - The gNB can also always trigger an RLC retransmission if the RLC status report is sent before the HARQ retransmissions are complete.

As the gNB is aware of the HARQ information there are ample ways using legacy/current specifications to trigger an early RLC retransmission. There is no gain to achieve in latency with autonomous retransmissions considering HARQ.

- There are ample ways using legacy mechanism to trigger an early RLC retransmission considering HARQ, there is no gain to achieve in latency.

Thus, we propose to not consider HARQ as a trigger for autonomous retransmission.

Do not pursue failed number of HARQ retransmissions as a trigger for autonomous retransmission.

Remaining time of an RLC PDU less than a threshold

In (B), considering that the gNB is not aware of the timing information of an RLC PDU at the UE, it would certainly help if autonomous retransmission were triggered based on the remaining time of the PDU.

- It would certainly help if autonomous retransmissions were triggered based on the remaining time threshold of an RLC PDU.

The remaining time threshold of the PDU is also applicable to the polling mechanism as the RLC status reports can be sent either too frequently (too early) or sparsely (too late). Frequent status reports are not always necessary because not all the RLC PDUs are delay critical at a given time i.e., overheads in transmission. On the other hand, it cannot be too sparse (too late) because this would result in the RLC PDU to be discarded if received too late. Essentially, the current polling mechanism might not always be synchronized with the delay requirements of the AMD PDU as the gNB is unaware of the remaining time to discard.

- The current polling mechanism cannot consider the delay criticality of the RLC PDU in the buffer, it is a trade-off between transmission overhead and timely reception of the RLC status PDU.

As a result, such enhancements to the polling mechanism could consider the delay criticality of the AMD PDU in the buffer. The advantages are that the polling is set only when required and it helps the Rx entity to send the RLC status report in a timely manner.

Consider the remaining time threshold of an RLC PDU as a trigger for both autonomous retransmissions and enhanced polling mechanism.

However, for the autonomous retransmission to be relevant, it is essential that the remaining time threshold allows for the HARQ retransmissions to complete. For example, with a BLER target of 10%, the success rate of the initial transmission is 90% and with subsequent HARQ retransmissions, the success rate is 99% making the autonomous retransmissions relevant for 1% of failure cases. Thus, it is natural to combine the autonomous retransmission with the polling enhancement as proposed in [1]. The procedure of this combined approach is as shown:

Figure 1: Combined polling and autonomous retransmission

As shown in Figure 1, when the remaining time is less than the threshold_polling, it triggers the polling bit to be set. Further, when the remaining time is less than the threshold_retx, UE can trigger an autonomous retransmission if an RLC status report was not received. Such a mechanism allows completion of the HARQ retransmissions avoiding the case when an autonomous retransmission is triggered even though the RLC PDU was successfully received at the gNB.

- In allowing the HARQ retransmissions to complete, avoids the case when an autonomous retransmission is triggered even though the RLC PDU was successfully received at the gNB.

In Figure 1, the two timers are only shown for the purposes of illustration, these can also be combined into one timer. This also gives us the flexibility to configure the UE with either the polling enhancement or autonomous retransmission by either setting threshold_polling or threshold_retx to zero. Hence, we propose:

For autonomous retransmission and polling enhancements considering the remaining time threshold of a PDU:

  - Configure two thresholds to trigger the enhanced polling mechanism and autonomous retransmissions respectively.

  - Configure a single timer to monitor the remaining time of an RLC PDU for the enhanced polling mechanism and autonomous retransmission. Or configure two timers respectively.

  - By setting one of the two thresholds to zero, the gNB can configure either the enhanced polling mechanism or autonomous retransmission.

### 2.2.2	Additional Triggers for Autonomous Retransmissions

Considering the impact on UL capacity, we believe that cell-edge users with bad radio channel conditions should not be allowed to perform autonomous retransmissions as the RLC PDU undergoes a lot of segmentation resulting in a 2X consumption of the UL resources per UE. For example, if segmented into 4 PDUs, it results in eight times the UL resources. In addition to segmentation, the link adaptation for such users is also quite robust and there is no need for autonomous retransmissions.

Network can configure the UEs to perform autonomous retransmission based on signal strength.

## 2.3	UE Capability

If time permits, RAN2 should also discuss the UE capability for the combined Tx-Rx approach. It is appropriate that the UE support for this approach is based on a new optional UE capability. But the combined Tx-Rx approach involves two sets of features, one, where the Tx side stops transmissions of an outdated SDU and two, where the Rx side abandons the outdated SDUs based on a local timer. Bundling the two features into a single capability forces the vendors to implement both features thereby delaying implementation and time to market.

- For UE capability, independent features should not be bundled into a single capability.

In general, features independent of each other as a matter of principle should not be bundled. Hence, we propose that RAN2 defines two capabilities to cover the Tx-side and Rx-side features allowing vendors to stagger their implementation of this feature resulting in faster time to market.

Define two UE capabilities for the combined Tx-Rx approach as follows:

  - Tx-side UE capability: Tx side stops the transmissions of an outdated SDUs, including new related procedures for e.g., RLF.

  - Rx-side UE capability: Rx side abandons the outdated SDU based on a new RLC timer, abandoned SDUs are positively acknowledged in the RLC STATUS report.

# 3 	Conclusion

Based on the discussion in the previous sections, we have the following observations:

Observation 1	There are ample ways using legacy mechanism to trigger an early RLC retransmission considering HARQ, there is no gain to achieve in latency.

Observation 2	It would certainly help if autonomous retransmissions were triggered based on the remaining time threshold of an RLC PDU.

Observation 3	The current polling mechanism cannot consider the delay criticality of the RLC PDU in the buffer, it is a trade-off between transmission overhead and timely reception of the RLC status PDU.

Observation 4	In allowing the HARQ retransmissions to complete, avoids the case when an autonomous retransmission is triggered even though the RLC PDU was successfully received at the gNB.

Observation 5	For UE capability, independent features should not be bundled into a single capability.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	The name t-WinAdvance is to be used for the new RLC timer.

Proposal 2	t-WinAdvance is started when RX_Next_Highest > RX_Next + 1.

Proposal 3	Introduce a new RLC state variable named RX_Win_Advance which holds the value of the SN following the SN of the RLC SDU which triggered t-WinAdvance.

Proposal 4	t-WinAdvance can be stopped/reset when the gap no longer exists i.e., RX_Win_Advance is equal to RX_Next.

Proposal 5	Upon expiry of t-WinAdvance, RX_Next is updated to the first RLC SN >= RX_Win_Advance, which is yet to be received and RX_Win_Advance is updated to RX_Next_Highest.

Proposal 6	t-WinAdvance can be configured as part of the DL-AM-RLC IE.

Proposal 7	Reuse the existing/legacy discard indication from the upper layer (e.g., PDCP) to RLC.

Proposal 8	Introduce a new parameter rlc-pdudiscard in the UL-AM-RLC IE. When configured, the Tx RLC side can stop transmission of outdated SDUs based on the upper layer indication even if the RLC SDUs or segments thereof have been submitted to the lower layers.

Proposal 9	Do not pursue failed number of HARQ retransmissions as a trigger for autonomous retransmission.

Proposal 10	Consider the remaining time threshold of an RLC PDU as a trigger for both autonomous retransmissions and enhanced polling mechanism.

Proposal 11	For autonomous retransmission and polling enhancements considering the remaining time threshold of a PDU:

a.	Configure two thresholds to trigger the enhanced polling mechanism and autonomous retransmissions respectively.

b.	Configure a single timer to monitor the remaining time of an RLC PDU for the enhanced polling mechanism and autonomous retransmission. Or configure two timers respectively.

c.	By setting one of the two thresholds to zero, the gNB can configure either the enhanced polling mechanism or autonomous retransmission.

Proposal 12	Network can configure the UEs to perform autonomous retransmission based on signal strength.

Proposal 13	Define two UE capabilities for the combined Tx-Rx approach as follows:

a.	Tx-side UE capability: Tx side stops the transmissions of an outdated SDUs, including new related procedures for e.g., RLF.

b.	Rx-side UE capability: Rx side abandons the outdated SDU based on a new RLC timer, abandoned SDUs are positively acknowledged in the RLC STATUS report.

# 4 	References

- R2-2409958, Discussion on Fast RLC Retransmission, Apple, Rel-19, November 18-22, 2024, Orlando, USA.

# TP – 38.323

START OF CHANGES

#### 5.2.3.2	Receive operations

##### 5.2.3.2.1	General

The receiving side of an AM RLC entity shall maintain a receiving window according to the state variable RX_Next as follows:

-	a SN falls within the receiving window if RX_Next <= SN < RX_Next + AM_Window_Size;

-	a SN falls outside of the receiving window otherwise.

When receiving an AMD PDU from lower layer, the receiving side of an AM RLC entity shall:

-	either discard the received AMD PDU or place it in the reception buffer (see clause 5.2.3.2.2);

-	if the received AMD PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly as needed (see clause 5.2.3.2.3).

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

-	update state variables and start t-Reassembly as needed (see clause 5.2.3.2.4).

##### 5.2.3.2.2	Actions when an AMD PDU is received from lower layer

When an AMD PDU is received from lower layer, where the AMD PDU contains byte segment numbers y to z of an RLC SDU with SN = x, the receiving side of an AM RLC entity shall:

-	if x falls outside of the receiving window; or

-	if byte segment numbers y to z of the RLC SDU with SN = x have been received before:

-	discard the received AMD PDU.

-	else:

-	place the received AMD PDU in the reception buffer;

-	if some byte segments of the RLC SDU contained in the AMD PDU have been received before:

-	discard the duplicate byte segments.

##### 5.2.3.2.3	Actions when an AMD PDU is placed in the reception buffer

When an AMD PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall:

-	if x >= RX_Next_Highest:

-	update RX_Next_Highest to x+ 1.

-	if all bytes of the RLC SDU with SN = x are received:

-	reassemble the RLC SDU from AMD PDU(s) with SN = x, remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer;

-	if x = RX_Highest_Status:

-	update RX_Highest_Status to the SN of the first RLC SDU with SN > current RX_Highest_Status for which not all bytes have been received.

-	if x = RX_Next:

-	update RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received.

-	if t-Reassembly is running:

-	if RX_Next_Status_Trigger = RX_Next; or

-	if RX_Next_Status_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or

-	if RX_Next_Status_Trigger falls outside of the receiving window and RX_Next_Status_Trigger is not equal to RX_Next + AM_Window_Size:

-	stop and reset t-Reassembly.

-	if t-Reassembly is not running (includes the case t-Reassembly is stopped due to actions above):

-	if RX_Next_Highest> RX_Next +1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.

-	if t-WinAdvance is running:

-   if RX_Win_Advance = RX_Next; or

-	if RX_Win_Advance = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	if RX_Win_Advance falls outside the receiving window and RX_Win_Advance is not equal to RX_Next + AM_Window_Size:

-	stop and reset t-WinAdvance.

-	if t-WinAdvance is not running (includes the case t-WinAdvance is stopped due to actions above):

-   if RX_Next_Highest > RX_Next + 1; or

-	if RX_Next_Highest = RX_Next + 1 and at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU and at least one RLC SDU has been delivered to upper layer with SN > RX_NEXT:

-	start t-WinAdvance.

-	set RX_Win_Advance to RX_Next_Highest.

##### 5.2.3.2.4	Actions when t-Reassembly expires

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

-	update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Status_Trigger for which not all bytes have been received;

-	if RX_Next_Highest> RX_Highest_Status +1: or

-	if RX_Next_Highest = RX_Highest_Status + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Highest_Status before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.

NEXT CHANGE

##### 5.2.3.2.4	Actions when t-WinAdvance expires

When t-WinAdvance expires, the receiving side of an AM RLC entity shall:

-	update RX_Next to the SN of the first RLC SDU with SN > RX_Win_Advance for which not all bytes have been received:

-	discard all bytes with SN < RX_Win_Advance;

-	if RX_Next_Highest > RX_Next +1; or

-   if RX_Next_Highest > RX_Next + 1 and at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU and at least one RLC SDU has been delivered to upper layer with SN > RX_NEXT:

-	start t-WinAdvance.

-	set RX_Win_Advance to RX_Next_Highest.

NEXT CHANGE

### 5.3.4	Status reporting

An AM RLC entity sends STATUS PDUs to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC SDUs (or portions of them).

Triggers to initiate STATUS reporting include:

-	Polling from its peer AM RLC entity:

-	When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall:

-	if the AMD PDU is to be discarded as specified in clause 5.2.3.2.2; or

-	if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size:

-	trigger a STATUS report.

-	else:

-	delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size.

NOTE 1:	This ensures that the RLC Status report is transmitted after HARQ reordering.

-	Detection of reception failure of an AMD PDU

-	The receiving side of an AM RLC entity shall trigger a STATUS report when t-Reassembly expires.

NOTE 2:	The expiry of t-Reassembly triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated.

When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall:

-	if t-StatusProhibit is not running:

-	at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and submit it to lower layer.

-	else:

-	at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and submit it to lower layer.

When a STATUS PDU has been submitted to lower layer, the receiving side of an AM RLC entity shall:

-	start t-StatusProhibit.

When constructing a STATUS PDU, the AM RLC entity shall:

-	for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer:

-	for an RLC SDU for which no byte segments have been received yet:

-	include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU.

-	for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN, SOstart and SOend.

-	for a continuous sequence of RLC SDUs that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN and NACK range;

-	include in the STATUS PDU, if required, a pair of SOstart and SOend.

-	set the ACK_SN to the SN of the next not received RLC SDU or the skipped SN from expiry of t-WinAdvance which is not indicated as missing in the resulting STATUS PDU.

NEXT CHANGE

## 5.4	SDU discard procedures

When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting entity shall:

-	if rlc-pdudiscard is configured:

-	the transmitting side of an AM RLC entity shall discard the indicated RLC SDU even if the RLC SDU or a segment thereof has been submitted to the lower layers.

-	if rlc-pdudiscard is not configured:

-	the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

END OF CHANGES

# Annex

The following were the agreements from the RLC AM enhancement discussions in RAN2#128:

Agreements on unnecessary RLC retransmissions avoidance - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced - Special handling to avoid PDCP control PDU discard is not needed. - A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. - The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report. Agreements on timely RLC retransmissions - Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.