3GPP TSG-RAN WG2 Meeting #129	R2-2500926

# Athens, Greece, 17th – 21st February 2025

Agenda Item:	8.7.6

Source:	Ericsson

Title:	Even More on XR Rate Control

Document for:	Discussion, Decision

# 1	Introduction

The discussion on the QoS flow vs DRB level granularity to perform XR rate control indication for the UL was dragging on for some time until it was agreed in the last meeting that the indication is per-QoS flow. Since it was agreed in the plenary that the MAC-layer indication is used to perform this rate control indication, this brings up the question on the exposure of QoS flow level information at such lower layers. Although strictly speaking, this is a violation of the protocol stack separation, companies claim that this should be the solution. Such exposure of QoS flow information at the MAC layer also has significant cross WG impacts, and an LS was sent to RAN3, SA4 and SA2 (in CC) to evaluate it. Thus, we believe this solution should not be discussed in RAN2 until the LS reply is received.

A much simpler yet effective solution, not violating the protocol stack separation and which has no cross WG impact, as proposed by us was to preconfigure the QoS flows candidates (via RRC) which are to be rate adapted then, reuse the recommended bit rate MAC CE per-DRB level indication to perform the rate control indication. We understand that a lot of the details of this solution were not available at the time, hence, in this contribution, we provide more information and insights into this solution.

# 2	Discussion

Agreement Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

From the agreement above, the rate indication to the UE on a per QoS flow level can be achieved in two ways:

- Flows are indicated by MAC CE

- By RRC while MAC CE is per DRB (RRC-based solution)

For 1), due to cross WG impacts, an LS [1] was sent to SA4/RAN3 (with SA2 in CC) hence, RAN2 should wait for the LS response before discussing this option.

- Due to cross WG impacts, RAN2 to wait for the LS response on [1] before discussing the option where QoS flows are indicated by MAC CE.

## 2.1 	RRC-Based Solution (RRC, with MAC CE per DRB)

For the RRC-based solution, we understand that a lot of the details were not present at the time of discussion, hence, the following describes the RRC-based solution.

How does the RRC-based solution work?

Given that the network is aware of which QoS flow (or QFI) needs to be adapted, the network can (pre)configure at the UE (via DL RRC signaling), the set of QoS flows that are candidates for being rate adapted. Then, upon reception of the recommended bit rate (RBR) MAC CE although indicating on a per DRB level, the UE performs rate adaption on the candidate QoS flows previously configured via RRC.

For example, if QoS flows A and B are mapped to the same DRB, the gNB configures QoS flow A to be a candidate for rate adaptation, but not QoS flow B. If/when the UE later gets the MAC CE, the UE rate adapts QoS flow A.

Is there a restriction that only one QoS flow of the DRB (with multiple QoS flows mapped) can be rate adapted?

No, there is no such restriction. It is up to network implementation how many candidate QoS flows are indicated in the DL RRC message.

How does the UE perform rate adaption across the configured QoS flows using the RBR MAC CE?

One option is that we reuse the legacy procedure and leave this to UE implementation. The other option is that in addition to the QoS flow IDs (QFIs) as indicated in the DL RRC message, the network can also include the ratio of the bit rates that need to be rate adapted across the configured QoS flows.

Thus, we propose the following for an RRC-based solution:

gNB can configure one or more QoS flows (mapped to the same DRB) to be rate adapted.

Reuse the recommended bit rate MAC CE to indicate the desired bit rate in the UL.

For rate adaptation of more than one QoS flow within a DRB, consider two options:

  - Up to UE implementation how to perform rate adaption between the configured QoS flows.

  - gNB indicates the ratio of rate adaption between the QoS flows as part of the configuration in P1.

It is clear from the description above that the RRC-based solution has no cross WG impacts and, also does not require QoS related information to be available on the MAC layer, which is a violation of the current principle of protocol stack separation. The RRC-based solution also achieves the required rate control on the QoS flow level without any hassles. Thus, we propose that this solution is used to perform rate indication from gNB to the UE on a per QoS flow level.

RRC-based, while MAC CE is per DRB is used to perform XR rate control indication from gNB to the UE on a per QoS flow level.

## 2.2	Indication of Rate-Adaptable QoS Flows to gNB

Agreement gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN

Another agreement made in the last meeting is related to how the gNB knows which QoS flows can be rate adapted. At least for the L4S mechanism, as specified by SA2 in Rel-18, the gNB is already aware if a DRB is carrying L4S-enabled traffic based on the information provided by CN.

- In L4S, the gNB is aware if a DRB is carrying L4S-enabled traffic based on information provided by the CN.

The same is also applicable to any application where the CN can obtain information about the rate adaptability, for example:

- CN obtains information on whether the application is VoLTE/VoNR and is thus aware of the rate-adaptability. This information is then passed on to the gNB.

- For a GBR QoS flow when the MFBR > GFBR, this is an indication that the application can be rate-adapted.

- The provision of an alternate QoS profile (AQP) is also an indication that the application is rate adaptable.

Hence, the CN has ample ways to figure out the rate adaptability of the application and this information can be passed on to the gNB. Thus, it is sufficient to rely on the CN to indicate if a QoS flow can be throttled.

CN can indicate to the gNB which QoS flows can be throttled.

## 2.3	Granularity and Range for XR Rate Control

As captured in the LS reply from SA4 [2], the range for an XR application can vary between 100 kbps and 40 Mbps. For specific applications like XR split rendering and cloud gaming, the UL data rate is 200 kbps and for AR conversational, it is 20 Mbps. Further on granularity, the RAN4 LS reply mentions that from an application perspective finer granularity e.g., 1% is preferred but coarser granularities e.g., 10% is also sufficient.

- SA4 reply LS points out that although 1% granularity is preferred, coarser granularities for e.g., 10% is also sufficient.

The recommended bit rate MAC CE has a bit rate index value of 6-bits i.e., 64 indices with 8 reserved values, resulting in 56 usable indices with the range between 9 kbps and 8 Mbps. In addition, with the multiplier values, the maximum bit rate value that can be indicated is 1.6 Gbps (with 200 as the multiplier value). Thus, this covers the required range of bit rates.

- The required range of bit rates as indicated in the SA4 LS is covered by the existing recommended bit rate values.

However, the current bit rates do not cover the 1% or 10% granularity as indicated. To obtain such granularities, a new table would need to be defined considering the range of 100 kbps and 40 Mbps. This brings up the question on whether the current recommended bit rate MAC CE format can be reused as only up to 64 indices can be indicated and maybe this is not sufficient to cover the range of bit rates. The format of this MAC CE should be such that it keeps the overhead (in terms of #indices) low whilst still covering the required range.

- The format of such a rate control MAC CE should be such that it keeps the overhead (in terms of #indices) low whilst still covering the required range.

Considering the 10% granularity and the range between 100 kbps and 40 Mbps, this can be covered with 64 indices assuming none of them are reserved, enabling the reuse of the current 6-bit bit rate index value. But considering the 1% granularity, the number of indices required to cover the whole range would be prohibitively large with a high signaling overhead and unnecessary given that the RAN4 LS mentions that a coarser granularity is also sufficient.

- With a 10% granularity, the range of 100 kbps to 40 Mbps can be covered with 64 indices. But with a 1% granularity, the number of indices required to cover the whole range is prohibitively large and unnecessary given that the SA4 LS mentions a coarser granularity is sufficient.

As a result, we propose that RAN2 only defines a new table for the bit rate field considering the 10% granularity and thus reuse the 6-bit bit rate index design.

Define a new bit rate table with 10% granularity for the ranges between 100 kbps and 40 Mbps.

Re-use the legacy 6-bit rate index design.

# 3 	Conclusion

Based on the discussion in the previous sections, we have the following observations:

Observation 1	Due to cross WG impacts, RAN2 to wait for the LS response on [1] before discussing the option where QoS flows are indicated by MAC CE.

Observation 2	In L4S, the gNB is aware if a DRB is carrying L4S-enabled traffic based on information provided by the CN.

Observation 3	SA4 reply LS points out that although 1% granularity is preferred, coarser granularities for e.g., 10% is also sufficient.

Observation 4	The required range of bit rates as indicated in the SA4 LS is covered by the existing recommended bit rate values.

Observation 5	The format of such a rate control MAC CE should be such that it keeps the overhead (in terms of #indices) low whilst still covering the required range.

Observation 6	With a 10% granularity, the range of 100 kbps to 40 Mbps can be covered with 64 indices. But with a 1% granularity, the number of indices required to cover the whole range is prohibitively large and unnecessary given that the SA4 LS mentions a coarser granularity is sufficient.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	gNB can configure one or more QoS flows (mapped to the same DRB) to be rate adapted.

Proposal 2	Reuse the recommended bit rate MAC CE to indicate the desired bit rate in the UL.

Proposal 3	For rate adaptation of more than one QoS flow within a DRB, consider two options:

a.	 Up to UE implementation how to perform rate adaption between the configured QoS flows.

b.	 gNB indicates the ratio of rate adaption between the QoS flows as part of the configuration in P1.

Proposal 4	RRC-based, while MAC CE is per DRB is used to perform XR rate control indication from gNB to the UE on a per QoS flow level.

Proposal 5	CN can indicate to the gNB which QoS flows can be throttled.

Proposal 6	Define a new bit rate table with 10% granularity for the ranges between 100 kbps and 40 Mbps.

Proposal 7	Re-use the legacy 6-bit rate index design.

# 4 	References

- R2-2411218, LS on XR UL bit rate control, 3GPP RAN2#128, November 18-22, 2024, Orlando, USA.

- S4-241803, Reply to LS on appropriate range and granularity of bit rate adaptation for XR applications, 3GPP SA4#120, November 18-22, 2024, Orlando, USA.

# 5	Annex

The following are the agreements from the last meeting on the XR rate control objective:

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.