3GPP TSG-RAN WG2 #129	R2-2500927

Athens, Greece, 17th – 21st February 2024

Agenda Item:	8.13.4

Source:	Ericsson

Title:	Intra-gNB Service Continuity for Multi-Hop Relays

Document for:	Discussion, Decision

# 1 	Introduction

The list of agreements made on service continuity and related aspects on the control plane procedure are in the Annex. An essential agreement which directly affects the service continuity objective was to support a baseline procedure where the intermediate relays are all transitioned to RRC_CONNECTED (if not already there). However, another agreement was made to continue discussions on whether/how to support intermediate relay UEs not in RRC_CONNECTED.

In this contribution, we discuss the behaviour for the first/last/intermediate relay UEs in terms of the measurement events and their reconfiguration for service continuity assuming the baseline procedure that the intermediate relay UEs are in RRC_CONNECTED state.

# 2 	Discussion

It was agreed in RAN plenary that service continuity scenarios that would be prioritized are:

- Intra-gNB multi-hop indirect to direct path switch (mh2d)

- Intra-gNB multi-hop indirect to single hop indirect path switch (mh2i)

For (A), it was already agreed in the last meeting that Events X1 (serving U2N Relay UE becomes worse than threshold1 and NR Cell becomes better than threshold2) and X2 (serving L2 U2N Relay UE becomes worse than threshold) are reused with the understanding that the first relay UE is the serving L2 U2N Relay UE.

There were also discussions on whether the signal strength information of the intermediate relay UEs should be considered to trigger a path switch to a direct path. Ideally, only the remote UE is configured with events that triggers a path switch. But if there is an issue with the links of the intermediate relay UEs, if this cannot be detectable, it would eventually result in an RLF. This is then notified downstream to reach the remote UE which then triggers reestablishment. This of course affects the QoE performance of a multi-hop link and probably helps to have early detection. In this case, RAN2 should first consider whether/if such events can be configured for these intermediate relay UEs.

For example, only event X2 can be configured considering that the relay UE upstream or downstream is considered the serving L2 U2N relay UE. In addition, considering that all the intermediate relay UEs are in the RRC_CONNECTED state, each of these relay UEs can be configured to send the measurement report directly to the gNB, it is then up to the network to take subsequent action. The same is also applicable for (B).

For both mh2d and mh2i path switch, confirm that the current specification allows the first/intermediate/last relay UEs to be configured with event X2 in a multi-hop indirect path to address service continuity.

Based on the two flow diagrams for as described in [1], for both (A) and (B), the gNB needs to reconfigure the first, intermediate and last relay UE once the path switch command is sent to the remote UE. It is up to network implementation when the relay UE(s) are configured.

For both mh2d and mh2i, it is up to network implementation when the first/last/intermediate relay UEs are reconfigured after the path switch command is sent to the remote UE.

# 3 	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	For both mh2d and mh2i path switch, confirm that the current specification allows the first/intermediate/last relay UEs to be configured with event X2 in a multi-hop indirect path to address service continuity.

Proposal 2	For both mh2d and mh2i, it is up to network implementation when the first/last/intermediate relay UEs are reconfigured after the path switch command is sent to the remote UE.

# 4	References

[1] R2-2410589, Discussion on service continuity for Multi-hop Relay, Huawei, HiSilicon, Rel-19, November 18-22, 2024, Orlando, USA.

# 5	Annex

The following are the agreements from RAN2#128 on the aspect of service continuity:

Agreements:

For scenario B, RAN2 supports the case that the target U2N relay UE is a new relay UE which is not on the source relay path, and existing Rel-18 indirect path to indirect path switching can be reused.

For scenario B, RAN2 will support the case that the target single-hop relay UE is the last relay UE on the source path using the existing Rel-18 i2i path switching.  Spec impact will be minimised.

For scenario B, RAN2 does not specify anything to support the case that that the target U2N relay UE is an intermediate relay UE which is on the source relay path using a single procedure.  This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the target intermediate relay UE.

The intra-gNB switching from indirect to direct path procedure defined in TS 38.300 for single hop case can be used as the baseline for multi-hop indirect to direct path switching.

Event X1 /X2 can be reused in multi-hop indirect to direct path switching with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

The intra-gNB switching from indirect path to indirect path procedure defined in TS 38.300 in single hop case can be used as baseline for multi-hop indirect to single hop indirect path switching procedure.

The following measurement events can be reused in multi-hop indirect to single-hop indirect path switching:

- Event Y2

- Event Z1 with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

Figure 1 of R2-2410589 can be used as a baseline for intra-gNB multi-hop indirect to direct path switching procedure to be captured in TS 38.300.

Figure 2 of R2-2410589 can be used as a baseline for intra-gNB multi-hop indirect to single-hop indirect path switching to be captured in TS 38.300.

In addition, the following agreements were made in the CP procedure regarding the RRC state of the intermediate relay UEs:

Agreement:

Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED.

1. Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell.

2. Continue to discuss whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment.

All agreements in this WI apply to both these cases unless otherwise specified.