3GPP TSG-RAN WG2 Meeting #129	R2-2500928

# Athens, Greece, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.323 | CR | 0146 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PDCP SN Gap Report – Triggering Condition and Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2024-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Issue-1: R2-2406598 Remaining Open Issues for PDCP SN Gap Reporting Ericsson discussion Rel-18 Proposal 1 Update the triggering conditions text to include DC and CA by adding “RLC entities” in the PDCP SN gap reporting procedure. Adopt the TP as shown in the Annex. - Nokia thinks that we can say “any”. LG and Apple think that RLC is already clear. Ericsson and Intel thinks that it is just making it consistent with other changes. => The change can be added when we have a PDCP CR as an editorial => Noted It was agreed in RAN#127 that the change to cover the case for DC and CA can be added when there is a PDCP CR and thus provided. Issue-2: The issue is related to the triggering and reporting mechanism of the PDCP SN gap report as currently specified. But as detailed in R2-2409852, there is an ambiguity in setting the FDC. Two options have been captured in R2-2409852: - Option 1: The discarded PDCP SDU included in the PDCP SN gap report should not include the PDCP SDU(s) which have been delivered to lower layer by RLC. - Option 2: The discarded PDCP SDU included in the PDCP SN gap report can include in the PDCP SDU(s) which have been delivered to lower layer by RLC. There was also an offline discussion in RAN2#128 to discuss the SN gap reporting as follows: *[AT128][010][XR] SN gap reporting (CATT) Intended outcome: agree to option and CR implementing the agreable option Deadline: Thursday The outcome of the offline/online was to postpone this issue to the next meeting. The correct approach is that the PDCP SN gap report should not include the PDCP SDU(s) which have been delivered to the lower layers by RLC i.e., Option 1 (as shown above). Option 2 (include PDCP PDUs already submitted to the lower layers) creates a race condition at the PDCP Rx entity between the reception of the SN gap report and the corresponding PDCP PDU. This leads to two cases: - If the SN gap report is received first, RX_DELIV is updated to a SN beyond the PDCP PDU yet to be received. Then, upon reception of the PDCP PDU, the PDCP Rx entity discards it. - If the PDCP PDU is received first, RX_DELIV is updated to the next missing COUNT value. Then, if a SN gap report is received including the SN of the successfully received PDCP PDU then, RX_DELIV is updated based on the SN gap report. In case 1, for traffic without PDU sets, this is not the desired behavior as this leads to early discarding of a valid PDCP PDU. For traffic with PDU sets, the assumption is that all PDUs belonging to a PDU set are irrelevant to the application even if one of the PDUs is discarded. However, it is in general better for the application to receive all possible PDUs even if they are delayed beyond the time budget as different applications react in different ways. In addition, there is no need to optimize for this case where a subset of the PDUs in a PDU set have already been transmitted but the rest are discarded. Given that Tx entity has already initiated transmissions/retransmissions, from an implementation standpoint it is better to exclude from the gap report those SNs already delivered to the lower layers. In addition, it is likely that case 1 occurs more often than case 2 because to keep the reordering times to a minimum, the SN gap reporting is prioritized. Thus, exacerbating the early discard issue. As a result, it is best to avoid such race conditions, corner cases and instead not include only the PDCP SDU(s) which have already been delivered to the lower layers by RLC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 5.16.1 - Include the phrase ‘any RLC entity’ to cover the CA/DC case. - Clarify that the SN gap report cannot include the discarded PDCP SDU(s) which have been submitted to the lower layers by RLC. Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: PDCP SN gap report Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there will be no inter-operability issues. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Leads to a race condition at the PDCP Rx entity between the reception of the valid PDCP PDU and the SN gap report. Depending on the outcome of the race condition, could lead to early discarding of a valid PDCP PDU. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.16.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

< Unmodified Parts Omitted >

## 5.16	SN gap report

### 5.16.1	Transmit operation

For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when:

-	the PDCP SDU(s) are discarded as specified in clause 5.3; and

-	there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and

-	the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers.

If a PDCP SN gap report is triggered for the discarded PDCP SDU(s) described above, the transmitting PDCP entity shall:

-	compile a PDCP SN gap report as indicated below by:

-	setting the FDC field to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s);

-	if more than one of the PDCP SDUs are discarded:

-	allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first discarded PDCP SDU up to and including the last discarded PDCP SDU, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first;

-	setting in the discard bitmap field as '0' for all PDCP SDUs that have not been discarded;

-	setting in the discard bitmap field as '1' for the PDCP SDUs that have been discarded;

-	submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface.

NOTE:	It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.

< Unmodified Parts Omitted >

END OF CHANGE