3GPP TSG-RAN WG2 #129	Tdoc R2-2500932

Athens, Greece, February 17 – 21, 2025

Agenda Item:	8.19

Source:	Ericsson

Title:	On UE capability for FDD-FDD inter-band CA simultaneous RxTx

Document for:	Discussion, Decision

# 1	Introduction

An LS was received from RAN4 (R4-2420410) on UE capability for FDD-FDD inter-band CA simultaneous RxTx. This contribution discusses the RAN4 LS and proposes a way forward.

# 2	Discussion

RAN4 intends to define a band combination (CA_n5-n8) where the specific band pair n5+n8 has overlapping bands and thus simultaneous (n5UL, n8DL) is supported but simultaneous (n8UL, n5DL) is not. In our understanding, such band combination is not used today and thus there should be no backwards compatibility issue.

No backwards compatibility issue is expected since the band combination (CA_n5-n8) is not currently used.

Moreover, we understand that such limitation should be present for all band combinations defined in the future in the same conditions as the band combination (CA_n5-n8). In other words, there would be no case where a UE supports simultaneous UL and DL on such overlapping bands.

Therefore, it should be sufficient to rely on the NW implementation to apply such restriction once configuring the UE with RAN4 defined FDD-FDD band combinations where simultaneous UL and DL is not possible. This, however, can only be done if such limitation is confined within special cases of FDD-FDD band combinations with overlapping bands – i.e. this limitation cannot be applicable to other existing band combinations that currently have no limitation, otherwise this could cause backwards compatibility issues. We should also inform RAN4 about the RAN2 understanding to avoid backwards compatibility issues in the future.

Inform RAN4 that enforcing non-simultaneous UL and DL to existing FDD-FDD band combinations would cause backwards compatibility issues, and thus it is expected that such restriction is limited to band combinations defined onwards and not applicable to already existing band combinations.

FDD-FDD band combinations where simultaneous UL and DL is not possible due to overlapping bands can be handled by NW implementation, as long as this restriction is limited to band combinations defined onwards and not applicable to already existing band combinations.

The above is also based on the assumption that RAN4 in the future will only define single band pairs with two FDD-FDD bands where simultaneous transmission is not possible. If any other arrangement would be discussed (e.g. 3 FDD bands where simultaneous transmission is not possible), then RAN2 would have to rediscuss the signaling options for this case, including the scenario of a single FDD-FDD band pair.

RAN2 assumes that, if there are other cases defined in the future, they would also be a single FDD-FDD band pair. If there is an intention of any different combination, then RAN2 may need to discuss it more on the signaling solutions for this non-simultaneous transmission (including the case of a single FDD-FDD band pair).

Lastly, we expect the restriction on non-simultaneous UL and DL to be captured in RAN4 since RAN2 will not define a new capability under the conditions above.

The restriction on non-simultaneous UL and DL for CA_n5-n8 is expected to be captured in RAN4 specifications.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	No backwards compatibility issue is expected since the band combination (CA_n5-n8) is not currently used.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Inform RAN4 that enforcing non-simultaneous UL and DL to existing FDD-FDD band combinations would cause backwards compatibility issues, and thus it is expected that such restriction is limited to band combinations defined onwards and not applicable to aleady existing band combinations.

Proposal 2	FDD-FDD band combinations where simultaneous UL and DL is not possible due to overlapping bands can be handled by NW implementation, as long as this restriction is limited to band combinations defined onwards and not applicable to already existing band combinations.

Proposal 3	RAN2 assumes that, if there are other cases defined in the future, they would also be a single FDD-FDD band pair. If there is an intention of any different combination, then RAN2 may need to discuss it more on the signaling solutions for this non-simultaneous transmission (including the case of a single FDD-FDD band pair).

Proposal 4	The restriction on non-simultaneous UL and DL for CA_n5-n8 is expected to be captured in RAN4 specifications.