3GPP TSG-RAN WG2 Meeting #129	R2-2500936

Athens, GR, February 17-21, 2025

Agenda Item:	8.17

Source:	THALES

Title:	Discussion of adaptions to support IoT-NTN TDD

Document for:	Discussion and Decision

# 1	Introduction

It is beneficial for IoT NTN market deployment to support TDD mode. The IoT-NTN TDD mode Work Item approved in RAN#105 in September was revised in RAN#106 [1] following a study phase in RAN1. It states the following objectives for RAN2:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period. RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2]

The RAN1 study phase agreements can be found in the WI status report [2].

The system parameters to consider in the normative phase are the followings:

| Parameters | Values/Comments |
| --- | --- |
| Orbit | LEO at 600 and 1200 km |
| Set in 3GPP TR 36.763 | Set-1 |
| Targeted band | 1616-1626.5 MHz |
| NB-IoT mode | Standalone with anchor and non-anchor carriers |
| Cell type | Earth-mixed and Earth-moving cells |
| Baseline system to design structure and procedures | NB-IoT NTN FDD |
| TDD pattern | Periodic pattern of N=9 radio frame constituted of U=8 non-overlapping set of usable contiguous UL subframe and D=8 set of usable contiguous DL subframe separated by guard periods |
| TDD pattern configuration | Fixed for the targeted band |

In this contribution, we present the definition of TDD pattern as discussed in RAN1, and the possible adaptions and associated UE procedures related to the new IoT-NTN TDD pattern:

- Description in Stage-2

- Definition of the TDD pattern

- NPBCH scheduling (MIB-NB)

- NPDCCH scheduling (DCI)

- SIB1-NB and other SIB scheduling in IoT-NTN TDD mode

- Paging and related DRX features

- NPRACH

- k-Mac extension

# 2	Discussion

## 2.1	Description in stage 2

It is the very first time we introduce with this work item the TDD mode to IoT-NTN. Therefore, the stage 2 specification needs to be updated to take into account the TDD of NB-IoT NTN system. Especially in [10]-§5, it is proposed to support IoT-NTN TDD pattern:

Downlink and uplink transmissions are organized into radio frames with 10 ms duration. Three radio frame structures are supported:

- Type 1, applicable to FDD and IoT-NTN TDD;

- Type 2, applicable to TDD;

- Type 3, applicable to LAA secondary cell operation only.

[…]

Uplink and downlink transmissions are separated in the frequency domain except for IoT-NTN TDD.

in [10]-§23.21.1, the following sentence is not yet relevant:

“In this release of the specification, NTN is only applicable to FDD system”

It is proposed to update it as follows:

“In this release of the specification, NTN is only applicable to FDD system. TDD can be supported for NB-IoT UEs with IoT-NTN TDD as described in §23.21.X”

Where the IoT-TDD mode pattern and adaptations, if any, are described in a new section 23.21.X.

### 23.21.X TDD mode for NB-IoT NTN The TDD mode is applicable for NB-IoT UEs over NTN. It is based on NB-IoT NTN FDD frame structure and procedures. The TDD pattern follows a periodic cycle of 9 radio frames wherein 8 non-overlapping downlink and 8 uplink subframes are usable, separated by a guard time. The pattern is fixed per band and applicable for both anchor and non-anchor carriers.

Update the stage 2 specification to support and describe the NB-IoT NTN TDD mode. The text proposal in Annex can be considered as a baseline.

## 2.2	Definition of the TDD pattern

As per the WID [1], new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames (RF). The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and a set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9.

The IoT-NTN TDD Pattern is discussed in RAN1 [2][3]. An example of configuration is given in the Figure below:

Figure 1 - Definition of the periodic TDD-DL-UL-Pattern

The TDD pattern parameters are at least the following:

- The TDD DL-UL-TransmissionPeriodicity which indicates the periodicity of the DL-UL-pattern

- The nbOfDownlinkSubframes which indicates the number of contiguous usable downlink subframes

- The nbOfUplinkSubframes which indicates the number of contiguous usable uplink subframes

- The Downlink-to-UplinkGuardPeriod which indicates the guard period between the DL and the UL transmission in the TDD pattern

According to the WID objective, the TDD DL-UL-TransmissionPeriodicity is equal to 9 radio frames, so 90 ms. The number of usable contiguous DL subframes is equal to 8 (nrOfDownlinkSubframes on the Figure) and 8 for UL subframes with a downlink-to-uplink guard period fixed per band. In total, the usable subframes for the considered TDD pattern are equal to 16/90=17.8%. The 74 other subframes should be considered as unusable/untransmitted. In the DL compared to FDD, 8 over 90 subframes or 8.9% are usable. Therefore, the essential SIB and signaling (NPSS, NSSS, MIB, Paging) should be transmitted on a usable subframe outside of the guard period (i.e. in active transmitting time).

Compared to the FDD, 8.9% of the DL subframes are usable in the proposed IoT-NTN TDD pattern. Therefore, adaptations are needed to ensure the transmission of the essential signaling and other messages occur on usable DL subframes.

The IoT-NTN TDD pattern parameters and possible adaptation of existing parameters are provided by RAN1

The MIB contains the 4 most significant bits of the SFN, the 6 least significant bits are implicitly encoded with NPBCH since it corresponds to 64 radio frames, i.e. the MIB scheduling periodicity. The SFN wrap-around corresponds to 10240 ms or 113.78 DL-UL-Transmission periodicities.

The H-SFN is used for eDRX. It is incremented by one when the SFN wraps around (every 10.24 sec). The 2 least significant bits are encoded in the MIB-NB and the 8 most significant bits are encoded in SIB1-NB. As illustrated below, it means that from one SFN period to another, there exists a shift where the active downlink radio frame of 8 subframes will not be on the same SFN from one H-SFN period to another.

Figure 2 - SFN, H-SFN, DL-UL-Transmission periodicity alignment

The issue is raised by RAN1 on the possible misalignment between the hyper SFN (H-SFN) wrap-around, which is 10240 ms and the TDD periodicity of 90 ms for N=9. Indeed, if we pose for H-SFN period#0, H-SFN=0 and first SFN of the TDD period=0, the first SFN of the TDD period at the H-SFN period#1 would be SF#2 and not 0 (1024%9 = 7). It can generate difficulties in the implementation of scheduling and UE in eDRX where H-SFN is used since the active radio frame while change from one H-SFN period to another.

The options to solve the misalignment are still discussed in RAN1:

The offset between the 90ms TDD structure and the 10240ms H-SFN is to be studied considering at least the following options: - Option 1: Always the same across H-SFNs. - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9. - Option 1-2: H-SFN duration is unchanged - Option 2: Different across some H-SFNs. - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB) - Option 2-2: Total number of H-SFN is changed to be a multiple of 9.

Wait for RAN1 conclusions on the possible misalignment issue between H-SFN and IoT-NTN TDD pattern with N=9 cycle periodicities

Another impact of the defined IoT-NTN TDD pattern is the truncated radio frame, where the usable contiguous subframes in a radio frame is configured to 8. It implies avoiding the transmission of 2 subframes in the radio frame. Regarding the constraints on the fixed transmission pattern for NPSS, NSSS, NPBCH, NPDSCH (SIB1-NB when transmitted) respectively on the 5th subframe of radio frame, the 9th subframe odd frame and the 0th subframe of radio frame, 4th subframe when transmitted according to the configuration, there are two solutions:

- 1) Move the transmission of NPSS/NSSS/NPBCH on a usable subframe

- 2) Skip subframes that are not used for NPSS/NSSS/NPBCH while keeping the subframe number continuity (to avoid complexities on the UE side to re-align subframe number after a subframe skipping).

Option 2) is the way forward in RAN1 with an agreement to downselect among 4 options where the SF#0, SF#5, SF#9 are kept:

The downlink subframes within D=8 are to be down-selected among the following: - Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames) - Option 2: [4 5 6 7 8 9 0 1] (across two consecutive radio frames) - Option 3: [8 9 0 1 2 3 4 5] (across two consecutive radio frames) - Option 4: [9 0 1 2 3 4 5 6] (across two consecutive radio frames)

Figure 3 - Modified frame structure for the new IoT-NTN TDD mode, Option 1, SIB1-NB with 16 repetitions

In all options, the contiguous usable downlink subframes are spanned over 2 radio frames as illustrated on the figure above (example with Option 1 selected and 16 repetitions for SIB1-NB). We see that an offset is created between the start of the radio frame marked SFN#2 and the first usable downlink subframe (which is SF#3 for Option 1). The discussion on the better option and how to signal to the UE according to the offset depicted above between the first usable subframe and the beginning of the radio frame is up to the RAN1. In all the cases, NPSS, NSSS, NPBCH and SIB1-NB subframe number for scheduling is not modified.

In the proposed way forward in RAN to support downlink subframes within D=8, the subframe number location of the NPSS, NSSS, NPBCH (MIB-NB) and NPDSCH (SIB1) remains unchanged compared to the FDD system

Wait for RAN1 on which downlink subframe option is selected and how to signal it

## 2.3	Downlink Time Domain Allocation

An important consequence of the TDD pattern is that only 1 over 9 DL radio frames are transmitted and only 8.9% of the DL subframes. In this section, we discuss the impact of the TDD pattern on the scheduling of the necessary information to allow to connect, i.e. MIB-NB, SIB1-NB, other SIBs and NPDCCH.

RAN1 agreed [2] to discuss the impact of the TDD pattern on the following channels/signals:

Agreement The impact of the TDD pattern at least on the following channels / signals are evaluated at least qualitatively (e.g. specification impact, number of available instances, etc.): - SIB1-NB - Other SIBs needed for basic NB-IoT NTN operation (e.g. SIB2, SIB31) - Paging - NPRACH / RAR - NPSS/NSSS/NPBCH - NPDCCH, NPUSCH, NPDSCH - Note: Study for the above channels / signals may continue during the normative phase. - It is up to companies to perform link level evaluations for some of the aspects above

RAN1 agreed to evaluate the impact of TDD pattern on SIB1-NB, other SIBs, paging, NPRACH and downlink/uplink control/data channels

### 2.3.1	NPBCH scheduling (MIB-NB)

The NPBCH containing the MIB-NB (N.B. the MasterInformationBlock-NB is in use according to the WID that states the IoT NTN TDD system is based on FDD and not the MasterInformationBlock-TDD-NB) is always transmitted in subframe #0 of each frame. The IoT-TDD pattern implies that only 1 over 9 of subframes are received by the UE. According to the MIB-NB transmission scheme, the MIB-NB uses a fixed scheduling with a periodicity of 640 ms [5]-5.2.12a. The 50 bits of the MIB-NB (34 bits and 16 bits of CRC) are encoded into 8 independently decodable blocks of 200 bits (for a total of 1600 bits). Each block is consecutively repeated 8 times at each radio frame.

Figure 4 - MIB-NB scheduling with TDD pattern with N=9

According to the pattern and the offset of the NPBCH period and the transmission scheme, the UE can expect to receive 7.125 (7 in 7 time over 8 and 8 in 1 time over 8) transmissions of a given MIB-NB version during the NPBCH period of 640 ms.

7.125 of the 64 blocks of a given MIB-NB are expected to be transmitted according to the TDD pattern with N=9

The below RAN1 observation shows that the link margin is sufficient to decode the MIB-NB with one block reception in many cases and in all the cases with the 7 or 8 blocks received in the 640 ms period.

RAN1#119 Observation RAN1 makes the following observations on downlink synchronization: Case 1: For N=9, D=8: - For NPBCH: - For 90ms combining, 4 sources provided simulation results, with a median required SNR of 3.48dB. The link budget margin is (median margin): - 2.03dB for LEO-1200 (0dBi antenna gain) - -4.07dB for LEO-600 (-5.5dBi antenna gain) - For 640ms combining, 6 sources provided results, with a median required SNR of -4.25dB. The link budget margin is (median margin): - 9.76dB for LEO-1200 (0dBi antenna gain) - 3.66dB for LEO-600 (-5.5dBi antenna gain) Observation Based on simulations submitted to this meeting, in terms of downlink synchronization (NPSS/NSSS/NPBCH), all the following cases meet the link budget requirements for IOT-NTN TDD and are feasible (from the downlink synchronization point of view) from RAN1 perspective: - Case 1: N=9, D=8

The NPBCH fixed schedule periodicity is misaligned on the DL-UL-periodicity, i.e. 64 mod 9 = 1. However, it seems to have no impact on the decoding capability and the MIB content since the blocks are independently decodable.

The MasterInformationBlock-NB can be decoded by the UE with the proposed IoT-NTN TDD pattern without modifications of the schedule periodicity of 640 ms

### 2.3.2	SIB1-NB

According to [5]-5.2.1.2a, “For FDD, SIB1-NB transmission occurs in subframe #4 of every other frame in 16 continuous frames. The starting frame for the first transmission of the SIB1-NB is derived from the cell PCID and the number of repetitions within the 2560 ms period, and repetitions are made, equally spaced, within the 2560 ms period”. The following parameters configure the SIB1-NB transmission in FDD:

- schedulingInfoSIB1-r13 (0..15): an index to table 16.4.1.3-3 in [4]. This configures the number of NPDSCH repetitions (4, 8, 16) and is used to set the ITBS.

- The PCI is used to determine the beginning offset of SIB1-NB (0 or 1 with 16 repetitions)

- AdditionalTransmissionSIB1 is configured to transmit SIB1-NB in SF#3 in addition to SF#4.

If we do not change the transmission pattern of SIB1-NB, we would have with 16 repetitions the following with the proposed TDD pattern:

Figure 5 - SIB1-NB transmissions in a 256 radio frames period with the proposed TDD pattern with N=9. Dark radioframe containing SF# are not transmitted.

With 16 repetitions, we can expect to receive 7.1 SIB1-NB transmissions in a time frame of 2560 ms. In the above example, the 8 blocks of SIB1-NB are received at least once, but 7 different blocks are more probable. With 8 repetitions, it would be only 3.5 transmissions. The additional transmission of SIB1 would increase the SIB1-NB decoding reliability on a given usable downlink radio frame but increase the overhead.

RAN1 concluded that SIB1-NB transmission with the TDD pattern is feasible at least when the configured number of repetitions is 16, but the maximum TBS supported is still discussed:

Conclusion For N=9, D=8, RAN1 concludes that SIB1-NB reception following current specifications is feasible for some SIB1-NB TBS values, at least when the configured number of repetitions is 16 and when SIB1-NB TBS value <=X bits. - When considering only NPSS/NSSS/NPBCH/SIB1-NB, and if the number of SIB1-NB repetitions is 16: - If additionalTransmissionSIB1 is set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 43.75% in the anchor carrier. - If additionalTransmissionSIB1 is not set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 37.5% in the anchor carrier. - NOTE: In the above calculations, overhead due to transmission of other SIBs has not been included. - The value of X may be further studied during the normative phase. Note: X is a TBS value supported by the specifications for SIB1-NB.

The SIB1-NB can be decoded by the UE for some SIB1-NB TBS values at least when configured number of repetitions is 16 with unchanged schedule periodicity of 2560 ms

Same as for the MIB-NB, the fixed scheduling period for SIB1-NB is not aligned on the DL-UL-Periodicity : 256 mod 9 = 4 radio frame. However, it seems to have no impact on the SIB1-NB decoding and content since block decoding does not have to be ordered to recover SIB1-NB content.

The SIB1-NB can be decoded by the UE with the proposed TDD pattern without modification of the fixed scheduling periodicity of 2560 ms at least when 16 repetitions are configured with a maximum TBS size to be determined by RAN1

### 2.3.3	Others SIBs scheduling

The SIBs other than SIB1-NB that are required for the NB-IoT NTN system are (including those for the mobility):

- SIB2-NB : contains common radio resource configuration

- SIB5-NB : contains neighbouring cell related information for inter-frequency cell re-selection (in IDLE mode)

- SIB22-NB : contains common radio resource configuration information for paging

- SIB31-NB : contains NTN configurations informations

The SIBs-NB are transmitted over NPDSCH in SI-Message within a periodically occurring SI-Window. Up to 8 SI-Message can be scheduled in SIB1-NB. The SIBs-NB transmitted in a SI-Window are indicated with the schedulingInfoList and the sib-MappingInfo. NB that SIB2-NB is always present in the first SI message listed in schedulingInfoList

The time-domain related configurations are:

- si-WindowLength (160 to 1600 ms) that indicates the SI-Window length where the SI-Message is transmitted

- si-RadioFrameOffset (0..15 radio frames) indicates the radio frame offset to calculate the SI-Window w.r.t. SFN

- si-Periodicity (640 to 40960 ms) indicated the periodicity of the SI-window in the SIB1-NB periodicity of 4096 radio frames

- si-RepetitionPattern (every 2nd RF to every 16th RF) indicates the repetition pattern within the SI-Window, e.g. every 2nd RF means that every 2 radio frames within the SI-Window, the SIB is transmitted. For a SI-WindowLenght of 160 ms, it implies 8 repetitions within the SI-Window of the SI-Message

- si-TB (56 to 680 bits) is used to determine the Transport Block Size of the SI-Message. It is related to the time domain since when the TBS <= 120 bits, the SI-Message is transmitted over the 2 first available SF of the RF that verifies the configurations above; otherwise, the SI-Message is transmitted over the 8 available SF in the radio frame.

The above configurations with example values are depicted below within the TDD pattern (note that any configuration of SI-WindowLength is aligned with the DL-UL-Periodicity of 90 ms):

Figure 6 - SI-Window within the TDD pattern with configurations : SI-WindowLenght=160ms; SI-Periodicity=64rf; repetitionPattern=every2ndRF

In the configuration above, on 2 SI-window repetitions, only one SI-Message radio frame is transmitted in a usable DL (in yellow). RAN1 still discusses the maximum TBS that can be decoded by the UE for a given configuration that determines the number of available repetitions within a BCCH modification period (see [5]-§5.2.1.3). However, it is possible to find a configuration that ensures a minimum transmission within the IoT-NTN TDD transmission pattern.

There are SI-Window configurations that ensure transmission of the SI-Message

RAN2 wait for RAN1 to discuss necessary enhancements, if needed, on SI-Window transmission configurations to permit the UE to decode SIBs-NB

Another issue raised by the SI-message transmission related to the TDD pattern comes from the TBS. Indeed, when the TBS <= 120 bits, 2 SFs within the SI-message radio frame are needed for transmission. TDD pattern can afford this even with 8 contiguous usable SF. But for TBS>120 bits, 8 SFs are needed. Only a maximum of 6 are available according to the DL subframe option selected. Therefore, two solutions:

- Either assumes that not all the 8 SF are needed for receiving the SI-Message. The maximum TBS has to be determined by RAN1

- Or introduce a postponing mechanism for SI-message, i.e. for maximum up to 680 bits (which may contain several SIBs-NB), describe a UE behavior where the UE receiving less than 8 SF in an active DL frame waits for the next active DL frame outside of the SI-Window to receive the remaining SF.

The Second option is illustrated below:

Figure 7 - Postponement of SIBs subframes principle

In the example given above (using option 1 of downlink subframe), we see that the 8 SFs corresponding to a SI-message transmission could not fit in a DL subframe. Moreover, there is an offset A between the beginning of the SFN and the beginning of the transmission. So the proposed expected UE behaviour is that, it starts to search for SIBs at the first subframe of the DL transmission in the SI-Window and according to the scheduling in the SIB1-NB, consider that the next 2 or 8 SFs contains the SIBs even if it is outside of the SI-Window. It is up to UE implementation to determine when occur the next downlink subframe tranmission according to the TDD pattern.

The TDD pattern of D=8 implies at most 6 usable subframes for SI-message transmission within a transmission period. For TBS > 120 bits, SI-message is transmitted over 8 SFs. Therefore, a SI-message transmission may need to be transmitted over 2 downlink transmissions.

According to [5]-§5.2.1.2a : “Within the SI-window, the corresponding SI message can be transmitted a number of times over 2 or 8 consecutive NBIoT”. The postponement implies that a SI message transmission of 8 SF (TBS>120 bits) could end outside of a SI-Window.

RAN1 agreed to discuss the following:

Agreement RAN1 to further discuss how to handle the overlap of NPDCCH/NPDSCH (other than the one carrying SIB1-NB) (including e.g. starting point, windows for SI or RAR and other window sizes for DL channels/signals, PO, etc.) with non-D NB-IoT subframes, including studying at least the following: - New periodicities to align with the TDD structure. - Postponement of a channel when it overlaps with non-D NB-IoT subframes. - Restriction of a channel to be fully confined within a single set of D NB-IoT subframes. - Dropping (full or partial) of channels when they overlap with non-D NB-IoT subframes. - The non-D NB-IoT subframes are not “NB-IoT DL subframes” from the specifications

According to RAN1 conclusion on others SIBs evaluation, RAN2 to discuss the postponing of the SI-message subframe to the next usable downlink subframe, including outside of the SI-Window

### 2.3.4	NPDCCH scheduling (DCI)

The NPDCCH is dedicated to the transmission of control information in downlink, e.g. DCI for DL and UL UE-specific resource allocation or paging. The DCI N0 contains the UL resource grant (23 bits), the N1 contains the DL resource scheduling (23 bits) and N2, paging and direct indication (15 bits). Two formats, format 0 and 1 are used according to the NCCE configuration.

| Search space types | NPDCCH formats | DCI types |
| --- | --- | --- |
| Type1 common | Format 1 | N2 |
| Type2 common | Format 1 | N0 and N1 |
| UE-Specific | Format 0 and 1 | N0 and N1 |
| NOTE 1: Format 0 for one NCCE (NCCE0 or NCCE1) and format 1 for the two NCCE NOTE 2: DCI format N0 for the uplink scheduling in one UL cell; DCI format N1 for the scheduling of one NPDSCH codeword per TTI in one cell and to trigger NPRACH; DCI format N2 for the paging [7]. |  |  |

The UE may retrieve the NPDCCH within a search space [4]-§16.6. An example configuration is given below:

Figure 8 - Search Space with TDD pattern N=9, with R=2, Rmax=4, G=4, alpha=1/4, D subrames Option 1

With parameters for NPDCCH Type 2 common and UE-specific search space such as:

| Parameter | Type2 common | UE-specific |
| --- | --- | --- |
| Rmax | npdcch-NumRepetitions-RA-r13 (1, 2, ...2048) | npdcch-NumRepetitions (1, 2,...2048) |
| Alpha (offset) | npdcch-Offset-RA-r13 (0..3/8) | npdcch-Offset-USS-r13 (0..3/8) |
| G | npdcch-StartSF-CSS-RA-r13 ([1.5, 2, 4, 8, 16, 32, 48, 64] in FDD mode) | npdcch-StartSF-USS-r13 ([1.5, 2, 4, 8, 16, 32, 48, 64] in FDD mode) |

For finding the starting subrame k0, the UE should apply the following formula:

10nf+ns2mod T= αoffset×T

Where:

- nf is the frame index

- ns∈{0,.., 19} is the slot index

- T=Rmax×G is the periodicity

Note that according to the formula and the available configurations, there is no combination that aligns the common search space to the TDD pattern with D=8 and N=9.

No available configurations can align all the NPDCCH transmissions with the IoT-NTN TDD pattern

RAN1 agreed to discuss on the followings:

Agreement RAN1 to further discuss how to handle the overlap of NPDCCH/NPDSCH (other than the one carrying SIB1-NB) (including e.g. starting point, windows for SI or RAR and other window sizes for DL channels/signals, PO, etc.) with non-D NB-IoT subframes, including studying at least the following: - New periodicities to align with the TDD structure. - Postponement of a channel when it overlaps with non-D NB-IoT subframes. - Restriction of a channel to be fully confined within a single set of D NB-IoT subframes. - Dropping (full or partial) of channels when they overlap with non-D NB-IoT subframes. - The non-D NB-IoT subframes are not “NB-IoT DL subframes” from the specifications

Wait RAN1 to conclude on the NPDCCH handling w.r.t. the IoT TDD pattern

According to [4]: “An NPDCCH search space ( ',RL ) NSk at aggregation level L ' ( L ' 2 = for TDD special subframe, L ∈ { }2,1' otherwise), and repetition level R ∈ { } ,8,4,2,1 16 ,32 ,64 ,128 ,256 ,512 ,1024 ,2048 is defined by a set of NPDCCH candidates where each candidate is repeated in a set of R consecutive NB-IoT downlink subframes excluding subframes used for transmission of SI messages starting with subframe k”

It is needed to clarify in the specifications the meaning to support IoT-NTN TDD pattern:

- Either consecutive NB-IoT downlink subframes, including subframes that are not transmitted (gap period) and UL subframe. Therefore, this transmission is truncated.

- Or consecutive NB-IoT downlink subframes, imply postponing to the next transmission period, even outside of the NPDCCH search space. The postponing is made by implementation.

If postponement is decided for NPDCCH, RAN2 to clarify in the specification that postponement is to be done to a next usable downlink subframes excluding subframes used for the transmission of SI messages

### Paging in I-DRX and eDRX mode

The IoT-NTN UE is expected to be configured with a discontinuous reception (DRX) for paging for energy saving purpose, i.e. Idle DRX (I-DRX) and/or extended DRX (eDRX). The UE is still reachable by the network through the paging. In I-DRX and eDRX, the paging is made through NPDCCH with Type 1 common search space (see in §2.3.4). Paging occurs at a limited number of occasions during Paging Occasions (PO) within Paging Frames (PF). One Paging Frame (PF) is one Radio Frame, which may contain one or multiple Paging Occasion(s). When DRX is used, the UE needs only to monitor one PO per DRX cycle. The paging occasions are configured through SIB2-NB in PCCH-Config-NB with:

- defaultPagingCycle-r13 (T) : 128, 256, 512, 1024 radio frame

- nB-r13 (nB for Paging frame) : (4T, 2T,… 1/1024T) paging cycle

- npdcch-NumRepetitionPaging-r13 (R) : number of repetition to be applied, (1..2048) repetitions

From which are derived the 3 scheduling parameters (in addition to the UE_ID):

- N the number of PFs within DRX cycle, i.e. min(T, nB)

- Ns the number of POs within PFs, i.e. max(1, Nb/T)

- i_s the index of PO table

The Paging frames are separated by the derived paging cycle (minimum between SIB2-NB default paging cycle and UE-specific paging cycle) and the paging occasions are determined from the Paging Occasion table in [8]-§7.2-1 (FDD NPDCCH). For Ns=1 (one paging occasion per paging frame), the PO is located on the SF#9, For Ns=2 the PO is located on SF#4 or 9 according to i_s and on SF#0, #4, #5, #9 for Ns=4.

In all DL subframes configuration options discussed in RAN1, the Paging Occasion(s) subframes within a Paging Frame are transmitted, i.e. SF#0, #4, #5, #9. No adaptation of the PO subframes is needed

After, there is the eDRX that was introduced for longer sleep of NB-IoT UEs. It is configured by the Core Network with 2 parameters [9]-§10.5.5.32 controlling the eDRX paging cycle:

- Paging Time Window (PTW) : 2.56 seconds (256 radio frames) to 40.96 seconds (4096 radio frames)

- eDRX value or T_eDRX : 10.24 seconds (1024 radio frames) to 10485.76 seconds (1024x1024 radio frames). The eDRX uses the H-SFN instead of SFN.

The eDRX is also used for system information update notification. So, for NB-IoT, the boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 1024 =0. In the case of de-synchronization between the UE and the eNB, the UE may reacquire H-SFN through MIB to avoid repeated paging failure.

I-DRX and eDRX cycles are depicted on the Figure below:

Figure 9 - I-DRX and eDRX with IoT-NTN TDD pattern with, Paging cycle = 128, nB = 1T, R = 1, PTW = 256, eDRX cycle = 1024, a given UE_id and TDD pattern N=9 (note that PF does not necessarily match with the beginning of the PTW)

We see that a paging frame does not necessarily match with the TDD pattern. There is no configuration that ensures always aligning the 90 ms IoT-NTN TDD pattern with the paging cycle.

There exists a set of configurations that allow transmission of paging but no configuration can align IoT-NTN TDD pattern N=9 and paging cycle

In [8], it is defined the following:

“In case of P-RNTI transmitted on NPDCCH, PO refers to the starting subframe of NPDCCH repetitions unless subframe determined by PO is not a valid NB-IoT downlink subframe then the first valid NB-IoT downlink subframe after PO is the starting subframe of the NPDCCH repetitions.”

The mechanism of postponement discussed for other SIBs-NB scheduling and NPDCCH can also be applied for the Paging Occasion. If the first subframe determined by PO is not a valid NB-IoT downlink subframe e.g. in the TDD guard time period, the PO is postponed to the next valid NB-IoT downlink subframe.

Upon RAN1 decision, if NPDCCH is postponed to a next usable D subframe, RAN2 to discuss how to clarify the UE behaviour on the postponement of a Paging Frame (so the Paging Occasion(s)) in the 3GPP TS 36.304

## Others

### 2.5.1	Random Access

Figure 10 - Random Access procedure for IoT NTN

The Random Access procedure is initiated by the UE after the reception of the necessary cell and network information in MIB-NB, SIB1-NB, SIB2-NB and SIB31-NB to connect to the network. The procedure is detailed in Figure 10. The impact of the TDD pattern on the NPRACH uplink channel is still discussed in RAN1 [2].

Wait for RAN1 inputs for any necessary enhancements on the NPRACH scheduling mechanism

After the NPRACH (msg1), the UE expects the msg2 after the estimated UE-gNB RTT in a Random Access Response (RAR) window configured by ra-ResponseWindowSize-r13 (RACH-Info-NB-r13). The Window size is configured in number of NPDCCH cycles from 2 to 10 cycles. With a sufficiently large configuration, the window can cover multiple DL transmission opportunities. The same reasoning can be applied to the mac-ContentionResolutionTimer-r13 for msg4 (from 1 to 64 NPDCCH cycles)

The RAR window and the MAC contention resolution timer can be configured to up to 10 NPDCCH cycles and 64 respectively. According to the NPDCCH cycles configuration is it sufficient to ensure multiple DL subframe transmissions w.r.t. the IoT-NTN TDD pattern

The RAR window and the MAC contention resolution timer configurations are sufficiently large to cope with limited DL transmission opportunities w.r.t the IoT NTN TDD pattern. Can be rediscussed to increase these timers according to RAN1 conclusions on NPRACH/NPDCCH evaluations

### 2.5.2	k-Mac range extension

According to the actual satellite constellation deployments that are likely to adopt the IoT-NTN TDD pattern and the constraints on the scheduling due to the scarce DL subframes, it would be beneficial to increase the k-Mac to allow more flexibilities to cope with extra latency. With one bit extension of k-Mac field, we would allow to use of the 513 to 1024 ms values. To ensure backward compatibility with actual deployments, a k-Mac_ext parameter of 1 bit to adress the new range of 513 to 1024 can be considered.

Increase k-Mmac of 1 bit to allow more flexibility on the scheduling. FFS if we define a new k-Mac_ext or if we extend the current k-Mac value broadcasted in SIB31/SIB31-NB

# 3	Conclusion

In this contribution, the followings observations and proposals was provided:

<Description in Stage 2>

- Update the stage 2 specification to support and describe the NB-IoT NTN TDD mode. The text proposal in Annex can be considered as a baseline.

<Definition of the TDD pattern>

- Compared to the FDD, 8.9% of the DL subframes are usable in the proposed IoT-NTN TDD pattern. Therefore, adaptations are needed to ensure the transmission of the essential signaling and other messages occur on usable DL subframes.

The IoT-NTN TDD pattern parameters and possible adaptation of existing parameters are provided by RAN1

Wait for RAN1 conclusions on the possible misalignment issue between H-SFN and IoT-NTN TDD pattern with N=9 cycle periodicities

In the proposed way forward in RAN to support downlink subframes within D=8, the subframe number location of the NPSS, NSSS, NPBCH (MIB-NB) and NPDSCH (SIB1) remains unchanged compared to the FDD system

Wait for RAN1 on which downlink subframe option is selected and how to signal it

<Downlink Time Domain Allocation>

RAN1 agreed to evaluate the impact of TDD pattern on SIB1-NB, other SIBs, paging, NPRACH and downlink/uplink control/data channels

<MIB-NB>

7.125 of the 64 blocks of a given MIB-NB are expected to be transmitted according to the TDD pattern with N=9

The MasterInformationBlock-NB can be decoded by the UE with the proposed IoT-NTN TDD pattern without modifications of the schedule periodicity of 640 ms

<SIB1-NB>

The SIB1-NB can be decoded by the UE for some SIB1-NB TBS values at least when configured number of repetitions is 16 with unchanged schedule periodicity of 2560 ms

The SIB1-NB can be decoded by the UE with the proposed TDD pattern without modification of the fixed scheduling periodicity of 2560 ms at least when 16 repetitions are configured with a maximum TBS size to be determined by RAN1

<Others SIBs>

There are SI-Window configurations that ensure transmission of the SI-Message

RAN2 wait for RAN1 to discuss necessary enhancements, if needed, on SI-Window transmission configurations to permit the UE to decode SIBs-NB

The TDD pattern of D=8 implies at most 6 usable subframes for SI-message transmission within a transmission period. For TBS > 120 bits, SI-message is transmitted over 8 SFs. Therefore, a SI-message transmission may need to be transmitted over 2 downlink transmissions.

According to RAN1 conclusion on others SIBs evaluation, RAN2 to discuss the postponing of the SI-message subframe to the next usable downlink subframe, including outside of the SI-Window

<NPDCCH>

No available configurations can align all the NPDCCH transmissions with the IoT-NTN TDD pattern

Wait RAN1 to conclude on the NPDCCH handling w.r.t. the IoT TDD pattern

If postponement is decided for NPDCCH, RAN2 to clarify in the specification that postponement is to be done to a next usable downlink subframes excluding subframes used for the transmission of SI messages

<Paging>

In all DL subframes configuration options discussed in RAN1, the Paging Occasion(s) subframes within a Paging Frame are transmitted, i.e. SF#0, #4, #5, #9. No adaptation of the PO subframes is needed

There exists a set of configurations that allow transmission of paging but no configuration can align IoT-NTN TDD pattern N=9 and paging cycle

Upon RAN1 decision, if NPDCCH is postponed to a next usable D subframe, RAN2 to discuss how to clarify the UE behaviour on the postponement of a Paging Frame (so the Paging Occasion(s)) in the 3GPP TS 36.304

<Random Access>

Wait for RAN1 inputs for any necessary enhancements on the NPRACH scheduling mechanism

The RAR window and the MAC contention resolution timer can be configured to up to 10 NPDCCH cycles and 64 respectively. According to the NPDCCH cycles configuration is it sufficient to ensure multiple DL subframe transmissions w.r.t. the IoT-NTN TDD pattern

The RAR window and the MAC contention resolution timer configurations are sufficiently large to cope with limited DL transmission opportunities w.r.t the IoT NTN TDD pattern. Can be rediscussed to increase these timers according to RAN1 conclusions on NPRACH/NPDCCH evaluations

<k-mac extension>

Increase k-Mac of 1 bit to allow more flexibility on the scheduling. FFS if we define a new k-Mac_ext or if we extend the current k-Mac value broadcasted in SIB31/SIB31-NB

# Text Proposal for 36.300

First Modified Subclause

## 5.0	Frame structures and channels

Downlink and uplink transmissions are organized into radio frames with 10 ms duration. Three radio frame structures are supported:

-	Type 1, applicable to FDD and IoT-NTN TDD;

-	Type 2, applicable to TDD;

-	Type 3, applicable to LAA secondary cell operation only.

Frame structure Type 1 is illustrated in Figure 5.1-1. Each 10 ms radio frame is divided into ten equally sized sub-frames. Each sub-frame consists of two equally sized slots. Each slot can further be divided into three subslots that may have different sizes. For FDD, 10 subframes, 20 slots, or up to 60 subslots are available for downlink and uplink transmission in each 10 ms interval. Uplink and downlink transmissions are separated in the frequency domain except for IoT-NTN TDD.

Figure 5.1-1: Frame structure type 1

Next Modified Subclause

### 23.21.1	General

Support for BL UEs, UEs in enhanced coverage and NB-IoT UEs over Non-Terrestrial Networks (see clause 4.12) is only applicable to E-UTRA connected to EPC. UEs not supporting NTN are barred from accessing an NTN cell.

In NTN, only BL UEs, UEs in enhanced coverage and NB-IoT UEs with GNSS capability are supported in this release of the specification.

To accommodate long propagation delays in NTN, increased timer values and window sizes, or delayed starting times are supported for the physical layer and for higher layers.

UL segmented transmission is supported for UL transmission with repetitions. The UE shall apply UE pre-compensation per segment of UL transmission of PUSCH/PUCCH/PRACH for BL UEs or UEs in enhanced coverage and NPUSCH/NPRACH for NB-IoT UEs from one segment to the next segment.

NTN is applicable to FDD system. TDD can be supported for NB-IoT UEs with IoT-NTN TDD as described in §23.21.X.

In NTN, the distance refers to Euclidean distance.

Next Modified Subclause

### 23.21.X	TDD mode for NB-IoT NTN

The TDD mode is applicable for NB-IoT UEs over NTN. It is based on NB-IoT NTN FDD frame structure and procedures. The TDD pattern follows a periodic cycle of 9 radio frames wherein 8 non-overlapping downlink and 8 uplink subframes are usable, separated by a guard time. The pattern is fixed per band and applicable for both anchor and non-anchor carriers.

End of Changes

# References

RP-243293, Revised WID on introduction of IoT-NTN TDD mode, RAN#106, 2024-12, Madrid, ES.

RP-242652, Status Report - WID on introduction of IoT-NTN TDD mode, Iridium, RAN#106, 2024-12, Madrid, ES.

R1-2409440, Discussion on IoT-NTN TDD mode, Thales, RAN1#119, 2024-11, Orlando, US.

3GPP TS 36.213, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures”, v18.3.0.

3GPP TS 36.331 “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification”, v18.4.0.

3GPP TS 36.306 “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities”, v18.4.0.

3GPP TS 36-212 “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding”, v18.1.0.

3GPP TS 36.304 “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode”, v18.3.0.

3GPP TS 24.008 “Mobile radio interface Layer 3 specification; Core network protocols; Stage 3”, v18.7.0.

3GPP TS 36.300 “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”, v18.4.0.