3GPP TSG-RAN WG2 Meeting #129	R2-2500945

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.6.4

Source:	Nokia

Title:	On Conditional LTM

WID/SID:	NR_Mob_Ph4-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#128 the following was agreed for Conditional LTM:

Agreements on C-LTM:

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

The primary motivation for CLTM is to introduce robustness to the LTM procedure where the UE may not be able to receive the cell-switch command at right radio condition which may lead to radio link failure. Compared to CHO, the conditional LTM is expected to bring the benefits of robustness along with the interruption time reduction benefits of LTM. In this contribution, we discuss the baseline procedure for CLTM support and identify the related key issues.

# 2	Discussion

We identify the following key aspects to be discussed in RAN2.

- Conditional cell switch condition configuration

- Conditional cell switch condition evaluation

- Early synchronization

- Conditional cell switch execution

- Conditional LTM with Dual Connectivity

## Conditional cell switch condition configuration

In Rel-18, it has been decided to limit the amount of LTM candidates to 8 candidate target cells on per serving cell. For the time being, we see no need to further increase this number. On the other hand, forcing the NW to configure solely Rel-18 LTM only candidate target cells or CLTM candidate target cells is limiting. In a result, it has been proposed in the ongoing RRC CR [1] to make CLTM candidate target cell to be Rel-18 LTM candidate target cell as well. Since, as agreed, set of candidate target cells is always prepared by current serving cell and future candidate target cells, NW is always aware of which candidate target cells are inter-CU and which of them are intra-CU.

Proposal 1: Handling of inter-CU LTM and CLTM shall be left to CU implementation. No changes in specification are needed.

Usage of CHO-like execution conditions has been discussed with no conclusion. Since not all the UE are capable of using L1-based execution conditions, L3-based execution conditions are needed. Using CHO conditions as a basis for execution conditions is rational, because of similarity between CHO and CLTM with regards to which entity decides about the switch. However, CHO execution conditions differ from what is expected from CLTM execution condition due to different characteristics of CHO and CLTM.

Observation 1: CondEvent is defined at the cell level while LTM/C-LTM considers beam-level measurements.

Proposal 2: RAN2 shall address the following aspects to support CHO-like conditions in Rel-19 C-LTM:

- CLTM capable UEs need to perform L3 measurements for evaluating C-LTM candidates

- Clarify whether L3 measurements are sufficient/suitable, considering frequency ranges

- LTM-dedicated configuration of CondEventAx would be needed to align with beam-level LTM approach

## Conditional cell switch condition evaluation

In Rel-18 LTM, cell switch is always triggered by the NW. In a result, this is NW which decides about the switch and can choose the right moment to do that. In Rel-19 CLTM, it is not the case anymore. It means that it starts to be important to specify the moment when UE starts the evaluation of conditions to avoid unwanted behavior.

Observation 2: UE can start evaluation of conditions without being synchronized with candidate target cell in both DL and UL. It leads to RACH-based admission which shall be avoided as much as it is possible.

In the current RRC CR for 38.331, the following procedural text is captured

2> stop the LTM conditions evaluation (based on L1 and/or L3 measurements), if any, for all the LTM candidate configurations associated with the cell group for which the LTM cell switch procedure is triggered; 2> if the LTM-Candidate IE for the selected LTM candidate configuration includes the field cltm-ExecutionConditions: 3> if the field l3-Conditions is included within cltm-ExecutionConditions: 4> perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.x according to the received cltm-ExecutionConditions; 3> else if the field l1-Conditions is included within cltm-ExecutionConditions: 4> inform lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the received field cltm-ExecutionConditions.

Observation 3: UE can start evaluation of conditions immediately after LTM switch. This is because, execution conditions can be all the time fulfilled (especially L1-based ones). In a result, UE can be stuck in continuous CLTM cell switch.

In Rel-18 network energy saving (NES) feature, the conditional evaluation is linked to NES mode of the serving cell. In other words, the conditional evaluation for the events will be triggered only when serving cell is in NES mode. Similar triggering can be considered for CLTM, where criteria such as availability of timing advance and early downlink synchronization may be used. This way, conditional LTM can benefit from the increased robustness of conditional cell switch execution without compromising on interruption time during cell switch. Nevertheless, since support of early synchronization procedures is optional for Rel-18 UEs (e.g., UE capabilities: rach-EarlyTA-Measurement-r18, ue-TA-Measurement-r18), condition evaluation for UEs supporting only RACH-based CTLM switches should be enabled. Furthermore, regardless of the UE capabilities,

Observation 4: For UEs supporting early synchronization, NW triggering of condition evaluation for CLTM allows for increased robustness of the conditional cell switch execution without compromising on interruption time during cell switch.

Proposal 3: For initial CLTM execution, UEs supporting only RACH-based CLTM cell switch can start the condition evaluation when they receive the LTM configuration. FFS for when UEs supporting RACH-less CLTM.

Proposal 4: For subsequent CLTM execution, NW triggering of condition evaluation should be supported.

To allow the network to start the early TA and early TCI state activation at appropriate time prior to CLTM, the network may configure separate LTM events with event parameters which may trigger events slightly earlier than the events defined as condition for CLTM. Alternatively additional offset for early reporting for the condition can be used to indicate that DU to start the early sync related actions for CLTM.

Proposal 5: CLTM event with additional offset can be used to trigger L1 measurement report prior to C-LTM execution condition.

## Early synchronization

### RACH-based early TA acquisition

In the last meeting, it was agreed to provide the TA information to the UE from the source using a new MAC CE:

RAN2#128 agreements: - The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. - The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM

The content of the MAC CE is not agreed yet. Before discussing the content of the MAC CE, a few considerations on the maintenance of the TA value it transfers should be made. After a TA value has been obtained, it must be ensured that at the time of the conditional cell switch, the TA value is valid. Regarding maintenance of the TA provided to the UE with the new MAC CE, in RAN2#128, the following agreement was made:

RAN2#128 agreement: Candidate cell TA is maintained by a new timer.

From the above agreements, it is evident that the UE may have to store and maintain multiple TAs for supporting RACH-less CLTM. In intra-CU setups it is likely that LTM candidates are collocated, thus it would be redundant for the UE to separately store a TA and maintain it with an associated TAT for such candidates.

Observation 5: For collocated LTM candidates, it is redundant for the UE to separately store a TA and maintain it with an associated TAT.

It is possible that early TA acquisition is performed for a candidate, but a conditional cell switch is triggered to another candidate that is collocated with the candidate for which early TA acquisition was performed. In addition, a conditional cell switch may be triggered to a candidate that is collocated with the current serving cell. It should be noted that this issue cannot be circumvented by the use the RACH-less HO IE in the candidates’ RRC configuration, as this would solve the issue for the initial cell switch but not for subsequent ones. In such scenarios, the UE will not use the available TA for accessing the target and will unnecessarily trigger a RACH-based CLTM cell switch.

Observation 6: The UE may unnecessarily trigger a RACH-based CLTM cell switch to a candidate, although a TA has been obtained for a collocated candidate or current serving cell.

To reduce TA maintenance effort on the UE side and avoid unnecessary RACH-based CLTM cell switches, it would be useful to provide an indication to the UE to allow it to reuse a TA among collocated candidates

Proposal 6: To reduce TA maintenance effort on the UE side and avoid unnecessary RACH-based CLTM cell switches, provide an indication to the UE to allow it to reuse a TA among collocated candidates

Proposal 7: RAN2 shall discuss how UE shall maintain CLTM TA timer(s). For example, UE can follow below scheme:

- CLTM TAT is started/re-started when UE receives TAC sent by candidate target cell through current serving cell

- CLTM TAT shall be stopped on CLTM cell switch execution to this cell

- Expiration of CLTM TAT related to TAG which candidate target cell belongs to, implies that TA is not valid anymore

- Previously received CLTM TAs and associated TATs become obsolete upon CLTM execution

### UE-based TA estimation

An alternative way for the UE to obtain the TA required for RACH-less cell switch is UE-based TA estimation. In Rel-18 LTM, if the UE is configured with Rel. 18 LTM and the TA in MAC CE is given as FFF, the UE may execute RACH-less LTM with its own measured TA. In RAN2#127bis, it was agreed to support UE-based TA measurement also for CLTM:

RAN2#127bis agreement: UE based TA measurement mechanism is supported for conditional intra-CU LTM.

As discussed in Section 2.3.1, in RAN2#128, it was agreed that the NW can provide the TA information of a candidate cell to the UE via a new MAC CE. In case the UE is configured with UE based TA; the network may trigger the RACH-based early UL synchronization procedure based on the PDCCH order for given candidate cell(s). This TA acquired from the candidate cell is to be shared to the UE by the network over the proposed new MAC CE as agreed above.

Proposal 8: In case CLTM is configured along with UE based TA estimation for given candidate cell(s), RAN2 to confirm that the TA information, which is obtained over the new MAC CE for the same candidate cell(s), if valid will get precedence over UE estimated TA.

For the maintenance of a TA associated with a serving cell, a TA validity timer (TAT) is configured over RRC and maintained at the UE. As discussed in Sec. 2.3.1, in RAN2#128 it was agreed that candidate cell TA will also by maintained by a new timer. Upon a conditional cell switch being triggered to a candidate, if the TA maintenance timer of the NW-given TA associated to that candidate has expired and the UE is not configured with UE-based TA estimation for that candidate, the UE will have no choice but to fall back to RACH-based CLTM cell switch execution. However, if the UE is configured with UE-based TA estimation, in case the TA value received via the new MAC CE has expired or it was set as ‘FFF’, or no TA-carrying MAC CE has been received, the UE can use the UE-estimated TA to perform RACH-less cell switch execution. Similarly, the TA value received at the UE over the new MAC CE is set as “FFF” or in case if no new MAC CE is received at the UE, the UE can use the UE estimated TA.

Proposal 9: In case of CLTM, RAN2 to agree that if the TA provided by the network over the new MAC CE or any network entity becomes invalid, or set as “FFF”, or no new MAC CE received, if UE is configured with UE based TA estimation, it may use the estimated TA value if the same is valid.

Another aspect that requires further discussion in RAN2 is the inter-working between CLTM and NW-triggered LTM. While the conditions for CLTM cell switch execution are being evaluated at the UE or before CLTM cell switch is initiated, the network may decide to trigger cell switch to a given LTM prepared candidate cell and send a MAC CE cell switch command (CSC MAC CE). The CSC MAC CE may or may not contain the Timing Advance information as per Release-19 behavior where it is Optional to include the TA in the cell switch command. In this scenario, if the UE is also configured with UE based TA estimation, the UE behavior to handle the UE based TA estimation is not clearly defined.

Proposal 10: RAN2 to discuss on the handling of the UE estimated TA in case of CLTM and network triggered non-conditional LTM co-existence.

## Conditional cell switch execution

As discussed above, allowing the UE to trigger a cell switch execution has significant impact on the actions that need to be taken before and after a conditional cell switch. An important consequence is the network’s uncertainty about the timing of the cell switch, which makes the preparation of the network for the UE’s access to the target cell more complicated. A possible way to overcome this challenge and simplify the procedures related to UE’s first access to the target is the transmission of a message from the UE to the source cell, informing the source cell that a conditional cell switch condition has been met. Upon receiving such a notification, the source cell can trigger procedures for UE’s access to the target cell as in Rel. 18 LTM, e.g., send a cell switch notification, or it can even decide whether a cell switch is allowed, in case more network control on the cell switch is desired, and inform the UE about it.

Observation 7: Autonomous UE’s decision on cell switch causes uncertainty at NW side with regards to timing of the cell switch and consequent preparation for UE admission.

Observation 8: Lack of any cell switch indication to the NW leads to inflexibilities in resource management at NW side.

Nevertheless, such a message may never reach the source cell as the UE-source cell link quality may already be too low soon after the conditional cell switch condition is met. In addition, while the UE will be waiting for a response to the message, the link may further deteriorate potentially resulting in radio link failure. Therefore, we should further study if and how this message should be transmitted. Alternatively, the UE could directly inform the target about its access, e.g. by sending a scheduling request to the target.

Observation 9: Early signalling with the source cell prior to cell switching can allow to mimic NW behaviour from Rel-18 LTM with regards to how candidate target cell is informed about cell switch.

Observation 10: Early signalling with the source cell prior to cell switching would be risky as it would need to be performed in potentially very bad radio conditions.

Observation 11: Early indication to the target cell before switching can allow to save PUSCH (CG) and/or PDCCH (DG) resources of target cell to be wasted or kept reserved for too long.

Proposal 11: RAN2 to decide whether UE shall inform the serving cell or the target cell about the switch before CLTM execution.

For RACH-less access to the target cell, apart from the TA, the UE must be provided with an UL grant. In Rel. 18 LTM, both CG and DG were supported as they have their own benefits. In RAN2#127bis, support of CG was agreed:

RAN2#127bis agreement: For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

Providing a CG may incur a large resource reservation overhead, especially when multiple CGs for multiple UEs are provided by the target cell, with the resources being reserved from RRC configuration to cell switch execution. Nevertheless, for non-conditional LTM, thanks to the network triggering of the cell switch and the transmission of cell switch notification, optimizations of the resource management on the network side are possible, as well as monitoring of the CG only after a cell switch has been decided. The lack of network awareness of when the cell switch in CLTM is triggered requires for alternative solutions to avoid monitoring of the CGs starting from RRC configuration and to enable more flexible resource management on the network side. To this end, as the RAR transmission may constitute the last signaling exchange between the UE and the source cell before a potential conditional cell switch, the RAR transmission and its preceding steps (RA preamble transmission, TA information transfer) may be used for the activation of a CG. Alternatively, a message from the UE to the NW can be used for the activation of a CG. Such a message would also facilitate the support of DG. Utilizing a DG mitigates issues related to increased resource reservation which were discussed earlier for CG.

Proposal 12: RAN2 to discuss CG activation based on RA-driven early TA acquisition procedure and/or message from the UE to the NW informing about cell switch triggering.

Proposal 13: DG-based admission is supported. RAN2 to discuss DG triggers.

A further aspect that must be considered regarding UE-triggered cell switch execution is that the conditional execution condition may be met while the early TA acquisition procedure is in progress, e.g., while the UE is waiting to receive the RAR. In such a case, it may be beneficial to allow for completion of the early TA acquisition before the cell switch is executed, so that a RACH-less cell switch can be performed, and an additional unnecessary random-access procedure is avoided. On the other hand, like the above considerations for the notification message from the UE to the source, while the UE is waiting for the RAR, the UE-source cell link may deteriorate, the RAR may never be received by the UE and the UE may experience a radio link failure, leading eventually to a larger interruption time compared to that of a RACH-based cell switch. Therefore, the details about if and how long the UE should wait for a RAR require further consideration.

Observation 12: Allowing for slight delay in cell switch while waiting for RAR is beneficial as it can increase the chances for RACH-less admission to target cell.

Proposal 14: If the conditional cell switch execution condition is met while early TA acquisition is in progress, allow for completion of the early TA acquisition procedure before triggering the cell switch.

Furthermore, it is likely that conditional cell switch execution conditions for multiple cells are met. In the current RRC CR we have captured the following procedure:

1> if this procedure is triggered due to fulfilment of LTM cell switch conditions: 2> if more than one LTM candidate configuration has triggered this procedure: 3> select one of the LTM candidate configurations as the selected cell for the LTM cell switch execution; Editor’s Note: FFS how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution.

Observation 13: Considering the procedural text in the current RRC CR, in case more than one LTM candidate configuration has triggered the LTM procedure, UE would select one of the LTM candidate configurations and execute CLTM towards it.

The actions that the UE has performed to prepare a cell switch to a candidate cell prior to the triggering of the condition may be different for each of the triggered candidates, e.g., the UE may have performed DL and UL synchronization with one of the triggered candidates, but only DL synchronization with another. To reduce cell switch interruption time, it is reasonable that the UE prioritizes execution to the former.

Proposal 15: When conditional cell switch execution conditions for multiple candidate cells have been triggered, the UE should trigger the cell switch to a candidate cell for which Early synchronisation steps are completed.

## CLTM with DC

In the current RRC CR, there was some discussion captured on the CLTM with DC aspects. In our understanding, progress on the topic depends on the TSG RAN checkpoint. In the approved WID, we have captured the following:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

Observation 14: Considering the current WID, RAN2 is requested to limit specifying CLTM to the non-DC scenario.

Proposal 16: Postpone discussions for CLTM with DC.

# 3	Conclusion

This document has made the following observations:

Observation 1: CondEvent is defined at the cell level while LTM/C-LTM considers beam-level measurements.

Observation 2: UE can start evaluation of conditions without being synchronized with candidate target cell in both DL and UL. It leads to RACH-based admission which shall be avoided as much as it is possible.

Observation 3: UE can start evaluation of conditions immediately after LTM switch. This is because, execution conditions can be all the time fulfilled (especially L1-based ones). In a result, UE can be stuck in continuous CLTM cell switch.

Observation 4: For UEs supporting early synchronization, NW triggering of condition evaluation for CLTM allows for increased robustness of the conditional cell switch execution without compromising on interruption time during cell switch.

Observation 5: For collocated LTM candidates, it is redundant for the UE to separately store a TA and maintain it with an associated TAT.

Observation 6: The UE may unnecessarily trigger a RACH-based CLTM cell switch to a candidate, although a TA has been obtained for a collocated candidate or current serving cell.

Observation 7: Autonomous UE’s decision on cell switch causes uncertainty at NW side with regards to timing of the cell switch and consequent preparation for UE admission.

Observation 8: Lack of any cell switch indication to the NW leads to inflexibilities in resource management at NW side.

Observation 9: Early signalling with the source cell prior to cell switching can allow to mimic NW behaviour from Rel-18 LTM with regards to how candidate target cell is informed about cell switch.

Observation 10: Early signalling with the source cell prior to cell switching would be risky as it would need to be performed in potentially very bad radio conditions.

Observation 11: Early indication to the target cell before switching can allow to save PUSCH (CG) and/or PDCCH (DG) resources of target cell to be wasted or kept reserved for too long.

Observation 12: Allowing for slight delay in cell switch while waiting for RAR is beneficial as it can increase the chances for RACH-less admission to target cell.

Observation 13: Considering the procedural text in the current RRC CR, in case more than one LTM candidate configuration has triggered the LTM procedure, UE would select one of the LTM candidate configurations and execute CLTM towards it.

Observation 14: Considering the current WID, RAN2 is requested to limit specifying CLTM to the non-DC scenario.

And proposed the following:

Proposal 1: Handling of inter-CU LTM and CLTM shall be left to CU implementation. No changes in specification are needed.

Proposal 2: RAN2 shall address the following aspects to support CHO-like conditions in Rel-19 C-LTM:

- CLTM capable UEs need to perform L3 measurements for evaluating C-LTM candidates

- Clarify whether L3 measurements are sufficient/suitable, considering frequency ranges

- LTM-dedicated configuration of CondEventAx would be needed to align with beam-level LTM approach

Proposal 3: For initial CLTM execution, UEs supporting only RACH-based CLTM cell switch can start the condition evaluation when they receive the LTM configuration. FFS for when UEs supporting RACH-less CLTM.

Proposal 4: For subsequent CLTM execution, NW triggering of condition evaluation should be supported.

Proposal 5: CLTM event with additional offset can be used to trigger L1 measurement report prior to C-LTM execution condition.

Proposal 6: To reduce TA maintenance effort on the UE side and avoid unnecessary RACH-based CLTM cell switches, provide an indication to the UE to allow it to reuse a TA among collocated candidates

Proposal 7: RAN2 shall discuss how UE shall maintain CLTM TA timer(s). For example, UE can follow below scheme:

- CLTM TAT is started/re-started when UE receives TAC sent by candidate target cell through current serving cell

- CLTM TAT shall be stopped on CLTM cell switch execution to this cell

- Expiration of CLTM TAT related to TAG which candidate target cell belongs to, implies that TA is not valid anymore

- Previously received CLTM TAs and associated TATs become obsolete upon CLTM execution

Proposal 8: In case CLTM is configured along with UE based TA estimation for given candidate cell(s), RAN2 to confirm that the TA information, which is obtained over the new MAC CE for the same candidate cell(s), if valid will get precedence over UE estimated TA.

Proposal 9: In case of CLTM, RAN2 to agree that if the TA provided by the network over the new MAC CE or any network entity becomes invalid, or set as “FFF”, or no new MAC CE received,  if UE is configured with UE based TA estimation, it may use the estimated TA value if the same is valid.

Proposal 10: RAN2 to discuss on the handling of the UE estimated TA in case of CLTM and network triggered non-conditional LTM co-existence.

Proposal 11: RAN2 to decide whether UE shall inform the serving cell or the target cell about the switch before CLTM execution.

Proposal 12: RAN2 to discuss CG activation based on RA-driven early TA acquisition procedure and/or message from the UE to the NW informing about cell switch triggering.

Proposal 13: DG-based admission is supported. RAN2 to discuss DG triggers.

Proposal 14: If the conditional cell switch execution condition is met while early TA acquisition is in progress, allow for completion of the early TA acquisition procedure before triggering the cell switch.

Proposal 15: When conditional cell switch execution conditions for multiple candidate cells have been triggered, the UE should trigger the cell switch to a candidate cell for which Early synchronisation steps are completed.

Proposal 16: Postpone discussions for CLTM with DC.

# 4	References

[1]	[POST128][107][MOB] RRC CR for LTM and CLTM (Ericsson), RAN2#128, Orlando, USA, November 18-22, 2024.