3GPP TSG-RAN WG2 Meeting #129	R2-2500947

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.3.3

Source:	Nokia

Title:	On the measurement event prediction

WID/SID:	FS_NR_AIML_Mob - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In the TP for TR 38.744 [1], the following is captured:

There are two methods to predict measurement event, namely indirect and direct measurement event prediction as illustrated in Figure 1-1 and Figure 1-2 respectively. Figure 1-1 Indirect measurement event prediction In indirect measurement event prediction for intra-frequency temporal domain case A, temporal domain case B or spatial domain, measurement result(s) is predicted by a RRM measurement prediction model at first. Afterwards, predicted and optionally actual historical measurement result(s) of the same cell are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. In indirect measurement event prediction for frequency domain, measurement result(s) is predicted by a RRM measurement prediction model for frequency domain at first. Afterwards, predicted and optionally actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Figure 1-2 Direct measurement event prediction As illustrated in Figure 1-2, the input of the model with direct prediction is the same as indirect prediction as illustrated in Figure 1-1 and additional other input is also allowed. Measurement event is predicted directly without any intermediate measurement results.

In this t-doc we present our views on indirect measurement prediction aspects, specifically focusing on the different ways for labelling the data and discussing the different approaches for selecting the time instance when the UE should perform the measurement event prediction. We also present some preliminary simulation results on both direct and indirect measurement event prediction approaches.

# 2	Discussion

## 2.1	Indirect Prediction Aspects

When considering indirect measurement event prediction, as shown in Figure 1-1, as highlighted in the earlier section, both cases A and B are possible. In temporal domain case B, the PW is typically short. For instance, the 50% MRRT skipping patterns considered either in Example 1 or Example 2 are depicted in Figures 2-1 and 2-2, respectively.

Figure 2-1 Example 1 of intra-frequency temporal domain case B

Figure 2-2 Example 2 of intra-frequency temporal domain case B

Considering the above figures, two options are possible for configuring temporal domain case B prediction for measurement event prediction. One option would be to configure long prediction intervals, e.g., equal to or larger than TTT, as shown with Example 1 in Figure 2-1. This example would suffer the already observed drawback that performance of AI/ML algorithms tends to degrade as PW increases, especially in time domain case B. The pattern in Example 2 with short PW was shown in evaluations to perform clearly better. For 50% MRRT, in Example 2 it is necessary to predict only one measurement. However, this might not be very useful in indirect measurement event prediction, since TTT values typically tend to be longer than the measurement period. Overall, the gain of performing measurement event prediction in time domain can in this case be expected to be low.

Observation 1: In RRM measurement time domain prediction case B, shorter PW and Example 2 were evaluated to perform better with the same MRRT.

Observation 2: The gain of performing indirect measurement event prediction with time domain case B RRM measurement prediction can be expected to be low, when Example 2 skipping pattern is used.

Proposal 1: Evaluate indirect measurement event prediction only with time domain case A measurement prediction.

## 2.2	Measurement Event Prediction Training Data Labelling

When discussing the actions taken in the simulations based on the predicted and the real A3 events, it is important to clarify how the training dataset is labelled. In our understanding, the ground truth that is being predicted is the legacy A3 measurement event that is reported with the agreed A3 configuration.

Proposal 2: RAN2 confirms that the ground truth that is being predicted are the legacy A3 measurement events that are reported with the agreed A3 configuration.

In the sequel, we define more precisely what it means to “predict the A3 event”. First, we set up the following terminology:

- A3 event entry: the time instance at which the condition for the A3 event entry is fulfilled. Typically, it takes the form of (RSRP_Diff > threshold + hysteresis), where RSRP_Diff is the difference between the strongest neighboring cell’s RSRP and the serving cell’s RSRP. This would correspond to the time when the UE starts the time-to-trigger (TTT).

- A3 event fulfilled: an A3 event is fulfilled if the exit condition (RSRP_Diff < threshold – hysteresis) is not met for the duration of the TTT. This is followed by the UE sending a measurement report to the network to prepare and execute a handover.

- A3 event unfulfilled: an A3 event is unfulfilled if the exit condition is met during the TTT. In this case, the UE does not send a measurement report.

There can be several points in time when the UE can trigger the prediction report, as well as several quantities associated with the A3 event that can be predicted. We summarize these cases below.

Observation 3: There can be several points in time when the UE can trigger the prediction report, as well as several quantities associated with the A3 event that can be predicted.

### 2.2.1 Option 1: Prediction before the A3 event entry

In this option, the UE predicts quantities associated with the A3 event at an arbitrary timestep before its entry (i.e. before the TTT is started in the UE). Figure 2-3 illustrates that option. The UE triggers the model prediction at timestep T, and predicts a quantity associated with the A3 event for the subsequent prediction window, which is larger than the time-to-trigger (TTT).

Figure 2-3 Measurement event prediction option 1

The time instances T at which the UE should perform the inference may be configured by the network as e.g. periodic, aperiodic, or conditional instances. The predicted quantities in this option (i.e. the output of the ML model) may involve if an A3 is predicted to occur in the PW and the ID of the target cell (the handover candidate). Additionally, the prediction may include the timestep within the PW at which the A3 event entry/fulfillment/unfulfillment are predicted to occur or the probability of the measurement event occurring in the PW. In case of direct measurement event prediction, these are predicted directly by an ML model. In case of indirect measurement event prediction, future measurements for the serving cell and target candidate cell(s) are predicted by an ML model and the predicted measurement events are derived from the predicted measurements.

Observation 4: Triggering measurement event prediction at any time before the A3 entry condition would require prediction windows to be larger than TTT.

Observation 5: Triggering measurement event prediction at any time before the A3 entry condition may require also predicting or determining the right target candidate cell.

### 2.2.2 Option 2: Prediction at the A3 event entry

In this option, the UE triggers the model’s prediction only at the time instance at which the A3 event entry condition is fulfilled. This is illustrated in Figure 2-4. Triggering the model’s inference at that time instance means that the UE already knows the candidate target cell, so it does not need to be predicted. Moreover, it knows the time instance at which the A3 event becomes “fulfilled”, which is one TTT later. The prediction quantity for that option, therefore, can simply be (for the direct prediction case) the probability that the A3 event will become fulfilled (i.e. the probability that the A3 exit condition does not occur during the TTT). This probability can then be used to e.g. inform the UE whether to send a prediction report to the network for an early preparation of a handover.

For the indirect prediction case, the predicted quantity can be the time series of future RSRP measurements in the TTT for both the serving cell and the target candidate cell.

Figure 2-4 Measurement event prediction option 2

Considering the two options above, we decide to use option 2 and believe that it is a better option overall for the following reasons:

- There is no need to predict the ID of the target candidate cell, as it’s already known at the A3 event entry. In option 1, predicting the candidate cell’s ID has the complication that an additional performance metric is needed for it (e.g. accuracy) along with the performance metrics related to the predictions of other quantities.

- The prediction window’s length is naturally fixed (it’s equal to the TTT), unlike option 1, so there’s no need to optimize for it to find the optimal prediction window length.

- There is no need to periodically or conditionally trigger the inference of the model at timesteps before the A3 event entry, as the time instance for inference is naturally the A3 event entry.

- There is no need to predict the time instance of the A3 event fulfillment or its probability of occurring inside a time window, because it’s always a TTT after the A3 event entry.

Observation 6: Predicting the A3 event’s fulfillment or unfulfillment at the A3 event entry time instance is a simpler approach than predicting it at an arbitrary time instance before the entry.

Proposal 3: Take the proposed option 2 (predicting the A3 event at the time instance of its entry) as a starting point for evaluations.

# 3	Evaluation Results

To evaluate the measurement event prediction use case, we set up a 7-site FR2 simulation scenario based on TR38.843 according to the Table 3-1.

Table 3-1 Simulation parameters for measurement event prediction

| Reported simulation assumptions | UE trajectory option | See Section 3.2 in our companion contribution [2] |
| --- | --- | --- |
|  | UE trajectory boundary processing option | See Section 3.2 in our companion contribution [2] |
|  | UE speed | 60/90/120 km/h |
|  | SoftLOS | Yes |
|  | Spatial consistency option | Option B |
|  | Number of TX beams | 8 |
|  | Number of RX beams | 4 |
|  | Measurement reduction rate | N/A |
|  | Sub-use-case | Case 2 (cell to cell) |
|  | Observation window (OW) | [20-2000] ms |
|  | Prediction window (PW) | 320 ms |
| Data Size (Number of Samples) | Training | Approximately 10K per speed |
|  | Testing | Approximately 7K per speed |
| AI/ML model input/output | Model input | L3 filtered RSRP measurements |
|  | Model output | L3 filtered RSRP measurements |
| AI/ML model description | Model type | Random Forest, Multi-Layer Perceptron |

## 3.2 Direct Prediction

For the direct prediction case, the input to the model is time series of historical L3-filtered RSRP measurements before the A3 event entry (and including it). The measurements can come from the any number of cells, but for this evaluation, we use the serving and candidate cell RSRPs and compare it with using all the 21 cells.

In the training data, each sample is labelled as either “fulfilled” (1) or “unfulfilled” (0), depending on whether the associated A3 event has been fulfilled or unfulfilled, according to the definitions laid out in Section 2.2. The problem then becomes a binary classification problem.

Figure 3-1 shows the test set F1 score of a random forest binary classifier (trained and tested on the 60 km/h UEs) for several lengths of the observation window, and when using either the serving and target cell RSRPs as input or when using all the 21 cells RSRPs as input. The F1 score improves slightly with the observation window length when using the 21 cells, but it increases significantly when using the source and target cells until it matches the F1 score of the 21-cell case at a 2000-millisecond observation window.

Figure 3-1 F1 score vs observation window length for the direct event prediction with 60 km/h UEs

Observation 7: Using all cells in the input of direct measurement even prediction gives better F1 score than using only the source and target cells, but only for short observation windows. For longer observations windows, both kinds of input are on-par with each other.

Table 3-2 shows the F1 score for different combinations of training and testing speeds to show the model’s generalization performance. The source and target cell RSRPs were used as an input. As expected, the F1 score decreases when we test the model on a UE speed that was not included in the training set. However, when some or all of the speeds are included in the training set, the F1 score improves, highlighting the importance of training set diversity. This is also reflected in the ROC AUC score of the model, shown in Table 3-3.

Observation 8: Including UEs of different speeds in the training set improves generalization performance.

Table 3-2 F1 score for different combinations of training and testing UE speeds for direct prediction

|  |  | Testing UE Speeds |  |  |
| --- | --- | --- | --- | --- |
|  |  | 60 | 90 | 120 |
| Training UE Speeds | 60 | 0.805 | 0.723 | 0.672 |
|  | 90 | 0.707 | 0.812 | 0.749 |
|  | 120 | 0.671 | 0.734 | 0.833 |
|  | 60, 90 | 0.798 | 0.823 | 0.758 |
|  | 60, 120 | 0.792 | 0.760 | 0.834 |
|  | 90, 120 | 0.707 | 0.813 | 0.843 |
|  | All | 0.792 | 0.823 | 0.840 |

Table 3-3 ROC AUC score for different combinations of training and testing UE speeds for direct prediction

|  |  | Testing UE Speeds |  |  |
| --- | --- | --- | --- | --- |
|  |  | 60 | 90 | 120 |
| Training UE Speeds | 60 | 0.886 | 0.783 | 0.690 |
|  | 90 | 0.759 | 0.897 | 0.828 |
|  | 120 | 0.668 | 0.775 | 0.910 |
|  | 60, 90 | 0.881 | 0.906 | 0.839 |
|  | 60, 120 | 0.875 | 0.835 | 0.914 |
|  | 90, 120 | 0.767 | 0.897 | 0.921 |
|  | All | 0.877 | 0.907 | 0.921 |

## 3.3	Indirect Prediction

For the indirect prediction case, the input to the model is the same as the direct prediction case above. The only difference is the output of the ML model, which is, in this case, the predicted future measurements of the serving and target cells in the TTT. These predicted measurements are used to determine whether the A3 event will become fulfilled or unfulfilled. If the A3 event exit condition is met (RSRP_Diff < threshold – hysteresis), the A3 event is predicted to become unfulfilled. Otherwise, it’s predicted to become fulfilled.

Figure 3-4 shows the test set F1 score of multi-layer perceptron classifier (trained and tested on the 60 km/h UEs) for several lengths of the observation window, and when using either the serving and target cell RSRPs as input or when using all the 21 cells RSRPs as input. As with the direct method, the F1 score improves slightly with the observation window length when using the 21 cells, but it increases significantly when using the source and target cells until it matches the F1 score of the 21-cell case at a 2000-millisecond observation window.

Observation 9: Using all cells in the input of direct and indirect measurement even prediction gives better F1 score than using only the source and target cells, but only for short observation windows. For longer observations windows, both kinds of input are on-par with each other.

Table 3-4 shows the F1 score for different combinations of training and testing speeds to show the model’s generalization performance. The observation window is 2000 ms, the model is a multi-layer perceptron of three layers and 128 neurons each, and the source and target cell RSRPs are used as an input. As before, the more UE speed classes included in the training data, the better the generalization performance.

Table 3-5 shows the ROC AUC score for the same experiment. By comparing the ROC AUC scores of direct and indirect methods, it tells us that the direct method is more performant. But if we compare the F1 scores instead, it tells us that the indirect method is more performant. We believe this is because the F1 score, being the harmonic mean of precision and recall, does not paint a full picture of the model’s performance, because the precision and recall depend on the classification threshold used, while the ROC AUC score does not.

Figure 3-4 F1 score vs observation window length for the indirect event prediction with 60 km/h UEs

Table 3-4 F1 score for different combinations of training and testing UE speeds for indirect prediction

|  |  | Testing UE Speeds |  |  |
| --- | --- | --- | --- | --- |
|  |  | 60 | 90 | 120 |
| Training UE Speeds | 60 | 0.863 | 0.734 | 0.615 |
|  | 90 | 0.660 | 0.882 | 0.774 |
|  | 120 | 0.604 | 0.734 | 0.888 |
|  | 60, 90 | 0.836 | 0.856 | 0.760 |
|  | 60, 120 | 0.835 | 0.779 | 0.864 |
|  | 90, 120 | 0.655 | 0.869 | 0.868 |
|  | All | 0.819 | 0.857 | 0.855 |

Table 3-5 ROC AUC score for different combinations of training and testing UE speeds for indirect prediction

|  |  | Testing UE Speeds |  |  |
| --- | --- | --- | --- | --- |
|  |  | 60 | 90 | 120 |
| Training UE Speeds | 60 | 0.847 | 0.640 | 0.523 |
|  | 90 | 0.609 | 0.870 | 0.715 |
|  | 120 | 0.547 | 0.698 | 0.890 |
|  | 60, 90 | 0.818 | 0.834 | 0.679 |
|  | 60, 120 | 0.792 | 0.707 | 0.844 |
|  | 90, 120 | 0.610 | 0.852 | 0.860 |
|  | All | 0.789 | 0.814 | 0.824 |

We note that the F1 score is defined as the harmonic mean of precision and recall. Therefore, the value of the F1 score depends on the way the classification threshold is selected, which makes comparing between the results reported by different companies more difficult. On the other hand, ROC AUC is not dependent on the selected classification threshold but can show the models performance in achieving the trade-off between precision and recall. In this sense, it is complementing the F1 score.

Observation 10: The direct prediction method achieves higher ROC AUC whereas the indirect prediction achieves higher F1 score.

Observation 11: The ROC AUC score can be used to complement the F1 score and obtain a better representation of the model’s performance.

Proposal 4: In addition to F1 score, consider also ROC AUC score of classifier methods as a performance metric.

# 3	Conclusion

This document has made the following observations:

Observation 1: In RRM measurement time domain prediction case B, shorter PW and Example 2 were evaluated to perform better with the same MRRT.

Observation 2: The gain of performing indirect measurement event prediction with time domain case B RRM measurement prediction can be expected to be low, when Example 2 skipping pattern is used.

Observation 3: There can be several points in time when the UE can trigger the prediction report, as well as several quantities associated with the A3 event that can be predicted.

Observation 4: Triggering measurement event prediction at any time before the A3 entry condition would require prediction windows to be larger than TTT.

Observation 5: Triggering measurement event prediction at any time before the A3 entry condition may require also predicting or determining the right target candidate cell.

Observation 6: Predicting the A3 event’s fulfillment or unfulfillment at the A3 event entry time instance is a simpler approach than predicting it at an arbitrary time instance before the entry.

Observation 7: Using all cells in the input of direct measurement even prediction gives better F1 score than using only the source and target cells, but only for short observation windows. For longer observations windows, both kinds of input are on-par with each other.

Observation 8: Including UEs of different speeds in the training set improves generalization performance.

Observation 9: Using all cells in the input of direct and indirect measurement even prediction gives better F1 score than using only the source and target cells, but only for short observation windows. For longer observations windows, both kinds of input are on-par with each other.

Observation 10: The direct prediction method achieves higher ROC AUC whereas the indirect prediction achieves higher F1 score.

Observation 11: The ROC AUC score can be used to complement the F1 score and obtain a better representation of the model’s performance.

And proposed the following:

Proposal 1: Evaluate indirect measurement event prediction only with time domain case A measurement prediction.

Proposal 2: RAN2 confirms that the ground truth that is being predicted are the legacy A3 measurement events that are reported with the agreed A3 configuration.

Proposal 3: Take the proposed option 2 (predicting the A3 event at the time instance of its entry) as a starting point for evaluations.

Proposal 4: In addition to F1 score, consider also ROC AUC score of classifier methods as a performance metric.

# 4	References

[1]	R2-2500287 Text Proposal of 38.744 3GPP RAN2#129, Athens, Greece, February 17 – 21, 2025

[2]	R2-2500948 Generalization of ML mobility use-cases, Nokia, Nokia Shanghai Bell, 3GPP RAN2#129, Athens, Greece, February 17 – 21, 2025