3GPP TSG RAN WG2 #129	R2-2500951

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.2.4

Source:	Fraunhofer HHI, Fraunhofer IIS

Title:	Discussion on data transmission and general aspects

Document for:	Discussion

# Introduction

In the last RAN#106 plenary, a new work item on solutions for Ambient-IoT (Internet of Things) in NR was approved [1]. Ambient-IoT is a new 3GPP IoT technology that relies on ultra-low complexity devices with ultra-low power consumption for very-low-end IoT applications. As a starting point, the following guidance was given to RAN2 WG:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signaling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included.

During the SI [2], functionalities related to data transmission have been studied. Among others, a particular focus has been set on failure handling and retransmission mechanisms. In alignment with the guidance outlined in the WID, and the TR [3] outcome of the SI, our contribution further explores these functionalities and discuss potential areas for improvement.

# Discussion

## Data retransmission and re-access for failure handling

In the TR [3], the following was agreed in regards data transmission and failure handling:

According to the protocol stack and functionalities in clause 6.3.2, more detailed data transmission functionalities are studied in this clause. The handling of data transmission failure has been studied. It is understood that the subsequent R2D data transmission after the D2R data transmission does not need to be always sent. In case of D2R data transmission failure, the A-IoT device follows the reader subsequent R2D instruction, if any. - For instance, the reader can repeat the R2D upper layer "command" to trigger the A-IoT device to re-send the same D2R upper layer "response" (i.e., the A-IoT device just follows the received R2D to transmit following D2R). - For instance, the reader can re-send A-IoT Msg2 to specific device(s) to echo the random ID(s), in case of failure reception of corresponding D2R data transmission (i.e. "Msg3") after the initial A-IoT Msg2, together with the corresponding D2R resource scheduling. This can trigger the A-IoT device to re-send the same D2R data transmission (i.e. "Msg3"). If it is supported to include the echoed random IDs for multiple devices, the re-sent A-IoT Msg2 only includes the random IDs of the devices, whose "Msg3" is not successfully received.

Figure 1. Scenarios for data transmission failure

Failures in D2R transmission primarily result from the unsuccessful reception of Msg3 by the reader, which can be attributed to two main scenarios: (i) Msg2 failure, where the device does not receive Msg2 and consequently does not respond with Msg3, leading the reader to never receive Msg3; and (ii) Msg3 failure, where the device correctly receives Msg2 and transmits Msg3, but the reader fails to receive Msg3 successfully. In either case, the reader is in charge for the latter retransmission.

To address such scenarios, two re-access approaches are derived from the agreed text in the TR [3]. The first approach, re-access within the same round, removes the need of paging and random access procedures, enabling the reader to trigger retransmissions either by allocating specific resources for retransmission in a new Msg2, or by re-sending the command in the same occasion or slot. While both methods provide faster failure recovery, both present some challenges.

First, allocating specific retransmission slots may lead to resource underutilization when failures occur infrequently, as they need to be specifically pre-reserved for retransmission in order to avoid devices to slect them. This inefficiency can be mitigated by dynamically adjusting the slot indicator number in subsequent frames, scaling the allocated retransmission resources proportionally to the number of A-IoT devices in the round, and the current network conditions. On the other hand, re-accessing within the same slot or occasion can potentially lead to synchronization issues for A-IoT devices scheduled in later slots. However, this can be addressed through periodic resynchronization mechanisms at predefined time steps within the round.

Even though both strategies may present challenges, they are better suited for A-IoT devices, specially if considering their limited capabilities, such as memory and/or energy resources. Hence, from our perspective, in-round retransmissions should be considered for failure handling.

Proposal 1: Support re-access within the same round, either immediately after a failure is detected or within a scheduled retransmission window.

Alternatively, re-access can be scheduled in a later rounds, which reduces immediate resource overhead but increases retransmission latency. This method is more resource-efficient since retransmissions are only triggered when necessary, rather than reserving retransmission slots in advance. However, the key drawback is the additional delay caused by the need to restart paging and random access procedures before retransmission can happen. Such approach results into a waste of energy from the device perspective.

Proposal 2: Re-access in different rounds should be deprioritized.

Another open issue is the definition of MAC layer parameters to control the retransmission behavior of devices. Parameters such as the maximum number of retransmissions and/or a maximum retransmission window must be standardized to ensure a correct operation. Besides, these parameters should be selected to align with the energy constraints of A-IoT devices, as excessive retransmissions or frequent attempts could drain device batteries and reduce the overall network efficiency. For example, in the case of segmentation, an A-IoT device may drain its energy if any of the segments require multiple retransmissions. In such scenario, the device could lose its segmentation point, forcing it to restart the entire frame transmission once it has enough energy. To mitigate this, it is preferable for the device to pause their retransmissions after a given threshold is reached, and then, resume them only after sufficient energy is restored.

Proposal 3: RAN2 should define MAC layer parameters regarding retransmission procedures such as maximum number of retransmissions and/or maximum time window for retransmissions.

## Feedback indication for segmentation in D2R transmission

It is studied on the functionality of segmentation. It is understood that segmentation and reassembly would add complexity. Therefore, the simplified segmentation in the A-IoT MAC layer is studied, considering following studies in physical layer: - In both R2D and D2R directions, there is no lower bound on the minimum TB size, and a maximum TB size of around 1000 bits can be supported. - How large TB that can be transported at a given time depends on target coverage/data rate, energy consumption/device availability, etc. The segmentation study focuses on D2R direction (it can be further considered on R2D direction). For the candidate solution for segmentation: - The sequence number, the segment number and the number of segments are not supported; - An indication is used to indicate to the reader on whether the data is segmented and whether the MAC PDU is the last segment. It needs to be further discussed on the size of this indication (one or two bits) and the corresponding further details; - It is beneficial for the reader to be able to trigger a re-transmission of a segment; - It is assumed that the A-IoT device will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all segment(s) are stored in upper layer(s).

In the TR [3], the following was agreed in regards of segmentation for the D2R:

One of the key issues arises when considering segmentation for D2R, is that given the limited computational resources and memory of AIoT tags, conventional segmentation and retransmission mechanisms are not viable. As agreed, sequence number is not supported. Therefore, a segment-by-segment feedback indication is necessary to ensure successful delivery. Such motivation relies on the limitations of A-IoT devices, since they lack the ability to keep information about previously transmitted segments. Therefore, if a segment is lost during transmission, the A-IoT device might be unable to detect which part of the data was not received successfully.

A segment-by-segment retransmission procedure can address this issue by incorporating the ACK/NACK signaling. After an A-IoT device transmits a segment, the reader immediately responds with either an acknowledgment (ACK) to confirm successful reception or a negative acknowledgment (NACK) to indicate that the segment was lost and needs to be resent. Since the tag does not maintain a memory of past transmissions, it simply reacts to the reader’s instruction, retransmitting the last segment if a NACK is received. This ensures that each segment is correctly delivered before the tag proceeds to the next one. Additionally, to minimize overhead, it could be considered an only NACK solution, where if no indication is received, after a time window has elapsed, the next segment is sent. However, such solution poses a challenge highly related to the previous section. If the indication is lost, A-IoT devices may consider that the segment has been successfully transmitted when it was not. Hence, we consider than an explicit indication is more suitable for A-IoT devices.

Proposal 4: Support for explicit feedback mechanisms, including ACK/NACK signaling, to ensure reliable segmentation and retransmission for A-IoT D2R communication.

# Conclusion

In this contribution we discussed the A-IoT data retransmission for failure handling. Based on the discussion in the previous sections we made the following observations and proposals:

Proposal 1: Support re-access within the same round, either immediately after a failure is detected or within a scheduled retransmission window.

Proposal 2: Re-access in different rounds should be deprioritized.

Proposal 3: RAN2 should define MAC layer parameters regarding retransmission procedures such as maximum number of retransmissions and/or maximum time window for retransmissions

Proposal 4: Support for explicit feedback mechanisms, including ACK/NACK signaling, to ensure reliable segmentation and retransmission for A-IoT D2R communication

# References

- 3GPP RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, 3GPP TSG-RAN #106, Madrid, Spain, Dec 9th-12th, 2024.

- 3GPP RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR” 3GPP TSG RAN #103, Maastricht, Netherlands, March 18th -21st, 2024.

- 3GPP TR 38.769 V2.0.1 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on solutions for ambient IoT (Internet of Things) “, 2024-12.