3GPP TSG-RAN WG2 Meeting #129 	R2-2500955

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.6.3

Source:	Panasonic

Title:	L1 measurement event configuration and reporting

Document for:	Discussion

# Introduction

In last meeting, RAN2 achieved the following agreements on LTM measurement event evaluation and reporting.

Agreements on L1 event-driven MR:

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

- TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

- TTT is applied to the leaving condition.

- Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

- Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

- For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

- A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

- Support the truncated measurement report MAC CE.

In this paper, we focus on addressing some remaining issues. In particular, we discuss RS type configuration and event trigger conditions when configured for both early synchronization and cell switch procedures.

# Discussion

## RS type configuration

In previous meeting, it has been agreed that network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation (see agreement 4 in section 1). In the current 38.331 running CR proposed in post meeting email discussion [1], this is reflected as RRC parameter eventEvaluationRS-Type-r19. However, it is only defined for eventLTM2. See the following,

where eventLTM2 concerns only serving cell. The list of LTM events are as follows:

- Event LTM2: Beam of serving becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than the beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

In the email discussion, there was a view that RS type configuration parameter is not needed for events other than eventLTM2. The argument is that the network can configure the same RS type for all candidate cells, either CSI-RS or SSB. Then the serving cell RS type just follows the candidate cell.

However, limiting one type of RS configuration for all candidate cells seems to be very restrictive. Imagine the situation where a new candidate cell needs to be added into the set of already configured candidate cells. If the new candidate cell has configured a different RS type for measurement, all the exiting candidate cells need to modify the RS configuration to ensure the same type? Therefore, a better way is to allow a candidate cell to configure both CSI-RS and SSB as measurement RS. Network can then use eventEvaluationRS-Type-r19 to configure which type to use for evaluation for a given event. This provides more flexibility and avoid additional signalling overhead to change the RS configuration.

As a matter of fact, the agreement 4 in the section 1 has been discussed and concluded in the context of ensuing same RS type between candidate cell and serving cell. Now the current spec implementation limits the usage to eventLTM2 which concerns only serving cell RS. This is the least motivated event to rely on eventEvaluationRS-Type-r19. For this eventLTM2, the current beam can simply follow the indicated TCI state. If the indicate TCI state has SSB as QCL RS, the SSB will be used for event evaluation; if the indicate TCI state has CSI-RS as QCL RS, the CSI-RS will be used for event evaluation. On the other hand, if the absolute threshold in the event evaluation differs between SSB beam and CSI-RS beam, it makes sense to use eventEvaluationRS-Type-r19 for eventLTM2 as well.

Although RS type configuration is applicable for all LTM events, it may not be the same configuration for all events. In other words, each event can be configured with different RS type for event evaluation.

Proposal 1: RS type configuration, eventEvaluationRS-Type-r19, is applicable for all LTM events and configured per event.

## LTMx event reporting for early synchronization.

In RAN2#126 meeting the following agreement was made regarding LTM measurement reporting design.

Event triggered L1 measurement should be designed for the following LTM purposes:-

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

Accordingly, if event based L1 measurements are to be triggered for early synchronization and cell switch procedures, it is a common understanding that network configures such trigger conditions at UE to report L1 measurements of candidate beam/cell, when LTM procedures are expected to be performed (like early synchronization or cell switch) towards one (or more) candidate beam/cell. The following figure illustrates such design requirement in LTM signalling flow diagram.

Figure 1: LTM procedure (source – TS38.300v18.4.0; 9.2.3.5.2-1 Signalling procedure for LTM)

Proposal 2: Signaling flow diagram for Rel-19 LTM to be updated with L1 measurement reporting flow from UE during early synchronization procedure as seen in Figure-1, possibly triggered and reported using event reporting conditions.

Further if network configures UE with event trigger condition to report L1 measurements during early synchronization and cell switch procedures, there could be multiple ways to provide such configurations to UE.

Proposal 3: RAN2 to consider following options to configure event trigger conditions to be reported during early synchronization and cell switch procedures.

Option1: One LTMx event can be configured at UE with event condition (i.e. entering condition) to report for both early synchronization and cell switch procedures.

- FFS: How entering condition is configured to trigger during different LTM procedure stages.

Option2: Multiple LTMx events can be used to configure event conditions at UE, where at least one LTMx event is configured dedicated to early synchronization procedure and the remaining (one or more) dedicated to cell switch procedure.

- FFS: How redundant measurement reports can be avoided when one beam/cell is triggered with multiple LTMx event conditions configured one for early synchronization and the other for cell switch.

# Conclusion

In this paper, we discuss some remaining issues for L1 measurement event. The proposals are as follows.

Proposal 1: RS type configuration, eventEvaluationRS-Type-r19, is applicable for all LTM events and configured per event.

Proposal 2: Signaling flow diagram for Rel-19 LTM to be updated with L1 measurement reporting flow from UE during early synchronization procedure as seen in Figure-1, possibly triggered and reported using event reporting conditions.

Proposal 3: RAN2 to consider following options to configure event trigger conditions to be reported during early synchronization and cell switch procedures.

Option1: One LTMx event can be configured at UE with event condition (i.e. entering condition) to report for both early synchronization and cell switch procedures.

- FFS: How entering condition is configured to trigger during different LTM procedure stages.

Option2: Multiple LTMx events can be used to configure event conditions at UE, where at least one LTMx event is configured dedicated to early synchronization procedure and the remaining (one or more) dedicated to cell switch procedure.

- FFS: How redundant measurement reports can be avoided when one beam/cell is triggered with multiple LTMx event conditions configured one for early synchronization and the other for cell switch.

# Reference

[1] [POST128][108][MOB], post email discussion, https://www.3gpp.org/ftp/Email_Discussions/RAN2/ [RAN2#128]/[POST128][108][MOB] RRC running CR (Huawei)