3GPP TSG RAN WG2 Meeting #129	R2-2500957

# Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 5095 | rev | - | Current version: | 16.18.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | First reconfiguration after reestablishment with SN-terminated bearers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to TS 36.331: - when the UE receives an RRCConnectionReestablishment message, the UE updates KeNB based on the current KeNB based on the KASME or KAMF to which the current KeNB is associated, using the received nextHopChainingCount value and derives the CP and UP security keys for ciphering and integrity protection. - when the UE receives the first RRCConnectionReconfiguration, the UE resumes DRBs that were suspended, including DRBs using the secondaryKey (unless they are released). This raises the question of the expected UE behaviour for those DRBs. The most appropriate would be that the network includes sk-counter in the RRCConnectionReconfiguration message to trigger AS key derivation but this is captured neither in TS 36.331 nor in TS 37.340 nor in any other 3GPP specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture in the description of sk-counter that it is always included in the first reconfiguration after RRC re-estalishment when one or more RBs with keyToUse set to secondary are configured and not released. For nr-Config described as "This field is used to configure (NG)EN-DC", add "(re)" and "or release" to better match with procedure text. Impact Analysis Impacted 5G architecture options: (NG)EN-DC Impacted functionality: RRC re-establishment Inter-operability: If the network is implemented according to the CR and UE is not, there is no interoperability issue. If UE is implemented according to the CR and the network is not, the first reconfiguration after re-establishment will fail when there are SN-terminated DRBs if the network does not include sk-counter. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The first reconfiguration after re-establishment will fail when there are SN-terminated DRBs and the network does not include sk-counter. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2.2	Message definitions

#### –	RRCConnectionReconfiguration

The RRCConnectionReconfiguration message is the command to modify an RRC connection. It may convey information for measurement configuration, mobility control, conditional reconfigurations (conditional handover, conditional PSCell addition or inter-SN conditional PSCell change), radio resource configuration (including RBs, MAC main configuration and physical channel configuration) including any associated dedicated NAS information and security configuration.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

RRCConnectionReconfiguration message

-- ASN1START

RRCConnectionReconfiguration ::=	SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

rrcConnectionReconfiguration-r8		RRCConnectionReconfiguration-r8-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReconfiguration-r8-IEs ::= SEQUENCE {

measConfig							MeasConfig						OPTIONAL,	-- Need ON

mobilityControlInfo					MobilityControlInfo				OPTIONAL,	-- Cond HO

dedicatedInfoNASList				SEQUENCE (SIZE(1..maxDRB)) OF

DedicatedInfoNAS			OPTIONAL,	-- Cond nonHO

radioResourceConfigDedicated		RadioResourceConfigDedicated	OPTIONAL, -- Cond HO-toEUTRA

securityConfigHO					SecurityConfigHO				OPTIONAL,	-- Cond HO-toEPC

nonCriticalExtension				RRCConnectionReconfiguration-v890-IEs	OPTIONAL

}

RRCConnectionReconfiguration-v890-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING RRCConnectionReconfiguration-v8m0-IEs)	OPTIONAL,

nonCriticalExtension				RRCConnectionReconfiguration-v920-IEs	OPTIONAL

}

-- Late non-critical extensions:

RRCConnectionReconfiguration-v8m0-IEs ::= SEQUENCE {

-- Following field is only for pre REL-10 late non-critical extensions

lateNonCriticalExtension			OCTET STRING								OPTIONAL,

nonCriticalExtension				RRCConnectionReconfiguration-v10i0-IEs	OPTIONAL

}

RRCConnectionReconfiguration-v10i0-IEs ::= SEQUENCE {

antennaInfoDedicatedPCell-v10i0	AntennaInfoDedicated-v10i0		OPTIONAL,	-- Need ON

nonCriticalExtension				RRCConnectionReconfiguration-v10l0-IEs		OPTIONAL

}

RRCConnectionReconfiguration-v10l0-IEs ::= SEQUENCE {

mobilityControlInfo-v10l0			MobilityControlInfo-v10l0			OPTIONAL,

sCellToAddModList-v10l0			SCellToAddModList-v10l0			OPTIONAL,	-- Need ON

-- Following field is only for late non-critical extensions from REL-10 to REL-11

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				RRCConnectionReconfiguration-v12f0-IEs		OPTIONAL

}

RRCConnectionReconfiguration-v12f0-IEs ::= SEQUENCE {

scg-Configuration-v12f0			SCG-Configuration-v12f0		OPTIONAL,	-- Cond nonFullConfig

-- Following field is only for late non-critical extensions from REL-12

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				RRCConnectionReconfiguration-v1370-IEs		OPTIONAL

}

RRCConnectionReconfiguration-v1370-IEs ::= SEQUENCE {

radioResourceConfigDedicated-v1370	RadioResourceConfigDedicated-v1370	OPTIONAL, -- Need ON

sCellToAddModListExt-v1370			SCellToAddModListExt-v1370	OPTIONAL,	-- Need ON

nonCriticalExtension					RRCConnectionReconfiguration-v13c0-IEs	OPTIONAL

}

RRCConnectionReconfiguration-v13c0-IEs ::= SEQUENCE {

radioResourceConfigDedicated-v13c0	RadioResourceConfigDedicated-v13c0	OPTIONAL, -- Need ON

sCellToAddModList-v13c0				SCellToAddModList-v13c0		OPTIONAL,	-- Need ON

sCellToAddModListExt-v13c0			SCellToAddModListExt-v13c0	OPTIONAL,	-- Need ON

scg-Configuration-v13c0				SCG-Configuration-v13c0		OPTIONAL,	-- Need ON

-- Following field is only for late non-critical extensions from REL-13 onwards

nonCriticalExtension					SEQUENCE {}					OPTIONAL

}

-- Regular non-critical extensions:

RRCConnectionReconfiguration-v920-IEs ::= SEQUENCE {

otherConfig-r9						OtherConfig-r9				OPTIONAL,	-- Need ON

fullConfig-r9						ENUMERATED {true}			OPTIONAL,	-- Cond HO-Reestab

nonCriticalExtension				RRCConnectionReconfiguration-v1020-IEs	OPTIONAL

}

RRCConnectionReconfiguration-v1020-IEs ::= SEQUENCE {

sCellToReleaseList-r10			SCellToReleaseList-r10			OPTIONAL,	-- Need ON

sCellToAddModList-r10				SCellToAddModList-r10				OPTIONAL,	-- Need ON

nonCriticalExtension				RRCConnectionReconfiguration-v1130-IEs	OPTIONAL

}

RRCConnectionReconfiguration-v1130-IEs ::= SEQUENCE {

systemInformationBlockType1Dedicated-r11	OCTET STRING (CONTAINING SystemInformationBlockType1)																			OPTIONAL,	-- Need ON

nonCriticalExtension				RRCConnectionReconfiguration-v1250-IEs	OPTIONAL

}

RRCConnectionReconfiguration-v1250-IEs ::= SEQUENCE {

wlan-OffloadInfo-r12				CHOICE {

release								NULL,

setup									SEQUENCE {

wlan-OffloadConfigDedicated-r12		WLAN-OffloadConfig-r12,

t350-r12								ENUMERATED {min5, min10, min20, min30, min60,

min120, min180, spare1}	OPTIONAL	-- Need OR

}

}																	OPTIONAL,		-- Need ON

scg-Configuration-r12				SCG-Configuration-r12		OPTIONAL,	-- Cond nonFullConfig

sl-SyncTxControl-r12				SL-SyncTxControl-r12			OPTIONAL,	-- Need ON

sl-DiscConfig-r12					SL-DiscConfig-r12				OPTIONAL,	-- Need ON

sl-CommConfig-r12					SL-CommConfig-r12				OPTIONAL,	-- Need ON

nonCriticalExtension				RRCConnectionReconfiguration-v1310-IEs	OPTIONAL

}

RRCConnectionReconfiguration-v1310-IEs ::= SEQUENCE {

sCellToReleaseListExt-r13			SCellToReleaseListExt-r13		OPTIONAL,	-- Need ON

sCellToAddModListExt-r13			SCellToAddModListExt-r13		OPTIONAL,	-- Need ON

lwa-Configuration-r13				LWA-Configuration-r13			OPTIONAL,	-- Need ON

lwip-Configuration-r13				LWIP-Configuration-r13			OPTIONAL,	-- Need ON

rclwi-Configuration-r13				RCLWI-Configuration-r13			OPTIONAL,	-- Need ON

nonCriticalExtension				RRCConnectionReconfiguration-v1430-IEs						OPTIONAL

}

RRCConnectionReconfiguration-v1430-IEs ::= SEQUENCE {

sl-V2X-ConfigDedicated-r14		SL-V2X-ConfigDedicated-r14		OPTIONAL,	-- Need ON

sCellToAddModListExt-v1430		SCellToAddModListExt-v1430		OPTIONAL,	-- Need ON

perCC-GapIndicationRequest-r14	ENUMERATED{true}					OPTIONAL,	-- Need ON

systemInformationBlockType2Dedicated-r14	OCTET STRING (CONTAINING SystemInformationBlockType2)											OPTIONAL,	-- Cond nonHO

nonCriticalExtension			RRCConnectionReconfiguration-v1510-IEs		OPTIONAL

}

RRCConnectionReconfiguration-v1510-IEs ::= SEQUENCE {

nr-Config-r15					CHOICE {

release							NULL,

setup							SEQUENCE {

endc-ReleaseAndAdd-r15	BOOLEAN,

nr-SecondaryCellGroupConfig-r15	OCTET STRING				OPTIONAL,	-- Need ON

p-MaxEUTRA-r15					P-Max						OPTIONAL	-- Need ON

}

}																	OPTIONAL,	-- Need ON

sk-Counter-r15					INTEGER (0.. 65535)					OPTIONAL,	-- Need ON

nr-RadioBearerConfig1-r15		OCTET STRING						OPTIONAL,	-- Need ON

nr-RadioBearerConfig2-r15		OCTET STRING						OPTIONAL,	-- Need ON

tdm-PatternConfig-r15			TDM-PatternConfig-r15			OPTIONAL,	-- Cond FDD-PCell

nonCriticalExtension			RRCConnectionReconfiguration-v1530-IEs		OPTIONAL

}

RRCConnectionReconfiguration-v1530-IEs ::= SEQUENCE {

securityConfigHO-v1530				SecurityConfigHO-v1530			OPTIONAL,	-- Cond HO-5GC

sCellGroupToReleaseList-r15		SCellGroupToReleaseList-r15			OPTIONAL,	-- Need ON

sCellGroupToAddModList-r15		SCellGroupToAddModList-r15			OPTIONAL,	-- Need ON

dedicatedInfoNASList-r15		SEQUENCE (SIZE(1..maxDRB-r15)) OF

DedicatedInfoNAS			OPTIONAL,	-- Cond nonHO

p-MaxUE-FR1-r15					P-Max								OPTIONAL,	-- Need OR

smtc-r15						MTC-SSB-NR-r15						OPTIONAL,	-- Need OP

nonCriticalExtension			RRCConnectionReconfiguration-v1610-IEs		OPTIONAL

}

RRCConnectionReconfiguration-v1610-IEs ::= SEQUENCE {

conditionalReconfiguration-r16			ConditionalReconfiguration-r16	OPTIONAL, -- Need ON

daps-SourceRelease-r16					ENUMERATED{true}				OPTIONAL, -- Need ON

tdm-PatternConfig2-r16						TDM-PatternConfig-r15			OPTIONAL, -- Need ON

sl-ConfigDedicatedForNR-r16					OCTET STRING					OPTIONAL, -- Need OR

sl-SSB-PriorityEUTRA-r16					INTEGER (1..8)					OPTIONAL, -- Need OR

nonCriticalExtension			RRCConnectionReconfiguration-v1700-IEs		OPTIONAL

}

RRCConnectionReconfiguration-v1700-IEs ::= SEQUENCE {

systemInformationBlockType31Dedicated-r17	OCTET STRING (CONTAINING SystemInformationBlockType31-r17)																			OPTIONAL,	-- Cond NTN

scg-State-r17						ENUMERATED{deactivated}			OPTIONAL, -- Need OP

nonCriticalExtension						SEQUENCE {}						OPTIONAL

}

SL-SyncTxControl-r12 ::=			SEQUENCE {

networkControlledSyncTx-r12				ENUMERATED {on, off}		OPTIONAL	-- Need OP

}

PSCellToAddMod-r12 ::=				SEQUENCE {

sCellIndex-r12						SCellIndex-r10,

cellIdentification-r12				SEQUENCE {

physCellId-r12						PhysCellId,

dl-CarrierFreq-r12					ARFCN-ValueEUTRA-r9

}																	OPTIONAL,	-- Cond SCellAdd

radioResourceConfigCommonPSCell-r12		RadioResourceConfigCommonPSCell-r12	OPTIONAL,	-- Cond SCellAdd

radioResourceConfigDedicatedPSCell-r12	RadioResourceConfigDedicatedPSCell-r12	OPTIONAL,	-- Cond SCellAdd2

...,

[[	antennaInfoDedicatedPSCell-v1280		AntennaInfoDedicated-v10i0	OPTIONAL	-- Need ON

]],

[[	sCellIndex-r13					SCellIndex-r13	OPTIONAL		-- Need ON

]],

[[	radioResourceConfigDedicatedPSCell-v1370	RadioResourceConfigDedicatedPSCell-v1370	OPTIONAL	-- Need ON

]],

[[	radioResourceConfigDedicatedPSCell-v13c0	RadioResourceConfigDedicatedPSCell-v13c0	OPTIONAL	-- Need ON

]]

}

PSCellToAddMod-v12f0 ::=				SEQUENCE {

radioResourceConfigCommonPSCell-r12		RadioResourceConfigCommonPSCell-v12f0	OPTIONAL

}

PSCellToAddMod-v1440 ::=				SEQUENCE {

radioResourceConfigCommonPSCell-r14		RadioResourceConfigCommonPSCell-v1440	OPTIONAL

}

PowerCoordinationInfo-r12 ::= SEQUENCE {

p-MeNB-r12							INTEGER (1..16),

p-SeNB-r12							INTEGER (1..16),

powerControlMode-r12				INTEGER (1..2)

}

SCellToAddModList-r10 ::=		SEQUENCE (SIZE (1..maxSCell-r10)) OF SCellToAddMod-r10

SCellToAddModList-v10l0 ::=		SEQUENCE (SIZE (1..maxSCell-r10)) OF SCellToAddMod-v10l0

SCellToAddModList-v13c0 ::=		SEQUENCE (SIZE (1..maxSCell-r10)) OF SCellToAddMod-v13c0

SCellToAddModList-r16 ::=		SEQUENCE (SIZE (1..maxSCell-r13)) OF SCellToAddMod-r16

SCellToAddModListExt-r13 ::=	SEQUENCE (SIZE (1..maxSCell-r13)) OF SCellToAddModExt-r13

SCellToAddModListExt-v1370 ::=	SEQUENCE (SIZE (1..maxSCell-r13)) OF SCellToAddModExt-v1370

SCellToAddModListExt-v13c0 ::=	SEQUENCE (SIZE (1..maxSCell-r13)) OF SCellToAddMod-v13c0

SCellToAddModListExt-v1430 ::=	SEQUENCE (SIZE (1..maxSCell-r13)) OF SCellToAddModExt-v1430

SCellGroupToAddModList-r15 ::=	SEQUENCE (SIZE (1..maxSCellGroups-r15)) OF SCellGroupToAddMod-r15

SCellToAddMod-r10 ::=			SEQUENCE {

sCellIndex-r10						SCellIndex-r10,

cellIdentification-r10				SEQUENCE {

physCellId-r10						PhysCellId,

dl-CarrierFreq-r10					ARFCN-ValueEUTRA

}																OPTIONAL,	-- Cond SCellAdd

radioResourceConfigCommonSCell-r10		RadioResourceConfigCommonSCell-r10	OPTIONAL,	-- Cond SCellAdd

radioResourceConfigDedicatedSCell-r10	RadioResourceConfigDedicatedSCell-r10	OPTIONAL,	-- Cond SCellAdd2

...,

[[	dl-CarrierFreq-v1090				ARFCN-ValueEUTRA-v9e0	OPTIONAL	-- Cond EARFCN-max

]],

[[	antennaInfoDedicatedSCell-v10i0		AntennaInfoDedicated-v10i0	OPTIONAL	-- Need ON

]],

[[	srs-SwitchFromServCellIndex-r14		INTEGER (0.. 31) OPTIONAL	-- Need ON

]],

[[	sCellState-r15						ENUMERATED {activated, dormant}	OPTIONAL	-- Need ON

]]

}

SCellToAddMod-v10l0 ::=			SEQUENCE {

radioResourceConfigCommonSCell-v10l0		RadioResourceConfigCommonSCell-v10l0	OPTIONAL

}

SCellToAddMod-v13c0 ::=			SEQUENCE {

radioResourceConfigDedicatedSCell-v13c0	RadioResourceConfigDedicatedSCell-v13c0	OPTIONAL

}

SCellToAddMod-r16 ::=			SEQUENCE {

sCellIndex-r16						SCellIndex-r13,

cellIdentification-r16				SEQUENCE {

physCellId-r16						PhysCellId,

dl-CarrierFreq-r16					ARFCN-ValueEUTRA-r9

}																OPTIONAL,	-- Cond SCellAdd

radioResourceConfigCommonSCell-r16		RadioResourceConfigCommonSCell-r10	OPTIONAL,	-- Cond SCellAdd

radioResourceConfigDedicatedSCell-r16	RadioResourceConfigDedicatedSCell-r10	OPTIONAL,	-- Cond SCellAdd2

antennaInfoDedicatedSCell-r16		AntennaInfoDedicated-v10i0	OPTIONAL,	-- Need ON

srs-SwitchFromServCellIndex-r16			INTEGER (0.. 31) OPTIONAL,	-- Need ON

sCellState-r16							ENUMERATED {activated, dormant}	OPTIONAL,	-- Need ON

...

}

SCellToAddModExt-r13 ::=			SEQUENCE {

sCellIndex-r13						SCellIndex-r13,

cellIdentification-r13				SEQUENCE {

physCellId-r13						PhysCellId,

dl-CarrierFreq-r13					ARFCN-ValueEUTRA-r9

}																OPTIONAL,	-- Cond SCellAdd

radioResourceConfigCommonSCell-r13		RadioResourceConfigCommonSCell-r10	OPTIONAL,	-- Cond SCellAdd

radioResourceConfigDedicatedSCell-r13	RadioResourceConfigDedicatedSCell-r10	OPTIONAL,	-- Cond SCellAdd2

antennaInfoDedicatedSCell-r13			AntennaInfoDedicated-v10i0		OPTIONAL	-- Need ON

}

SCellToAddModExt-v1370 ::=			SEQUENCE {

radioResourceConfigCommonSCell-v1370		RadioResourceConfigCommonSCell-v10l0	OPTIONAL

}

SCellToAddModExt-v1430 ::=			SEQUENCE {

srs-SwitchFromServCellIndex-r14			INTEGER (0.. 31)			OPTIONAL,	-- Need ON

...,

[[	sCellState-r15					ENUMERATED {activated, dormant}		OPTIONAL	-- Need ON

]]

}

SCellGroupToAddMod-r15 ::=			SEQUENCE {

sCellGroupIndex-r15					SCellGroupIndex-r15,

sCellConfigCommon-r15				SCellConfigCommon-r15			OPTIONAL,	-- Need ON

sCellToReleaseList-r15				SCellToReleaseListExt-r13		OPTIONAL,	-- Need ON

sCellToAddModList-r15				SCellToAddModListExt-r13		OPTIONAL	-- Need ON

}

SCellToReleaseList-r10 ::=			SEQUENCE (SIZE (1..maxSCell-r10)) OF SCellIndex-r10

SCellToReleaseListExt-r13 ::=			SEQUENCE (SIZE (1..maxSCell-r13)) OF SCellIndex-r13

SCellGroupToReleaseList-r15 ::=			SEQUENCE (SIZE (1..maxSCellGroups-r15)) OF SCellGroupIndex-r15

SCellGroupIndex-r15 ::=			INTEGER (1..maxSCellGroups-r15)

SCellConfigCommon-r15 ::= SEQUENCE {

radioResourceConfigCommonSCell-r15		RadioResourceConfigCommonSCell-r10	OPTIONAL,	-- Need ON

radioResourceConfigDedicatedSCell-r15	RadioResourceConfigDedicatedSCell-r10	OPTIONAL,-- Need ON

antennaInfoDedicatedSCell-r15			AntennaInfoDedicated-v10i0		OPTIONAL	-- Need ON

}

SCG-Configuration-r12 ::=			CHOICE {

release								NULL,

setup								SEQUENCE {

scg-ConfigPartMCG-r12				SEQUENCE {

scg-Counter-r12						INTEGER (0.. 65535)			OPTIONAL,	-- Need ON

powerCoordinationInfo-r12			PowerCoordinationInfo-r12	OPTIONAL,	-- Need ON

...

}																OPTIONAL,	-- Need ON

scg-ConfigPartSCG-r12				SCG-ConfigPartSCG-r12		OPTIONAL	-- Need ON

}

}

SCG-Configuration-v12f0 ::=			CHOICE {

release								NULL,

setup								SEQUENCE {

scg-ConfigPartSCG-v12f0				SCG-ConfigPartSCG-v12f0		OPTIONAL	-- Need ON

}

}

SCG-Configuration-v13c0 ::=			CHOICE {

release								NULL,

setup								SEQUENCE {

scg-ConfigPartSCG-v13c0				SCG-ConfigPartSCG-v13c0		OPTIONAL	-- Need ON

}

}

SCG-ConfigPartSCG-r12 ::=			SEQUENCE {

radioResourceConfigDedicatedSCG-r12	RadioResourceConfigDedicatedSCG-r12	OPTIONAL,	-- Need ON

sCellToReleaseListSCG-r12			SCellToReleaseList-r10		OPTIONAL,	-- Need ON

pSCellToAddMod-r12					PSCellToAddMod-r12			OPTIONAL,	-- Need ON

sCellToAddModListSCG-r12			SCellToAddModList-r10		OPTIONAL,	-- Need ON

mobilityControlInfoSCG-r12			MobilityControlInfoSCG-r12	OPTIONAL,	-- Need ON

...,

[[

sCellToReleaseListSCG-Ext-r13			SCellToReleaseListExt-r13		OPTIONAL,	-- Need ON

sCellToAddModListSCG-Ext-r13			SCellToAddModListExt-r13		OPTIONAL	-- Need ON

]],

[[

sCellToAddModListSCG-Ext-v1370		SCellToAddModListExt-v1370	OPTIONAL	-- Need ON

]],

[[

pSCellToAddMod-v1440				PSCellToAddMod-v1440		OPTIONAL	-- Need ON

]],

[[	sCellGroupToReleaseListSCG-r15	SCellGroupToReleaseList-r15	OPTIONAL,	-- Need ON

sCellGroupToAddModListSCG-r15	SCellGroupToAddModList-r15	OPTIONAL	-- Need ON

]],

[[	-- NE-DC addition for setup/ modification and release SN configured measurements

measConfigSN-r15				MeasConfig						OPTIONAL,	-- Need ON

-- NE-DC additions concerning DRBs/ SRBs are within RadioResourceConfigDedicatedSCG

tdm-PatternConfigNE-DC-r15		TDM-PatternConfig-r15			OPTIONAL	-- Cond FDD-PSCell

]],

[[	p-MaxEUTRA-r15					P-Max							OPTIONAL	-- Need ON

]]

}

SCG-ConfigPartSCG-v12f0 ::=			SEQUENCE {

pSCellToAddMod-v12f0				PSCellToAddMod-v12f0		OPTIONAL,	-- Need ON

sCellToAddModListSCG-v12f0			SCellToAddModList-v10l0		OPTIONAL	-- Need ON

}

SCG-ConfigPartSCG-v13c0 ::=			SEQUENCE {

sCellToAddModListSCG-v13c0			SCellToAddModList-v13c0		OPTIONAL,	-- Need ON

sCellToAddModListSCG-Ext-v13c0		SCellToAddModListExt-v13c0	OPTIONAL	-- Need ON

}

SecurityConfigHO ::=				SEQUENCE {

handoverType						CHOICE {

intraLTE							SEQUENCE {

securityAlgorithmConfig				SecurityAlgorithmConfig		OPTIONAL,	-- Cond fullConfig

keyChangeIndicator					BOOLEAN,

nextHopChainingCount				NextHopChainingCount

},

interRAT							SEQUENCE {

securityAlgorithmConfig				SecurityAlgorithmConfig,

nas-SecurityParamToEUTRA			OCTET STRING (SIZE(6))

}

},

...

}

SecurityConfigHO-v1530 ::=		SEQUENCE {

handoverType-v1530				CHOICE {

intra5GC							SEQUENCE {

securityAlgorithmConfig-r15			SecurityAlgorithmConfig		OPTIONAL,	-- Cond HO-toEUTRA

keyChangeIndicator-r15				BOOLEAN,

nextHopChainingCount-r15			NextHopChainingCount,

nas-Container-r15					OCTET STRING	OPTIONAL	-- Need ON

},

fivegc-ToEPC						SEQUENCE {

securityAlgorithmConfig-r15			SecurityAlgorithmConfig,

nextHopChainingCount-r15			NextHopChainingCount

},

epc-To5GC						SEQUENCE {

securityAlgorithmConfig-r15		SecurityAlgorithmConfig,

nas-Container-r15				OCTET STRING

}

},

...

}

-- ASN1STOP

RRCConnectionReconfiguration field descriptions

conditionalReconfiguration This field is used to configure the UE with a conditional reconfiguration. The reconfiguration is applied when the execution condition(s) is fulfilled. The field is absent if daps-HO is configured for any DRB or if MobilityControlInfo is included in the RRCConnectionReconfiguration message. The conditionalReconfiguration is not configured in the RRCConnectionReconfiguration message included in a conditionalReconfiguration.

daps-SourceRelease A one-shot field that indicates that the UE shall release the resources associated with source PCell at a DAPS HO, including reconfiguration of the PDCP entity to release DAPS.

dedicatedInfoNASList This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for each PDU in the list. If dedicatedInfoNASList-r15 is present, UE shall ignore the dedicatedInfoNASList (without suffix).

endc-ReleaseAndAdd A one-shot field indicating whether the UE simultaneously releases and adds all the NR SCG related configuration within nr-Config, i.e. the configuration set by the NR RRCReconfiguration message (e.g. secondaryCellGroup, SRB3 and measConfig).

fullConfig Indicates the full configuration option is applicable for the RRC Connection Reconfiguration message for intra-system intra-RAT handover. For inter-RAT handover from NR to E-UTRA, fullConfig indicates whether or not delta signalling of SDAP/PDCP from source RAT is applicable. This field is absent when the RRCConnectionReconfiguration message is generated by the E-UTRA SCG.

keyChangeIndicator If UE is connected to EPC, true is used only in an intra-cell handover when a KeNB key is derived from a KASME key taken into use through the latest successful NAS SMC procedure, as described in TS 33.401 [32] for KeNB re-keying. false is used in an intra-LTE handover when the new KeNB key is obtained from the current KeNB key or from the NH as described in TS 33.401 [32]. If UE is connected to 5GC, with keyChangeIndicator-r15, true is used in an intra-cell handover when a KeNB key is derived from a KAMF key taken into use through the latest successful NAS SMC procedure, as described in TS 33.501 [86] for KeNB re-keying. False is used for intra-system handover when the new KeNB key is obtained from the current KeNB key or from the NH as described in TS 33.501 [86]. True is also used in NG based handover procedure with KAMF change, when a KeNB key is derived from the new KAMF key as described in TS 33.501 [86].

lwa-Configuration This field is used to provide parameters for LWA configuration. E-UTRAN does not simultaneously configure LWA with DC, LWIP or RCLWI for a UE.

lwip-Configuration This field is used to provide parameters for LWIP configuration. E-UTRAN does not simultaneously configure LWIP with DC, LWA or RCLWI for a UE.

measConfig Measurements that E-UTRAN may configure when the UE is not configured with NE-DC.

measConfigSN Measurements that E-UTRAN may configure when the UE is configured with NE-DC and for which reports are carried within an NR RRC message.

nas-Container This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for this field, although, if included, it affects activation of AS- security after handover within E-UTRA/5GC. The content is defined in TS 24.501 [95]. In case of NG based handover, the content of nas-Container is. the Intra N1 mode NAS transparent container IE. In case of inter-system handover to from 5GS to EPS, the content of NAS-Container is. the S1 mode to N1 mode NAS transparent container IE.

nas-securityParamToEUTRA This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for this field, although, if included, it affects activation of AS- security after inter-RAT handover to E-UTRA/EPC or inter-system handover to E-UTRA/EPC. The content is defined in TS 24.301 [35]. This field is not used for handover from 5GC.

networkControlledSyncTx This field indicates whether the UE shall transmit synchronisation information (i.e. become synchronisation source). Value On indicates the UE to transmit synchronisation information while value Off indicates the UE to not transmit such information.

nextHopChainingCount Parameter NCC: See TS 33.401 [32] if UE is connected to EPC, else see 33.501 [86] if UE is connected to 5GC.

nr-Config Includes the NR related configurations. This field is used to (re)configure or release (NG)EN-DC, possibly in conjunction with fields sk-Counter and nr-RadioBearerConfig1/ 2. NOTE 1.

nr-RadioBearerConfig1, nr-RadioBearerConfig2 Includes the NR RadioBearerConfig IE as specified in TS 38.331 [82]. The field includes the configuration of RBs configured with NR PDCP.

nr-SecondaryCellGroupConfig Includes the NR RRCReconfiguration message as specified in TS 38.331 [82]. In this version of the specification, the NR RRC message only includes fields secondaryCellGroup, conditionalReconfiguration, otherConfig, bap-Config, iab-IP-AddressConfigurationList and/ or measConfig. If nr-SecondaryCellGroupConfig is configured, the network always includes this field upon MN handover to initiate an NR SCG reconfiguration with sync and key change.

perCC-GapIndicationRequest Indicates that UE shall include perCC-GapIndicationList and numFreqEffective in the RRCConnectionReconfigurationComplete message. numFreqEffectiveReduced may also be included if frequencies are configured for reduced measurement performance.

p-MaxEUTRA Indicates the maximum power available for LTE.

p-MaxUE-FR1 The maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1) across all cell groups. The maximum transmit power that the UE may use may be additionally limited on cell- or cell-group level. The field is optionally present, if (NG)EN-DC (nr-Config-r15) has been configured. It is absent otherwise.

p-MeNB Indicates the guaranteed power for the MeNB, as specified in TS 36.213 [23]. The value N corresponds to N-1 in TS 36.213 [23].

powerControlMode Indicates the power control mode used in DC. Value 1 corresponds to DC power control mode 1 and value 2 indicates DC power control mode 2, as specified in TS 36.213 [23].

p-SeNB Indicates the guaranteed power for the SeNB as specified in TS 36.213 [23], Table 5.1.4.2-1. The value N corresponds to N-1 in TS 36.213 [23].

rclwi-Configuration WLAN traffic steering command as specified in 5.6.16.2. E-UTRAN does not simultaneously configure RCLWI with DC, LWA or LWIP for a UE.

sCellConfigCommon Indicates the common configuration for the SCell group.

sCellGroupIndex Indicates the identity of SCell groups for which a common configuration is provided.

sCellIndex The sCellIndex is unique within the scope of the UE. In case of DC, an SCG cell can not use the same value as used for an MCG cell. For pSCellToAddMod, if sCellIndex-r13 is present the UE shall ignore sCellIndex-r12.

sCellGroupToAddModList, sCellGroupToAddModListSCG Indicates the SCell group to be added or modified. E-UTRAN only configures at most 4 SCell groups per UE over all cell groups. SCell groups can only be configured for LTE SCells, and all SCells in an SCell group must belong to the same cell group.

sCellGroupToReleaseList Indicates the SCell group to be released.

sCellState A one-shot field that indicates whether the SCell shall be considered to be in activated or dormant state upon SCell configuration.

sCellToAddModList, sCellToAddModListExt Indicates the SCell to be added or modified. E-UTRAN uses field sCellToAddModList-r10 to add or modify SCells (with sCellIndex-r10) for a UE that does not support carrier aggregation with more than 5 component carriers. If E-UTRAN includes sCellToAddModListExt-v1430 it includes the same number of entries, and listed in the same order, as in sCellToAddModListExt-r13. If E-UTRAN includes sCellToAddModList-v10l0 it includes the same number of entries, and listed in the same order, as in sCellToAddModList-r10. If E-UTRAN includes sCellToAddModListExt-v1370 it includes the same number of entries, and listed in the same order, as in sCellToAddModListExt-r13. If E-UTRAN includes sCellToAddModListExt-v13c0 it includes the same number of entries, and listed in the same order, as in sCellToAddModListExt-r13.

sCellToAddModListSCG, sCellToAddModListSCG-Ext Indicates the SCG cell to be added or modified. The field is used for SCG cells other than the PSCell (which is added/ modified by field pSCellToAddMod). E-UTRAN uses field sCellToAddModListSCG-r12 to add or modify SCells (with sCellIndex-r10) for a UE that does not support carrier aggregation with more than 5 component carriers. If E-UTRAN includes sCellToAddModListSCG-v10l0 it includes the same number of entries, and listed in the same order, as in sCellToAddModListSCG-r12. If E-UTRAN includes sCellToAddModListSCG-Ext-v1370 it includes the same number of entries, and listed in the same order, as in sCellToAddModListSCG-Ext-r13. If E-UTRAN includes sCellToAddModListSCG-Ext-v13c0 it includes the same number of entries, and listed in the same order, as in sCellToAddModListSCG-Ext-r13.

sCellToReleaseList, sCellToReleaseListExt Indicates the SCell to be released. E-UTRAN uses field sCellToReleaseList-r10 to release SCells for a UE that does not support carrier aggregation with more than 5 component carriers.

sCellToReleaseListSCG, sCellToReleaseListSCG-Ext Indicates the SCG cell to be released. The field is also used to release the PSCell e.g. upon change of PSCell, upon system information change for the PSCell. E-UTRAN uses field sCellToReleaseListSCG-r12 to release SCells for a UE that does not support carrier aggregation with more than 5 component carriers.

scg-Configuration Covers the SCG configuration as used in case of DC and NE-DC. When the UE is configured with NE-DC, E-UTRAN neither applies value release nor configures scg-ConfigPartMCG. When resuming a connection with NE-DC, this field is included, containing at least the mobilityControlInfoSCG.

scg-Counter A counter used upon initial configuration of SCG security as well as upon refresh of S-KeNB. E-UTRAN includes the field upon SCG change when one or more SCG DRBs are configured. Otherwise E-UTRAN does not include the field.

scg-State Indicates that the NR SCG is deactivated. The field is absent if CPA or CPC is configured for the UE, or if the RRCConnectionReconfiguration message is contained in condReconfigurationToApply.

securityConfigHO This field contains the parameters required to update the security keys at handover. If E-UTRAN includes the securityConfigHO (i.e., without suffix), the choice intraLTE is used for handover within E-UTRA/EPC while the choice interRAT is used for handover from GERAN or UTRAN to E-UTRA/EPC. If E-UTRAN includes the securityConfigHO-v1530 (i.e., with suffix), the choice intra5GC is used for handover from NR or E-UTRA/5GC to E-UTRA/5GC while the choice fivegc-ToEPC is used for inter-system handover from NR or E-UTRA/5GC to E-UTRA/EPC and the choice epc-To5GC is used for inter-system handover from E-UTRA/EPC to E-UTRA/5GC.

sk-Counter A one-shot counter used upon initial configuration of S-KgNB as well as upon refresh of S-KgNB. E-UTRAN always provides this field either upon initial configuration of an NR SCG, or upon configuration of the first (SN terminated) RB using S-KgNB, whichever happens first. E-UTRAN always includes this field in the first reconfiguration after RRC re-establishment when one or more RB(s) configured with NR PDCP with keyToUse set to secondary are configured and not released.

sl-ConfigDedicatedForNR Container for providing the dedicated configurations for NR sidelink communication, the octet string contains the NR RRCReconfiguration message as specified in TS 38.331 [82]. In this version of the specification, the NR RRC message only includes fields related to NR sidelink communication, i.e. sl-ConfigDedicatedNR, measConfig and/or otherConfig. If the UE is configured by the current Pcell with sl-ScheduledConfig set to setup (i.e., NR sidelink communication mode 1), the network only includes sl-PrioritizationThres and sl-ConfiguredGrantConfig that only includes the configurations of configured sidelink grant Type 1 in the field sl-ScheduledConfig. This field is not applicable to 5GS Proximity based Services (ProSe) as defined in TS 23.304 [112] in this release.

sl-SSB-PriorityEUTRA Indicates the priority of LTE PSSS/SSSS/PSBCH transmission and reception. NOTE 3.

sl-V2X-ConfigDedicated Indicates sidelink configuration for non-P2X related V2X sidelink communication as well as P2X related V2X sidelink communication.

smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell addition and SN change. It is based on timing reference of EUTRA PCell. NOTE 2. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

srs-SwitchFromServCellIndex Indicates the serving cell whose UL transmission may be interrupted during SRS transmission on a PUSCH-less cell. During SRS transmission on a PUSCH-less cell, the UE may temporarily suspend the UL transmission on a serving cell with PUSCH in the same CG to allow the PUSCH-less cell to transmit SRS. The PUSCH-less cell is always a TDD cell but the serving cell with PUSCH may be either a FDD or TDD cell.

systemInformationBlockType1Dedicated This field is used to transfer SystemInformationBlockType1 or SystemInformationBlockType1-BR to the UE.

systemInformationBlockType2Dedicated This field is used to transfer BR version of SystemInformationBlockType2 to BL UEs or UEs in CE or SystemInformationBlockType2 to non-BL UEs.

systemInformationBlockType31Dedicated This field is used to transfer SystemInformationBlockType31 to BL UEs or UEs in CE for an NTN cell.

t350 Timer T350 as described in clause 7.3. Value minN corresponds to N minutes.

tdm-PatternConfig This field is used when power control or IMD issues require single UL transmission in (NG)EN-DC as specified in TS 38.101-3 [101] and TS 38.213 [88].

tdm-PatternConfig2 This field is used for dual UL transmission in EN-DC with LTE FDD PCell and for single UL transmission in EN-DC with LTE FDD/TDD PCell, as specified in TS 38.101-3 [101] and TS 38.213 [88]. The network sets at most one of tdm-PatternConfig and tdm-PatternConfig2 to setup. When this field is configured in EN-DC with LTE TDD PCell, it is not applicable if TDD configuration is sa0 or sa6 in SIB1.

tdm-PatternConfigNE-DC This field is used when power control or IMD issues require single UL transmission in NE-DC as specified in TS 38.101-3 [101] and TS 38.213 [88].

| Conditional presence | Explanation |
| --- | --- |
| EARFCN-max | The field is mandatory present if dl-CarrierFreq-r10 is included and set to maxEARFCN. Otherwise the field is not present. |
| FDD-PCell | This field is optionally present, need ON, for a FDD PCell if there is no SCell with configured uplink. Otherwise, the field is not present. |
| FDD-PSCell | This field is optionally present, need ON, for a FDD PSCell if there is no SCell with configured uplink. Otherwise, the field is not present. |
| fullConfig | This field is mandatory present for handover within E-UTRA when the fullConfig is included; otherwise it is optionally present, Need OP. |
| HO | The field is mandatory present in case of handover within E-UTRA or to E-UTRA and in a message contained in a NR DLInformationTransferMRDC message; otherwise the field is not present. The field is not present if source PCell resources after a DAPS handover have not been released. |
| HO-Reestab | The field is mandatory present in case of inter-system handover within E-UTRA or handover from NR to E-UTRA/EPC; it is optionally present, need ON, in case of intra-system handover within E-UTRA or upon the first reconfiguration after RRC connection re-establishment; or for intra-system handover from NR to E-UTRA, otherwise the field is not present. |
| HO-5GC | The field is mandatory present in case of handover within E-UTRA/5GC, handover to E-UTRA/5GC, handover from NR to E-UTRA/EPC, or handover from E-UTRA/5GC to E-UTRA/EPC, otherwise the field is not present. |
| HO-toEPC | The field is mandatory present in case of handover within E-UTRA/EPC or to E-UTRA/EPC, except handover from NR or E-UTRA/5GC, otherwise the field is not present. |
| HO-toEUTRA | The field is mandatory present in case of handover to E-UTRA or for reconfigurations when fullConfig is included; otherwise the field is optionally present, need ON. |
| nonFullConfig | The field is not present when the fullConfig is included or in case of handover to E-UTRA; otherwise it is optional present, need ON. |
| nonHO | The field is not present in case of handover within E-UTRA or to E-UTRA; otherwise it is optional present, need ON. |
| NTN | The field is mandatory present in case of handover to a NTN cell. Otherwise the field is optionally present, Need ON, in a NTN cell. |
| SCellAdd | The field is mandatory present upon SCell addition; otherwise it is not present. |
| SCellAdd2 | The field is mandatory present upon SCell addition; otherwise it is optionally present, need ON. |

NOTE 1:	Fields sk-Counter and nr-RadioBearerConfig1/ 2 are placed outside nr-Config, as these may be configured while the UE is not configured with (NG)EN-DC.

NOTE 2:	It is not specified whether the timing reference for the SMTC configuration is the source EUTRA PCell or the target EUTRA PCell in case the NR PSCell addition or SN change takes place simultaneously with handover. As a consequence, explicit SMTC configuration is only supported when the source EUTRA PCell and the target EUTRA PCell of the handover are SFN/subframe-synchronized.

NOTE 3:	For UEs in RRC_IDLE, RRC_INACTIVE or out-of coverage, and for the case that sl-SSB-PriorityEUTRA is absent, it is up to UE implementation to decide the priority of LTE PSSS/SSSS/PSBCH transmission and reception.