3GPP TSG-RAN WG2 #129	R2-2500969

# Athens, Greece, 17th – 21th February 2024

Agenda Item:	 8.1.2.3

Source:	Ericsson

Title:	LCM For Positioning

Document for:	Discussion

# 1	Introduction

The following agreements on the topic of LCM for the positioning Case 1 in RAN2#127bis and RAN2#128.

In this paper we discuss the below aspects for whether to work on 2nd priority Case 2b and the 1st priority Case 1 (UE-based positioning with UE-side model, direct AI/ML positioning):

- RAN2’s opinion on working on 2nd priority Case 2b

- New Method or update of existing method

- Stage 2 Draft CR

# 2	Discussion

## 2nd Priority Case 2b

There is an LS (S2-2501133) from SA2 requesting RAN1 and RAN2 to provide current progress and future work plans on data types and procedures defined for Case 2b. Case 2b is 2nd priority case, and RAN2 has not yet started discussion on Case 2b. Considering the current focus is to complete Case 1 and only few meetings left, RAN2 should suggest SA2 to drop the study of Case 2b.

RAN2 should not study Case 2b in Release 19. Indicate this to SA2.

## Stage 2 Description “New Method For case 1”

Based on the [POST128][026][AIML] LCM Procedure for Positioning Case1 (Ericsson) email discussion, majority companies favour to take Case 1 as a new method as case 1 cannot be integrated with legacy method and also for leaner design, better forward compatibility consideration. The potential stage2 procedure for Case 1 as a new positioning method is described below. It can be captured in a new section in TS 38.305 Section 8.X

#### 8.X	UE-based direct AI/ML positioning with UE-side model

For the positioning use case, UE-sided model:

- the baseline LCM procedure is:

  - Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP RequestCapabilities message.

  - Step 2: UE sends LPP ProvideCapabilities message to LMF with the supported functionalities at the UE side.

  - Step 3: LMF sends the LPP ProvideAssistanceData message (which may contain network side additional condition).

  - Step 4: UE reports the applicable functionality to the LMF by the LPP ProvideCapabilities message in response to Step 3 (“reactive” way).

    - The UE can report the applicable functionality also in “proactive” way by sending an unsolicited LPP ProvideCapabilities message to LMF when the applicability changes.

  - Step 5: The LMF requests the inferred location information using the LPP RequestLocationInformation message.

  - Step 6: UE reports the inferred location using LPP ProvideLocationInformation message.

As a baseline, if a functionality (e.g., AIML-based positioning method) becomes non-applicable in the UE, when LMF requests UE location estimation, the UE cannot perform the AIML-based positioning and reply with LPP ProvidelocationInformation message with error cause.

The proposed Stage 2 procedures is taken as baseline for Case 1 as a new positioning method.

## Training Data Collection for Case 1

### 2.3.1 Using Legacy Procedure for AD request for training

As per RAN1 agreements (Only keeping option for Case 1):

Agreement For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

The training data collection for Case 1, which only requires PRS configuration, is similar to existing procedure where UE requests assistance data of PRS configuration for legacy positioning measurements. Thus legacy procedure/ways for UE to request PRS configuration including UE-initiated assistance data request and UE-initiated on-demand PRS procedure can be reused. The training data collection for Case 1 is part of procedure for AI/ML positioning, and the PRS configuration request for this should not be either prioritised or deprioritized compared with legacy positioning, thus UE does not include the purpose of request when sending request for PRS configuration.

For data collection for AI/ML models on the UE side, the UE may request assistance data to LMF using UE-initiated assistance data request procedure as specified in clause 8.12.3.1.2.2 of TS 38.305 or UE-initiated on-demand PRS procedure as specified in clause 7.6.2 of TS 38.305 without the need to include the purpose of request.

Legacy procedure can be reused for data collection for case1.

### 2.3.2 Generating Label for data collection and associated parameters

RAN1 Agreements Working Assumption For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Working Assumption For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of the label and its related data is out of RAN1 scope.

AI/ML is used in area where there can be NLOS. Further, the PRS symbols may not be transmitted with perfect sync and there may be Real Time Difference. The LMF should be able to guide what is the expected positioning error during training phase to the UE based upon related Integrity parameters (e.g., standard deviation of RTD) so that the UE can understand if the obtained positioning can be used as a label (ground truth) or should be discarded. Similar to legacy expectedRSTD where LMF provides the expected RSTD value so that the UE can filter any outliers, the LMF for AI/ML training may provide maximum uncertainty/error so that the UE can judge if the obtained positioning can be considered as ground truth or should be discarded.

Similar to legacy expectedRSTD where LMF provides the expected RSTD value so that the UE can filter any outliers, the LMF for the case of AI/ML training can provide maximum uncertainty/error so that the UE can judge if the obtained positioning can be considered as ground truth or should be discarded.

LMF provides the guidance for trustworthy label based upon Positioning Integrity (maximum allowed Positioning error or uncertainty for a trustworthy label)

# Conclusion

In the previous sections we made the following observations:

Observation 1	For data collection for AI/ML models on the UE side, the UE may request assistance data to LMF using UE-initiated assistance data request procedure as specified in clause 8.12.3.1.2.2 of TS 38.305 or UE-initiated on-demand PRS procedure as specified in clause 7.6.2 of TS 38.305 without the need to include the purpose of request.

Observation 2	Similar to legacy expectedRSTD where LMF provides the expected RSTD value so that the UE can filter any outliers, the LMF for the case of AI/ML training can provide maximum uncertainty/error so that the UE can judge if the obtained positioning can be considered as ground truth or should be discarded.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 should not study Case 2b in Release 19. Indicate this to SA2.

Proposal 2	The proposed Stage 2 procedures is taken as baseline for Case 1 as a new positioning method.

Proposal 3	Legacy procedure can be reused for data collection for case1.

Proposal 4	LMF provides the guidance for trustworthy label based upon Positioning Integrity (maximum allowed Positioning error or uncertainty for a trustworthy label)

# References

TS 38.305, Stage2 for Positioning