3GPP TSG RAN2 Meeting #129							R2-2500970

Athens, Greece, February 17-21st, 2025

Agenda Item:	8.18

Source:	Ericsson, AT&T, FirstNet, Deutsche Telekom, MediaTek Inc.

Title:	Introduction of Location Coordinate Reference Systems

Document for:	Discussion, Decision

# 1	Global vs Regional Coordinate Reference Systems

A geodetic coordinate reference system defines coordinates in relation to the earth – a datum. Geodetic coordinates in 3GPP are defined in WGS 84 and the most recent versions of WGS84 are aligned to the ITRF datum. This means that they are well defined globally. However, nationally and regionally, coordinates are defined in different datums – reference surfaces that represent the specific region well - to ensure that coordinates match regional and national maps, considering tectonic plate movements horizontally and vertically.

For example, the North American Datum, NAD83, differs from ITRF by about 2 meters, and the European Terrestrial Reference Frame, ETRF89, differs from ITRF by about 1 meter. Such a difference was negligible when positioning accuracy requirements were rather loose, which was the case when 3GPP decided to represent coordinates only in WGS84/ITRF. However, today, positioning accuracy requirements are much tighter and the positioning capabilities much better – plentiful of use cases rely on ca few centimeters precision in relation to maps, e.g. TR 22.872, TR 38.857. Therefore, it is highly relevant that the coordinate representation in 3GPP becomes more flexible with alternatives of coordinates in regional coordinate reference systems to adapt to use case needs and regional maps. Indeed, it is also important to know which version (epoch) of WGS84 has been adopted in the 3GPP network implementation to enable translations through time at global and regional scales.

3GPP LPP introduced support for assistance of high accuracy Global Navigation Satellite Systems (GNSS) in Rel 15 based on RTCM SC 104 v.3.3 [1], released in October 2016, to enable centimeter precision. Since then, 3GPP Rel 16 and 17 has introduced support for decimeter precision cellular 5G positioning, and Rel 17 and 18 has introduced support for integrity to define error sources and mechanisms to assess positioning reliability.

The current 3GPP RAN support for only WGS84 CRS has the following shortcomings:

- Coordinates only defined in WGS84 without an associated epoch time are not well-defined.

- Site surveys of TRP antenna locations are typically performed in regional CRS so the network (RAN OAM or E-SMLC/LMF) needs to translate these coordinates to WGS84.

- Regional GNSS corrections providers typically provides data in regional CRS, so either they need to instead provide data in WGS84 or network needs to transform the reference station coordinates from regional CRS to WGS84.

- UEs needs to transform its estimated position in WGS84 to a regional CRS in order to align to maps.

- RTCM has the ability to align all served devices by data in a regional CRS or via a signaled CRS transformation in an eco-system, while 3GPP LPP will not be able to align devices causing a reliability risk with potentially different understandings of the CRS and relations to maps.

3GPP RAN only representing coordinates in WGS84 implies extensive coordinate transformations in different nodes, and it is not possible to remain within a regional CRS that is the established CRS in a region/nation.

Coordinate transformations in network and devices without a common set of transformation parameters implies a reliability and integrity risk

RTCM SC 104 v3.4 [2] was released in December 2023, and it features messages to represent regional and national coordinate reference systems (CRS) and parameterized coordinate transformation formulas to enable conversion between WGS 84 and regional/national CRS that are specific to countries and parts of countries that will stay relevant over long time.

RTCM has published messages for regional CRS identification and CRS transformation parameters that 3GPP RAN can leverage to introduce support for regional CRS and CRS transformations.

It is therefore timely to make sure that 3GPP RAN, leveraged by RTCM [2], also supports regional CRS and CRS transformations which are relevant for both RAT-dependent and RAT-independent positioning methods including handling of location coordinates of in assistance data and of location information in LPP. A short TEI19 work is therefore very timely and suitable, ensuring support for both 4G/LTE and 5G/NR via LPP, while also being ready with the 3GPP representation to be transferred to 6G.

Benefits with regional CRS and CRS transformations support in 3GPP RAN are summarized below.

- 3GPP coordinate representations can be aligned with regional maps and avoid the meter level difference between regional maps and WGS84/ITRF

- 3GPP LPP assistance data representation will be on par with RTCM – industry standard for representing GNSS assistance data in many verticals.

- 3GPP LPP assistance data representation will also be on par with the representation of the data in regional CRS available from GNSS correction providers in a region/country.

- 3GPP distributed regional CRS or CRS transformations will align coordinate representation and avoid different understandings and assumptions of what the coordinates represent and how they should be converted on the device application layer – a difference in understanding that otherwise would be a positioning error source impacting positioning reliability.

- 3GPP regional CRS coordinate representation will mean that the mobile network operator will be able to represent base station antenna coordinates in a CRS that will remain relevant and associated to maps over long time and would require the base station antenna coordinates to be re-determined regularly causing higher OPEX.

TRP location information is provided in location coordinates, which also should be able to be represented in regional/national CRS, and after completed work in RAN2, the implications on RAN3 with respect to LPPa and NRPPa can be triggered via an LS.

Introduce support for regional CRS and CRS transformations in 3GPP RAN2.

Draft CRs are provided in text proposals in Appendix B (TS 38.305), Appendix B (TS 36.305) and Appendix C (TS 37.355 LPP).

# 2	Scope of location CRS and CRS transformations

The scope of the needed additions is summarized in the following sections

## 2.1 Location CRS information

An optional location CRS IE, identifying the CRS of the provided location coordinates is the key part to allow association of a location coordinate to a different coordinate reference system than WGS84.This information is provided as database links to databases managed by ISO [3] and EPRG [4]. The parameters provided in the IE CoordinateReferenceSystem are used as specified for message type 1300 and 1302 in [2].

CoordinateReferenceSystem-r19 ::= SEQUENCE {

coordinateEpoch-r19		- 	INTEGER (0..65535)					OPTIONAL,  -- Cond CE

plateNumber-r19				INTEGER (0..31)						OPTIONAL,  -- Cond Plate

crs-Name-r19				VisibleString (SIZE (1..31))		OPTIONAL,

crs-DatabaseLinks-r19		CRS-DatabaseLinks-r19				OPTIONAL,

...

}

CRS-DatabaseLinks-r19::= SEQUENCE (SIZE (1..maxDBLinks-r19)) OF CRS-DatabaseLinkElement-r19

CRS-DatabaseLinkElement-r19 ::= SEQUENCE {

crs-Database-r19			ENUMERATED {epsg, iso, ...},

crs-Category-r19			ENUMERATED {crs, datum, ...}		OPTIONAL,

crs-Code-r19				INTEGER (1..65536)					OPTIONAL,

...

}

maxDBLinks-r19 INTEGER ::= 7

## 2.2 Optional Association of location coordinates to a location CRS

The baseline will be WGS84, but in case location CRS is supported, an additional attribute identifying the location CRS can be provided in

- IE CommonIEsProvideLocationInformation to define location CRS of reported location coordinates.

- IE NR-PositionCalculationAssistance-r16 to define location CRS of the provided assistance data for UE-based positioning.

- IE GNSS-RTK-ReferenceStationInfo-r15 to define location CRS of reference station coordinates.

- IE GNSS-SSR-CorrectionPoints-r16 to define location CRS of GNSS SSR correction points.

- IE GNSS-LOS-NLOS-GridPoints-r18 to define location CRS of GNSS NLOS/LOS grid points.

- IE GNSS-SSR-OrbitCorrections-r15 to define location CRS of orbit corrections.

- IEs TBS-AssistanceDataList-r14, Sensor-AssistanceDataList-r14, LocationDataLCI-r14 (of IE WLAN-AP-Location-r14), BT-BeaconInfo-r18 to define location CRS for RAT-independent positioning.

- posSibType8-1 representing the IE CoordinateReferenceSystem-r19.

## 2.3 Optional Coordinate Transformations

In addition to the location CRS, it is relevant to also add support for coordinate transformations. There are a separate addition from the location CRS.

The IE CoordinateTransformations specifies coordinate transformation between a source CRS and a target CRS as specified in messages 1021-1027 and 1301 in [2]. The transformations are defined based on three components – coordinate transformation information, residuals and projections, and the components can be combined in sequence. The components the combine into a transformation from a source CRS to a target CRS are provided with the same system ID, and realized by the following IE

CoordinateTransformations-r19::= SEQUENCE (SIZE (1..maxCT-r19)) OF CoordTransElement-r19

CoordTransElement-r19 ::= SEQUENCE {

systemID-r19				INTEGER (0..255)					OPTIONAL,

coordTransInfo-r19			CoordTransInfo-r19					OPTIONAL,

coordTransResidual-19 		CHOICE {

residualEllipsGridRepr-r19		CoordTransResidualEllipsoidalGridRepr-r19,

residualPlaneGridRepr-r19		CoordTransResidualPlaneGridRepr-r19,

...

}																OPTIONAL,

coordTransProjection-r19 	CHOICE {

projTypesBaseline-r19		CoordTransProjectionTypesBaseline-r19,

projLambertConicConf-r19	CoordTransProjectionLambertConicConformal-r19,

projObliqueMercator-r19		CoordTransProjectionObliqueMercator-r19,

...

}																OPTIONAL,

...

}

CoordTransInfo-r19::= SEQUENCE  {

sourceCRS-r19					CoordinateReferenceSystem-r19,

targetCRS-r19					CoordinateReferenceSystem-r19,

coordTrans-r19 				CHOICE {

helmertAbrMolodenskiBadekas-r19		CoordTransHelmertAbrMolodenskiBadekas-r19,

timeDepLinExprHelmert-r19			CoordTransTimeDepLinExprHelmert-r19,

...

},

...

}

maxCT-r19 INTEGER ::= 255

Coordinate transformation components

- The coordinate transformation information is either

  - linear or strict Helmert, the abridged Molodenski (message 1021) or the Molodenski-Badekas (1022). These are supported via one and the same IE:

CoordTransHelmertAbrMolodenskiBadekas -r19 ::= SEQUENCE {

computationIndicator-r19		ENUMERATE { linearForm, strictForm, abridgedMolodenski,

molodenskiBadekas, ... },

heightIndicator-r19				ENUMERATE { geometricHeight, physHeightTarget, physHeightSource,

...},

areaValidityOriginLatitude-r19		INTEGER (-324000..324000),

areaValidityOriginLongitude-r19		INTEGER (-648000..648000),

areaValidity-N-S-Extension-r19		INTEGER (0..32766),

areaValidity-E-W-Extension-r19		INTEGER (0..32766),

xTranslation-r19					INTEGER (-4194303..4194303),

yTranslation-r19					INTEGER (-4194303..4194303),

zTranslation-r19					INTEGER (-4194303..4194303),

xRotation-r19						INTEGER (-2147483647.. 2147483647),

yRotation-r19						INTEGER (-2147483647.. 2147483647),

zRotation-r19						INTEGER (-2147483647.. 2147483647),

scaleCorrection-r19					INTEGER (-16777215..16777215),

x-M-B-RotationPoint-r19				INTEGER (-17179869183..17179869183)	OPTIONAL, --Cond molBad

y-M-B-RotationPoint-r19				INTEGER (-17179869183..17179869183)	OPTIONAL, --Cond molBad

z-M-B-RotationPoint-r19				INTEGER (-17179869183..17179869183)	OPTIONAL, --Cond molBad

sourceSemiMajorAxis-r19				INTEGER (0..16777215),

sourceSemiMinorAxis-r19				INTEGER (0..33554431),

targetSemiMajorAxis-r19				INTEGER (0..16777215),

targetSemiMinorAxis-r19				INTEGER (0..33554431),

horizHelmertMolodenskiQuality-r19	INTEGER (0..7),

vertHelmertMolodenskiQuality-r19	INTEGER (0..7),

...

}

  - time-dependent linear Helmert transformation (message 1301 in [2]), supported via the IE

CoordTransTimeDepLinExprHelmert-r19 ::= SEQUENCE {

referenceEpoch-r19					INTEGER (0..65535),

xTranslation-r19					INTEGER (-4194303..4194303),

yTranslation-r19					INTEGER (-4194303..4194303),

zTranslation-r19					INTEGER (-4194303..4194303),

xRotation-r19						INTEGER (-2147483647.. 2147483647),

yRotation-r19						INTEGER (-2147483647.. 2147483647),

zRotation-r19						INTEGER (-2147483647.. 2147483647),

scaleCorrection-r19					INTEGER (-16777215..16777215),

dot-xTranslation-r19				INTEGER (-65535..65535),

dot-yTranslation-r19				INTEGER (-65535..65535),

dot-zTranslation-r19				INTEGER (-65535..65535),

dot-xRotation-r19					INTEGER (-65535..65535),

dot-yRotation-r19					INTEGER (-65535..65535),

dot-zRotation-r19					INTEGER (-65535..65535),

dot-scaleCorrection-r19				INTEGER (-8191..8191),

...

}

- The coordinate transformation residuals can for example be used to translate the CRS from one epoch to another to capture drift of tectonic plates. They can be defined in ellipsoidal grid representation (message 1023  in [2]) or plane grid representation (message 1024 in [2])

  - Ellipsoidal grid representation is supported by the IE

CoordTransResidualEllipsoidalGridRepr-r19 ::= SEQUENCE {

horizontalShift-r19					ENUMERATED { true }					OPTIONAL,	-- Need ON

verticalShift-r19					ENUMERATED { true }					OPTIONAL,	-- Need ON

gridOriginLatitude-r19				INTEGER (-648000..648000),

gridOriginLongitude-r19				INTEGER (-1296000..1296000),

grid-N-S-Delta-r19					INTEGER (0..4095),

grid-E-W-Delta-r19					INTEGER (0..4095),

meanLatOffset-r19					INTEGER (-127..127),

meanLongOffset-r19					INTEGER (-127..127),

meanHeightOffset-r19				INTEGER (-16383..16383),

ellipsoidalGridResiduals-r19		SEQUENCE (SIZE(16)) OF EllipsoidalGridResidualElements-r19,

horizInterpMethodIndicator-r19		INTEGER (0..3),

vertInterpMethodIndicator-r19		INTEGER (0..3),

horizResidualQualityIndicator-r19	INTEGER (0..7),

vertResidualQualityIndicator-r19	INTEGER (0..7),

modifiedJulianDayNumber-r19			INTEGER (0..65535),

...

}

EllipsoidalGridResidualElements-r19 ::= SEQUENCE {

residualLat-r19						INTEGER (-255..255),

residualLong-r19					INTEGER (-255..255),

residualHeight-r19					INTEGER (-255..255)

}

  - Plane grid representation is supported by the IE

CoordTransResidualPlaneGridRepr-r19 ::= SEQUENCE {

horizontalShift-r19					ENUMERATED { true }					OPTIONAL,	-- Need ON

verticalShift-r19					ENUMERATED { true }					OPTIONAL,	-- Need ON

gridOriginNorthing-r19				INTEGER (-16777215..16777215),

gridOriginEasting-r19				INTEGER (0..67108863),

grid-N-S-Delta-r19					INTEGER (0..4095),

grid-E-W-Delta-r19					INTEGER (0..4095),

meanLocalNorthingOffset-r19			INTEGER (-511..511),

meanLocalEastingOffset-r19			INTEGER (-511..511),

meanLocalHeightOffset-r19			INTEGER (-16383..16383),

planeGridResiduals-r19				SEQUENCE (SIZE(16)) OF PlaneGridResidualElements-r19,

horizInterpMethodIndicator-r19		INTEGER (0..3),

vertInterpMethodIndicator-r19		INTEGER (0..3),

horizResidualQualityIndicator-r19	INTEGER (0..7),

vertResidualQualityIndicator-r19	INTEGER (0..7),

modifiedJulianDayNumber-r19			INTEGER (0..65535),

...

}

PlaneGridResidualElements-r19 ::= SEQUENCE {

residualLocalNorthing-r19			INTEGER (-255..255),

residualLocalEasting-r19			INTEGER (-255..255),

residualLocalHeight-r19				INTEGER (-255..255)

}

- The projections are grouped in different IEs, where the projections are represented as in [2] and defined in [5]. The different IEs are representing a baseline set of projections (message 1025 in [2]), Lambert conic conformal projection (message 1026 in [2]) and Oblique Mercator projection (message 1027 in [2]).

  - The baseline set of projections is supported by the IE

CoordTransProjectionTypesBaseline-r19 ::= SEQUENCE {

projectionType-r19					ENUMERATED { unknown, tm, tms, lcc1sp, lccw, cs, om, os, mc,

ps, ds, ...},

latNaturalOrigin-r19				INTEGER (-8181818181.. 8181818181),

longNaturalOrigin-r19				INTEGER (-16363636362.. 16363636362),

scaleFactorNaturalOrigin-r19		INTEGER (0..1073741823),

falseNorthing-r19					INTEGER (0..68719476735),

falseEasting-r19					INTEGER (0..17179869183),

...

}

  - The Lambert conic conformal projection is supported by the IE

CoordTransProjectionLambertConicConformal-r19 ::= SEQUENCE {

latFalseOrigin-r19					INTEGER (-8181818181..8181818181),

longFalseOrigi-r19					INTEGER (-16363636362..16363636362),

latStdParallelNo1-r19				INTEGER (-8181818181..8181818181),

latStdParallelNo2-r19				INTEGER (-8181818181..8181818181),

northingFalseOrigin-r19				INTEGER (0..68719476735),

eastingFalseOrigin-r19				INTEGER (-17179869183..17179869183),

...

}

  - The Oblique Mercator projection is supported by the IE

CoordTransProjectionObliqueMercator-r19 ::= SEQUENCE {

rectificationFlag-r19			ENUMERATED { true },					OPTIONAL	-- Need ON

latProjectionCenter-r19			INTEGER (-8181818181..8181818181),

longProjectionCenter-r19		INTEGER (-16363636362..16363636362),

azIL-r19						INTEGER (0..32727272724),

diggARSG-r19					INTEGER (-33554431..33554431),

addSIL-r19						INTEGER (0..1073741823),

northingProjectionCenter-r19	INTEGER (0..68719476735),

eastingProjectionCenter-r19		INTEGER (-17179869183..17179869183),

...

}

## 2.4 Optional distribution of CRS transformations

The CRS transformations can be distributed to target devices in two different ways

- via the optional attribute coordTransformations of IE CommonIEsProvideAssistanceData.

- Vis the posSibType8-2 representing the IE CoordinateTransformations-r19.

# 3	Summary

Based on the discussion and the text proposals in Appendix A, B and C, we have the following proposal

Agree to text proposals in Appendix A, B and C.

# 4	Conclusion

We propose the following:

Proposal 1	Introduce support for regional CRS and CRS transformations in 3GPP RAN2.

Proposal 2	Agree to text proposals in Appendix A, B and C.

# References

- RTCM Standard 10403.3: "Differential GNSS (Global Navigation Satellite Systems) Services" – Version 3, October 7, 2016.

- RTCM Standard 10403.4: "Differential GNSS (Global Navigation Satellite Systems) Services" – Version 3, December 1, 2023.

- European Petroleum Survey Group (EPSG) Geodetic Parameter Dataset EPSG CRS database , International Association of Oil & Gas Producers (IOGP), https://epsg.org.

- ISO Geodetic Registry for ISO 19127, ISO/TC 211, https://geodetic.isotc211.org.

- IOGP Publication 373-7-2 – Geomatics Guidance Note number 7, part 2, Coordinate Conversions and Transformations including Formulas, September 2019.

# Appendix A. Text Proposal to TS 38.305

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501 "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 22.071: "Location Services (LCS); Service description, Stage 1".

[4]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[5]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[6]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[7]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[8]	Galileo OS Signal in Space ICD (OS SIS ICD), Draft 0, Galileo Joint Undertaking, May 23rd, 2006.

[9]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5, 2002.

[10]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.0, June 17, 2008.

[11]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[12]	RTCM 10402.3, RTCM Recommended Standards for Differential GNSS Service (v.2.3), August 20, 2001.

[13]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[14]	3GPP TS 38.331: "NR Radio Resource Control (RRC) protocol specification".

[15]	OMA-AD-SUPL-V2_0: "Secure User Plane Location Architecture Approved Version 2.0".

[16]	OMA-TS-ULP-V2_0_6: "UserPlane Location Protocol Approved Version 2.0.6".

[17]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer – Measurements".

[18]	3GPP TS 36.302: "Evolved Universal Terrestrial Radio Access (E-UTRA); Services provided by the physical layer".

[19]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[20]	BDS-SIS-ICD-B1I-3.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1I (Version 3.0)", February, 2019.

[21]	IEEE 802.11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications"

[22]	Bluetooth Special Interest Group: "Bluetooth Core Specification v4.2", December 2014.

[23]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance", May 2015.

[24]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[25]	3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRA".

[26]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[27]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[28]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[29]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[30]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[31]	RTCM 10403.3, "RTCM Recommended Standards for Differential GNSS Services (v.3.3)", October 7, 2016.

[32]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[33]	3GPP TS 29.572: "Location Management Services; Stage 3".

[34]	BDS-SIS-ICD-B1C-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (Version 1.0)", December, 2017

[35]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[36]	IS-QZSS-L6-001, Quasi-Zenith Satellite System Interface Specification – Centimetre Level Augmentation Service, Cabinet Office, November 5, 2018.

[37]	3GPP TS 38.215: "NR; Physical layer – Measurements".

[38]	3GPP TS 38.401: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NG-RAN; Architecture description".

[39]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[40]	3GPP TS 38.212: "NR; Multiplexing and channel coding".

[41]	3GPP TS 24.571: "Control plane Location Services (LCS) procedures".

[42]	3GPP TS 37.355: "Technical Specification Group Radio Access Network; LTE Positioning Protocol (LPP)".

[43]	IRNSS Signal-In-Space (SPS) Interface Control Document (ICD) for standard positioning service version 1.1, August 2017.

[44]	BDS-SIS-ICD-B2a-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B2a (Version 1.0)", December, 2017.

[45]	BDS-SIS-ICD-B3I-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B3I (Version 1.0)", February, 2018.

[xx]	RTCM Standard 10403.4: "Differential GNSS (Global Navigation Satellite Systems) Services" – Version 3, December 1, 2023.

[yy]	European Petroleum Survey Group (EPSG) Geodetic Parameter Dataset EPSG CRS database , International Association of Oil & Gas Producers (IOGP), https://epsg.org.

[zz]	ISO Geodetic Registry for ISO 19127, ISO/TC 211, https://geodetic.isotc211.org.

[aa]	IOGP Publication 373-7-2 – Geomatics Guidance Note number 7, part 2, Coordinate Conversions and Transformations including Formulas, September 2019.

#### […]

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5GS	5G System

A-AoA	Azimuth-Angle of Arrival

ADR	Accumulated Delta Range

AL	Alert Limit

AoA	Angle of Arrival

AP	Access Point

ARP	Antenna Reference Point

BDS	BeiDou Navigation Satellite System

BSSID	Basic Service Set Identifier

CID	Cell-ID (positioning method)

CLAS	Centimetre Level Augmentation Service

CRS	Coordinate Reference System

DL-AoD	Downlink Angle-of-Departure

DL-PRS	Downlink Positioning Reference Signal

DL-TDOA	Downlink Time Difference Of Arrival

DNU	Do Not Use

E-SMLC	Enhanced Serving Mobile Location Centre

E-CID	Enhanced Cell-ID (positioning method)

ECEF	Earth-Centered, Earth-Fixed

ECI	Earth-Centered-Inertial

EGNOS	European Geostationary Navigation Overlay Service

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDMA	Frequency Division Multiple Access

FKP	Flächenkorrekturparameter (Engl: Area Correction Parameters)

GAGAN	GPS Aided Geo Augmented Navigation

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System)

GMLC	Gateway Mobile Location Centre

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

GRS80	Geodetic Reference System 1980

HESSID	Homogeneous Extended Service Set Identifier

LCS	LoCation Services

LMF	Location Management Function

LPP	LTE Positioning Protocol

MAC	Master Auxiliary Concept

MBS	Metropolitan Beacon System

MO-LR	Mobile Originated Location Request

MT-LR	Mobile Terminated Location Request

Multi-RTT	Multi-Round Trip Time

NavIC	NAVigation with Indian Constellation

NG-C	NG Control plane

NG-AP	NG Application Protocol

NI-LR	Network Induced Location Request

N-RTK	Network – Real-Time Kinematic

NRPPa	NR Positioning Protocol A

OTDOA	Observed Time Difference Of Arrival

PDU	Protocol Data Unit

posSI	Positioning System Information

posSIB	Positioning SIB

PPP	Precise Point Positioning

PPP-RTK	Precise Point Positioning – Real-Time Kinematic

PRS	Positioning Reference Signal (for E-UTRA)

PRU	Positioning Reference Unit

QZSS	Quasi-Zenith Satellite System

RP	Reception Point

RRM	Radio Resource Management

RSRP	Reference Signal Received Power

RSRPP	Reference Signal Received Path Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTK	Real-Time Kinematic

SBAS	Space Based Augmentation System

SDT	Small Data Transmission

SET	SUPL Enabled Terminal

SIB	System Information Block

SLP	SUPL Location Platform

SP	Semi-Persistent

SRS	Sounding Reference Signal

SSB	Synchronization Signal Block

SSID	Service Set Identifier

SSR	State Space Representation

STEC	Slant TEC

SUPL	Secure User Plane Location

TADV	Timing Advance

TBS	Terrestrial Beacon System

TEC	Total Electron Content

TEG	Timing Error Group

TP	Transmission Point

TRP	Transmission-Reception Point

TTA	Time To Alert

TxTEG	Tx Timing Error Group

UE	User Equipment

UL-AoA	Uplink Angle of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

UL-TDOA	Uplink Time Difference of Arrival

URA	User Range Accuracy

WAAS	Wide Area Augmentation System

WGS-84	World Geodetic System 1984

WLAN	Wireless Local Area Network

Z-AoA	Zenith Angles of Arrival

#### […]

## 4.x	Coordinate Reference Systems and Transformations

### 4.x.1	Coordinate Reference Systems

The baseline Coordinate Reference System (CRS) for 5GS, including NG-RAN, is WGS84 as defined in [4]. Optionally, location coordinates in location information and assistance data can be provided in other CRS. A typical example is a national or regional CRS that is adapted to the regional conditions and follow the tectonic plate movements, which means that the coordinates will remain aligned with maps over long time without calibration. In this case, the location coordinates are accompanied by an optional CRS information as defined in [xx]. This CRS information consists of database links to databases managed by ISO [yy] and EPRG [zz].

The CRS information can be accompanied via the assistance data from the location server as well as the location information from the device. The CRS information can also be provided as a posSIB.

### 4.x.2	Coordinate Transformations

Coordinates in one source CRS can be transformed into coordinates in a target CRS via coordinate transformations as specified in [xx. The transformations are defined based on three components – coordinate transformation information, residuals and projections, and the components can be combined in sequence. The components that combine into a transformation from a source CRS to a target CRS are provided with the same system identifier, to allow them to be distributed separately.

The coordinate transformation components and their subtypes are:

- The coordinate transformation information is either

  - linear or strict Helmert, the abridged Molodenski (message 1021 in [xx]) or the Molodenski-Badekas (1022) transformation.

  - time-dependent linear Helmert transformation (message 1301 in [xx])

- The coordinate transformation residuals can for example be used to translate the CRS from one epoch to another to capture drift of tectonic plates. They can be defined in either

  - ellipsoidal grid representation (message 1023  in [xx])

  - plane grid representation (message 1024 in [xx]

- The projections are defined in three groups, where the projections are represented as in [xx] and defined in [aa], chapter 1.4:

  - a baseline set of projections (message 1025 in [xx]), including Transverse Mercator (TM), Transverse Mercator, South-Oriented (TMS), Lambert Conic Conformal, 1SP (LCC1SP), Lambert Conic Conformal (LCCW), Casini-Soldner (CS), Oblique Stereographic (OS), Mercator (MC), Polar Stereographic (PS), Double Stereographic (DS).

  - Lambert conic conformal, 2SP, projection (message 1026 in [xx]) and

  - Oblique Mercator projection (message 1027 in [xx]).

# Appendix B. Text Proposal to TS 36.305

Essentially the same text proposal as in Appendix A, except the first line of the new section 4.x:

## 4.x	Coordinate Reference Systems and Transformations

### 4.x.1	Coordinate Reference Systems

The baseline Coordinate Reference System (CRS) for EPS, including E-UTRAN, is WGS84 as defined in [4].

# Appendix C. Text Proposal to TS 37.355

START OF CHANGE

# 2	References

#### […]

[54]	3GPP TS 38.101-5: "User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements".

[xx]	RTCM-SC104, RTCM Recommended Standards for Differential GNSS Service (v.3.4 with Amendment 1), November 1, 2024.

[yy]	European Petroleum Survey Group (EPSG) Geodetic Parameter Dataset EPSG CRS database , International Association of Oil & Gas Producers (IOGP), https://epsg.org.

[zz]	ISO Geodetic Registry for ISO 19127, ISO/TC 211, https://geodetic.isotc211.org.

[aa]	IOGP Publication 373-7-2 – Geomatics Guidance Note number 7, part 2, Coordinate Conversions and Transformations including Formulas, September 2019.

#### […]

NEXT CHANGE

### 6.4.1	Common Lower-Level IEs

#### […]

#### –	CellGlobalIdGERAN

The IE CellGlobalIdGERAN specifies the global Cell Identifier for GERAN, the globally unique identity of a cell in GERAN.

-- ASN1START

CellGlobalIdGERAN ::= SEQUENCE {

plmn-Identity		SEQUENCE {

mcc		SEQUENCE (SIZE (3))	OF INTEGER (0..9),

mnc		SEQUENCE (SIZE (2..3))	OF INTEGER (0..9)

},

locationAreaCode		BIT STRING (SIZE (16)),

cellIdentity			BIT STRING (SIZE (16)),

...

}

-- ASN1STOP

CellGlobalIdGERAN field descriptions

plmn-Identity This field identifies the PLMN of the cell.

locationAreaCode This field is a fixed length code identifying the location area within a PLMN.

cellIdentity This field specifies the cell Identifier which is unique within the context of the GERAN location area.

#### –	CoordinateReferenceSystem

The IE CoordinateReferenceSystem specifies the name and/or further details of the considered coordinate reference system (CRS). The parameters provided in the IE CoordinateReferenceSystem are used as specified for message type 1300 and 1302 in [xx].

-- ASN1START

CoordinateReferenceSystem-r19 ::= SEQUENCE {

coordinateEpoch-r19			INTEGER (0..65535),					OPTIONAL,  -- Cond CE

plateNumber-r19				INTEGER (0..31),					OPTIONAL,  -- Cond Plate

crs-Name-r19				VisibleString (SIZE (1..31))		OPTIONAL,

crs-DatabaseLinks-r19		CRS-DatabaseLinks-r19				OPTIONAL,

...

}

CRS-DatabaseLinks-r19::= SEQUENCE (SIZE (1..maxDBLinks-r19)) OF CRS-DatabaseLinkElement-r19

CRS-DatabaseLinkElement-r19 ::= SEQUENCE {

crs-Database-r19			ENUMERATED {epsg, iso, ...},

crs-Category-r19			ENUMERATED {crs, datum, ...}		OPTIONAL,

crs-Code-r19				INTEGER (1..65536)					OPTIONAL,

...

}

maxDBLinks-r19 INTEGER ::= 7

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| CE | The field is present, need ON, if the CoordinateReferenceSystem IE is used to define the CRS of associated coordinates, but not present if the CoordinateReferenceSystem IE is used to define a source or target CRS of a coordinate transformation. |
| Plate | The field is optionally present, need ON, if the CoordinateReferenceSystem IE is used to define the CRS of associated coordinates or a target CRS of a coordinate transformation, but not present if the CoordinateReferenceSystem IE is used to define a source CRS of a coordinate transformation. |

CoordinateReferenceSystem field descriptions

coordinateEpoch This field identifies the epoch of the coordinate reference system, where: 0: indicates the current epoch. >0: indicates that the coordinate reference system is not in the current epoch, and encodes the coordinate epoch in decimal year, scale factor 0.01 years, after the start of year 1900. For example, coordiateEpoch = 12500 means the start of year 2025.

plateNumber This field the associated tectonic plate number if the CRS, where: 0: unknown, used for global CRS. 1: AFRC – Africa. 2: ANTA – Antarctica. 3: ARAB – Arabia. 4: AUST – Australia. 5: CARB – Caribbean. 6: COCO – Cocos. 7: EURA – Eurasia. 8: INDI – India. 9: NOAM – N. America 10: NAZCA – Nazca 11: PCFC – Pacific 12: SOAM – S. America 13: JUFU – Juan de Fuca 14: PHIL – Philippine 15: RIVR – Rivera 16: SCOT – Scotia 17-31: Reserved

crs-Name This field specifies the name of the CRS, If available, the EPSG [yy] identification code has to be used with the format example “EPSG:7912” indicating ITRF2014.

crs-Database This field specifies the database, where: epsg: indicates EPSG [yy]. Iso: indicates ISO [zz].

crs-Categoty This field specifies the database table or category, where: crs: indicates a CRS database table/category. datum: indicates a datum database table/category.

crs-Code This field specifies the CRS or datum code in the database.

NOTE 1:	If a database link is provided via the triplet cre-Database, crs-Category and crs-Code, the string representation of the Database-Link attribute of message 1302 in [xx] is compiled as “[crs-Database].[crs-Category]:[crs-Code]”, for example as “EPSG.DATUM:1165” for the ITRF2014 datum or “ISO.CRS:425” for the ITRF2014 geodetic CRS.

#### –	CoordinateTransformations

The IE CoordinateTransformations specifies coordinate transformation between a source CRS and a target CRS as specified in messages 1021-1027 and 1301 in [xx].

-- ASN1START

CoordinateTransformations-r19::= SEQUENCE (SIZE (1..maxCT-r19)) OF CoordTransElement-r19

CoordTransElement-r19 ::= SEQUENCE {

systemID-r19				INTEGER (0..255)					OPTIONAL,

coordTransInfo-r19			CoordTransInfo-r19					OPTIONAL,

coordTransResidual-19 		CHOICE {

residualEllipsGridRepr-r19		CoordTransResidualEllipsoidalGridRepr-r19,

residualPlaneGridRepr-r19		CoordTransResidualPlaneGridRepr-r19,

...

}																OPTIONAL,

coordTransProjection-r19 	CHOICE {

projTypesBaseline-r19		CoordTransProjectionTypesBaseline-r19,

projLambertConicConf-r19	CoordTransProjectionLambertConicConformal-r19,

projObliqueMercator-r19		CoordTransProjectionObliqueMercator-r19,

...

}																OPTIONAL,

...

}

CoordTransInfo-r19::= SEQUENCE  {

sourceCRS-r19					CoordinateReferenceSystem-r19,

targetCRS-r19					CoordinateReferenceSystem-r19,

coordTrans-r19 				CHOICE {

helmertAbrMolodenskiBadekas-r19		CoordTransHelmertAbrMolodenskiBadekas-r19,

timeDepLinExprHelmert-r19			CoordTransTimeDepLinExprHelmert-r19,

...

},

...

}

maxCT-r19 INTEGER ::= 255

-- ASN1STOP

CoordinateTransformations field descriptions

systemID This field identifies sets of coordinate transformation components, where components with the same systemID can be combined together for coordinate transformations.

coordTransInfo This field specifies the fundamental coordinate transformation.

sourceCRS This field specifies the source CRS of the coordinate transformation.

targetCRS This field specifies the target CRS of the coordinate transformation.

#### –	CoordTransHelmertAbrMolodenskiBadekas

The IE CoordTransHelmertAbrMolodenskiBadekas specifies four different sets of transformations – the linear form of the Helbert Transformation, the strict form of the Helbert Transformation, the abridged Molodenski Transformation, and the Molodenski-Badekas as specified in messages 1021 and 1022 in [xx].

-- ASN1START

CoordTransHelmertAbrMolodenskiBadekas -r19 ::= SEQUENCE {

computationIndicator-r19		ENUMERATE { linearForm, strictForm, abridgedMolodenski,

molodenskiBadekas, ... },

heightIndicator-r19				ENUMERATE { geometricHeight, physHeightTarget, physHeightSource,

...},

areaValidityOriginLatitude-r19		INTEGER (-324000..324000),

areaValidityOriginLongitude-r19		INTEGER (-648000..648000),

areaValidity-N-S-Extension-r19		INTEGER (0..32766),

areaValidity-E-W-Extension-r19		INTEGER (0..32766),

xTranslation-r19					INTEGER (-4194303..4194303),

yTranslation-r19					INTEGER (-4194303..4194303),

zTranslation-r19					INTEGER (-4194303..4194303),

xRotation-r19						INTEGER (-2147483647.. 2147483647),

yRotation-r19						INTEGER (-2147483647.. 2147483647),

zRotation-r19						INTEGER (-2147483647.. 2147483647),

scaleCorrection-r19					INTEGER (-16777215..16777215),

x-M-B-RotationPoint-r19				INTEGER (-17179869183..17179869183)	OPTIONAL, --Cond molBad

y-M-B-RotationPoint-r19				INTEGER (-17179869183..17179869183)	OPTIONAL, --Cond molBad

z-M-B-RotationPoint-r19				INTEGER (-17179869183..17179869183)	OPTIONAL, --Cond molBad

sourceSemiMajorAxis-r19				INTEGER (0..16777215),

sourceSemiMinorAxis-r19				INTEGER (0..33554431),

targetSemiMajorAxis-r19				INTEGER (0..16777215),

targetSemiMinorAxis-r19				INTEGER (0..33554431),

horizHelmertMolodenskiQuality-r19	INTEGER (0..7),

vertHelmertMolodenskiQuality-r19	INTEGER (0..7),

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| molBad | The field is present, need ON, if the computationIndicator is set to 'molodenskiBadekas', otherwise it is not present. |

CoordTransHelmertAbrMolodenskiBadekas field descriptions

computationIndicator This field identifies the used coordinate transformation method: linearForm: indicates the standard seven parameter transformation with linear approximation. strictForm: indicates the standard seven parameter transformation with a strict formula. abridgedMolodenski: indicates the abridged Molodenski transformation. molodenskiBadekas: indicates the Molodenski-Badekas transformation.

heightIndicator This field indicates the height transformation to be used, where: geometricHeight: geometric height results. physHeightTarget: physical heights derived via Helmert/Molodenski transformation and target system height. physHeightSource: physical height definition from the source system.

areaValidityOriginLatitude This field specifies latitude origin in arc seconds of the area of validity of the Helmert/Molodenski transformation, see Figure 3.5-3 of [xx]. Scale factor 2 arc seconds.

areaValidityOriginLongitude This field specifies longitude origin in arc seconds of the area of validity of the Helmert/Molodenski transformation, see Figure 3.5-3 of [xx]. Scale factor 2 arc seconds.

areaValidity-N-S-Extension This field specifies extension in north-south directions in arc seconds of the area of validity of the Helmert/Molodenski transformation, see Figure 3.5-3 of [xx]. Scale factor 2 arc seconds.

areaValidity-E-W-Extension This field specifies extension in east-west directions in arc seconds of the area of validity of the Helmert/Molodenski transformation, see Figure 3.5-3 of [xx]. Scale factor 2 arc seconds.

xTranslation, yTranslation, zTranslation These fields specify the x, y and z coordinates of the translation vector to be added to the position in the source CRS In order to transform to the target CRS. Scale factor 0.001 m.

xRotation, yRotation, zRotation These fields specify the rotation around the x, y and z axes to be applied to position in the source CRS. Scale factor 0.00002 arc seconds.

scaleCorrection This field specifies the scale corrections in parts per million. Scale factor 0.00001 ppm.

x-M-B-RotationPoint, y-M-B-RotationPoint, z-M-B-RotationPoint These fields specify the x, y and z coordinates of the Molodenski-Badekas rotation point. Scale factor 0.001 m.

sourceSemiMajorAxis, targetSemiMajorAxis These fields specify semi-major axis of the source and target system ellipsoid, encoded as 6370000 + *SemiMajorAxis, and if *SemiMajorAxis = 0, the the conversion to ellipsoidal coordinates, the projection and the local transformation have to be omitted. Scale factor 0.001 m.

sourceSemiMinorAxis, targetSemiMinorAxis These fields specify semi-minor axis of the source and target system ellipsoid, encoded as 6350000 + *SemiMinorAxis, and if *SemiMinorAxis = 0, the the conversion to ellipsoidal coordinates, the projection and the local transformation have to be omitted. Scale factor 0.001 m.

horizHelmertMolodenskiQuality, vertHelmertMolodenskiQuality These fields specify the Helmert-Molodenski quality indicator as the maximum approximation error after the application of the transformation within the area of validity. The accuracy can be further improved by residual information. NOTE 1.

NOTE 1:	Maximum approximation error of the Helmert/Molodenski transformation in the table below:

Relationship between Helmert/Molodenski quality indicator and physical quantity

| Index | Helmert/Molodenski Quality Indicator Q [mm] |
| --- | --- |
| 0 | Unknown quality |
| 1 | Q < 21 |
| 2 | 21 ≤ Q ≤ 50 |
| 3 | 51 ≤ Q ≤ 200 |
| 4 | 201 ≤ Q ≤ 500 |
| 5 | 501 ≤ Q ≤ 2000 |
| 6 | 2001 ≤ Q ≤ 5000 |
| 7 | 5001 < Q |

#### –	CoordTransTimeDepLinExprHelmert

The IE CoordTransTimeDepLinExprHelmert specifies the time-dependent linear expression of the Helbert Transformation as specified in message 1301 in [xx].

-- ASN1START

CoordTransTimeDepLinExprHelmert-r19 ::= SEQUENCE {

referenceEpoch-r19					INTEGER (0..65535),

xTranslation-r19					INTEGER (-4194303..4194303),

yTranslation-r19					INTEGER (-4194303..4194303),

zTranslation-r19					INTEGER (-4194303..4194303),

xRotation-r19						INTEGER (-2147483647.. 2147483647),

yRotation-r19						INTEGER (-2147483647.. 2147483647),

zRotation-r19						INTEGER (-2147483647.. 2147483647),

scaleCorrection-r19					INTEGER (-16777215..16777215),

dot-xTranslation-r19				INTEGER (-65535..65535),

dot-yTranslation-r19				INTEGER (-65535..65535),

dot-zTranslation-r19				INTEGER (-65535..65535),

dot-xRotation-r19					INTEGER (-65535..65535),

dot-yRotation-r19					INTEGER (-65535..65535),

dot-zRotation-r19					INTEGER (-65535..65535),

dot-scaleCorrection-r19				INTEGER (-8191..8191),

...

}

-- ASN1STOP

CoordTransTimeDepLinExprHelmert field descriptions

referenceEpoch This field specifies the reference epoch t0 for the transformation, given as the modified Julian day number minus 44244.

xTranslation, yTranslation, zTranslation These fields specify the x, y and z coordinates of the translation vector for reference epoch t0. Scale factor 0.001 m.

xRotation, yRotation, zRotation These fields specify the rotation around the x, y and z axes for reference epoch t0. Scale factor 0.00002 arc seconds.

scaleCorrection This field specifies the scale corrections for reference epoch t0 in parts per million. Scale factor 0.00001 ppm.

xTranslation, yTranslation, zTranslation These fields specify the rate of change in the x, y and z coordinates of the translation vector for reference epoch t0. Scale factor 0.001 m.

xRotation, yRotation, zRotation These fields specify the rate of change of the rotation around the x, y and z axes for reference epoch t0. Scale factor 0.00002 arc seconds.

scaleCorrection This field specifies the rate of change of the scale corrections for reference epoch t0 in parts per million. Scale factor 0.00001 ppm.

#### –	CoordTransResidualEllipsoidalGridRepr

The IE CoordTransResidualEllipsoidalGridRepr specifies residuals with an ellipsoidal grid representation as part of the coordinate transformation as specified in message 1023 in [xx].

-- ASN1START

CoordTransResidualEllipsoidalGridRepr-r19 ::= SEQUENCE {

horizontalShift-r19					ENUMERATED { true }					OPTIONAL,	-- Need ON

verticalShift-r19					ENUMERATED { true }					OPTIONAL,	-- Need ON

gridOriginLatitude-r19				INTEGER (-648000..648000),

gridOriginLongitude-r19				INTEGER (-1296000..1296000),

grid-N-S-Delta-r19				INTEGER (0..4095),

grid-E-W-Delta-r19				INTEGER (0..4095),

meanLatOffset-r19					INTEGER (-127..127),

meanLongOffset-r19					INTEGER (-127..127),

meanHeightOffset-r19				INTEGER (-16383..16383),

ellipsoidalGridResiduals-r19		SEQUENCE (SIZE(16)) OF EllipsoidalGridResidualElements-r19,

horizInterpMethodIndicator-r19		INTEGER (0..3),

vertInterpMethodIndicator-r19		INTEGER (0..3),

horizResidualQualityIndicator-r19	INTEGER (0..7),

vertResidualQualityIndicator-r19	INTEGER (0..7),

offsetModifiedJulianDayNumber-r19	INTEGER (0..65535),

...

}

EllipsoidalGridResidualElements-r19 ::= SEQUENCE {

residualLat-r19						INTEGER (-255..255),

residualLong-r19					INTEGER (-255..255),

residualHeight-r19					INTEGER (-255..255)

}

-- ASN1STOP

CoordTransResidualEllipsoidalGridRepr field descriptions

horizontalShift, verticalShift These fields specify whether horizontal and vertical shifts, respectively, are considered.

gridOriginLatitude This field specifies latitude of the grid origin in arc seconds, see Figure 3.5-4 of [xx]. Scale factor 0.5 arc seconds.

gridOriginLongitude This field specifies longitude of the grid origin in arc seconds, see Figure 3.5-4 of [xx]. Scale factor 0.5 arc seconds.

grid-N-S-Delta, grid-E-W-Delta These fields specify grid delta between adjacent grid points in north-south and east-west directions, respectively, in arc seconds, see Figure 3.5-4 of [xx]. Scale factor 0.5 arc seconds, and value 0 is undefined. Delta coordinates defined in target system, which in this context means directly after the coordinate transformation via any of the IEs CoordTransHelmertAbrMolodenskiBadekas or CoordTransTimeDepLinExprHelmert.

meanLatOffset, meanLongOffset These fields specify the mean offset of latitude and longitude, respectively, over the grid. Scale factor 0.001 arc seconds.

meanHeightOffset These fields specify the mean height offset over the grid. Scale factor 0.01 m. If the field heightIndicator of IE CoordTransHelmertAbrMolodenskiBadekas is equal to 2, the height offset is in the source CRS, otherwise it is in target CRS.

ellipsoidalGridResiduals This field specifies the grid with residuals. The grid contains 16 grid points, with the first grid point as the north-west corner, and the following grid points row-wise from north to south.

residualLat, residualLong These fields specify the residual latitude and longitude, respectively, for the grid point i. Scale factor 0.00003 arc seconds.

residualHeight This field specifies the residual height for the grid point i. Scale factor 0.001 m. If the field heightIndicator of IE CoordTransHelmertAbrMolodenskiBadekas is equal to 2, the residual height is in the source CRS, otherwise it is in target CRS.

horizInterpMethodIndicator, vertInterpMethodIndicator These fields specify the interpolation method horizontally and vertically, respectively, to be used over the grid, where: 0: bi-linear interpolation. 1: bi-quadratic interpolation. 2: bi-spline interpolation 3: reserved

horizResidualQualityIndicator, vertResidualQualityIndicator These fields specify the quality indicator as the maximum approximation error after the application of the transformation and residuals within the area of validity and replaces the quality indicator of the transformation. NOTE 1.

offsetModifiedJulianDayNumber This field specifies the modified Julian dat number as the continuous count of day numbers since November 17, 1858 minus 44244 days. For example, the first day in GPS week 0 has MJD 44244, and the offset MJD is 0. At day 65536, the counter will start at 0 again.

NOTE 1:	Maximum approximation error of the transformation and grid adjustments in the table below:

Relationship between quality indicator and physical quantity after grid adjustments

| Index | Quality Indicator Q [mm] |
| --- | --- |
| 0 | Unknown quality |
| 1 | Q < 10 |
| 2 | 11 ≤ Q ≤ 20 |
| 3 | 21 ≤ Q ≤ 50 |
| 4 | 51 ≤ Q ≤ 100 |
| 5 | 101 ≤ Q ≤ 200 |
| 6 | 201 ≤ Q ≤ 500 |
| 7 | 501 < Q |

#### –	CoordTransResidualPlaneGridRepr

The IE CoordTransResidualPlaneGridRepr specifies residuals with a plane grid representation as part of the coordinate transformation as specified in message 1024 in [xx].

-- ASN1START

CoordTransResidualPlaneGridRepr-r19 ::= SEQUENCE {

horizontalShift-r19					ENUMERATED { true }					OPTIONAL,	-- Need ON

verticalShift-r19					ENUMERATED { true }					OPTIONAL,	-- Need ON

gridOriginNorthing-r19				INTEGER (-16777215..16777215),

gridOriginEasting-r19				INTEGER (0..67108863),

grid-N-S-Delta-r19					INTEGER (0..4095),

grid-E-W-Delta-r19					INTEGER (0..4095),

meanLocalNorthingOffset-r19			INTEGER (-511..511),

meanLocalEastingOffset-r19			INTEGER (-511..511),

meanLocalHeightOffset-r19			INTEGER (-16383..16383),

planeGridResiduals-r19				SEQUENCE (SIZE(16)) OF PlaneGridResidualElements-r19,

horizInterpMethodIndicator-r19		INTEGER (0..3),

vertInterpMethodIndicator-r19		INTEGER (0..3),

horizResidualQualityIndicator-r19	INTEGER (0..7),

vertResidualQualityIndicator-r19	INTEGER (0..7),

offsetModifiedJulianDayNumber-r19	INTEGER (0..65535),

...

}

PlaneGridResidualElements-r19 ::= SEQUENCE {

residualLocalNorthing-r19			INTEGER (-255..255),

residualLocalEasting-r19			INTEGER (-255..255),

residualLocalHeight-r19				INTEGER (-255..255)

}

-- ASN1STOP

CoordTransResidualPlaneGridRepr field descriptions

horizontalShift, verticalShift These fields specify whether horizontal and vertical shifts, respectively, are considered.

gridOriginNorthing, gridOriginLongitude This field specifies Northing and Easting, respectively, of the grid origin in meters, see Figure 3.5-4 of [xx]. Coordinates defined in local system after projection. Scale factor 10m.

grid-N-S-Delta, grid-E-W-Delta These fields specify grid delta between adjacent grid points in north-south and east-west directions, respectively, in meters, see Figure 3.5-4 of [xx]. Scale factor 10m and value 0 is undefined. Delta coordinates defined in local system after projection.

meanLocalNorthingOffset, meanLocalEastingOffset These fields specify the mean offset of Northing and Easting, respectively, over the local system grid. Scale factor 0.01 m.

meanLocalHeightOffset These fields specify the mean height offset over the local system grid. Scale factor 0.01 m. If the field heightIndicator of IE CoordTransHelmertAbrMolodenskiBadekas is equal to 2, the height offset is in the source CRS, otherwise it is in target CRS.

planeGridResiduals This field specifies the local system grid with residuals. The grid contains 16 grid points, with the first grid point as the north-west corner, and the following grid points row-wise from north to south.

residualLocalNorthing, residualLocalEasting These fields specify the residual local Northing and Easting, respectively, for the grid point i. Scale factor 0.001 m.

residualLocalHeight This field specifies the residual height for the grid point i. Scale factor 0.001 m. If the field heightIndicator of IE CoordTransHelmertAbrMolodenskiBadekas is equal to 2, the residual height is in the source CRS, otherwise it is in target CRS.

horizInterpMethodIndicator, vertInterpMethodIndicator These fields specify the interpolation method horizontally and vertically, respectively, to be used over the grid, where: 0: bi-linear interpolation. 1: bi-quadratic interpolation. 2: bi-spline interpolation 3: reserved

horizResidualQualityIndicator, vertResidualQualityIndicator These fields specify the quality indicator as the maximum approximation error after the application of the transformation and residuals within the area of validity and replaces the quality indicator of the transformation. See NOTE 1 of IE CoordTransResidualEllipsoidalGridRepr.

offsetModifiedJulianDayNumber This field specifies the modified Julian dat number as the continuous count of day numbers since November 17, 1858 minus 44244 days. For example, the first day in GPS week 0 has MJD 44244, and the offset MJD is 0. At day 65536, the counter will start at 0 again.

#### –	CoordTransProjectionTypesBaseline

The IE CoordTransProjectionTypesBaseline specifies a baseline set of projection types as specified in message 1025 in [xx], and with definitions of projections in [aa].

-- ASN1START

CoordTransProjectionTypesBaseline-r19 ::= SEQUENCE {

projectionType-r19					ENUMERATED { unknown, tm, tms, lcc1sp, lccw, cs, om, os, mc,

ps, ds, ...},

latNaturalOrigin-r19				INTEGER (-8181818181.. 8181818181),

longNaturalOrigin-r19				INTEGER (-16363636362.. 16363636362),

scaleFactorNaturalOrigin-r19		INTEGER (0..1073741823),

falseNorthing-r19					INTEGER (0..68719476735),

falseEasting-r19					INTEGER (0..17179869183),

...

}

-- ASN1STOP

CoordTransProjectionTypesBaseline field descriptions

projectionType This field specifies the projection type, where: unknown: unknown project type tm: Transverse Mercator (TM), [aa] 1.4.5.1 EPSG dataset coordinate operation method code 9807. tms: Transverse Mercator, South-Oriented (TMS), [aa] 1.4.5.3 EPSG dataset coordinate operation method code 9808. lcc1sp: Lambert Conic Conformal, 1SP (LCC1SP), [aa] 1.4.1.2 EPSG dataset coordinate operation method code 9801. lccw: Lambert Conic Conformal, West-Oriented (LCCW), [aa] 1.4.1.3 EPSG dataset coordinate operation method code 9826. cs: Casini-Soldner (CS), [aa] 1.4.4 EPSG dataset coordinate operation method code 9806. os: Oblique Stereographic (OS), [aa] 1.4.7.1 EPSG dataset coordinate operation method code 9809. mc: Mercator (MC), [aa] 1.4.3 EPSG dataset coordinate operation method code 9804 or 9805. ps: Polar Stereographic (PS), [aa] 1.4.7.2 EPSG dataset coordinate operation method code 9810. ds: Double Stereographic (DS). If the projection type is unknown, only the 7-parameter transformation and the ellipsoid residual interpolation can be performed, not the local system residual interpolation nor projections.

latNaturalOrigin This field specifies latitude of the natural origin in degrees for projection methods TM, TMS, LCC1SP, LCCW, CS, OS, PS and DS. Scale factor 0.000000011 degrees.

longNaturalOrigin This field specifies longitude of the natural origin in degrees for projection methods TM, TMS, LCC1SP, LCCW, CS, OS, MC, PS and DS. Scale factor 0.000000011 degrees.

scaleFactorNaturalOrigin This field specifies the scale factor at the natural origin. Scale factor 0.00001 ppm, and the scale factor is the specified value + 993000 ppm.

falseNorthing, falseEasting These fields specify false Northing and Easting, respectively. Scale factor 0.001 m.

#### –	CoordTransProjectionLambertConicConformal

The IE CoordTransProjectionLambertConicConformal specifies a Lambert conic conformal projection as specified in message 1026 in [xx], and with definitions of projections in [aa], specifically 1.4.1.1 EPSG dataset coordinate operation method code 9802.

-- ASN1START

CoordTransProjectionLambertConicConformal-r19 ::= SEQUENCE {

latFalseOrigin-r19					INTEGER (-8181818181..8181818181),

longFalseOrigi-r19					INTEGER (-16363636362..16363636362),

latStdParallelNo1-r19				INTEGER (-8181818181..8181818181),

latStdParallelNo2-r19				INTEGER (-8181818181..8181818181),

northingFalseOrigin-r19				INTEGER (0..68719476735),

eastingFalseOrigin-r19				INTEGER (-17179869183..17179869183),

...

}

-- ASN1STOP

CoordTransProjectionLambertConicConformal field descriptions

latFalseOrigin This field specifies latitude of the false origin in degrees. Scale factor 0.000000011 degrees.

longFalseOrigin This field specifies longitude of the false origin in degrees. Scale factor 0.000000011 degrees.

latStdParallelNo1, latStdParallelNo2 These fields specify the latitude of the standard papallels No1 and No2, respectively. Scale factor 0.000000011 degrees.

northingFalseOrigin, eastingFalseOrigin These fields specify the Northing and Easting, respectively, of the false origin. Scale factor 0.001 m.

#### –	CoordTransProjectionObliqueMercator

The IE CoordTransProjectionObliqueMercator specifies a oblique mercator projection as specified in message 1027 in [xx], and with definitions of projections in [aa], specifically 1.4.6 EPSG dataset coordinate operation method code 9815.

-- ASN1START

CoordTransProjectionObliqueMercator-r19 ::= SEQUENCE {

rectificationFlag-r19			ENUMERATED { true },					OPTIONAL	-- Need ON

latProjectionCenter-r19			INTEGER (-8181818181..8181818181),

longProjectionCenter-r19		INTEGER (-16363636362..16363636362),

azIL-r19						INTEGER (0..32727272724),

diggARSG-r19					INTEGER (-33554431..33554431),

addSIL-r19						INTEGER (0..1073741823),

northingProjectionCenter-r19	INTEGER (0..68719476735),

eastingProjectionCenter-r19		INTEGER (-17179869183..17179869183),

...

}

-- ASN1STOP

CoordTransProjectionObliqueMercator field descriptions

rectificationFlag If present, rectified Oblique Mercator projection is considered, otherwise Oblique Mercator without rectification is considered.

latProjectionCenter, longProjectionCenter This field specifies latitude and longitude, respectively, of the projection center. Scale factor 0.000000011 degrees.

azIL This field specifies the azimuthof the initial line. Scale factor 0.000000011 degrees.

diffARSG This field specifies the difference from azimuth of initial line to angle from restricted to skew gris. Scale factor 0.000000011 degrees. ARSG = azIL + diffARSG.

addSIL This field specifies the additional scale factor on initial line, where SIL = 933000 + addSIL ppm. Scale factor 0.00001 ppm.

northingProjectionCenter, eastingProjectionCenter These fields specify the Northing and Easting, respectively, of the projection center. Scale factor 0.001 m.

NEXT CHANGE

### 6.4.2	Common Positioning

#### […]

#### –	CommonIEsProvideAssistanceData

The CommonIEsProvideAssistanceData carries common IEs for a Provide Assistance Data LPP message Type.

-- ASN1START

CommonIEsProvideAssistanceData ::= SEQUENCE {

...,

[[

segmentationInfo-r14		SegmentationInfo-r14		OPTIONAL	-- Need ON

]],

[[

periodicAssistanceData-r15	PeriodicAssistanceDataControlParameters-r15

OPTIONAL	-- Cond PerAD

]],

[[

coordTransformations-r19	CoordinateTransformations-r19																									OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| PerAD | The field is mandatory present in a periodic assistance data delivery session. Otherwise it is not present. |

CommonIEsProvideAssistanceData field descriptions

segmentationInfo This field indicates whether this ProvideAssistanceData message is one of many segments, as specified in clause 4.3.5.

periodicAssistanceData This field indicates a periodic assistance data delivery, as specified in clauses 5.2.1a and 5.2.2a.

coordTransformations This field provides coordinate transformations information.

#### […]

#### –	CommonIEsProvideLocationInformation

The CommonIEsProvideLocationInformation carries common IEs for a Provide Location Information LPP message Type.

-- ASN1START

CommonIEsProvideLocationInformation ::= SEQUENCE {

locationEstimate			LocationCoordinates		OPTIONAL,

velocityEstimate			Velocity				OPTIONAL,

locationError				LocationError			OPTIONAL,

...,

[[	earlyFixReport-r12		EarlyFixReport-r12		OPTIONAL

]],

[[	locationSource-r13		LocationSource-r13		OPTIONAL,

locationTimestamp-r13	UTCTime					OPTIONAL

]],

[[

segmentationInfo-r14	SegmentationInfo-r14	OPTIONAL		-- Cond Segmentation

]],

[[

integrityInfo-r17		IntegrityInfo-r17		OPTIONAL

]],

[[

locationCRS-r19			CoordinateReferenceSystem-r19	OPTIONAL

]]

}

LocationCoordinates ::= CHOICE {

ellipsoidPoint								Ellipsoid-Point,

ellipsoidPointWithUncertaintyCircle			Ellipsoid-PointWithUncertaintyCircle,

ellipsoidPointWithUncertaintyEllipse		EllipsoidPointWithUncertaintyEllipse,

polygon										Polygon,

ellipsoidPointWithAltitude					EllipsoidPointWithAltitude,

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid

EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

ellipsoidArc								EllipsoidArc,

...,

highAccuracyEllipsoidPointWithUncertaintyEllipse-v1510

HighAccuracyEllipsoidPointWithUncertaintyEllipse-r15,

highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-v1510

HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15,

ha-EllipsoidPointWithScalableUncertaintyEllipse-v1680																			HA-EllipsoidPointWithScalableUncertaintyEllipse-r16,

ha-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-v1680

HA-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-r16,

local2dPointWithUncertaintyEllipse-v1800	Local2dPointWithUncertaintyEllipse-r18,

local3dPointWithUncertaintyEllipsoid-v1800	Local3dPointWithUncertaintyEllipsoid-r18

}

Velocity ::= CHOICE {

horizontalVelocity							HorizontalVelocity,

horizontalWithVerticalVelocity				HorizontalWithVerticalVelocity,

horizontalVelocityWithUncertainty			HorizontalVelocityWithUncertainty,

horizontalWithVerticalVelocityAndUncertainty

HorizontalWithVerticalVelocityAndUncertainty,

...

}

LocationError ::= SEQUENCE {

locationfailurecause			LocationFailureCause,

...

}

LocationFailureCause ::= ENUMERATED {

undefined,

requestedMethodNotSupported,

positionMethodFailure,

periodicLocationMeasurementsNotAvailable,

...

}

EarlyFixReport-r12 ::= ENUMERATED {

noMoreMessages,

moreMessagesOnTheWay

}

LocationSource-r13 ::= BIT STRING {	a-gnss				(0),

wlan				(1),

bt					(2),

tbs					(3),

sensor				(4),

ha-gnss-v1510		(5),

motion-sensor-v1550 (6),

dl-tdoa-r16			(7),

dl-aod-r16			(8) } (SIZE(1..16))

IntegrityInfo-r17 ::= SEQUENCE {

horizontalProtectionLevel-r17		INTEGER (0..50000),

verticalProtectionLevel-r17			INTEGER (0..50000)				OPTIONAL,

achievableTargetIntegrityRisk-r17	INTEGER (10..90)				OPTIONAL,

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Segmentation | This field is optionally present, need OP, if lpp-message-segmentation-req has been received from the location server with bit 1 (targetToServer) set to value 1. The field shall be omitted if lpp-message-segmentation-req has not been received in this location session, or has been received with bit 1 (targetToServer) set to value 0. |

CommonIEsProvideLocationInformation field descriptions

locationEstimate This field provides a location estimate using one of the geographic shapes defined in TS 23.032 [15]. Coding of the values of the various fields internal to each geographic shape follow the rules in TS 23.032 [15]. The conditions for including this field are defined for the locationInformationType field in a Request Location Information message. NOTE: If the locationInformationType field in a Request Location Information message was set to 'locationEstimateAndMeasurementsRequired' and this field is absent, it means that the PRU location is not available.

velocityEstimate This field provides a velocity estimate using one of the velocity shapes defined in TS 23.032 [15]. Coding of the values of the various fields internal to each velocity shape follow the rules in TS 23.032 [15].

locationError This field shall be included if and only if a location estimate and measurements are not included in the LPP PDU. The field includes information concerning the reason for the lack of location information. The LocationFailureCause 'periodicLocationMeasurementsNotAvailable' shall be used by the target device if periodic location reporting was requested, but no measurements or location estimate are available when the reportingInterval expired.

earlyFixReport This field shall be included if and only if the ProvideLocationInformation message contains early location measurements or an early location estimate. The target device shall set the values of this field as follows: - noMoreMessages: This is the only or last ProvideLocationInformation message used to deliver the entire set of early location information. - moreMessagesOnTheWay: This is one of multiple ProvideLocationInformation messages used to deliver the entire set of early location information (if early location information will not fit into a single message). If this field is included, the IE SegmentationInfo shall not be included.

locationSource This field provides the source positioning technology for the location estimate. NOTE 1: In this version of the specification, the entry 'tbs' is used only for TBS positioning based on MBS signals. NOTE 2: The entry 'sensor' is used only for positioning technology that uses barometric pressure sensor. The entry 'motion-sensor' is used for positioning technology that uses sensor(s) to detect displacement and movement, e.g. accelerometers, gyros, magnetometers.

locationTimestamp This field provides the UTC time when the location estimate is valid and should take the form of YYMMDDhhmmssZ.

segmentationInfo This field indicates whether this ProvideLocationInformation message is one of many segments, as specified in clause 4.3.5

integrityInfo This field provides the integrity result for the locationEstimate. - horizontalProtectionLevel provides the HPL for the locationEstimate along the semi-major axis of the error ellipse. Scale factor 0.01 metre; range 0 – 500 metres. - verticalProtectionLevel provides the VPL for the locationEstimate. Scale factor 0.01 metre; range 0 – 500 metres. - achievableTargetIntegrityRisk indicates the achievable TIR for which the HPL and VPL are provided. The achievable TIR is given by P=10-0.1n [hour-1] where n is the value of achievableTargetIntegrityRisk and the range is 10-1 to 10-9 per hour. If this field is absent, the achievable TIR is the same as the targetIntegrityRisk in CommonIEsRequestLocationInformation.

locationCRS This field specifies the coordinate reference system (CRS) the location coordinates are related to. In case of Local 2D/3D coordinates, the specifies CRS relates to the reference point attribute. If not present, the location coordinates are related to a CRS as defined in TS 23.032 [15].

NOTE:	Void.

NEXT CHANGE

### 6.4.3	Common NR Positioning Information Elements

#### […]

#### –	NR-PositionCalculationAssistance

The IE NR-PositionCalculationAssistance is used by the location server to provide assistance data including integrity information to enable UE-based downlink positioning.

-- ASN1START

NR-PositionCalculationAssistance-r16 ::= SEQUENCE {

nr-TRP-LocationInfo-r16			NR-TRP-LocationInfo-r16				OPTIONAL,	-- Need ON

nr-DL-PRS-BeamInfo-r16			NR-DL-PRS-BeamInfo-r16				OPTIONAL,	-- Need ON

nr-RTD-Info-r16					NR-RTD-Info-r16						OPTIONAL,	-- Need ON

...,

[[

nr-TRP-BeamAntennaInfo-r17		NR-TRP-BeamAntennaInfo-r17			OPTIONAL,	-- Need ON

nr-DL-PRS-Expected-LOS-NLOS-Assistance-r17

NR-DL-PRS-ExpectedLOS-NLOS-Assistance-r17

OPTIONAL,	-- Need ON

nr-DL-PRS-TRP-TEG-Info-r17		NR-DL-PRS-TRP-TEG-Info-r17			OPTIONAL	-- Need ON

]],

[[

nr-IntegrityServiceParameters-r18	NR-IntegrityServiceParameters-r18	OPTIONAL,	-- Need OR

nr-IntegrityServiceAlert-r18		NR-IntegrityServiceAlert-r18		OPTIONAL,	-- Need OR

nr-IntegrityRiskParameters-r18		NR-IntegrityRiskParameters-r18		OPTIONAL,	-- Need OR

nr-IntegrityParametersTRP-LocationInfo-r18																								NR-IntegrityParametersTRP-LocationInfo-r18

OPTIONAL, -- Cond Integrity1

nr-IntegrityParametersDL-PRS-BeamInfo-r18

NR-IntegrityParametersDL-PRS-BeamInfo-r18

OPTIONAL, -- Cond Integrity2

nr-IntegrityParametersRTD-Info-r18																										NR-IntegrityParametersRTD-Info-r18

OPTIONAL, -- Cond Integrity3

nr-IntegrityParametersTRP-BeamAntennaInfo-r18																							NR-IntegrityParametersTRP-BeamAntennaInfo-r18

OPTIONAL, -- Cond Integrity4

nr-PRU-DL-Info-r18					NR-PRU-DL-Info-r18				OPTIONAL	-- Need ON

]],

[[

locationCRS-r19					CoordinateReferenceSystem-r19	OPTIONAL -- Need OP

]]

}

NR-IntegrityParametersTRP-LocationInfo-r18 ::= SEQUENCE {

trp-ErrorCorrelationTime-r18					INTEGER(0..255)		OPTIONAL, -- Need ON

dl-PRS-ResourceSetARP-ErrorCorrelationTime-r18	INTEGER(0..255)		OPTIONAL, -- Need ON

dl-PRS-ResourceARP-ErrorCorrelationTime-r18		INTEGER(0..255)		OPTIONAL, -- Need ON

...

}

NR-IntegrityParametersDL-PRS-BeamInfo-r18 ::= SEQUENCE {

dl-PRS-BeamInfoErrorCorrelationTime-r18		INTEGER (0..255),

...

}

NR-IntegrityParametersRTD-Info-r18 ::= SEQUENCE {

rtd-ErrorCorrelationTime-r18		INTEGER (0..255),

...

}

NR-IntegrityParametersTRP-BeamAntennaInfo-r18 ::= SEQUENCE {

trp-BeamAntennaInfoErrorCorrelationTime-r18		INTEGER (0..255),

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Integrity1 | The field is optionally present, need OR, if nr-TRP-LocationInfo is present and nr-IntegrityTRP-LocationBounds is present in IE NR-TRP-LocationInfo; otherwise it is not present. |
| Integrity2 | The field is optionally present, need OR, if nr-DL-PRS-BeamInfo is present and nr-IntegrityBeamInfoBounds is present in IE NR-DL-PRS-BeamInfo; otherwise it is not present. |
| Integrity3 | The field is optionally present, need OR, if nr-RTD-Info is present and nr-IntegrityRTD-InfoBounds is present in IE NR-RTD-Info; otherwise it is not present. |
| Integrity4 | The field is optionally present, need OR, if nr-TRP-BeamAntennaInfo is present and nr-IntegrityBeamPowerBounds is present in IE NR-TRP-BeamAntennaInfo; otherwise it is not present. |

NR-PositionCalculationAssistance field descriptions

nr-TRP-LocationInfo This field provides the location coordinates of the TRPs and location coordinates of antenna reference points for DL-PRS Resource Set(s) and DL-PRS Resources of the TRPs.

nr-DL-PRS-BeamInfo This field provides the spatial directions of DL-PRS Resources for TRPs.

nr-RTD-Info This field provides the time synchronization information between the reference TRP and neighbour TRPs.

nr-TRP-BeamAntennaInfo This field provides the relative DL-PRS Resource power between DL-PRS Resources per angle per TRP.

nr-DL-PRS-ExpectedLOS-NLOS-Assistance This field provides the expected likelihood of a LOS propagation path from a TRP to the target device. The information is provided per TRP or per DL-PRS Resource.

nr-DL-PRS-TRP-TEG-Info This field provides the TRP Tx TEG ID associated with the transmission of each DL-PRS Resource of the TRP.

nr-IntegrityServiceParameters This field specifies the range of Integrity Risk (IR) for which the integrity assistance data are valid.

nr-IntegrityServiceAlert This field indicates whether the corresponding assistance data can be used for integrity related applications.

trp-ErrorCorrelationTime This field specifies the TRP Error Correlation Time which is the upper bound of the correlation time of the TRP error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

dl-PRS-ResourceSetARP-ErrorCorrelationTime This field, if present, specifies the DL-PRS Resource Set ARP Error Correlation Time which is the upper bound of the correlation time of the DL-PRS Resource Set ARP error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

dl-PRS-ResourceARP-ErrorCorrelationTime This field, if present, specifies the DL-PRS Resource ARP Error Correlation Time which is the upper bound of the correlation time of the DL-PRS Resource ARP error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

rtd-ErrorCorrelationTime This field specifies the inter-TRP synchronization error Correlation Time which is the upper bound of the correlation time of the inter-TRP synchronization error. The correlation time is calculated using: t=10i, 1≤&i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, 234<i [s] Where i is the value given by rtdErrorCorrelationTime. Range is 1-28,200 s.

dl-PRS-BeamInfoErrorCorrelationTime This field specifies the Beam Boresight Direction Angle Error Correlation Time which is the upper bound of the correlation time of the DL-PRS Resource angle error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

trp-BeamAntennaInfoErrorCorrelationTime This field specifies the Mean Beam Power Error Correlation Time which is the upper bound of the correlation time of the mean beam power error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

nr-PRU-DL-Info This field provides the measurements reported by a PRU to the target UE.

locationCRS This field specifies the coordinate reference system (CRS) the location coordinates are related to in this IE If not present, the coordinates are defined as specified in the corresponding field descriptions.

NEXT CHANGE

#### 6.5.2.2	GNSS Assistance Data Elements

#### […]

#### –	GNSS-RTK-ReferenceStationInfo

The IE GNSS-RTK-ReferenceStationInfo is used by the location server to provide the Earth-centered, Earth-fixed (ECEF) coordinates of the antenna reference point (ARP) of the stationary reference station for which the GNSS-RTK-Observations assistance data are provided together with reference station antenna description.

The parameters provided in IE GNSS-RTK-ReferenceStationInfo are used as specified for message type 1006, 1033 and 1032 in [30].

-- ASN1START

GNSS-RTK-ReferenceStationInfo-r15 ::= SEQUENCE {

referenceStationID-r15					GNSS-ReferenceStationID-r15,

referenceStationIndicator-r15			ENUMERATED {physical, non-physical},

antenna-reference-point-ECEF-X-r15		INTEGER (-137438953472..137438953471),

antenna-reference-point-ECEF-Y-r15		INTEGER (-137438953472..137438953471),

antenna-reference-point-ECEF-Z-r15		INTEGER (-137438953472..137438953471),

antennaHeight-r15						INTEGER (0..65535)					OPTIONAL, -- Need ON

antennaDescription-r15					AntennaDescription-r15				OPTIONAL, -- Need ON

antenna-reference-point-unc-r15			AntennaReferencePointUnc-r15		OPTIONAL, -- Need ON

physical-reference-station-info-r15		PhysicalReferenceStationInfo-r15	OPTIONAL, -- Cond NP

...,

[[

equalIntegerAmbiguityLevel-r16			EqualIntegerAmbiguityLevel-r16		OPTIONAL -- Need ON

]],

[[

locationCRS-r19						CoordinateReferenceSystem-r19		OPTIONAL -- Need ON

]]

}

AntennaDescription-r15 ::= SEQUENCE {

antennaDescriptor-r15					VisibleString (SIZE (1..256)),

antennaSetUpID-r15						ENUMERATED { non-zero }				OPTIONAL, -- Need OP

...

}

AntennaReferencePointUnc-r15 ::= SEQUENCE {

uncertainty-X-r15						INTEGER (0..255),

confidence-X-r15						INTEGER (0..100),

uncertainty-Y-r15						INTEGER (0..255),

confidence-Y-r15						INTEGER (0..100),

uncertainty-Z-r15						INTEGER (0..255),

confidence-Z-r15						INTEGER (0..100),

...

}

PhysicalReferenceStationInfo-r15 ::= SEQUENCE {

physicalReferenceStationID-r15			GNSS-ReferenceStationID-r15,

physical-ARP-ECEF-X-r15					INTEGER (-137438953472..137438953471),

physical-ARP-ECEF-Y-r15					INTEGER (-137438953472..137438953471),

physical-ARP-ECEF-Z-r15					INTEGER (-137438953472..137438953471),

physical-ARP-unc-r15					AntennaReferencePointUnc-r15		OPTIONAL, -- Need ON

...

}

EqualIntegerAmbiguityLevel-r16 ::= CHOICE {

allReferenceStations-r16				NULL,

referenceStationList-r16				ReferenceStationList-r16

}

ReferenceStationList-r16 ::= SEQUENCE (SIZE(1..16)) OF GNSS-ReferenceStationID-r15

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NP | The field is optionally present, need ON, if the referenceStationIndicator has the value ′non-physical′; otherwise it is not present. |

| GNSS-RTK-ReferenceStationInfo field descriptions |
| --- |
| referenceStationID The Reference Station ID is determined by the RTK service provider. |
| referenceStationIndicator This fields specifies type of reference station. Enumerated value physical indicates a real, physical reference station; value non-physical indicates a non-physical or computed reference station. |
| antenna-reference-point-ECEF-X This field specifies the antenna reference point X-coordinate in the datum associated to the locationCRS if present: otherwise in the World Geodetic System 1984 (WGS 84) datum. Scale factor 0.0001 m; range ±13,743,895.3471 m. |
| antenna-reference-point-ECEF-Y This field specifies the antenna reference point Y-coordinate in the datum associated to the locationCRS if present: otherwise in the World Geodetic System 1984 (WGS 84) datum. Scale factor 0.0001 m; range ±13,743,895.3471 m. |
| antenna-reference-point-ECEF-Z This field specifies the antenna reference point Z-coordinate in the datum associated to the locationCRS if present: otherwise in the World Geodetic System 1984 (WGS 84) datum. Scale factor 0.0001 m; range ±13,743,895.3471 m. |
| antennaHeight This field specifies the height of the Antenna Reference Point above the marker used in the survey campaign. Scale factor 0.0001 m; range 0–6.5535 m. |
| antennaDescriptor This field provides an ASCII descriptor of the reference station antenna using IGS naming convention [31]. The descriptor can be used to look up model specific phase centre corrections of that antenna. |
| antennaSetUpID This field, if present, indicates that the standard IGS Model is not valid (≠ 0 [30]). If this field is absent the standard IGS Model is valid (′0 = Use standard IGS Model′ [30]). |
| antenna-reference-point-unc This field specifies the uncertainty of the ARP coordinates. uncertainty-X, uncertainty-Y, and uncertainty-Z correspond to the encoded high accuracy uncertainty of the X, Y, and Z-coordinate, respectively, as defined in TS 23.032 [15]. confidence-X, confidence-Y, and confidence-Z corresponds to confidence as defined in TS 23.032 [15]. |
| physical-reference-station-info This field provides the earth-centred, earth-fixed (ECEF) coordinates of the antenna reference point (ARP) for the real (or "physical") reference station used. This field may be used in the case of the non-physical reference station approach to allow the target device to refer baseline vectors to a physical reference rather than to a non-physical reference without any connection to a physical point. |
| physicalReferenceStationID This field specifies the station ID of a real reference station, when the referenceStationIndicator has the value ′non-physical′. |
| physical-ARP-ECEF-X This field specifies the antenna reference point X-coordinate in the datum associated to the locationCRS if present: otherwise in the World Geodetic System 1984 (WGS 84) datum. Scale factor 0.0001 m; range ±13,743,895.3471 m. |
| physical-ARP-ECEF-Y This field specifies the antenna reference point Y-coordinate in the datum associated to the locationCRS if present: otherwise in the World Geodetic System 1984 (WGS 84) datum. Scale factor 0.0001 m; range ±13,743,895.3471 m. |
| physical-ARP-ECEF-Z This field specifies the antenna reference point Z-coordinate in the datum associated to the locationCRS if present: otherwise in the World Geodetic System 1984 (WGS 84) datum. Scale factor 0.0001 m; range ±13,743,895.3471 m. |
| physical-ARP-unc This field specifies the uncertainty of the ARP coordinates. |
| equalIntegerAmbiguityLevel This field specifies the integer ambiguity level of this reference station in relation to other reference stations. Either, the presence or absence of allReferenceStations indicates whether the integer ambiguity level may be assumed to be aligned between all reference stations or not (interpreted as no alignment is facilitated from the location server), or referenceStationList provides a list of reference stations for which the integer ambiguity level may be assumed to be the same. |  |
| locationCRS This field specifies the coordinate reference system (CRS) the location coordinates of this IE are related to. |  |

#### […]

#### –	GNSS-SSR-CorrectionPoints

The IE GNSS-SSR-CorrectionPoints is used by the location server to provide a list of correction point coordinates or an array of correction points ("grid") for which the GNSS-SSR-GriddedCorrection are valid.

-- ASN1START

GNSS-SSR-CorrectionPoints-r16 ::= SEQUENCE {

correctionPointSetID-r16			INTEGER (0..16383),

correctionPoints-r16				CHOICE {

listOfCorrectionPoints-r16			GNSS-SSR-ListOfCorrectionPoints-r16,

arrayOfCorrectionPoints-r16			GNSS-SSR-ArrayOfCorrectionPoints-r16

},

...,

[[

locationCRS-r19					CoordinateReferenceSystem-r19	OPTIONAL -- Cond SSRorbit

]]

}

GNSS-SSR-ListOfCorrectionPoints-r16 ::= SEQUENCE {

referencePointLatitude-r16		INTEGER (-16384..16383),

referencePointLongitude-r16		INTEGER (-32768..32767),

relativeLocationsList-r16		SEQUENCE (SIZE (0..63)) OF RelativeLocationElement-r16,

...

}

RelativeLocationElement-r16 ::= SEQUENCE {

deltaLatitude-r16				INTEGER (-512..511),

deltaLongitude-r16				INTEGER (-1024..1023),

...

}

GNSS-SSR-ArrayOfCorrectionPoints-r16 ::=SEQUENCE {

referencePointLatitude-r16			INTEGER (-16384..16383),

referencePointLongitude-r16			INTEGER (-32768..32767),

numberOfStepsLatitude-r16			INTEGER (0..63),

numberOfStepsLongitude-r16			INTEGER (0..63),

stepOfLatitude-r16					INTEGER (1..511),

stepOfLongitude-r16					INTEGER (1..1023),

bitmaskOfGrids-r16					BIT STRING (SIZE(64))				OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

GNSS-SSR-CorrectionPoints field descriptions

correctionPointSetID This field provides the ID of the Atmospheric Correction Point set. It is a regionally unique arbitrary number that is used by the UE to ensure that the atmospheric corrections are being applied to the correct set of points.

referencePointLatitude This field specifies the latitude for the reference point, expressed in the range of -90° , +90°, coded as a number between -214 and 214-1, coded in 2's complement binary on 15 bits. The relation between the latitude X in the range [-90°, 90°] and the coded number N is: where denotes the greatest integer less than or equal to x (floor operator). For the listOfCorrectionPoints, the reference point defines the 1st correction point location. For the arrayOfCorrectionPoints, the reference point defines the northwest corner of the correction point array.

referencePointLongitude This field specifies the longitude for the reference point, expressed in the range -180°, +180°, coded as a number between -215 and 215-1, coded in 2's complement binary on 16 bits. The relation between the longitude X in the range [-180°, 180°) and the coded number N is: For the listOfCorrectionPoints, the reference point defines the 1st correction point location. For the arrayOfCorrectionPoints, the reference point defines the northwest corner of the correction point array.

relativeLocationsList This field specifies the 2nd, 3rd, …, 64th correction point location.

deltaLatitude This field specifies the delta value in latitude of this correction point location relative to the previous point on the list or the reference point in the case of the first additional point, defined as "correction point location" minus "previous correction point location" in units of 0.01 degrees.

deltaLongitude This field specifies the delta value in longitude of this correction point location relative to the previous point on the list or the reference point in the case of the first additional point, defined as "correction point location" minus "previous correction point location" in units of 0.01 degrees.

numberOfStepsLatitude, numberOfStepsLongitude These fields specify the number of steps for latitude and longitude direction respectively.

stepOfLatitude, stepOfLongitude These fields specify the spacing of the correction points for latitude and longitude respectively. The unit and scale factor is 0.01 degrees.

bitmaskOfGrids This field specifies the availability of correction data at the correction points in the array. If a specific bit is enabled (set to '1'), the correction is available. Only the first (numberOfStepsLatitude + 1)×(numberOfStepsLongitude + 1) bits are used, the remainder are set to '0'. Starting with the northwest corner of the array (top left on a north oriented map) the correction points are enumerated with row precedence – first row west to east, second row west to east, until last row west to east – ending with the southeast corner of the array. If the field is omitted all correction points are used and none omitted.

locationCRS This field specifies the coordinate reference system (CRS) the location coordinates of this IE are related to. If the field gnss-SSR-OrbitCorrections is present in any instance of the IE GNSS-GenericAssistDataElement the CRS of the location coordinates is the same as of this field gnss-SSR-OrbitCorrections; otherwise, the location coordinates are related to WGS 84.

| Conditional presence | Explanation |
| --- | --- |
| SSRorbit | This field is mandatory present if there is no associated IE GNSS-SSR-OrbitCorrections present; otherwise it is not present. |

#### […]

#### –	GNSS-LOS-NLOS-GridPoints

The IE GNSS-LOS-NLOS-GridPoints is used by the location server to provide a list of grid point coordinates or an array of correction points ("grid") for which specific assistance data can be provided.

-- ASN1START

GNSS-LOS-NLOS-GridPoints-r18 ::= SEQUENCE {

gridPointsSetID-r18				INTEGER (0..16383),

horizontalGridPoints-r18		ArrayOfGridPoints-r18,

referenceAltitudeFine-r18		INTEGER (0..9)					OPTIONAL, -- Need OP

verticalGridPoints-r18			VerticalGridPoints-r18			OPTIONAL, -- Cond 3D

...,

[[

locationCRS-r19				CoordinateReferenceSystem-r19	OPTIONAL -- Need OP

]]

}

ArrayOfGridPoints-r18 ::= SEQUENCE {

referencePointLatitude-r18		INTEGER (-16777216.. 16777215),

referencePointLongitude-r18		INTEGER (-33554432.. 33554431),

numberOfStepsSouth-r18			INTEGER (0.. 255),

numberOfStepsEast-r18			INTEGER (0.. 255),

stepSouth-r18					SpatialDelta-r18,

stepEast-r18					SpatialDelta-r18,

bitmaskOfGrids-r18				CHOICE {

bog16-r18						BIT STRING (SIZE(16)),

bog64-r18						BIT STRING (SIZE(64)),

bog256-r18						BIT STRING (SIZE(256)),

...

}																OPTIONAL, -- Need OP

...

}

VerticalGridPoints-r18 ::=SEQUENCE {

referenceAltitudeCoarse-r18		INTEGER (-50..900),

numberOfStepsDown-r18			INTEGER (1..3),

stepDown-r18					SpatialDelta-r18,

upperValidityDeltaAltitude-r18	SpatialDelta-r18				OPTIONAL, -- Need OP

lowerValidityDeltaAltitude-r18	SpatialDelta-r18				OPTIONAL, -- Need OP

...

}

SpatialDelta-r18 ::= ENUMERATED {n1, n2, n3, n4, n5, n10, n20, n50, n100}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| 3D | This field is mandatory present if a 3D grid is provided; otherwise it is absent. |

GNSS-LOS-NLOS-GridPoints field descriptions

gridPointsSetID This field provides the ID of the spatial grid point set. It is a regionally unique arbitrary number that is used by the UE to ensure that provided assistance data associated to a spatial grid point set is being applied to the correct set of points. The grid point set ID identifies a grid defined by a reference point corresponding to the northwest corner (of the upper layer in case of a 3D grid), where the rest of the grid is defined by a number of steps and step lengths in the south, east (down in case of a 3D grid). The grid is valid within the horizontal perimeter of the grid and between an upper and lower validity altitude, where the upper validity altitude is (referenceAltitudeFine + 10* referenceAltitudeCoarse + upperValidityDeltaAltitude), and the lower validity altitude is (referenceAltitudeFine + 10* referenceAltitudeCoarse – numberOfStepsDown*stepDown – lowerValidityDeltaAltitude).

referencePointLatitude This field specifies the latitude for the reference point, expressed in the range of -90° , +90°, coded as a number between -224 and 224-1, coded in 2's complement binary on 25 bits. The relation between the latitude X in the range [-90°, 90°] and the coded number N is: N=X90∘224 where denotes the greatest integer less than or equal to x (floor operator). The reference point defines the northwest corner of the grid point array.

referencePointLongitude This field specifies the longitude for the reference point, expressed in the range -180°, +180°, coded as a number between -225 and 225-1, coded in 2's complement binary on 26 bits. The relation between the longitude X in the range [-180°, 180°) and the coded number N is: N=X180∘225 The reference point defines the northwest corner of the grid point array.

numberOfStepsSouth, numberOfStepsEast, numberOfStepsDown These fields specify the number of steps for south, east and down direction respectively.

stepSouth, stepEast, stepDown These fields specify the spacing of the grid points for south, east and down respectively. Enumerated values n1, n2, n3, n4, n5, n10, n20, n50, n100 correspond to 1, 2, 3, 4, 5, 10, 20, 50, 100 meters, respectively.

bitmaskOfGrids This field specifies the availability of grid data at the horizontal grid points in the array and applies to all altitude layers of the grid. If a specific bit is enabled (set to '1'), the grid is available. Only the first (numberOfStepsSouth+1)×(numberOfStepsEast+1) bits are used, the remainder are set to '0'. Starting with the northwest corner of the array (top left on a north oriented map) the grid points are enumerated with row precedence – first row west to east, second row west to east, until last row west to east – ending with the southeast corner of the array. If the field is omitted all grid points are used and none omitted.

referenceAltitudeFine If this field is present and - the field referenceAltitudeCoarse is also present, it provides the fine resolution of the 3D grid altitude of the upmost layer, or - the field referenceAltitudeCoarse is absent, it provides the altitude above ground level of the 2D grid, with a scale factor of 1m. If the field is absent, the default value is 0m.

referenceAltitudeCoarse This field is present if a 3D grid is provided and specifies the coarse altitude, scale factor 10m, of the upmost layer of the grid relative to the WGS84 ellipsoid. If this field is absent, a 2D grid is provided, valid for ground level.

upperValidityDeltaAltitude This field, if present, specifies the upper validity altitude relative to the grid upper layer altitude. Enumerated values n1, n2, n3, n4, n5, n10, n20, n50, n100 correspond to 1, 2, 3, 4, 5, 10, 20, 50, 100 meters, respectively. If the field is absent, the default value is 0m.

lowerValidityDeltaAltitude This field, if present, specifies the lower validity altitude relative to the lowest grid layer altitude. Enumerated values n1, n2, n3, n4, n5, n10, n20, n50, n100 correspond to 1, 2, 3, 4, 5, 10, 20, 50, 100 meters, respectively. If the field is absent, the default value is 0m.

locationCRS This field specifies the coordinate reference system (CRS) the location coordinates of this IE are related to. Otherwise, the location coordinates are related to WGS 84.

#### […]

#### –	GNSS-SSR-OrbitCorrections

The IE GNSS-SSR-OrbitCorrections is used by the location server to provide radial, along-track and cross-track orbit corrections together with integrity information. The target device may use the SSR-OrbitCorrectionList to compute a satellite position correction to be combined with the satellite position calculated from broadcast ephemeris.

The parameters provided in IE GNSS-SSR-OrbitCorrections – except for ORBIT-IntegrityParameters and SSR-IntegrityOrbitBounds – are used as specified for SSR Orbit Messages (e.g., message type 1057 and 1063) in [30] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-OrbitCorrections-r15 ::= SEQUENCE {

epochTime-r15						GNSS-SystemTime,

ssrUpdateInterval-r15				INTEGER (0..15),

satelliteReferenceDatum-r15			ENUMERATED { itrf, regional, ..., locationCRS },

iod-ssr-r15							INTEGER (0..15),

ssr-OrbitCorrectionList-r15			SSR-OrbitCorrectionList-r15,

...,

[[

orbit-IntegrityParameters-r17	ORBIT-IntegrityParameters-r17	OPTIONAL -- Need OR

]],

[[

locationCRS-r19					CoordinateReferenceSystem-r19	OPTIONAL – Cond locCRS

]]

}

SSR-OrbitCorrectionList-r15 ::= SEQUENCE (SIZE(1..64)) OF SSR-OrbitCorrectionSatelliteElement-r15

SSR-OrbitCorrectionSatelliteElement-r15 ::= SEQUENCE {

svID-r15							SV-ID,

iod-r15								BIT STRING (SIZE(11)),

delta-radial-r15					INTEGER (-2097152..2097151),

delta-AlongTrack-r15				INTEGER (-524288..524287),

delta-CrossTrack-r15				INTEGER (-524288..524287),

dot-delta-radial-r15				INTEGER (-1048576..1048575)		OPTIONAL, -- Need ON

dot-delta-AlongTrack-r15			INTEGER (-262144..262143)		OPTIONAL, -- Need ON

dot-delta-CrossTrack-r15			INTEGER (-262144..262143)		OPTIONAL, -- Need ON

...,

[[

ssr-IntegrityOrbitBounds-r17	SSR-IntegrityOrbitBounds-r17	OPTIONAL  -- Cond Integrity1

]]

}

ORBIT-IntegrityParameters-r17 ::= SEQUENCE {

probOnsetConstFault-r17					INTEGER (0..255),

meanConstFaultDuration-r17				INTEGER (1..3600),

probOnsetSatFault-r17					INTEGER (0..255),

meanSatFaultDuration-r17				INTEGER (1..3600),

orbitRangeErrorCorrelationTime-r17		INTEGER (0..255)			OPTIONAL, -- Need OR

orbitRangeRateErrorCorrelationTime-r17	INTEGER (0..255)			OPTIONAL, -- Cond Integrity2

...

}

SSR-IntegrityOrbitBounds-r17 ::= SEQUENCE {

meanOrbitError-r17				RAC-OrbitalErrorComponents-r17,

stdDevOrbitError-r17			RAC-OrbitalErrorComponents-r17,

meanOrbitRateError-r17			RAC-OrbitalErrorComponents-r17,

stdDevOrbitRateError-r17		RAC-OrbitalErrorComponents-r17,

...

}

RAC-OrbitalErrorComponents-r17	::= SEQUENCE {

radial-r17			INTEGER (0..255),

alongTrack-r17		INTEGER (0..255),

crossTrack-r17		INTEGER (0..255)

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Integrity1 | The field is mandatory present if ORBIT-IntegrityParameters is present; otherwise it is not present. |
| Integrity2 | The field is mandatory present if orbitRangeErrorCorrelationTime is present; otherwise it is not present. |
| locCRS | This field is mandatory present if the satelliteReferenceDatum field is set to “locationCRS”, otherwise it is not present. |

GNSS-SSR-OrbitCorrections field descriptions

epochTime This field specifies the epoch time of the orbit corrections. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation below. NOTE 1.

satelliteReferenceDatum This field specifies the satellite refence datum for the orbit corrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

svID This field specifies the satellite for which the orbit corrections are provided.

iod This field specifies the IOD value of the broadcast ephemeris for which the orbit corrections are valid (see IE GNSS-NavigationModel). NOTE 2, NOTE 4.

delta-radial This field specifies the radial orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.1 mm; range ±209.7151 m.

delta-AlongTrack This field specifies the along-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.4 mm; range ±209.7148 m.

delta-CrossTrack This field specifies the cross-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.4 mm; range ±209.7148 m.

dot-delta-radial This field specifies the velocity of radial orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.001 mm/s; range ±1.048575 m/s.

dot-delta-AlongTrack This field specifies the velocity of along-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.004 mm/s; range ±1.048572 m/s.

dot-delta-CrossTrack This field specifies the velocity of cross-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.004 mm/s; range ±1.048572 m/s.

probOnsetConstFault This field specifies the Probability of Onset of Constellation Fault per Time Unit where a constellation fault is at least two satellites being faulty simultaneously due to the same event. This field specifies the onset probability that the residual range or range rate error exceeds a bound created using the minimum allowed inflation factor Kmin, and bounding parameters as mean + Kmin * stdDev where Kmin = normInv(irMaximum / 2), with irMaximum as provided in IE GNSS-Integrity-ServiceParameters. The probability is calculated by P=10-0.04n [hour-1] where n is the value of probOnsetConstFault and the range is 10-10.2 to 1 per hour.

meanConstFaultDuration This field specifies the Mean Constellation Fault Duration which is the mean duration between when a constellation fault occurs, and the user is alerted by IE GNSS-RealTimeIntegrity (or the integrity violation is over). Scale factor 1 s; range 1-3600 s.

probOnsetSatFault This field specifies the Probability of Onset of Satellite Fault per Time Unit which is the probability of occurrence of satellite error to exceed the residual error bound for more than the Time to Alert (TTA). This field specifies the onset probability that the residual range or range rate error exceeds a bound created using the minimum allowed inflation factor Kmin, and bounding parameters as mean + Kmin * stdDev where Kmin = normInv(irMaximum / 2), with irMaximum as provided in IE GNSS-Integrity-ServiceParameters. The probability is calculated by P=10-0.04n [hour-1] where n is the value of probOnsetSatFault and the range is 10-10.2 to 1 per hour.

meanSatFaultDuration This field specifies the Mean Satellite Fault Duration which is the mean duration between when a satellite fault occurs, and the user is alerted by IE GNSS-RealTimeIntegrity (or the integrity violation is over). Scale factor 1 s; range 1-3,600 s.

orbitRangeErrorCorrelationTime This field specifies the Orbit Range Error Correlation Time which is the upper bound of the correlation time of the satellite residual range error due to orbit. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

orbitRangeRateErrorCorrelationTime This field specifies the Orbit Range Rate Error Correlation Time which is the upper bound of the correlation time of the satellite residual range rate error due to orbit. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

meanOrbitError This field specifies the Mean Orbit Error bound in satellite radial, along-track and cross-track coordinates, which are the mean values for a set of three overbounding models that bound the residual orbit error in satellite radial, along-track and cross-track directions. Each mean is calculated using: μ=0.01i, &i≤2002+0.1(i-200), 200<&i≤230 5+0.5i-230, &i>230 [m] Range is 0-17.5 m.

stdDevOrbitError This field specifies the Standard Deviation Orbit Error bound in satellite radial, along-track and cross-track coordinates, which are the standard deviation values for a set of three overbounding models that bound the residual orbit error in satellite radial, along-track and cross-track directions. Each standard deviation is calculated using: σ=0.01i, &i≤2002+0.1(i-200), 200<&i≤230 5+0.5i-230, &i>230 [m] Range is 0-17.5 m.

meanOrbitRateError This field specifies the Mean Orbit Rate Error in satellite radial, along-track and cross-track coordinates, which are the mean values for a set of three overbounding models that bound the residual satellite orbit rate error in satellite radial, along-track and cross-track directions. Scale factor 0.001 m/s; range 0-0.255 m/s.

stdDevOrbitRateError This field specifies the Standard Deviation Orbit Rate Error in satellite radial, along-track and cross-track coordinates, which are the standard deviation values for a set of three overbounding models that bound the residual satellite orbit rate error in satellite radial, along-track and cross-track directions. Scale factor 0.001 m/s; range 0-0.255 m/s.

locationCRS This field specifies the coordinate reference system (CRS) for the satellite reference datum.

NOTE 1:	The update intervals are aligned to the GPS time scale for all GNSSs in order to allow synchronous operation for multiple GNSS services. This means that the update intervals may not be aligned to the beginning of the day for another GNSS. Due to the leap seconds, this is generally the case for GLONASS.

NOTE 2:	In the cases that gnss-ID indicates 'gps', 'qzss' or 'bds', the iod refers to the NAV broadcast ephemeris (GPS L1 C/A, QZSS QZS-L1 or BDS B1I/B3I, respectively, in table GNSS to iod Bit String(11) relation in IE GNSS-NavigationModel).

NOTE 3:	The reference time t0 is epochTime + ½ × ssrUpdateInterval. The reference time t0 for ssrUpdateInterval '0' is epochTime.

NOTE 4:	In the cases that the GNSS-SSR-OrbitCorrection IE is contained within a GNSS-SSR-OrbitCorrectionsSet2 IE, then the reference ephemeris is denoted by the refEph field within the GNSS-SSR-OrbitCorrectionsSet2 IE, not the default as per NOTE 2. In the case gnss-ID indicates 'bds' and if refEph is set to b1c, the iod refers to the B-CNAV1 broadcast ephemeris (BDS B1C/B2a in table GNSS to iod Bit String (11) relation in IE GNSS-NavigationModel).

Value of ssrUpdateInterval to SSR Update Interval relation

| Value of ssrUpdateInterval | SSR Update Interval |
| --- | --- |
| 0 | 1 second |
| 1 | 2 seconds |
| 2 | 5 seconds |
| 3 | 10 seconds |
| 4 | 15 seconds |
| 5 | 30 seconds |
| 6 | 60 seconds |
| 7 | 120 seconds |
| 8 | 240 seconds |
| 9 | 300 seconds |
| 10 | 600 seconds |
| 11 | 900 seconds |
| 12 | 1800 seconds |
| 13 | 3600 seconds |
| 14 | 7200 seconds |
| 15 | 10800 seconds |

NEXT CHANGE

#### 6.5.4.8	TBS Assistance Data Elements

#### –	TBS-AssistanceDataList

The IE TBS-AssistanceDataList is used by the location server to provide the TBS specific assistance data to the UE.

-- ASN1START

TBS-AssistanceDataList-r14 ::= SEQUENCE {

mbs-AssistanceDataList-r14		MBS-AssistanceDataList-r14		OPTIONAL,	-- Need ON

...,

[[

locationCRS-r19				CoordinateReferenceSystem-r19	OPTIONAL	-- Need ON

]]

}

MBS-AssistanceDataList-r14 ::= SEQUENCE (SIZE (1..maxMBS-r14)) OF MBS-AssistanceDataElement-r14

MBS-AssistanceDataElement-r14 ::= SEQUENCE {

mbs-AlmanacAssistance-r14		MBS-AlmanacAssistance-r14			OPTIONAL,	-- Need ON

mbs-AcquisitionAssistance-r14	MBS-AcquisitionAssistance-r14		OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

TBS-AssistanceDataList field descriptions

locationCRS This field specifies the coordinate reference system (CRS) the location coordinates of this IE are related to.

#### –	MBS-AlmanacAssistance

The IE MBS-AlmanacAssistance is used by the location server to provide LLA of MBS transmitters to enable position estimation at the UE.

-- ASN1START

MBS-AlmanacAssistance-r14 ::= SEQUENCE {

transmitterID-r14			INTEGER (0..32767),

transmitterLatitude-r14		BIT STRING (SIZE (26)),

transmitterLongitude-r14	BIT STRING (SIZE (27)),

transmitterAltitude-r14		BIT STRING (SIZE (15)),

timeCorrection-r14			INTEGER (0..25)		OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

MBS-AlmanacAssistance field descriptions

transmitterID This field specifies the MBS transmitter ID [24].

transmitterLatitude This field specifies latitude of the MBS transmitter, degrees. Scale factor 4/220 decimal degrees, added to -90°. Valid range -90° to 90° [24].

transmitterLongitude This field specifies longitude of the MBS transmitter, degrees. Scale factor 4/220 decimal degrees, added to -180°. Valid range -180° to 180° [24].

transmitterAltitude This field specifies altitude of the MBS transmitter, metres. Scale factor 0.29 metres, added to -500 metres. Valid range -500 to 9002.43 metres [24].

timeCorrection This field contains the residual timing error for a particular beacon, in units of nano-seconds, in the range from 0 to 25. This field is used for UE-based mode only, by subtracting from the codePhase measurement made by the target device [24].

#### –	MBS-AcquisitionAssistance

The IE MBS-AcquisitionAssistance is used by the location server to provide parameters that support acquisition of the MBS signals [24].

-- ASN1START

MBS-AcquisitionAssistance-r14 ::= SEQUENCE {

transmitterID-r14				INTEGER (0..32767)						OPTIONAL,	-- Need ON

mbsConfiguration-r14			ENUMERATED {tb1, tb2, tb3, tb4, ...}	OPTIONAL,	-- Need ON

pnCodeIndex-r14					INTEGER (1..128)						OPTIONAL,	-- Need ON

freq-r14						INTEGER (919750000..927250000)			OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

MBS-AcquisitionAssistance field descriptions

transmitterID This field contains the MBS transmitter identifier [24].

mbsConfiguration This field specifies MBS configuration as defined in the MBS ICD [24].

pnCodeIndex This field specifies the index of the MBS PN code [24].

freq This field specifies the MBS signal centre frequency in units of Hz [24].

NEXT CHANGE

#### 6.5.5.8	Sensor Assistance Data Elements

#### –	Sensor-AssistanceDataList

The IE Sensor-AssistanceDataList is used by the location server to provide the Sensor specific assistance data to the UE.

-- ASN1START

Sensor-AssistanceDataList-r14::= SEQUENCE {

refPressure-r14		INTEGER (-20000..10000),

refPosition-r14		EllipsoidPointWithAltitudeAndUncertaintyEllipsoid	OPTIONAL,	-- Need ON

refTemperature-r14	INTEGER (-64..63)									OPTIONAL,	-- Need ON

...,

[[

period-v1520		SEQUENCE {

pressureValidityPeriod-v1520	PressureValidityPeriod-v1520,

referencePressureRate-v1520		INTEGER	(-128..127)					OPTIONAL,	-- Need ON

...

}																		OPTIONAL,	-- Need ON

area-v1520			SEQUENCE {

pressureValidityArea-v1520		PressureValidityArea-v1520,

gN-pressure-v1520				INTEGER (-1024..1023)				OPTIONAL,	-- Need ON

gE-pressure-v1520				INTEGER (-1024..1023)				OPTIONAL,	-- Need ON

...

}																		OPTIONAL	-- Need ON

]],

[[

locationCRS-r19					CoordinateReferenceSystem-r19		OPTIONAL	-- Need ON

]]

}

PressureValidityArea-v1520 ::= SEQUENCE {

centerPoint-v1520				Ellipsoid-Point,

validityAreaWidth-v1520			INTEGER (1..128),

validityAreaHeight-v1520		INTEGER (1..128),

...

}

PressureValidityPeriod-v1520 ::= SEQUENCE {

beginTime-v1520					GNSS-SystemTime,

beginTimeAlt-v1520				INTEGER (0..2881)						OPTIONAL,	-- Need ON

duration-v1520					INTEGER (1..2881),

...

}

-- ASN1STOP

Sensor-AssistanceDataList field descriptions

refPressure This field specifies the atmospheric pressure (Pa) nominal at sea level, EGM96 [29] to the target. The scale factor is 1 Pa. The value is added to the nominal pressure of 101325 Pa.

refPosition This field specifies the reference position at which the pressure measurement is made, as an ellipsoid point with altitude and uncertainty ellipsoid.

refTemperature Local temperature measurement at the reference where the pressure measurement is made. The scale factor 1K. The value is added to 273K.

period This field specifies the pressure validity period and reference pressure rate.

pressureValidityPeriod - beginTime: this field specifies the start time of the pressure validity period in GNSS System Time. - beginTimeAlt: this field specifies an alternative start time. It may be used by the target device if GNSS-System Time is not available. The alternative start time is relative to the time the message was received. The scale factor is 15 min. The range is from 0 minutes to 43215 minutes = 30 days. - duration: this field specifies the duration of the validity period after the begin time. The scale factor is 15 minutes. The range is from 15 minutes to 43215 minutes = 30 days.

referencePressureRate This field specifies the rate of change of pressure. When this field is included, the reference pressure applies only at the start of the pressure validity period. The scale factor is 10Pa/hour.

area This field specifies the area within which the provided atmospheric reference pressure is valid and any spatial drift.

pressureValidityArea - centerPoint: this field specifies the coordinates of the centre of the rectangular validity area. - validityAreaWidth: this field specifies the width of the rectangular validity area. Width is measured from the centre along the latitude and is measured as the total width of the rectangle. The scale factor is 1km. The range is from 1km to 128km. - validityAreaHeight: this fields specifies the height of the rectangular validity area. Height is measured from the centre along the longitude and is measured as the total height of the rectangle. The scale factor is 1km. The range is from 1km to 128km. If this field is present, refPosition should not be provided by the location server and if provided, shall be ignored by the target device.

gN-pressure This field specifies the northward gradient of the reference pressure calculated from the centre of the pressureValidityArea. The scale factor is 1 Pa/km. If this field is not provided, the gradient is assumed to be zero.

gE-pressure This field specifies the eastward gradient of the reference pressure calculated from the centre of the pressureValidityArea. The scale factor is 1 Pa/km. If this field is not provided, the gradient is assumed to be zero.

locationCRS This field specifies the coordinate reference system (CRS) the location coordinates of this IE are related to.

NEXT CHANGE

#### 6.5.6.8	WLAN Assistance Data Elements

#### –	WLAN-DataSet

The IE WLAN-DataSet is used by the location server to provide WLAN AP information for one set of WLAN APs.

-- ASN1START

WLAN-DataSet-r14 ::= SEQUENCE {

wlan-AP-List-r14				SEQUENCE (SIZE (1..maxWLAN-AP-r14)) OF WLAN-AP-Data-r14,

supportedChannels-11a-r14		SupportedChannels-11a-r14		OPTIONAL,	-- Need ON

supportedChannels-11bg-r14		SupportedChannels-11bg-r14		OPTIONAL,	-- Need ON

...

}

SupportedChannels-11a-r14 ::= SEQUENCE {

ch34-r14		BOOLEAN,

ch36-r14		BOOLEAN,

ch38-r14		BOOLEAN,

ch40-r14		BOOLEAN,

ch42-r14		BOOLEAN,

ch44-r14		BOOLEAN,

ch46-r14		BOOLEAN,

ch48-r14		BOOLEAN,

ch52-r14		BOOLEAN,

ch56-r14		BOOLEAN,

ch60-r14		BOOLEAN,

ch64-r14		BOOLEAN,

ch149-r14		BOOLEAN,

ch153-r14		BOOLEAN,

ch157-r14		BOOLEAN,

ch161-r14		BOOLEAN

}

SupportedChannels-11bg-r14 ::= SEQUENCE {

ch1-r14			BOOLEAN,

ch2-r14			BOOLEAN,

ch3-r14			BOOLEAN,

ch4-r14			BOOLEAN,

ch5-r14			BOOLEAN,

ch6-r14			BOOLEAN,

ch7-r14			BOOLEAN,

ch8-r14			BOOLEAN,

ch9-r14			BOOLEAN,

ch10-r14		BOOLEAN,

ch11-r14		BOOLEAN,

ch12-r14		BOOLEAN,

ch13-r14		BOOLEAN,

ch14-r14		BOOLEAN

}

-- ASN1STOP

WLAN-DataSet field descriptions

wlan-AP-List This field provides information for WLAN APs in the data set.

supportedChannels-11a This field defines the superset of all channels supported by all WLAN APs in the data set of type 801.11a (5GHz band).

supportedChannels-11bg This field defines the superset of all channels supported by all WLAN APs in the data set of type 801.11b or 802.11g (2.4 GHz band).

#### –	WLAN-AP-Data

The IE WLAN-AP-Data is used by the location server to provide information for one WLAN AP as part of WLAN AP assistance data.

-- ASN1START

WLAN-AP-Data-r14 ::= SEQUENCE {

wlan-AP-Identifier-r14				WLAN-AP-Identifier-r13,

wlan-AP-Location-r14				WLAN-AP-Location-r14		OPTIONAL,	-- Need ON

...

}

WLAN-AP-Location-r14 ::= SEQUENCE {

locationDataLCI-r14					LocationDataLCI-r14,

...

}

LocationDataLCI-r14 ::= SEQUENCE {

latitudeUncertainty-r14				BIT STRING (SIZE (6)),

latitude-r14						BIT STRING (SIZE (34)),

longitudeUncertainty-r14			BIT STRING (SIZE (6)),

longitude-r14						BIT STRING (SIZE (34)),

altitudeUncertainty-r14				BIT STRING (SIZE (6))		OPTIONAL,	-- Need ON

altitude-r14						BIT STRING (SIZE (30))		OPTIONAL,	-- Need ON

datum-r14							BIT STRING (SIZE (8)),

...,

[[

locationCRS-r19					CoordinateReferenceSystem-r19	OPTIONAL -- Need ON

]]

}

-- ASN1STOP

WLAN-AP-Data field descriptions

wlan-AP-Location - locationDataLCI This field provides the location of the WLAN AP in the form of Location Configuration Information (LCI) defined in [27] and includes the following subfields: latitudeUncertainty: 6-bits quantifying the amount of uncertainty in latitude. A value of 0 is reserved to indicate that the uncertainty is unknown; values greater than 34 are reserved. Its relation with the corresponding value in degrees is expressed with the following formula: latitudeUncertainty = 8 - ceil(log2(uncertainty in degrees)) latitude: A 34-bits fixed point value consisting of 9-bits of integer and 25-bits of fraction indicating the Latitude (+/- 90 degrees) of the AP. longitudeUncertainty: 6-bits quantifying the amount of uncertainty in longitude. A value of 0 is reserved to indicate that the uncertainty is unknown; values greater than 34 are reserved. Its relation with the corresponding value in degrees is expressed with the following formula: longitudeUncertainty = 8 - ceil(log2(uncertainty in degrees)) longitude: A 34-bits fixed point value consisting of 9-bits of integer and 25-bits of fraction indicating the Longitude (+/- 180 degrees) of the AP. altitudeUncertainty: 6-bits value quantifying the amount of uncertainty in the altitude value. A value of 0 is reserved to indicate that the uncertainty is unknown; values greater than 30 are reserved. Its relation with the corresponding value in metres is expressed with the following formula: altitudeUncertainty = 21 - ceil(log2( uncertainty in metres)) altitude: A 30-bit fixed point value consisting of 22-bits of integer and 8-bits of fraction indicating the altitude of the AP in metres. datum: 8-bits indicating the map datum used for the coordinates. Defined codes are: Bit 1: World Geodetic System 1984 (WGS-84) Bit 2: North American Datum 1983 (NAD-83) with North American Vertical Datum 1988 (NAVD-88) Bit 3: North American Datum 1983 (NAD-83) with Mean Lower Low Water (MLLW) vertical datum. Bit 4: As specified by field locationCRS. Bits 5 – 8 are reserved. locationCRS: This coordinate reference system (CRS) the location coordinates of this IE.

NEXT CHANGE

#### 6.5.7.8	Bluetooth Assistance Data Elements

#### –	BT-BeaconInfo

The IE BT-BeaconInfo is used by the location server to provide Bluetooth beacon information for one set of Bluetooth beacons.

-- ASN1START

BT-BeaconInfo-r18 ::= SEQUENCE {

referencePoint-r18					ReferencePoint-r16,

bt-BeaconInfoList-r18				SEQUENCE (SIZE (1..maxBT-BeaconAD-r18)) OF

BT-BeaconInfoElement-r18,

...,

[[

locationCRS-r19					CoordinateReferenceSystem-r19	OPTIONAL -- Need ON

]]

}

BT-BeaconInfoElement-r18 ::= SEQUENCE {

bt-Addr-r18							BIT STRING (SIZE (48)),

bt-BeaconLocation-r18				RelativeLocation-r16	OPTIONAL,	-- Need OP

bt-LCS-GCS-TranslationParameter-r18	LCS-GCS-TranslationParameter-r16

OPTIONAL,	-- Cond NotSameAsPrev1

bt-antArrayConfig-r18				BT-AntArrayConfig-r18	OPTIONAL,	-- Cond NotSameAsPrev2

bt-antElementList-r18		SEQUENCE (SIZE (2..maxBT-BeaconAntElt-r18)) OF

BT-AntElement-r18		OPTIONAL,	-- Cond NotSameAsPrev3

bt-antSwitchingPattern-r18	SEQUENCE (SIZE (2..maxBT-BeaconAntElt-r18)) OF

BT-AntSwitchElement-r18	OPTIONAL,	-- Cond NotSameAsPrev4

bt-AoD-TransmConfig-r18				BT-AoD-TransmConfig-r18	OPTIONAL,	-- Cond NotSameAsPrev5

...

}

BT-AntArrayConfig-r18 ::= CHOICE {

bt-UniformLinearArray-r18			BT-UniformLinearArray-r18,

bt-UniformRectangularArray-r18		BT-UniformRectangularArray-r18,

bt-UniformCircularArray-r18			BT-UniformCircularArray-r18,

bt-GenericArray-r18					BT-GenericArray-r18

}

BT-AntElement-r18 ::= SEQUENCE {

polarization-r18				ENUMERATED {m45, zero, p45, p90, circ},

...

}

BT-AntSwitchElement-r18 ::= SEQUENCE {

antElementIndexShort-r18		INTEGER (1..16),

antElementIndexOffset-r18		ENUMERATED {o16, o32, o48, o64}		OPTIONAL,	-- Need OP

...

}

BT-AoD-TransmConfig-r18 ::= SEQUENCE {

primaryAdvInterval-r18		INTEGER (32..16777),

secondAdvInterval-r18		INTEGER (6..65535),

cte-Length-r18				INTEGER (2..20),

cte-Count-r18				INTEGER (1..16),

cte-Type2us-r18				ENUMERATED { true }					OPTIONAL,	-- Need OP

tx-PHY-M2-r18				ENUMERATED { true }					OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameAsPrev1 | The field is mandatory present in the first element of the bt-BeaconInfoList list; otherwise if not present, the bt-LCS-GCS-TranslationParameter of this element is the same as the bt-LCS-GCS-TranslationParameter of the previous element in the bt-BeaconInfoList list. |
| NotSameAsPrev2 | The field is mandatory present in the first element of the bt-BeaconInfoList list; otherwise if not present, the bt-antArrayConfig of this element is the same as the bt-antArrayConfig of the previous element in the bt-BeaconInfoList list. |
| NotSameAsPrev3 | The field is mandatory present in the first element of the bt-BeaconInfoList list; otherwise if not present, the bt-antElementList of this element is the same as the bt-antElementList of the previous element in the bt-BeaconInfoList list. |
| NotSameAsPrev4 | The field is mandatory present in the first element of the bt-BeaconInfoList list; otherwise if not present, the bt-antSwitchingPattern of this element is the same as the bt-antSwitchingPattern of the previous element in the bt-BeaconInfoList list. |
| NotSameAsPrev5 | The field is mandatory present in the first element of the bt-BeaconInfoList list; otherwise if not present, the bt-AoDTransmConfig of this element is the same as the bt-AoDTransmConfig of the previous element in the bt-BeaconInfoList list. |

BT-BeaconInfo field descriptions

bt-Addr This field specifies the Bluetooth public address of the Bluetooth beacon [53].

referencePoint This field specifies the reference point used to define the locations of the set of Bluetooth beacons.

bt-BeaconLocation This field provides the location of the Bluetooth beacon relative to the referencePoint location. If this field is absent the Bluetooth beacon location coincides with the referencePoint location.

bt-LCS-GCS-TranslationParameter This field provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [44].

polarization This field specifies the antenna element polarization in degrees relative the positive y-axis, where m45, zero, p45, p90 represents -45, zero, 45 and 90 degrees respectively towards the z-axis, and circ represents circular polarization.

bt-AoD-TransmConfig This field specifies Bluetooth beacon AoD transmission configuration in terms of advertising periodicities and CTE configuration to support the device to configure its scan windows and intervals.

primaryAdvInterval This field specifies the Bluetooth primary advertisement channel periodicity used by the Bluetooth beacon, scaling factor 0.625 ms [53].

secondAdvInterval This field specifies the Bluetooth periodic advertising interval on secondary advertisement channels used by the Bluetooth beacon, scaling factor 0.625 ms [53].

cte-Length This field specifies the configured CTE length to be used by the beacon in number of 8 µs segments.

cte-Count This field specifies the number of Bluetooth packets that include a CTE that are transmitted each periodic advertising event.

cte-Type2us This field, if present, indicates that 2 µs antenna switching slot duration is used by the beacon, otherwise 1 µs antenna switching slot duration is used.

tx-PHY-M2 This field, if present, indicates that Bluetooth TX PHY 2 Megasymbols/s is used by the beacon, otherwise Bluetooth TX PHY 1 Megasymbols/s is used.

bt-antSwitchingPattern This field specifies the Bluetooth antenna switching pattern as a list of indices, where each index is the order value of a specific antenna element in the bt-antElementList-r18 attribute of the IE BT-BeaconInfoElement-r18 – first element in the list corresponds to index 1 and so on. If the antenna switching pattern is shorter than the number of available sample slots, then the antenna switching patterns continues from the beginning of the bt-antSwitchingPattern-r18. If antenna switching pattern is longer than the number of available sample slots, then the elements in bt-antSwitchingPattern-r18 are discarded. If this field is not present, the target device can assume an antenna switching pattern with the configured antenna element in the same order as in the bt-antElementList-r18.

antElementIndexShort This field specifies short part of the antenna element index.

antElementIndexOffset This field specifies offset of the antenna element index, where o16, o32, o48 and o64 respresents 16, 32, 48 and 64 respectively to offset the short part of the antenna element index. If not present, the offset is zero.

locationCRS This field specifies the coordinate reference system (CRS) for the location coordinates of this IE.

NEXT CHANGE

## 7.2	Mapping of posSibType to assistance data element

The supported posSibType's are specified in Table 7.2-1. The GNSS Common and Generic Assistance Data IEs are defined in clause 6.5.2.2. The OTDOA Assistance Data IEs and NR DL-TDOA/DL-AoD Assistance Data IEs are defined in clause 7.4.2. The Barometric Assistance Data IEs are defined in clause 6.5.5.8. The TBS (based on MBS signals) Assistance Data IEs are defined in clause 6.5.4.8. The Coordinate Reference System and Transformations IEs are defined in clause 6.4.1.

Table 7.2-1: Mapping of posSibType to assistanceDataElement

|  | posSibType | assistanceDataElement |
| --- | --- | --- |
| GNSS Common Assistance Data (clause 6.5.2.2) | posSibType1-1 | GNSS-ReferenceTime |
|  | posSibType1-2 | GNSS-ReferenceLocation |
|  | posSibType1-3 | GNSS-IonosphericModel |
|  | posSibType1-4 | GNSS-EarthOrientationParameters |
|  | posSibType1-5 | GNSS-RTK-ReferenceStationInfo |
|  | posSibType1-6 | GNSS-RTK-CommonObservationInfo |
|  | posSibType1-7 | GNSS-RTK-AuxiliaryStationData |
|  | posSibType1-8 | GNSS-SSR-CorrectionPoints |
|  | posSibType1-9 | GNSS-Integrity-ServiceParameters |
|  | posSibType1-10 | GNSS-Integrity-ServiceAlert |
|  | posSibType1-11 | GNSS-LOS-NLOS-GridPoints |
|  | posSibType1-12 | GNSS-SSR-IOD-Update |
| GNSS Generic Assistance Data (clause 6.5.2.2) | posSibType2-1 | GNSS-TimeModelList |
|  | posSibType2-2 | GNSS-DifferentialCorrections |
|  | posSibType2-3 | GNSS-NavigationModel |
|  | posSibType2-4 | GNSS-RealTimeIntegrity |
|  | posSibType2-5 | GNSS-DataBitAssistance |
|  | posSibType2-6 | GNSS-AcquisitionAssistance |
|  | posSibType2-7 | GNSS-Almanac |
|  | posSibType2-8 | GNSS-UTC-Model |
|  | posSibType2-9 | GNSS-AuxiliaryInformation |
|  | posSibType2-10 | BDS-DifferentialCorrections |
|  | posSibType2-11 | BDS-GridModelParameter |
|  | posSibType2-12 | GNSS-RTK-Observations |
|  | posSibType2-13 | GLO-RTK-BiasInformation |
|  | posSibType2-14 | GNSS-RTK-MAC-CorrectionDifferences |
|  | posSibType2-15 | GNSS-RTK-Residuals |
|  | posSibType2-16 | GNSS-RTK-FKP-Gradients |
|  | posSibType2-17 | GNSS-SSR-OrbitCorrections |
|  | posSibType2-17a | GNSS-SSR-OrbitCorrectionsSet2 |
|  | posSibType2-18 | GNSS-SSR-ClockCorrections |
|  | posSibType2-18a | GNSS-SSR-ClockCorrectionsSet2 |
|  | posSibType2-19 | GNSS-SSR-CodeBias |
|  | posSibType2-20 | GNSS-SSR-URA |
|  | posSibType2-20a | GNSS-SSR-URA-Set2 |
|  | posSibType2-21 | GNSS-SSR-PhaseBias |
|  | posSibType2-22 | GNSS-SSR-STEC-Correction |
|  | posSibType2-23 | GNSS-SSR-GriddedCorrection |
|  | posSibType2-24 | NavIC-DifferentialCorrections |
|  | posSibType2-25 | NavIC-GridModelParameter |
|  | posSibType2-26 | GNSS-LOS-NLOS-GriddedIndications |
|  | posSibType2-27 | GNSS-SSR-SatellitePCVResiduals |
| OTDOA Assistance Data (clause 7.4.2) | posSibType3-1 | OTDOA-UE-Assisted |
| Barometric Assistance Data (clause 6.5.5.8) | posSibType4-1 | Sensor-AssistanceDataList |
| TBS Assistance Data (clause 6.5.4.8) | posSibType5-1 | TBS-AssistanceDataList |
| NR DL-TDOA/DL-AoD Assistance Data (clauses 6.4.3, 7.4.2) | posSibType6-1 | NR-DL-PRS-AssistanceData |
|  | posSibType6-2 | NR-UEB-TRP-LocationData |
|  | posSibType6-3 | NR-UEB-TRP-RTD-Info |
|  | posSibType6-4 | NR-TRP-BeamAntennaInfo |
|  | posSibType6-5 | NR-DL-PRS-TRP-TEG-Info |
|  | posSibType6-7 | NR-PRU-DL-Info |
| On-demand DL-PRS Configurations (clause 6.4.3) | posSibType6-6 | NR-On-Demand-DL-PRS-Configurations |
| Integrity Assistance Data for NR Positioning Methods (clause 6.4.3) | posSibType7-1 | NR-IntegrityRiskParameters |
|  | posSibType7-2 | NR-IntegrityServiceParameters |
|  | posSibType7-3 | NR-IntegrityServiceAlert |
|  | posSibType7-4 | NR-IntegrityParameters |
| Coordinate Reference System (clause 6.4.1) | posSibType8-1 | CoordinateReferenceSystem |
|  | posSibType8-2 | CoordinateTransformations |

END OF CHANGE