3GPP TSG-RAN WG2 #129	R2-2500972

Athens, Greece, 2025-02-17 - 2025-02-21

Agenda Item:	8.15

Source:	Ericsson, Reliance Jio

Title:	NavIC broadcast ephemeris to refer to for GNSS SSR corrections

Document for:	Discussion, Decision

# Introduction

3GPP Rel 16 introduced support for NavIC as an A-GNSS positioning method, first in relation to the L5 signal, and now in Rel-19 also support for the L1 signal.. Since GNSS assistance data is generic across constellations, there is thus NavIC support also for GNSS SSR. SSR orbit and clock assistance data are provided in relation to a nominal model that the target device can retrieve from the NAV message from the satellites, or the corresponding A-GNSS assistance data - the broadcast ephemeris. Since NavIC supports multiple signal; L5 and L1, there needs to be a note specifying which signal broadcast ephemeris the SSR orbit and clock corrections are related to.

# 2	Discussion

The information about which NavIC signal broadcast ephemeris SSR clock and orbit corrections should be added to the NOTE 2 of the IE GNSS-SSR-OrbitCorrections.

To follow the example of GPS, QZSS and BDS, the selected signal should be the one with most comprehensive support now and onwards, and also the signal that will be most widely deployed on the satellites of the NavIC system.

The signal expected to be most widely supported by devices, to be most deployed on satellites and will be most future compatible for the NavIC is the L1 signal. Therfore, we have the following observation and proposal.

NavIC L1 will for the forseable future be the most supported NavIC signal, and SSR orbit and clock correctiosn should be related to NavIC L1 for best uptake and compatability

RAN2 to agree to below Text Proposal for updating Note2 of IE GNSS-SSR-OrbitCorrections.

Another aspect is the name of GNSS ID corresponding to NavIC. Since, it was introduced in an enum as extension of Rel-16 field; hence following ASN.1 rule the version number was added. Thus for the first occurence it is fine; however the suffix does not need to be repeated again; for example in GNSS-ID-Bitmap.

#### – GNSS-ID The IE GNSS-ID is used to indicate a specific GNSS. -- ASN1START GNSS-ID ::= SEQUENCE { gnss-id ENUMERATED{ gps, sbas, qzss, galileo, glonass, ..., bds, navic-v1610 }, ... } -- ASN1STOP

However, it is not essential to name GNSS ID with version number as it appears confusing in GNSS-ID-Bitmap. It is possible to map it unambigously to NavIC even without the suffix.

Hence, we propose to correct at least from Rel-19 to remove version tag from the GNSS ID of NavIC.

GNSS ID of NavIC is replaced from navic-v1610 to navic.

#### – GNSS-ID-Bitmap The IE GNSS-ID-Bitmap is used to indicate several GNSSs using a bit map. -- ASN1START GNSS-ID-Bitmap ::= SEQUENCE { gnss-ids BIT STRING { gps (0), sbas (1), qzss (2), galileo (3), glonass (4), bds (5), navic (6) } (SIZE (1..16)), ... } -- ASN1STOP GNSS-ID-Bitmap field descriptions. gnss-ids This field specifies the GNSS(s). This is represented by a bit string, with a one-value at the bit position means the particular GNSS is addressed; a zero-value means not addressed..

## 2.2	Text Proposal

#### 6.5.2.1	GNSS Assistance Data

#### –	A-GNSS-ProvideAssistanceData

The IE A-GNSS-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-based and UE-assisted A-GNSS. It may also be used to provide GNSS positioning specific error reasons.

<Skipped unmodified changes>

#### –	GNSS-SSR-OrbitCorrections

The IE GNSS-SSR-OrbitCorrections is used by the location server to provide radial, along-track and cross-track orbit corrections. The target device may use the parameters to compute a satellite position correction to be combined with the satellite position calculated from broadcast ephemeris.

The parameters provided in IE GNSS-SSR-OrbitCorrections are used as specified for SSR Clock Messages (e.g., message type 1057 and 1063) in [30] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-OrbitCorrections-r15 ::= SEQUENCE {

epochTime-r15						GNSS-SystemTime,

ssrUpdateInterval-r15				INTEGER (0..15),

satelliteReferenceDatum-r15			ENUMERATED { itrf, regional, ... },

iod-ssr-r15							INTEGER (0..15),

ssr-OrbitCorrectionList-r15			SSR-OrbitCorrectionList-r15,

...

}

SSR-OrbitCorrectionList-r15 ::= SEQUENCE (SIZE(1..64)) OF SSR-OrbitCorrectionSatelliteElement-r15

SSR-OrbitCorrectionSatelliteElement-r15 ::= SEQUENCE {

svID-r15							SV-ID,

iod-r15								BIT STRING (SIZE(11)),

delta-radial-r15					INTEGER (-2097152..2097151),

delta-AlongTrack-r15				INTEGER (-524288..524287),

delta-CrossTrack-r15				INTEGER (-524288..524287),

dot-delta-radial-r15				INTEGER (-1048576..1048575)		OPTIONAL, -- Need ON

dot-delta-AlongTrack-r15			INTEGER (-262144..262143)			OPTIONAL, -- Need ON

dot-delta-CrossTrack-r15			INTEGER (-262144..262143)			OPTIONAL, -- Need ON

...

}

-- ASN1STOP

GNSS-SSR-OrbitCorrections field descriptions

epochTime This field specifies the epoch time of the orbit corrections. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation below. NOTE 1.

satelliteReferenceDatum This field specifies the satellite refence datum for the orbit corrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

svID This field specifies the satellite for which the orbit corrections are provided.

iod This field specifies the IOD value of the broadcast ephemeris for which the orbit corrections are valid (see IE GNSS-NavigationModel). NOTE 2.

delta-radial This field specifies the radial orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.1 mm; range ±209.7151 m.

delta-AlongTrack This field specifies the along-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.4 mm; range ±209.7148 m.

delta-CrossTrack This field specifies the cross-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.4 mm; range ±209.7148 m.

dot-delta-radial This field specifies the velocity of radial orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.001 mm/s; range ±1.048575 m/s.

dot-delta-AlongTrack This field specifies the velocity of along-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.004 mm/s; range ±1.048572 m/s.

dot-delta-CrossTrack This field specifies the velocity of cross-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.004 mm/s; range ±1.048572 m/s.

NOTE 1:	The update intervals are aligned to the GPS time scale for all GNSSs in order to allow synchronous operation for multiple GNSS services. This means that the update intervals may not be aligned to the beginning of the day for another GNSS. Due to the leap seconds, this is generally the case for GLONASS.

NOTE 2:	In the cases that gnss-ID indicates 'gps', 'qzss', 'bds' or ‘navic’, the iod refers to the NAV broadcast ephemeris (GPS L1 C/A, QZSS QZS-L1, BDS B1I, or NavIC L1 respectively, in table GNSS to iod Bit String(11) relation in IE GNSS-NavigationModel).

NOTE 3:	The reference time t0 is epochTime + ½ × ssrUpdateInterval. The reference time t0 for ssrUpdateInterval '0' is epochTime.

Value of ssrUpdateInterval to SSR Update Interval relation

| Value of ssrUpdateInterval | SSR Update Interval |
| --- | --- |
| 0 | 1 second |
| 1 | 2 seconds |
| 2 | 5 seconds |
| 3 | 10 seconds |
| 4 | 15 seconds |
| 5 | 30 seconds |
| 6 | 60 seconds |
| 7 | 120 seconds |
| 8 | 240 seconds |
| 9 | 300 seconds |
| 10 | 600 seconds |
| 11 | 900 seconds |
| 12 | 1800 seconds |
| 13 | 3600 seconds |
| 14 | 7200 seconds |
| 15 | 10800 seconds |

# Conclusion

In the previous sections we made the following observations:

Observation 1	NavIC L1 will for the forseable future be the most supported NavIC signal, and SSR orbit and clock correctiosn should be related to NavIC L1 for best uptake and compatability

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to agree to below Text Proposal for updating Note2 of IE GNSS-SSR-OrbitCorrections.

Proposal 2	GNSS ID of NavIC is replaced from navic-v1610 to navic.

# References

TS 37.355 LPP Specification