3GPP TSG-RAN WG2 #129	R2-2500975

# Athens, Greece, 17th – 21th February 2025

Agenda Item:	8.12.2

Source:	Ericsson

Title:	Asymmetric DL/UL mTRP user plane impact from MIMO ph. 5

Release:	19

Document for:	Discussion

# 1	Introduction

Release 19 MIMO with its justification and objective has been endorsed as one of the features for NR MIMO Phase 5 [1].

The subtopic objectives relevant to this WI are:

- UE initiated/event-triggered beam reporting,

- increased number of ports (up to 128) for CSI measurement and reporting,

- inter-TRP calibration for CJT,

- 3Tx for UL,

- asymmetric DL sTRP/UL mTRP.

Agreements on asymmetric DL sTRP and UL mTRP, RAN2#128

New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID.

Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB.

In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

This contribution discusses user plane impacts of Asymmetric DL sTRP/UL mTRP that has clear impact to RAN2 specification, also considering some RAN1 related agreements.

# 2	Discussion

2.1 Background Asymmetric DL sTRP UL mTRP

Figure 1 illustrates an asymmetric DL sTRP UL mTRP deployment. It consists of an Anchor TRP (providing both DL and UL service), and a TRP which will be called an UL-only TRP (for ease of reference) to provide UL service, where the downlink reference signals, the downlink control channels, and the downlink data transmissions for a UE are only provided from the Anchor TRP.

Figure 1: UL-only TRP in an asymmetric DL sTRP UL mTRP deployment

The main motivation for asymmetric DL sTRP UL mTRP deployment is to improve UL throughput by UE transmitting UL data to the UL-only TRP. For example, the Anchor TRP in Figure 1 provides coverage with both DL and UL transmissions, and the UL-only TRP is deployed to improve UL performance. Other motivations would be to reduce network energy consumption, by avoid sending downlink data from the UL-only TRP. It is envisioned that a UE in the cell may first perform initial access via the Anchor TRP, and then, if the UE is closer to the UL-only TRP than to the Anchor TRP (i.e. better UL link budget), or if the NW decides to move the UE to an UL-only TRP, e.g., to reduce the UL interference for the Anchor TRP, or to improve resource usage efficiency, the NW may direct the UE to transmit towards the UL-only TRP.

Enhancement on power control is needed to support the asymmetric deployment.

Apart from the power control aspect, another important aspect is how to set the UL transmission timing towards the Anchor TRP and UL-only TRP respectively. In a practical deployment the propagation delay differences and synchronization timing error between the Anchor TRP and the non-collocated UL-only TRP can be significantly large that will impact the performance of the asymmetric TRP deployment.

In addition, one should keep in mind that multiple UL-only TRPs might be deployed per Anchor TRP, and that the specified enhancements related to UL-only TRPs should preferably be applicable for all the following cases:

UE performs UL transmission to only the UL-only TRP

UE performs UL transmission to two different UL-only TRPs

UE performs UL transmission to one UL-only TRP and to the Anchor TRP

## 2.2	User Plane Impacts

### 2.2.1		MAC-CE update of PL Offset value for Asymmetric DL sTRP UL mTRP

In RAN1 a PL Offset has been introduced to be configured per TCI State. This PL Offset is associated with one of the indicated joint/UL TCI State for the UL TRP in the unified TCI framework and is applied PDCCH order PRACH transmission. Here, details of the design are still to be defined.

As stated in the LS in the previous meeting, MAC-CE is assumed to be used to update the PL Offset value for a UL TCI state (or PL offsets for TCI-states/joint TCI-states). The RAN1 agreement also assumes that the asymmetric DL sTRP/UL mTRP deployment scenario reuse the rel-17 and rel-18 unified TCI framework:

For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework:

- When rel-17 unified TCI/ICBM is configured:

  - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied.

  - For FR2: {one DL TCI state + one UL TCI state} can be applied.

- When rel-18 unified TCI is configured:

  - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied.

  - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

In RAN2#128, RAN2 agreed as a first base line that “In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states”

The PL offset in Asymmetric DL/UL can be associated to an UL TCI state, and the UE should determine the output power for transmitted UL signals/channels associated with the UL TCI state by applying this PL offset to the open loop power control. The impact to existing legacy PHR triggers from this is discussed in 2.2.3 below.

Given the current agreements is RAN1 and RAN2, a resulting MAC CE can be found in Fig.  The Ti in this example would indicate the PL Offset for a TCI state with TCI-StateId “i” (c.f. 38.331). Currently Joint/unified TCI states are configured for a Serving Cell BWP and the UL BWP is correspondingly following the unifiedTCI-State-Type in that it may either be associated with a DL or UL BWP. For the MAC CE signaling, a BWP ID field should as a result indicate BWP ID either for joint or separate UL TCI states. That is, only UL TCI state IDs and UL BWP indication is used for a configuration where unifiedTCI-State-Type is configured with separate TCI-States, and joint DL TCI state IDs and DL BWP ID when unifiedTCI-State-Type is configured with joint TCI-States.

Resulting example of MAC CEs to update PL offset for TCI states for mTRP operation:

Fig 2

The Ti in this example indicate if the UL or Joint TCI state with TCI-StateId “i” (c.f. 38.331) should have an updated pathloss offset or not. The format can of course be optimized, e.g. using R-bits, although a baseline could be to represent a number N equal to the maximum number of Joint/UL TCI states configured. This also since it is yet not clear currently, what limitation exist in how many PL Offsets the UE will be able to maintain etc.

PL Offset update signalling MAC CE(s) is defined so that it includes a TCI-State Id and BWP ID association to configured joint/TCI States per Serving Cell.

UL TCI state IDs and a corresponding UL BWP indication is used for a configuration where unifiedTCI-State-Type is configured with separate TCI-States, and joint DL TCI state IDs and a corresponding DL BWP ID when unifiedTCI-State-Type is configured with joint TCI-States.

In the Unified TCI state framework, a PL-RS can be associated with an UL TCI state through RRC configuration, and the UE should then use the PL-RS associated with an UL TCI state when transmitting UL signals/channels associated with that UL TCI state. Since the UE uses the estimated pathloss to the Anchor TRP to determine output power for an UL TCI state associated with the UL-only TRP, it is important that the pathloss estimation towards the Anchor TRP reflects the actual pathloss. When the UE moves around in a cell in FR2, the downlink beams and the associated DL TCI state from Anchor TRP must be updated frequently. In a similar way, the pathloss reference signal that the UE uses from Anchor TRP also needs to be update frequently, since pathloss reference signals are expected to be beamformed in FR2 (for example the pathloss reference signal can be one of the beamformed SSBs or TRSs).

Considering mobility also in asymmetric Rel-19 deployments, the DL-TCI state towards the current serving TRP may thus need to be updated frequently. Similarly, the pathloss reference signal and PL Offset that the UE uses may also need to be updated frequently. Since the pathloss reference signal for the UL TCI state targeting the UL-only TRP is currently assumed to be RRC configured, this may require RRC signalling to frequently update the pathloss reference signal towards the serving TRP.

As a result, the MAC CE used for updating the PL Offset may need to also include an association to a PL-RS.  It is however not clear yet in RAN1 what the details are w.r.t PL-RS, i.e. to what combination of e.g. PL offset, the PL offset association to an UL TCI state (UL) and joint DL TCI state (serving TRP) while the PL-RS is related to the joint TCI state of a serving TRP.

For FR2, when a UE moves around in the cell and need to update the serving Anchor TRP beam, the pathloss reference signal configured in an UL-TCI state used for UL transmission towards the UL-only TRP may need to be updated with RRC signaling, which generates a significant amount of overhead signaling and latency.

This somewhat defeats the usefulness of additional MAC CE signaling for updated PL Offset as frequent RRC signaling anyway could cater for this in addition.

To solve this issue, it would be preferred if in addition to the PL Offset, also the PL-RS associated with an UL-TCI state targeting UL-only TRP could be linked with a DL-RS associated to an indicated DL TCI state, thus the pathloss reference signal used for UL-only TRP will get updated automatically and follow the change of indicated DL TCI. With this approach, the output power (and PL Offset) towards UL-only TRP would be automatically updated when the UE moves around and changes serving Anchor TRP beams. The additional significant overhead and latency introduced by higher layer signaling to update the PL-RS/PL Offset could then be limited for the UL-TCI state to follow the rapid DL beam change.

Ask RAN1 if a linkage of PL-RS of an UL TCI state with a DL-RS associated with an indicated DL TCI state is beneficial to include with a PL Offset MAC CE.

The format then may depend on the PLOffset and PL-RS association to TCI-State(s):

Fig 3
MAC CE to update pathloss offsets, and DL PL RS per PL-RS associated with UL-TCI state for UL-only TRP linked with a DL-RS associated to an indicated DL TCI state

### 2.2.2	Miscellaneous aspects

As a result of Asymmetric deployment, 38.331 will include a absolute PL Offset value for joint/ UL TCI-State along with the new MAC CE for updating the path loss offset. In RAN2 it has not yet been determined in which way the UE handles the TCI State and PL Offset configuration upon RRC reconfiguration or MAC CE signaling. In general terms, it can be expected that the UE maintain a “latest” configured TCI-State/PL Offset configuration. In here, we suggest adopting this baseline principle until the detailed RRC procedures and parameters are defined.

As a baseline, a UE maintain a latest UL TCI State PL Offset configuration for a BWP as configured by RRC or updated by MAC CE.

### 2.2.3	PHR

In asymmetric DL sTRP/ UL mTRP deployment an UL TRP is receive only and hence does not transmit any DL signals. Without a DL PL-RS (reference) transmission, the pathloss to the UL-only cannot be estimated to determine the UL transmit power for the transmission towards the UL-only TRP. However, it is possible for a UE to determine the pathloss based on the pathloss of the serving (DL) TRP using the pathloss offset (PL∆ [dB]) indicated by the NW.

UL/DL TRP

UL only TRP

UE

PUSCH/PUCCH/SRS

DL-RS for path loss calculations

PL0

PL1

The current formula is thus updated to incorporate the determination for pathloss so that:

P=P0 +  α*(PLDL_RS+PL∆)+ PCL

PL∆=PL1-PL0

Fig. 6

Potential changes/impact to 38.321:

Concerning impact in 38.321, the PHR trigger may have impact from the addition of a PL Offset. In 38.321 the current trigger includes a change to pathloss more than PowerFactorChange DB.

The question is then if the legacy PHR triggering condition should consider the per-TCI state PL offset value. In here the assumption is that the existing PHR MAC CE (fomat) can be used as in legacy.

The UE should determine the output power for transmitted UL signals/channels associated with the UL TCI state by applying the PL offset to the open loop power control.  This as the UL Tx power the UE should use adjustments according to the PL offset. Thus, the UE should trigger a PHR when the UL Tx power changes more than PowerFactorChange +/- PL Offset.

If the above is the case, for Rel-19, either a new threshold configuration may be needed, e.g.  “phr-Tx-PowerFactorChange-Asym” should there be a need for different threshold values, or if not, a possibility is to use a conditional presence of the PL Offset {plOffset} as different PL offset values may be configured to TCI-states acc. to the RAN1 agreement for asymmetric mTRP deployment, and due to that the triggering calculation is based on the use on the serving DL/UL TCI State and the corresponding reference PL-RS. That is, if plOffset is configured for an associated PL-RS, the path loss calculation follows an updated formula that includes a PL Offset (according to the configuration, or as updated with MAC CE signalling) and this path loss calculation result is then measured toward an updated “PowerFactorChange” threshold.

As RAN1 have not yet decided if other trigger then for PHR Type 1 is to be supported for UL Only, as well as details w.r.t the configurable combinations that include DL sTRP or UL mTRP scenarios (e.g. twoPHRMode, multipanelSchemeSDM, multipanelSchemeSFN etc), RAN2 should wait with the detailed actual procedure impacts. However, it may be beneficial to know to what extent or if the current trigger and conditions can be maintained from a RAN2 p.o.v.

RAN2 to discuss impact on legacy triggers and thresholds if a PL Offset trigger is needed for asymmetric mTRP.

Ran2 to wait with procedural updates until RAN1 concludes on PHR reporting.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	For FR2, when a UE moves around in the cell and need to update the serving Anchor TRP beam, the pathloss reference signal configured in an UL-TCI state used for UL transmission towards the UL-only TRP may need to be updated with RRC signaling, which generates a significant amount of overhead signaling and latency.

Based on the discussion in the previous sections we propose the following:

Proposal 1	PL Offset update signalling MAC CE(s) is defined so that it includes a TCI-State Id and BWP ID association to configured joint/TCI States per Serving Cell.

Proposal 2	UL TCI state IDs and a corresponding UL BWP indication is used for a configuration where unifiedTCI-State-Type is configured with separate TCI-States, and joint DL TCI state IDs and a corresponding DL BWP ID when unifiedTCI-State-Type is configured with joint TCI-States.

Proposal 3	Ask RAN1 if a linkage of PL-RS of an UL TCI state with a DL-RS associated with an indicated DL TCI state is beneficial to include with a PL Offset MAC CE.

Proposal 4	As a baseline, a UE maintain a latest UL TCI State PL Offset configuration for a BWP as configured by RRC or updated by MAC CE.

Proposal 5	RAN2 to discuss impact on legacy triggers and thresholds if a PL Offset trigger is needed for asymmetric mTRP.

Proposal 6	Ran2 to wait with procedural updates until RAN1 concludes on PHR reporting.

# 4	References

- RP-242394 NR MIMO Phase 5, Samsung (Moderator), RAN meeting #102 Scotland, December 11-15, 2023