3GPP TSG-RAN WG2 Meeting #129	R2-2500978

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.10.4

Source:	Nokia

Title:	Area Scope for the logged MDT for NTN

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this paper we discuss the question on MDT for NTN from RAN3 in the incoming LS from RAN3 [R2-2500026/R3-247908].

# 2	Discussion

## 2.1	Background

RAN2 had received an LS from RAN3 [R2-2500026/R3-247908] in which they ask for RAN2 feedback to help them make decision on how to define the area scope for logged MDT for NTN. RAN3 assumes the geographic area and/or mapped cell ID can be used as area scope for logged MDT for NTN over NGAP. They would like to confirm with RAN2 if geographical area defined for MBS over NTN can be reused as area scope for logged MDT for NTN over Uu. This would help RAN3 to make to decision on whether to specify geographical area and/or mapped cell ID over NGAP.

## 2.2	Defining an area scope

RAN3 is asking us if it is feasible to use the same format that was used for the so called “intended broadcast service area” from the MBS NTN.

First, in legacy MDT, the area scope for logged MDT in RRC signalling protocol is defined using an area configuration that involves an area indicated by a list of TACs or a list of TAIs or a list of CGIs. Area configuration for logged MDT can also be defined in terms of a list of CAG IDs for CAG or in terms of SNPN configuration like SNPN cell ID list, SNPN TAC list or SNPN TAI list for SNPNs. So, these are basically signalling of a list of identities that identify some form of an area, like a cell, tracking area etc. that defines the area scope.

Observation 1: Signalling a list of identities that identify some form of an area, like a cell, tracking area etc. that defines the area scope is already feasible in legacy MDT.

Next, legacy NTN, SIB25 broadcast in an NTN cell contains TN coverage information to assist with neighbour cell measurements for the UEs. Here, NTN coverage is expressed in terms of TN coverage area where a TN coverage area is identified by an area ID (range 1 to 32) and the area is defined in terms of a reference location and a distance (up to 65535 meters) from the reference location. So, there can be 32 TN coverage areas under an NTN coverage area.

Observation 2: RAN2 had already specified a solution for representing an entire NTN cell area in terms of TN coverage area and defined a format for representing a unit of TN area bigger than a TN cell coverage using a circular TN areas.

As part of Rel-19 NTN enhancements for support of MBS, RAN2 had agreed on the concept of “intended broadcast service area”. The UE acquisition of MCCH and establishment of MBS radio bearer for an MBS session takes place while the UE is inside the intended broadcast service area. Intended broadcast service area information is broadcast in a system information message. The intended broadcast service area can cover the area of more than one NTN cells (or portions thereof). RAN2#127 agreed that the intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

Observation 3: In Rel-19, NTN supports an area scope for MBS service reception defined by either a circular area or a polygon which may cover a part of the NTN cell or may span multiple NTN cells.

In the work on QoE for MBS, the QoE measurement collection (QMC) is also associated with a specific area where the area is defined either a cell list or a location filter, where the location filter points to an area defined by a circle or polygon.

Observation 4: QoE for MBS provides support for defining an area scope for measurement collection where the area is defined using either list of cells or a location area defined by a circle or polygon.

Based on the above discussion and observations, we think it is feasible to define an area scope for MDT measurement logging for NTN using either a list of cell IDs or a geographic area defined by either a circle or a polygon. Given that NTN already supports the mapping of an NTN cell to TN areas, defined by a TN area ID which are circular areas and given that RAN2 now supports a circular or polygon intended broadcast service areas for MBS for NTN it is feasible to reuse the geographical area defined for MBS for NTN for Area Scope of logged MDT. RAN2 will specify signalling details for intended broadcast service area for broadcast over Uu for NTN as part of stage-3 work for MBS for NTN.

Proposal 1: RAN2 to reply to RAN3 that it is feasible to reuse the geographical area (intended broadcast service area) defined for MBS for NTN as an area scope for logged MDT for NTN and consider the draft reply LS given in annex for further discussion and decision. RAN2 will specify in Rel-19 signalling details for intended broadcast service area for broadcast over Uu for NTN as part of stage-3 work for MBS for NTN.

# 3	Conclusion

Always echo in the conclusion all proposals made above.

This document has made the following observations:

Observation 1: Signalling a list of identities that identify some form of an area, like a cell, tracking area etc. that defines the area scope is already feasible in legacy MDT.

Observation 2: RAN2 had already specified a solution for representing an entire NTN cell area in terms of TN coverage area and defined a format for representing a unit of TN area bigger than a TN cell coverage using a circular TN areas.

Observation 3: In Rel-19, NTN supports an area scope for MBS service reception defined by either a circular area or a polygon which may cover a part of the NTN cell or may span multiple NTN cells.

Observation 4: QoE for MBS provides support for defining an area scope for measurement collection where the area is defined using either list of cells or a location area defined by a circle or polygon.

And proposed the following:

Proposal 1: RAN2 to reply to RAN3 that it is feasible to reuse the geographical area (intended broadcast service area) defined for MBS for NTN as an area scope for logged MDT for NTN and consider the draft reply LS given in annex for further discussion and decision. RAN2 will specify in Rel-19 signalling details for intended broadcast service area for broadcast over Uu for NTN as part of stage-3 work for MBS for NTN.

# Annex

3GPP TSG-RAN WG2 Meeting #129	R2-250xxxx

Athens, Greece, 17 – 21 February 2025

Title:	[DRAFT] Reply LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT

Response to:	R2-25xxxxx/R3-247908

Release:	Release 19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG3

Cc:

Contact Person:

Name:	Mani Thyagarajan

E-mail Address:	mani.thyagarajan@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 thanks RAN3 for the LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT.

Regarding SON/MDT for NTN, RAN2 discussed the issue of reuse of geographical area defined for MBS NTN, for Area Scope of logged MDT over Uu for NTN. RAN2 had already agreed to define geographical areas for MBS NTN, referred to as intended broadcast service areas, using a (set of) reference location(s) and radius or by a (set of) polygon(s). Also, in legacy NTN, SIB25 broadcast in an NTN cell contains TN coverage information to assist with neighbour cell measurements for the UEs where a TN coverage area is indicated by a reference location and a distance from the reference location. Since these area representations for NTN tries to address the problem of too big an NTN coverage area, which equally applies for logged MDT for NTN, it is feasible to reuse how the geographical area is represented for MBS for NTN, also as an area scope for logged MDT for NTN. That is, an area for logged MDT for NTN can be defined using a reference location and radius, or a polygon.

2. Actions:

To RAN3 group.

ACTION: 	RAN2 respectfully asks RAN3 to take the provided information into account in their work on MDT for NTN.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#129-bis	from 2025-04-07	to 2025-04-11		China, CN

RAN2#130	from 2025-05-19	to 2025-05-23		Malta, MT

RAN2#131	from 2025-08-25	to 2025-08-29		India (TBC), IN