3GPP TSG-RAN WG2 Meeting #129	R2-2500983

# Athens, Greece, 17th – 21st	February 2025

Agenda Item:	8.7.4.1

Source:	Ericsson

Title:	LCP enhancements

Document for:	Discussion, Decision

# 1	Introduction

At RAN#105 the XR WID [1] for Rel-19 was updated with the following objective on the scheduling enhancements related to LCP:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

In this contribution we continue the discussion on remaining details of the enhanced LCP solution. Furthermore, we provide a TP for how the enhanced LCP mechanism can be modelled.

# 2	Discussion on LCP enhancements

For the LCP enhancement it has been agreed to introduce an independent per-LCH remaining time threshold for applying adjusted priority. The adjusted priority will be applied for at least the first round of LCP as long as there is LCH priority-adjusted data available for transmission in the LCH. The legacy LCP procedure also consist of other parameters than the priority, most importantly the Prioritized Bit Rate (PBR) and the BSD parameter, which impacts how much data that is selected from each LCH during the first round of the LCP procedure. These parameters coupled with the priority adjustment allow for intricate procedure behaviours depending on the traffic served. To finalize the priority-adjusting LCP enhancement it is necessary to consider how these aspects should be coupled together for a complete solution. Thus before deciding on the final details of the enhancement, it is desirable to first have look at the relevant traffic scenarios.

## 2.1 Scenarios to utilize priority-adjusting LCP

The priority-adjusting LCP enhancement will enable the LCP to support the prioritizations between multiple LCH in complex multi traffic flow scenarios. One of the most basic and likely scenario is a two-traffic flow case, where one flow consists of latency critical XR traffic while the other flow consists of traditional eMBB traffic. Even with such simple traffic setup there are a multitude of possible solutions how to construct a priority-adjusting LCP procedure to best support the traffic. The following sections go over a few of those possibilities.

### 2.1.1 Scenario 1: Adjust XR flow priority and override PBR

In the first scenario the XR flow has a high priority setting and the eMBB flow has a low priority setting while the PBR settings are being balanced between the flows, i.e. the XR flow will be able to use most of the transmission resources but the eMBB flow will still get to transmit some data to not get starved. Eventually there may be instances where data from the XR traffic has not been served fast enough and data becomes extra time critical, i.e. there is LCH priority adjusted data in the LCH queue. At this time adjusting the priority of the LCH does not matter, since this LCH already has the highest priority, but it may be beneficial to override the PBR setting and allow the transmission of all the LCH priority adjusted data as fast as possible, i.e. the XR traffic will take all the transmission resources. In this way the solution will have a higher likelihood to meet the target delay budget. However a drawback with these PBR settings can potentially be unnecessary limiting of the general transmission rate of the XR traffic and the QoE of the service.

Figure 1: XR flow has high priority, eMBB flow has low priority and XR flow has balanced PBR setting to allow eMBB flow to also transmit data. During priority adjustment the PBR value is overridden.

### 2.1.2 Scenario 2: Adjust eMBB flow priority and override PBR

In the second scenario the XR flow is again configured with higher priority than the eMBB flow, but the PBR settings is also higher on the XR flow to maximum the throughput and QoE of this traffic. With these settings the higher priority XR traffic use up most of the grant resources (with high PBR and BSD settings) and the low priority eMBB traffic rarely gets to transmit any data. Thus at some point in time the eMBB flow can become starved. The eMBB traffic may still need to fulfil some minimum requirements and a time threshold can be configured to detect LCH priority-adjusted data for this flow. Figure 2 depicts an outcome with these settings. The transmission of the XR traffic will be maximised most of the time but the eMBB traffic will eventually be served and not completely starved. However as can be seen this will impact the XR flow negatively during the adjusted priority period as the XR flow will not be allowed to transmit any data until all the LCH priority-adjusted data of the eMBB traffic has been served.

Figure 2: XR flow has high priority, eMBB flow has low priority and XR flow has high PBR setting. During priority adjustment the PBR value is overridden.

### 2.1.3 Scenario 3: Adjust eMBB flow priority and adjust PBR value

In the third scenario also the PBR value is adjusted together with the priority. Similar settings are used as in the second scenario but instead of overriding the PBR value the value is changed to a new lower value. This will allow the eMBB traffic to be prioritized but restrict it from using all the transmission resources and thus XR traffic will also be served. This is shown in Figure 3. A consequence of this LCP procedure behaviour is that the adjusted priority period will continue for a longer time, as it will take more time to transmit all the LCH priority-adjusted data. However this could still be the preferred scenario. With careful setting of the threshold value for determining the LCH priority-adjusted data and PBR, the data can still be delivered according to the requirements. Furthermore it is expected that rate adaption will play a large role in handling high rate XR traffic and thus when being in this congested state (i.e. not all traffic can be served according to the requirements) the XR flow can potentially run at a lower rate during this time period. Still with this solution the XR traffic will be able to run with a high rate most of the time.

Figure 3: XR flow has high priority, eMBB flow has low priority and XR flow has high PBR setting. During priority adjustment period also the PBR is adjusted to a new value.

### 2.1.4 Scenario 4: Adjust eMBB flow priority with fallback

Another aspect to consider when handling the multi flow scenarios is the second round of LCP. In previous meetings it has been agreed that based on UE capability the priority of the LCH in the second round of LCP can fall back to the default priority. The grant size is dynamic and there can be scenarios where network is able to provide a larger grant, even during the adjusted priority period. It may then be preferred that the extra transmission resources are still used by the XR traffic. The XR traffic is dynamic with a tight relationship between bitrate and QoE. On the other hand, the PBR configured for the eMBB traffic is already expected to provide enough bitrate to fulfil the traffic requirements and thus the extra resources will have a larger positive impact on the QoE of the XR traffic. Thus as an option the network should be able to configure the fallback to default priority for the second round of LCP if the PBR is fulfilled in the first round, to limit the rate of the priority adjusted flows. An example of such scenario is demonstrated in Figure 4 and Figure 5. In Figure 4 the network provides a large grant for second transmission occasion when in the adjusted priority state. Without fallback to default priority the eMBB flow will use the extra resources and finish the transmission of the delay critical data earlier. However the XR traffic still takes the same amount of time to transmit the buffered PDU Set. With fallback option in Figure 5 the extra resources provided in the larger grant are used by the XR traffic and thus the buffered PDU Set are delivered faster and potentially improving the QoE.

Figure 4: Large grant without priority fallback option.

Figure 5: Large grant with priority fallback option.

### 2.1.5 Conclusion

When considering the possible scenarios there are pros and cons with different LCP solutions. Depending on the preferred handling of the traffic there may be different solutions wanted. For simplicity, as a baseline solution at least the PBR overriding should be allowed, as only adjusting the LCH priority will not always be enough to ensure the prioritization of the wanted traffic. For a more flexible solution that can support different handling of traffic a possibility to adjust also the PBR value should be considered. Furthermore it is beneficial if network are able to configure the conditions for the fallback to default priority in the 2nd round of LCP for capable UEs.

- Depending on the desired treatment of the traffic in the multiple traffic scenarios, different LCP solutions may be preferred.

Overriding the PBR parameter during priority adjustment can be considered a baseline solution for the priority-adjusting LCP enhancement.

The network can adjust the PBR value during the priority adjustment period.

The network can configure the conditions for fallback to default priority for the second round of LCP when having a priority adjusted LCH and UE capable of priority fallback.

## 2.2 Priority-adjusting LCP enhancement in specification

To introduce the baseline solution in proposal 1 there is a need to apply some changes to the LCP procedure in MAC specification. To keep track of the transmission of the LCH priority-adjusted data a new counter, Rj, can be applied. This counter can be configured so that only when all of the LCH priority-adjusted data is transmitted from the LCH the resource allocation moves on to another lower priority LCH. This solution assures that the time critical data is being included in the transmission before any other low important data. An example of how this new counter Rj is tracking the data in the LCH is shown in Figure 6.

Figure 6: Examples of how the counter Rj can be calculated for two different LCHs.

We thus propose for the priority-adjusting LCP enhancements:

A priority-adjusting LCP solution uses a new counter variable to keep track of the scheduling of the LCH priority-adjusted data.

The described priority-adjusting LCP solution is simple to implement and understand in the specification. It ensures a fair LCP procedure while at the same time enabling that the LCH priority-adjusted data is transmitted before other data when possible. A text proposal implementing the priority-adjusting LCP solution using the new counter Rj and overriding the PBR can be found in Appendix 1.

Implement the TP provided in Appendix 1.

Please note that the text proposal is written such that the priority-adjusting LCP behaviour is controlled by the additional RRC parameters.

## 2.3 Configuration of priority-adjusting LCP

Each LCH that acts on LCH priority-adjusted data will need to be configured via RRC with an alternative priority value and a threshold for remaining time. This can be done in the same IE that is used for LCP configuration today, LogicalChannelConfig.

For each LCH that contains time critical data, the network can configure an alternative priority value and a remaining time threshold in RRC IE LogicalChannelConfig.

Some data, e.g. pose and signallling should always have higher priority than the LCH priority-adjusted data, even if the time critical data is close to be discarded. This can be solved by the network via configuration of the default priority levels and the alternative priority levels. Since we can’t predict exactly which data that need to be prioritized we shouldn’t specify how these priority levels will be configured, it can be left for network implementation.

It is up to network implementation to configure priority values, so that e.g. pose and signalling always have higher priority than LCH priority-adjusted data.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	Depending on the desired treatment of the traffic in the multiple traffic scenarios, different LCP solutions may be preferred.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Overriding the PBR parameter during priority adjustment can be considered a baseline solution for the priority-adjusting LCP enhancement.

Proposal 2	The network can adjust the PBR value during the priority adjustment period.

Proposal 3	The network can configure the conditions for fallback to default priority for the second round of LCP when having a priority adjusted LCH and UE capable of priority fallback.

Proposal 4	A priority-adjusting LCP solution uses a new counter variable to keep track of the scheduling of the LCH priority-adjusted data.

Proposal 5	Implement the TP provided in Appendix 1.

Proposal 6	For each LCH that contains time critical data, the network can configure an alternative priority value and a remaining time threshold in RRC IE LogicalChannelConfig.

Proposal 7	It is up to network implementation to configure priority values, so that e.g. pose and signalling always have higher priority than LCH priority-adjusted data.

# References

RP-241771 Revised WID: XR (eXtended Reality) for NR Phase 3, September 2024

# Appendix 1

Changes to specification TS 38.321, additions highlighted in yellow:

### 5.4.3	Multiplexing and assembly

#### 5.4.3.1	Logical Channel Prioritization

##### 5.4.3.1.1	General

The Logical Channel Prioritization (LCP) procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission;

-	allowedCG-List which sets the allowed configured grant(s) for transmission;

-	allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission;

-	allowedHARQ-mode which sets the allowed UL HARQ mode for transmission.

-	timeCriticalPriority an alternative priority value.

-	timeCriticalTimeThreshold which set the remainingTimeThreshold (RETT).

The following UE variable is used for the Logical channel prioritization procedure:

-	Bj which is maintained for each logical channel j.

-	if timeCriticalPriority is configured, Rj which is maintained for each logical channel j.

The MAC entity shall initialize Bj and Rj of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented;

1>	if the value of Bj is greater than the bucket size (i.e. PBR × BSD):

2>	set Bj to the bucket size.

1>	if timeCriticalPriority is configured:

2>	if the remaining time until discard for any MAC SDU in logical channel j is less than or equal to RETT:

3>	set Rj to the size in bytes for MAC SDUs with remaining time less or equal to RETT.

3>	adjust priority of logical channel j to timeCriticalPriority

2>	if the remaining time until discard for all MAC SDU in logical channel j is greater than RETT:

3>	set Rj to zero.

3>	adjust priority of logical channel j to priority.

NOTE:	The exact moment(s) when the UE updates Bj and Rj between LCP procedures is up to UE implementation, as long as Bj is up to date at the time when a grant is processed by LCP.

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

1>	select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	configuredGrantType1Allowed, if configured, is set to true in case the UL grant is a Configured Grant Type 1; and

2>	allowedServingCells, if configured, includes the Cell information associated to the UL grant. Does not apply to logical channels associated with a DRB configured with PDCP duplication within the same MAC entity (i.e. CA duplication) when CA duplication is deactivated for this DRB in this MAC entity; and

2>	allowedCG-List, if configured, includes the configured grant index associated to the UL grant; and

2>	allowedPHY-PriorityIndex, if configured, includes the priority index (as specified in clause 9 of TS 38.213 [6]) associated to the dynamic UL grant; and

2>	allowedHARQ-mode, if configured, includes the allowed UL HARQ mode for the HARQ process associated to the UL grant.

NOTE:	The Subcarrier Spacing index, PUSCH transmission duration, Cell information, and priority index are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

##### 5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover.

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 or Rj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2> decrement Rj by the total size of MAC SDUs with remaining time less or equal to RETT served to logical channel j above;

2>	if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

NOTE 1:	The value of Bj can be negative.