3GPP TSG-RAN WG2 #128	R2-2500990

# Orlando, USA, 18th – 22nd November 2024

Agenda Item:	7.2

Source:	Ericsson

Title:	Summary of RRC proposals for feMob

Document for:	Discussion

# 1	Introduction

In this contribution is provide a summary of the RRC proposals for FeMob with some possible conclusion for each of them. Please, note that MAC corrections are left out from this document.

# 2	Discussion

Hereafter the RRC related proposals submitted to AI 7.2 are listed and a possible outcome for what is proposed is suggested:

| Tdoc # | Company | Proposal(s) | Suggested conclusion |
| --- | --- | --- | --- |
| R2-2500151 | MediaTek | P1: The network always signals fields frequencyBandList and absoluteFrequencyPointA when FrequencyInfoUL IE is used within EarlyUL-SyncConfig IE P2: The UE maintains fields subCarrierSpacing, absoluteFrequencyPointA, and cyclicPrefix in case the fields are absent in NZP-CSI-RS-Resource IE used within LTM-Candidate IE. P3: RAN2 to discuss usage of the LTM reference configuration. P4: Based on the outcome of the discussion (P3), RAN2 to agree on which fields can and which fields cannot be present in the LTM reference configuration. P5: Based on the outcome of the discussion (P3), RAN2 to agree on which fields can and which fields cannot be present at the same time in the LTM reference configuration and in a non-complete LTM candidate configuration. P6: Based on the outcome of the discussion (P3), RAN2 to agree on whether the fields which can be present at the same time in the LTM reference configuration and in a non-complete LTM candidate configuration, if any, can configure the same entities (same radio bearer, same SCell, etc.). P7: RAN2 to discuss whether the agreements (from P4, P5, and P6), if any, should be captured in 38.331. P8: Within LTM Cell Switch Command MAC CE, the indicated joint TCI state in the TCI state ID field is the same or QCL to the SSB indicated in the SS/PBCH index field of the contention-free Random Access Resources. P9: For UE capability pdcch-RACH-AffectedBandsList-r18, RAN2 to discuss the issue of UE reporting this UE capability for bands for which the UE does not support PDCCH-ordered RACH for LTM (Two options are listed). | P1: To be agreed. It seems this is correct and maybe something it was overlooked when the configuration for ealy sync was discussed. P2: To be agreed. The need code seems correct and anyway this should be the expected UE behaviour. P3-P7: Not pursued. We agree that at the moment there is full flexibility for the network to split the complete RRC configuration between the reference and the LTM candidate configuration. However, discussing now what can be included in the reference sounds like a tricky and long discussion. We think the flexibility was intentional from the beginning, but now it too late to change this. If too difficult to support the reference configuration, we have anyway a UE capability for it. P8: To be agreed. This seems straightforward. Do we need to check with RAN1 about this? P9: To be discussed. The issue may be valid but probably is good to check this with RAN4 as RAN2 cannot really decide this. |
| R2-2500173 | Samsung | P1: Upon successful completion of cell switch, UE releases configured contention-free Random Access Resources in rach-ConfigDedicated if configured in the reconfigurationWithSync. P2: Propose following change: “select the set of Random Access resources configured in EarlyUL-SyncConfig of the selected UL carrier, corresponding to the cell indicated by the field Cell indicator in the PDCCH order as specified in TS 38.212 [9].” | P1: To be discussed. In general, P1 seems to be correct, even if we needs to specify that the random access resources to be released are the one in “current UE configuration”. The Random access resources within the LTM candidate configuration should not be released as those can be used for subsequent LTM. However, we am wondering whether something is harmful even if we don’t do this clarification. P2: To be discussed in the MAC summary. |
| R2-2500228 | CATT | P1: Add the procedure description about RRC layer indicates lower layer which candidate cell could be performed UE-based TA measurement in case of L3 SpCell change execution and with ltm-ServingCellUE-MeasuredTA-ID but without ltm-CandidateToAddModList is configured within the reconfiguration message. | P1. To be discussed. Probably the scenario targeted is a corner case. Because the issue is only if the UE performs a intra-CU HO with CHO, L3, or CPAC and, at the same time, the new serving cell is not an LTM candidate cell. Only in this case the UE-based TA may not work. However, for the other cases (inter-CU) the LTM candidate cell may need to be reconfigured or released, as these will be useless for the UE. But in principle the proposed change can be okay (also looking at Rel-19). |
| R2-2500333 | Nokia | P1: Clarify that PDCP recovery is only done for DRB which RLC entity has been re-established. P2: RAN2 does not consider any further the addition of an explicit information regarding the number of SpCells and SCells within the LTM candidate configuration. P3: RAN2 adopts the changes suggested by the RAN4 LS about the FG 39-6 prerequisites pre-requisites. P4: The use of ltm-UE-MeasuredTA in the 5.3.5.18.6 shall not be allowed when LTM is executed due to the LTM candidate selection in accordance with 5.3.7.3. P5: In addition to the number of LTM cells and L1 measurement configuration, MN informs SN about the number of cells for which early synchronization procedures can be performed. Adopt the TP in Annex D in TS 38.331. | P1. To be agreed. The change seems to make sense as there is no need to do PDCP recovery for DRBs which RLC entity has not been re-established. However, the change could be slightly different, in the sense that we can append the suggested text in the existing once, as it makes sense to keep “current UE configuration”. P2: To be discussed. It seems there are different views on this, so better to discuss this during the meeting. P3: To be agreed. The change seems straightforward as this is what RAN4 is asking us to do. P4: Not pursued. The interpretation seems to be not correct. The part of text which is referred is “after” the LTM cell switch is done (because of fast RLF recovery) and to configure the UE to do UE-based TA for the “next” LTM cell switch. This is not in contrast with the agreement. However, maybe we can have a note in the LTM execution section to clarify that, even if the UE has done UE-based TA, the UE should always do RACH if the LTM execution is due to a cell selection. P5: Not pursued. According to stage 2 we have “the MN indicates the maximum number of LTM candidate configurations the SN is allowed to configure for SCG LTM, to ensure that UE capabilities are not exceeded”. This already implies that MN is responsible that capabilities are not exceeded. We don’t need additional signalling for the early TA acquisition procedure. |
| R2-2500356 | Vivo | P1: Correct current specification to make UE only perform SCG LTM configuration release procedure once when UE performs RRC re-establishment. P2: The terms that “CHO with candidate SCG(s)” and “subsequent CPAC” are distinguished from the “CHO” and “CPA/CPC” in the whole RRC specification. | P1: Not pursued. The procedural text looks correct. The UE will only release the LTM configuration once but, even if it was to release twice, then I guess there is not issue about this. P1: Not pursued. The term “CHO” can indeed cover both “CHO” and “CHO with SCGs” and same goes for the term “CPA/CPC”. We should not repeat 4 terms all over the spec but we should rather use “CHO with SCGs” and “Subsequent CPAC” only when something applies specifically to these cases. |
| R2-2500374 | Ofinno | P1: RAN2 to confirm that the UE reverts ltm-ServingCellUE-MeasuredTA-ID used in the source PCell when T304 is expired. | P1: Not pursued. When we say UE reverts back to the previous UE configuration it shall also include the UE variable (which are not normative text anyway). Also, at the moment of the procedural text, the UE has not yet released the LTM configuration so the UE variables should also not be released. |
| R2-2500540 | MediaTek | P1: Removal of prerequisite of ltm-FastProcessingConfig-r18 according to the received RAN4 LS. | P1: To be agreed. RAN4 asked us to do this, so the change seems straightforward. |
| R2-2500728 | Google | P1: The UE does not discard the CFRA resources explicitly signalled in the LTM Cell Switch Command in the MAC reset, and discards the CFRA resources only when the UE successfully completes the LTM cell switch | P1. To be discussed in the MAC summary. |
| R2-2500962 | Huawei | P1: Correct the Interruption time of Figure G-2 (make it end with TLTM-IU). P2: Replace the Tfirst-data by TLTM-IU of Figure G-3. P3: Add ‘Mobility latency’ in Figure G-2 and G-3. P4: For ssb-Periodicity-r18, change to Need S and use a new IE SSB-Periodicity with the description currently in ServingCellConfigCommon (and use this IE in ServingCellConfigCommon too). P5: For ssb-PositionsInBurst-r18, capture that it is always present and use a new IE SSB-PositionsInBurst with the description currently in ServingCellConfigCommon (and use this IE in ServingCellConfigCommon too). P6: For ss-PBCH-BlockPower-r18, capture that it is always present and use a new IE SS-PBCH-BlockPower with the description currently in ServingCellConfigCommon (and use this IE in ServingCellConfigCommon too). P7: Capture that the unifiedTCI-StateType in LTM-TCI-Info should be the same like the unifiedTCI-StateType in the candidate cell configuration (ltm-CandidateConfig), as already agreed in RAN2#126. P8: In the LTM cell switch execution procedure, remove the procedure text for the case when ltm-UE-MeasuredTA-ID is not configured for the LTM-Candidate to which the UE is switching. P9: Replace the last action at the end of the LTM cell switch execution procedure with "release the radio bearer(s) without pdcp-Config and the logical channel(s) without servedRadioBearer". P10: Capture in TS 38.331 clause 5.3.5.3 that, when RACH-less LTM cell switch is completed, the UE shall release the uplink configured grant for RACH-less LTM cell switch, if configured (this is like RACH-less handover and does not affect the RRCReconfiguration message in ltm-CandidateConfig). P11: Capture that cg-LTM-Configuration is optionally present in the default uplink BWP of the SpCell in the RRCReconfiguration message contained in ltm-CandidateConfig, and cannot be included in more than one ConfiguredGrantConfig in that uplink BWP. | P1-P3: To be discussed. The discussion is whether we want to introduce the RAN4 interruption in Figure G-2 and G-3, but probably since those figures are indicative there is no need to modify them further. If we introduce the “mobility latency” then we would need to clarify the difference between “mobility latency” and “interruption”. P4-P6: To be agreed. Agree that makes sense to have such field descriptions but maybe we should not create a new IE as it could be a bit cumbersome. We have multiple definition of SSB configuration, and this new IE would be anyway a subset of the whole SSB configuration. Therefore, better to include the single IEs. P7: To be agreed. This seems straightforward. P8: To be discussed. It seems that it is anyway necessary to handle the case where initially the UE is configured with UE-based TA and then is switched to a candidate where the UE-based TA is not configured. Maybe one option would be to say “UE-based TA measurements for any configured LTM-Candidate” P9: To be discussed. This could be one option. Another option would be to either mention the logical channel identity or the radio bearer identity. P10: Not pursued. We discussed this in the work item and we agreed that network needs to release this explicitly: - Observation: In cases for which it is desired that CG used for LTM is not used further once the UE has made the cell its new serving cell, it is assumed that the network could release Type1 CG resource on LTM completion (existing functionality) Therefore, would be good to not modify now this principle. P11: To be discussed. This was never discussed within the work item and we don’t see why having such restriction as the UE can be switched directly to a BWP which is from the default BWP. We don’t think this change needs to be done now. Also, we never discussed whether this field can be configured in more than one ConfiguredGrantConfig (even is maybe is reasonable to say that we can configured in only one). |
| R2-2500963 | Huawei | MAC CR rapporteur summary | To be handled in the MAC summary |
| R2-2500988 | Ericsson, MTK | P1: Clarify that since the UE keeps the associated PDCP and SDAP entities but releases all fields related to the SRB/DRB configuration except for srb-Identity/drb-Identity, the network should provide new SRB/DRB configuration (signal again all fields) for the same srb-Identity/drb-Identity in order for the radio bearer to remain configured after the LTM cell switch procedure. The new SRB/DRB configuration should contain the same values (in comparison to the UE configuration for the radio bearer before this procedure is executed) for those fields which cannot be modified without PDCP re-establishment according to the field descriptions and conditions of PDCP-Config and TS 38.323. | P1: To be agreed. This comes basically from the agreements made in the last meeting which indicate to the network that the PDCP and SDAP entity need to be signalled again. When doing this, it would be good to understand how network should set certain field, according to what UE has in its current UE configuration. |
| R2-2500989 | Ericsson, MTK | P1: Add a new field within LTM-Candidate IE to indicate the number of serving cells configured for the LTM candidate configuration | P1: To be discussed. There are divergent opinions on this so better to discuss during the meeting. |
| R2-2501196 | Sharp | P1: Clarify that a PDCP entity of the DRB which is associated with the cell group for which the LTM cell switch procedure is triggered will perform data recovery. | P1: To be agreed. This makes sense and is also proposed by another company. |
| R2-2501294 | ZTE, Ericsson | This is an in-principle agreed CR in R2-2410801 which was forgotten in the last RAN2 meeting. | To be agreed. |
| R2-2501295 | ZTE | P1: Clarify in the field description of selectedCondRRCReconfig that selectedCondRRCReconfig can be used upon the execution of subseq uent CPAC P2: Clarify in the field description of reportSlotConfig-r18 that UE shall ignore the offset provided by the reportSlotConfig-r18 in the semiPersistentOnPUSCH-r18 | P1. To be agreed. Even if current text may already cover subsequent CPAC P2: Not pursued. We understand current spec is correct. Otherwise, this should be checked with RAN1. |

According to what is discussed above, the following proposals are made.

The following proposals are suggested to be agreed:

The network always signals fields frequencyBandList and absoluteFrequencyPointA when FrequencyInfoUL IE is used within EarlyUL-SyncConfig IE

The UE maintains fields subCarrierSpacing, absoluteFrequencyPointA, and cyclicPrefix in case the fields are absent in NZP-CSI-RS-Resource IE used within LTM-Candidate IE.

Within LTM Cell Switch Command MAC CE, the indicated joint TCI state in the TCI state ID field is the same or QCL to the SSB indicated in the SS/PBCH index field of the contention-free Random Access Resources.

Clarify that PDCP recovery is only done for DRB which RLC entity has been re-established.

RAN2 adopts the changes suggested by the RAN4 LS about the FG 39-6 prerequisites pre-requisites.

Removal of prerequisite of ltm-FastProcessingConfig-r18 according to the received RAN4 LS.

For ssb-Periodicity-r18, change to Need S and use a new IE SSB-Periodicity with the description currently in ServingCellConfigCommon (and use this IE in ServingCellConfigCommon too).

For ssb-PositionsInBurst-r18, capture that it is always present and use a new IE SSB-PositionsInBurst with the description currently in ServingCellConfigCommon (and use this IE in ServingCellConfigCommon too).

For ss-PBCH-BlockPower-r18, capture that it is always present and use a new IE SS-PBCH-BlockPower with the description currently in ServingCellConfigCommon (and use this IE in ServingCellConfigCommon too).

Capture that the unifiedTCI-StateType in LTM-TCI-Info should be the same like the unifiedTCI-StateType in the candidate cell configuration (ltm-CandidateConfig), as already agreed in RAN2#126.

Clarify that since the UE keeps the associated PDCP and SDAP entities but releases all fields related to the SRB/DRB configuration except for srb-Identity/drb-Identity, the network should provide new SRB/DRB configuration (signal again all fields) for the same srb-Identity/drb-Identity in order for the radio bearer to remain configured after the LTM cell switch procedure. The new SRB/DRB configuration should contain the same values (in comparison to the UE configuration for the radio bearer before this procedure is executed) for those fields which cannot be modified without PDCP re-establishment according to the field descriptions and conditions of PDCP-Config and TS 38.323.

Clarify that a PDCP entity of the DRB which is associated with the cell group for which the LTM cell switch procedure is triggered will perform data recovery.

Clarify in the field description of selectedCondRRCReconfig that selectedCondRRCReconfig can be used upon the execution of subseq uent CPAC

The following proposals are suggested to be discussed ONLINE:

For UE capability pdcch-RACH-AffectedBandsList-r18, RAN2 to discuss the issue of UE reporting this UE capability for bands for which the UE does not support PDCCH-ordered RACH for LTM (Two options are listed).

Upon successful completion of cell switch, UE releases configured contention-free Random Access Resources in rach-ConfigDedicated if configured in the reconfigurationWithSync.

Add the procedure description about RRC layer indicates lower layer which candidate cell could be performed UE-based TA measurement in case of L3 SpCell change execution and with ltm-ServingCellUE-MeasuredTA-ID but without ltm-CandidateToAddModList is configured within the reconfiguration message.

RAN2 does not consider any further the addition of an explicit information regarding the number of SpCells and SCells within the LTM candidate configuration.

Correct the Interruption time of Figure G-2 (make it end with TLTM-IU).

Replace the Tfirst-data by TLTM-IU of Figure G-3.

Add ‘Mobility latency’ in Figure G-2 and G-3.

In the LTM cell switch execution procedure, remove the procedure text for the case when ltm-UE-MeasuredTA-ID is not configured for the LTM-Candidate to which the UE is switching.

Replace the last action at the end of the LTM cell switch execution procedure with "release the radio bearer(s) without pdcp-Config and the logical channel(s) without servedRadioBearer".

Capture that cg-LTM-Configuration is optionally present in the default uplink BWP of the SpCell in the RRCReconfiguration message contained in ltm-CandidateConfig, and cannot be included in more than one ConfiguredGrantConfig in that uplink BWP.

Add a new field within LTM-Candidate IE to indicate the number of serving cells configured for the LTM candidate configuration

The remaining proposals are suggested to be discussed OFFLINE:

Discuss the remaining RRC proposals (including the ones suggested to not be pursued) offline.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

The following proposals are suggested to be agreed:

Proposal 1	The network always signals fields frequencyBandList and absoluteFrequencyPointA when FrequencyInfoUL IE is used within EarlyUL-SyncConfig IE

Proposal 2	The UE maintains fields subCarrierSpacing, absoluteFrequencyPointA, and cyclicPrefix in case the fields are absent in NZP-CSI-RS-Resource IE used within LTM-Candidate IE.

Proposal 3	Within LTM Cell Switch Command MAC CE, the indicated joint TCI state in the TCI state ID field is the same or QCL to the SSB indicated in the SS/PBCH index field of the contention-free Random Access Resources.

Proposal 4	Clarify that PDCP recovery is only done for DRB which RLC entity has been re-established.

Proposal 5	RAN2 adopts the changes suggested by the RAN4 LS about the FG 39-6 prerequisites pre-requisites.

Proposal 6	Removal of prerequisite of ltm-FastProcessingConfig-r18 according to the received RAN4 LS.

Proposal 7	For ssb-Periodicity-r18, change to Need S and use a new IE SSB-Periodicity with the description currently in ServingCellConfigCommon (and use this IE in ServingCellConfigCommon too).

Proposal 8	For ssb-PositionsInBurst-r18, capture that it is always present and use a new IE SSB-PositionsInBurst with the description currently in ServingCellConfigCommon (and use this IE in ServingCellConfigCommon too).

Proposal 9	For ss-PBCH-BlockPower-r18, capture that it is always present and use a new IE SS-PBCH-BlockPower with the description currently in ServingCellConfigCommon (and use this IE in ServingCellConfigCommon too).

Proposal 10	Capture that the unifiedTCI-StateType in LTM-TCI-Info should be the same like the unifiedTCI-StateType in the candidate cell configuration (ltm-CandidateConfig), as already agreed in RAN2#126.

Proposal 11	Clarify that since the UE keeps the associated PDCP and SDAP entities but releases all fields related to the SRB/DRB configuration except for srb-Identity/drb-Identity, the network should provide new SRB/DRB configuration (signal again all fields) for the same srb-Identity/drb-Identity in order for the radio bearer to remain configured after the LTM cell switch procedure. The new SRB/DRB configuration should contain the same values (in comparison to the UE configuration for the radio bearer before this procedure is executed) for those fields which cannot be modified without PDCP re-establishment according to the field descriptions and conditions of PDCP-Config and TS 38.323.

Proposal 12	Clarify that a PDCP entity of the DRB which is associated with the cell group for which the LTM cell switch procedure is triggered will perform data recovery.

Proposal 13	Clarify in the field description of selectedCondRRCReconfig that selectedCondRRCReconfig can be used upon the execution of subseq uent CPAC

The following proposals are suggested to be discussed ONLINE:

Proposal 14	For UE capability pdcch-RACH-AffectedBandsList-r18, RAN2 to discuss the issue of UE reporting this UE capability for bands for which the UE does not support PDCCH-ordered RACH for LTM (Two options are listed).

Proposal 15	Upon successful completion of cell switch, UE releases configured contention-free Random Access Resources in rach-ConfigDedicated if configured in the reconfigurationWithSync.

Proposal 16	Add the procedure description about RRC layer indicates lower layer which candidate cell could be performed UE-based TA measurement in case of L3 SpCell change execution and with ltm-ServingCellUE-MeasuredTA-ID but without ltm-CandidateToAddModList is configured within the reconfiguration message.

Proposal 17	RAN2 does not consider any further the addition of an explicit information regarding the number of SpCells and SCells within the LTM candidate configuration.

Proposal 18	Correct the Interruption time of Figure G-2 (make it end with TLTM-IU).

Proposal 19	Replace the Tfirst-data by TLTM-IU of Figure G-3.

Proposal 20	Add ‘Mobility latency’ in Figure G-2 and G-3.

Proposal 21	In the LTM cell switch execution procedure, remove the procedure text for the case when ltm-UE-MeasuredTA-ID is not configured for the LTM-Candidate to which the UE is switching.

Proposal 22	Replace the last action at the end of the LTM cell switch execution procedure with "release the radio bearer(s) without pdcp-Config and the logical channel(s) without servedRadioBearer".

Proposal 23	Capture that cg-LTM-Configuration is optionally present in the default uplink BWP of the SpCell in the RRCReconfiguration message contained in ltm-CandidateConfig, and cannot be included in more than one ConfiguredGrantConfig in that uplink BWP.

Proposal 24	Add a new field within LTM-Candidate IE to indicate the number of serving cells configured for the LTM candidate configuration

The remaining proposals are suggested to be discussed OFFLINE:

Proposal 25	Discuss the remaining RRC proposals (including the ones suggested to not be pursued) offline.