3GPP TSG-RAN WG2 Meeting #129	R2-2501020

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.5.3

Source:	CMCC

Title:	Discussion on On-demand SIB1

Document for:	Discussion

Introduction

In RAN#102 meeting, a new WID on enhancements of network energy saving has been approved [1], one of the objectives is on-demand SIB1 for UEs in idle/inactive mode, the scope of this objective has been discussed and revised in RAN#105 [2]. RAN2#127bis meeting is the first meeting of the normative phase, following are the related agreements [3]:

Agreements on OD-SIB1

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

- New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

- Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

- Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

- The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

- The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

- If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

- A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

- Working assumption: UL-WUS configuration in RRC release is not supported.

In this contribution, we provide our views on the open issues related OD-SIB1.

Discussion

WUS configuration

In last meeting, RAN2 agreed to use a new SIB for WUS configuration delivery, but it’s not clear the  maximum number of WUS configurations. Considering that Cell A is more possibility to be deployed on lower frequency for better coverage, while NES cell is deployed on higher frequency as a hotspot. Therefore, it is likely that Cell A and NES cell are on the different carrier frequency. Based on this, the maximum number of configured inter frequency cells can be considered as the maximum  number of WUS configuration, i.e., 16.

Proposal 1: The maximum number of configured inter frequency cells can be considered as the maximum  number of WUS configuration, i.e., 16.

In last meeting, one related discussion is whether Cell A can provide its own WUS configuration if it intends to be a NES cell.  In our opinion, though it’s network implementation to provide WUS configuration of any NES cell, but for a cell turns to be NES cell, it means that this cell intends to save its energy, and the rational network behaviour is to move the UEs in its coverage to other cells. If it provides its WUS configuration, then it will be waked up by UEs reselects to it. It’s not rational. Besides, as we discussed in the previous paragraph, in commercial network, the more possible case is that one Cell A is deployed with multiple NES cells, if Cell A is turned into NES cell, it will bring big impact since there’s no other Cell A to ensure coverage. Therefore, we prefer to not introduce spec impact on this issue.

Proposal 2: There’s no need to introduce extra spec impact on the issue that Cell A provides its WUS configuration if it intends to be a NES cell. It can be network implementation.

As to the content of WUS configuration, at least RACH related configurations are included in WUS configuration, as discussed in both RAN1 and RAN2. Following are the related RAN1 agreements:

Agreement in RAN1#119 [4]: The parameter “n-TimingAdvanceOffset” is included in the UL-WUS configuration. Agreement in RAN1#119: For parameters agreed for UL WUS, whether it is mandatory or optional is for further discussion. Agreement in RAN1#119: At least for RO validation determination for WUS transmission, the parameter “ssb-PeriodicityServingCell” is included in the UL-WUS configuration at least for TDD system. - FFS: FDD system Agreement in RAN1#119: Confirm the working assumption below from RAN1 #118-bis to include it in the UL-WUS configuration. - (working assumption) ULSubCarrierSpacing Agreement in RAN1#119: CORESET0 of NES cell is used for RAR CORESET of that NES cell. Agreement in RAN1#119: The mapping rule between ra-PreambleStartIndex for OD-SIB1 and SSB follows the mapping rule between ra-PreambleStartIndex for OSI request and SSB as in legacy specification. Agreement in RAN1#117 [5]: For further study of on-demand SIB1 in idle/inactive mode, use the following Table I (from R1-2405106, Ericsson) as a starting point to discuss the required parameters/contents inside the UL WUS configuration. Table I. Purpose; Parameters. To which cell does the config applies; NES-CellId; PhysCellId. ARFCN-ValueNR. WUS transmission; SIB1-RequestConfig; ss-PBCH-BlockPower. rach-OccasionsSIB1; Prach-ConfigurationIndex. msg1-FDM. msg1-FrequencyStart. zeroCorrelationZoneConfig. preambleReceivedTargetPower. preambleTransMax. powerRampingStep. ra-ResponseWindow. ssb-perRACH-Occasion. sib1-RequestPeriod. sib1-RequestResource; ra-PreambleStartIndex. ra-AssociationPeriodIndex. ra-ssb-OccasionMaskIndex. rsrp-ThresholdSSB. prach-RootSequenceIndex. msg1-SubcarrierSpacing. restrictedSetConfig. tdd-UL-DL-ConfigurationCommon. frequencyInfoUL; frequencyBandList. absoluteFrequencyPointA. offsetToCarrier. p-Max. frequencyShift7p5khz. SIB1 reception; pdcch-ConfigSIB1; ssb-SubcarrierOffset. controlResourceSetZero. searchSpaceZero. RAR Reception; pdcch-ConfigOD-SIB1-RAR; controlResourceSet. monitoringSlotPeriodicityAndOffset. Duration. monitoringSymbolsWithinSlot. aggregationLevels.

What’s more, as RAN2 agreed that NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy. Therefore, for other cases, the CORESET0 configuration and the actual value of K_SSB should be indicated to the NES UE for SIB1 reception. Furthermore, as discussed in previous meeting, the duration of SIB1 reception time window shall also be indicated. Though one way is to deliver the related configuration in RAR, in our view, the second way is preferred to avoid changes on RAR and to reduce the overhead on MAC PDU transmission.

Proposal 3: SIB1 reception related parameters including the configuration of COREST#0, the actual value of K_SSB and Sib1 reception window can be indicated in WUS configuration.

On-demand SIB1 acquisition procedure

Based on the agreements made in RAN2, the procedure for a UE to acquire on-demand SIB1 can be described as following:

- The UE camps on a cell (Cell A) and get NES cells’ WUS configuration provided in the new SIB.

- The UE reselects to a cell using legacy cell reselection criterion and identifies whether it’s a NES cell. If it’s a NES cell, and it has its WUS configuration, the UE will initiate on-demand SIB1 acquisition procedure using the WUS configuration. Else, if the UE doesn’t have the related WUS configuration, it will treat the cell as barred.

One issue should be discussed is the validity of the WUS configuration, since the UE may moves among cells, and not all of the cells are anchor cells and even for an anchor cell it may not broadcast the new SIB to provide WUS configuration of NES cell(s). From our point of view, it’s reasonable that the WUS configuration is semi-static, therefore, the UE can treat the WUS configuration as valid unless the network indicated a system information updating.

Proposal 4: The UE treats the WUS configuration as valid during its mobility unless indicated by the network for system information updating.

What’s more, another related issue is how to identify WUS configuration is broadcast in a cell by the UE. Since the WUS configuration is provided in the new SIB, the legacy way for the UE to identify whether a SIB is broadcasted can be used, and there’s no need to introduce a separate broadcast status indication.

Proposal 5: The UE can identify whether the WUS configuration is provided by legacy way for system information broadcasting indication.

Conclusions

In this contribution, we analyse the OD-SIB1 related open issues , following are the proposals and observations:

Proposal 1: The maximum number of configured inter frequency cells can be considered as the maximum  number of WUS configuration, i.e., 16.

Proposal 2: There’s no need to introduce extra spec impact on the issue that Cell A provides its WUS configuration if it intends to be a NES cell. It can be network implementation.

Proposal 3: SIB1 reception related parameters including the configuration of COREST#0, the actual value of K_SSB and SIB1 reception window can be indicated in WUS configuration.

Proposal 4: The UE treats the WUS configuration as valid during its mobility unless indicated by the network for system information updating.

Proposal 5: The UE can identify whether the WUS configuration is provided by legacy way for system information broadcasting indication.

References

- RP-234065, New WID: Enhancements of network energy savings for NR, 3GPP TSG RAN Meeting #102;

- RP-242354, Revised WID: Enhancements of network energy savings for NR, 3GPP TSG RAN Meeting #105;

- Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA;

- Chair notes RAN1#119;

- Chair notes RAN1#117.