3GPP TSG-RAN WG2 Meeting #129	R2-2501022

Athens, Greece, Feb. 17th – 21st, 2025

# Agenda item:	8.6.2

# Source:	CMCC

# Title:	Discussion on inter-CU LTM

# Document for:	Discussion, Decision

- Introduction

In RAN2#128 meeting, RAN2 reached the following agreements on inter-CU LTM [1]:

Agreements on inter-CU LTM:

- Rel-19 Set ID is configured for a candidate configuration. For DRB:

- Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> UE performs PDCP re-establishment to all radio bearers.

- Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

> For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

> If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

- RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

- RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

- From RAN2 perspective, the following coexistence cases in NR-DC can be supported:

- Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM

- Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM

7.	In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations.

8.	RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

9.	RAN2 to support intra-CU MCG LTM with SCG configuration.

10.	It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

11.	RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

In this paper, we further discuss the open issues of inter-CU LTM.

- Discussion

  - Inter-CU MCG LTM

    - UE-based TA measurement

In Rel-18 intra-CU LTM, except for L2 handing, the UE’s execution of UE-based TA measurement is also based on the comparison of related IDs of the serving cell and candidate cell. In the current spec, it’s UE implementation on how to measure TA when this way is configured by the network, and there’s no restrictions on the scenarios  in which UE can be configured with UE-based TA measurement. In other words, whether to configure UE-based TA measurement to the UE is a kind of network implementation, maybe it is decided based on the prior information in the serving cell.

Observation 1: In Rel-18 intra-CU LTM, whether to configure UE-based TA measurement is up to network implementation.

When it comes to Rel-19 inter-CU LTM, we think the same principle can be applied, the serving cell can configure UE-based TA measurement to the UE. If UE cannot access to the target cell successfully, it can fallback to legacy RACH procedure.

Proposal 1: UE-based  TA  measurement can be supported in Rel-19 inter-CU LTM.

    - L3 measurement results triggered inter-CU LTM

In the discussion of decision node of inter-CU LTM in RAN2#127 meeting [2], L3 measurement results triggered LTM is mentioned, which is also discussed for Rel-18 intra-CU LTM. The reason to support L3 measurement results triggered LTM is that the UE capability of LTM and L1 measurement reporting is decoupled, so that UE supporting LTM may have no L1 measurement results. To progress L3 measurement results triggered LTM, an LS was sent to RAN3 [5]. However, since it is triggered by intra-CU LTM, it’s not clear whether L3 measurement results triggered inter-CU LTM  is supported. In our view, since RAN2 agreed to support the coexistence of intra-CU LTM and inter-CU LTM in a UE, there’s no need to preclude the LTM supporting UE without L1 measurement capability. Therefore, we think inter-CU LTM can be triggered by L3 measurement results.

Proposal 2: RAN2 clarifies that inter-CU LTM can be triggered by L3 measurement results.

For L1 measurement results triggered LTM, the gNB-DU sends the DU-CU CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch command to the UE including the target cell ID and the TCI state ID after sending LTM cell switch command to the UE.

Observation 2: In current LTM procedure, the gNB-DU sends the DU-CU CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch command to the UE including the target cell ID and the TCI state ID after sending LTM cell switch command to the UE.

However, when it comes to L3 measurement result triggered LTM, it’s rational that gNB-CU only indicates the target cell ID and related information but not L3 measurement results, in this case, there’s no need for the gNB-DU to send target cell ID to the gNB-CU again.

Proposal 3: For L3 measurement results triggered LTM, it’s not needed for the source gNB-DU to indicate cell ID to the source gNB-CU after it sends LTM cell switch command to the UE.

The motivation to support L3 measurement based LTM is for the UE without L1 measurement capability. But when it comes to the UE with L1 measurement capability, it’s not clear, whether the gNB-CU can request the gNB-DU to send LTM cell switch command based on L3 measurement results. If the gNB-CU can request the gNB-DU to send LTM switch command for the UE with L1 measurement capability, it may cause conflict between the gNB-CU and the gNB-DU on the target cell selection. In our opinion, LTM is to take advantage of L1 measurement, therefore, for  the UE with L1 measurement capability, the gNB-CU should not request LTM based on L3 measurement results.

Proposal 4: For the UE with capability of L1 measurement, the source gNB-CU cannot request the source gNB-DU to trigger LTM cell switch command based on L3 measurement results.

    - CFRA-based inter-CU LTM

In Rel-18 intra-CU LTM, both RACH-less LTM and RACH based LTM are supported. For the RACH based LTM, RACH resources can be indicated in LTM cell switch command. For inter-CU LTM, it was agreed in RAN2#125bis meeting [3] that R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM. Therefore, it should be clarified that CFRA-based LTM is supported in inter-CU cases.

Proposal 5: CFRA-based LTM is supported in inter-CU cases.

  - Inter-CU SCG LTM

    - Coexistence of inter-CU SCG LTM and other mechanism(s)

Coexistence with intra-CU SCG LTM

In last meeting, RAN2 made clarification for the supporting scenarios in DC:

2. From RAN2 perspective, the following coexistence cases in NR-DC can be supported: - Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM - Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM

For Case2, how to configure both intra-CU SCG LTM and inter-CU SCG LTM should be discussed. In Rel-18, intra-CU SCG LTM is configured by SN in SN format. However, for inter-CU SCG LTM, the configuration is provided to UE by MN, with other MN RRC configurations, that’s to say, inter-CU SCG LTM is configured in MN format. Similar issue is discussed in Rel-18 SCPAC, in which only MN format is used for both intra-SN and inter-SN candidate PsCell configurations. The unified format can satisfied with different UE moving trajectory, i.e. only moves among the candidate cells of the same SN or moves among cells of different SN.  This can be inherited in the coexistence of intra-CU SCG LTM and inter-CU SCG LTM.

Proposal 6: Intra-CU SCG LTM can be provided to the UE in MN format.

Proposal 7: For the coexistence of intra-CU SCG LTM and inter-CU SCG LTM, LTM configuration is configured to the UE in MN format.

Coexistence with L3 SN handling

It was agreed in RAN2#127bis meeting that L3 mobility can be configured to the UE with inter-CU LTM configured. Following are the related agreements with regard to the coexistence of  L3 mobility and inter-CU MCG LTM:

9. L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM): - When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network. - The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations. - For the execution order between CHO and LTM, Rel-18 principle is applied.

Therefore, when it comes to SCG LTM, the coexistence of inter-CU SCG LTM and L3 SCG handling like SN change/PsCell change/(S-)CPAC should also be discussed. And we think the similar principle can be applied, including:

- Network triggered SN change/PsCell change and (S-)CPAC can be configured to UE, when (SN initiated) inter-CU SCG LTM is configured.

  - When performing network triggered SN change/PsCell change or (S-)CPAC, the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

  - The RRCReconfiguration message to execute an SN change/PsCell change/(S-)CPAC procedure may reconfigure inter-CU LTM configurations.

  - For the execution order between (S-)CPAC and inter-CU SCG LTM, similar principle for CHO and inter-CU MCG LTM is reused.

Proposal 8: Network triggered SN change/PsCell change and (S-)CPAC can be configured to UE, when (SN initiated) inter-CU SCG LTM is configured.

-	When performing network triggered SN change/PsCell change or (S-)CPAC, the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

-	The RRCReconfiguration message to execute an SN change/PsCell change/(S-)CPAC procedure may reconfigure inter-CU LTM configurations.

-	For the execution order between (S-)CPAC and inter-CU SCG LTM, similar principle for CHO and inter-CU MCG LTM is reused.

    - Inter-CU SCG LTM configuration

When it comes to inter-CU SCG LTM configuration, there’s two issues for further clarification:

- Whether a separate IE is used to configure inter-CU SCG LTM except for ltm-config;

- How to provide sk-counters?

Whether a separate IE is used to configure inter-CU SCG LTM except for ltm-config

Based on RAN2 conclusion, the coexistence of intra-CU MCG and inter-CU MCG LTM is supported, the two types of LTM can be configured  in the same IE ltm-config, and the UE can identify whether the LTM procedures is a inter-CU LTM based on the new Rel-19 ID. Besides, only one reference configuration is configured for both types of LTM.

Observation 3: For the coexistence of intra-CU MCG and inter-CU MCG LTM, only one IE ltm-config is used to provide LTM candidate configurations, including only one reference configuration and Rel-19/Rel-18 ID for UE to identify the LTM type is inter-CU LTM or intra-CU LTM.

If the IE ltm-config is also used to config inter-CU SCG LTM, though the coexistence of inter-CU MCG LTM and inter-CU SCG LTM is not supported, it may bring extra complexity of network configuration and UE understanding. For example, it may increase difficulty to achieve a common reference configuration and CSI-resource configuration, which can be used for both MCG LTM and SCG LTM.

Observation 4: It’s hard to achieve a common reference configuration and CSI-resource configuration, which can be used for both MCG LTM and SCG LTM.

Besides, for the case of co-existence of inter-CU MCG LTM and intra-CU SCG LTM, one scell of source SCG may be configured as inter-CU MCG LTM candidate cell and intra-CU SCG LTM candidate, we don’t think the configuration for the different  type LTM could be the same, though it’s the same cell.

Observation 5: For the same Scell of SCG, the candidate configuration could be different in different type LTM.

Based on the discussion above, it is proposed to use a separate IE for inter-CU SCG LTM configuration. And it can also be used for intra-CU SCG LTM.

Proposal 9: A separate IE is used for inter-CU SCG LTM configuration.

How to provide sk-counters

In the running CR discussion, one issue is about the how to provide sk-counters: FFS whether the list of sk-counters is part of the LTM-Config or is provided outside (same level as ltm-Config in the RRCReconfiguration message).  In S-CPAC, sk-counters are provided as a part of conditional reconfiguration, as shown in the following [4]:

And as RAN2 agreed that the SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration. Therefore, we prefer to use the same principle for sk-counter configuration in inter-CU SCG LTM.

Proposal 10: sk-counters for inter-CU SCG LTM is configured as a part of SCG LTM configuration.

- Conclusion

In this contribution, we discuss inter-CU LTM related issue. Following are the proposals and observations made in this contribution:

Observations:

Observation 1: In Rel-18 intra-CU LTM, whether to configure UE-based TA measurement is up to network implementation.

Observation 2: In current LTM procedure, the gNB-DU sends the DU-CU CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch command to the UE including the target cell ID and the TCI state ID after sending LTM cell switch command to the UE.

Observation 3: For the coexistence of intra-CU MCG and inter-CU MCG LTM, only one IE ltm-config is used to provide LTM candidate configurations, including only one reference configuration and Rel-19/Rel-18 ID for UE to identify the LTM type is inter-CU LTM or intra-CU LTM.

Observation 4: It’s hard to achieve a common reference configuration and CSI-resource configuration, which can be used for both MCG LTM and SCG LTM.

Observation 5: For the same Scell of SCG, the candidate configuration could be different in different type LTM.

Proposals:

Proposal 1: UE-based  TA  measurement can be supported in Rel-19 inter-CU LTM.

Proposal 2: RAN2 clarifies that inter-CU LTM can be triggered by L3 measurement results.

Proposal 3: For L3 measurement results triggered LTM, it’s not needed for the source gNB-DU to indicate cell ID to the source gNB-CU after it sends LTM cell switch command to the UE.

Proposal 4: For the UE with capability of L1 measurement, the source gNB-CU cannot request the source gNB-DU to trigger LTM cell switch command based on L3 measurement results.

Proposal 5: CFRA-based LTM is supported in inter-CU cases.

Proposal 6: Intra-CU SCG LTM can be provided to the UE in MN format.

Proposal 7: For the coexistence of intra-CU SCG LTM and inter-CU SCG LTM, LTM configuration is configured to the UE in MN format.

Proposal 8: Network triggered SN change/PsCell change and (S-)CPAC can be configured to UE, when (SN initiated) inter-CU SCG LTM is configured.

-	When performing network triggered SN change/PsCell change or (S-)CPAC, the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

-	The RRCReconfiguration message to execute an SN change/PsCell change/(S-)CPAC procedure may reconfigure inter-CU LTM configurations.

-	For the execution order between (S-)CPAC and inter-CU SCG LTM, similar principle for CHO and inter-CU MCG LTM is reused.

Proposal 9: A separate IE is used for inter-CU SCG LTM configuration.

Proposal 10: sk-counters for inter-CU SCG LTM is configured as a part of SCG LTM configuration.

- References

- Report of 3GPP TSG RAN WG2 meeting #128;

- Report of 3GPP TSG RAN WG2 meeting #127;

- Report of 3GPP TSG RAN WG2 meeting #125bis;

- TS 38.331.